3GPP TSG-RAN WG2 Meeting #130											R2-2503512

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.7.6

Source: 	Sharp

Title:  	Open Issues on Rate Control

Document for: 	Discussion

# Introduction

In RAN2#129bis meeting, RAN2 made the following agreements:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. - FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

This document discusses remaining open issues on XR rate control enhancements, captured as open issues or FFS [1][2].

# Discussion

## ID in Rel-19 Rate Control MAC CE

- [MAC-04] Which type of ID should be used to identify a QoS flow, e.g. DRB ID + QFI ID or some other identifier

In RAN2#129bis meeting, RAN2 agreed per flow indication as the preference. QFI is indicated in the Rel-19 MAC CE anyway. A question is whether DRB ID is necessary or not. The benefit of DRB indication could be that the same QFI is shared over different PDU sessions of different DRBs. However, considering RAN QFI space (up to 64) and DRB ID spaces (up to 29), this situation is not common. Moreover, most of practical configuration uses only a few QFIs, and we do not expect that we have a QFI shortage in XR. In that sense, we see that DRB ID is not needed, and it would be better to save a byte.

QFI without DRB ID is included in Rel-19 Rate Control MAC CE.

## Support of DL Rate Control in Rel-19

- [MAC-11] Whether to apply the same design for UL rate control design to DL data transmission

In Rel-19 XR, RAN2 focused on UL rate control, since the WID specified only uplink rate control. Now, this feature is almost being finalized. The current form of the rate control mechanism is very similar to Rel-15 Recommended Bit Rate procedure which supports both UL and DL. It means that there is no technical difficulty nor DL specific issue foreseen. We see that DL support is a low hanging fruit we can easily have at least from the signaling perspective.

Rel-19 Rate Control MAC CE can be used for DL as well as UL.

## Granularity of Query

- Issue1: FFS how to indicate whether bit rate query is enabled based on which granularity (QoS flow level or DRB level)

This open issue is merely on configuration of bit rate query. Since RAN2 agreed to prefer QoS flow level indication, the same granularity can be used for the configuration. The exact location of the configuration can be further discussed, but we can consider LogicalChannelConfig as a baseline, similar to Rel-15 query.

Bit rate query is based on QoS flow level and configured in LogicalChannelConfig.

## Index 0 in Rel-19 Rate Control MAC CE

Editor’s note: FFS whether the index 0 has any special meaning.

There is a difference of bit rate field between Rel-15 MAC CE and the current draft of Rel-19 MAC CE, as follows (some irrelevant parts are omitted):

| Rel-15 RBR bit rate | Running CR for Rel-19 MAC CE |
| --- | --- |
| Index; NR Recommended Bit Rate value [kbit/s]. 0; Note 1. 1; 0. 2; 9. Note 1: For bit rate recommendation message this index is used for indicating that no new recommendation on bit rate is given. | Index; Bit Rate. 0; 0. 1; ≤ 100. 2; ≤ 102. |

The main difference of usage of Index 0. In Rel-15, Index 0 is dedicated for the case that there is no new recommendation. It is separated from the case that zero bit rate is recommended by the network. Thus, we can keep the same principle in Rel-15, and we cannot find a reason to make the difference between those similar features.

Index 0 is used for indicating that no new recommendation on bit rate is given, same as Rel-15.

## Multiple QF Reporting

- [MAC-05] Whether the Rate Control MAC CE includes single or multiple QoS flows

In Rel-19 XR WI, RAN2 agreed to prefer per QoS flow indication. This assumes that multiple QoS flows are being used for multi-modal traffic. In this case, it is likely that multiple QoS flows require adaptive rate control. In this case, individual signaling is not efficient. Instead, a unified signaling for multiple QoS flows is desirable and efficient from the overhead perspective. Currently, the priority of the uplink query message is lower than that of data, so multiple query messages could be sent together in the same MAC PDU. Smaller size is important for timely transmission of the query message.

The Rate Control MAC CE includes multiple QoS flows.

# Conclusions

Based on the discussion above, RAN2 is requested to agree the following proposals:

- QFI without DRB ID is included in Rel-19 Rate Control MAC CE.

- Rel-19 Rate Control MAC CE can be used for DL as well as UL.

- Bit rate query is based on QoS flow level and configured in LogicalChannelConfig.

- Index 0 is used for indicating that no new recommendation on bit rate is given, same as Rel-15.

- The Rate Control MAC CE includes multiple QoS flows.

# Reference

[1] R2-25003361	List of open issues in MAC, Qualcomm Incorporated

[2] R2-2503788	Summary of [POST129bis][503][XR] RRC running CR and open issues (Huawei), Huawei, HiSilicon