3GPP TSG-RAN WG2 Meeting #130											R2-2503513

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.11.2

Source: 	Sharp

Title:  	Open Issues on SBFD Random Access

Document for: 	Discussion

# Introduction

In RAN2#129bis meeting, RAN2 made the following agreements:

On RO type signaling for CFRA - 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. - 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated. When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type. Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM. NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

This document discusses remaining open issues on SBFD random access.

# Discussion

  - Msg3 Early Indication

In RAN2#128 meeting, whether an early indication of SBFD capability during the random access using legacy RO was discussed but no conclusion has been reached, as follows:

Early indication via msg3 R2-2410385 Random access for SBFD Operation Sony discussion Rel-19 NR_duplex_evo-Core Noted Proposal 2: Early identification of SBFD-aware UEs in message 3 is not needed during RA procedure. R2-2410088 RA Aspects for SBFD Sharp discussion Rel-19 NR_duplex_evo-Core Noted Proposal 1 Msg3-based early indication in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability. DISCUSSION - Sony do not think this is critical and think we should not spend too much time on this. - Apple think today we have many features that use msg3 based EI and if we do so here it requires more time to discuss.

In RAN2#129 and RAN2#129bis meetings, there was no discussion on this issue. We expect a relevant discussion in this meeting.

We see that an early indication of SBFD capability during the random access is beneficial. One of the most important benefits of SBFD is latency reduction compared to legacy TDD. An early indication enables the benefit of latency reduction during the random access.

Regarding how to support the early indication, RAN2 concluded that there is no need to introduce SBFD as a feature combination. It means that Msg1-based early indication is not supported. Instead, we can consider Msg3-based early indication which still has a benefit to start transmission and reception early on SBFD symbols. It can be realized by using LCID field.

Msg3-based early indication with using LCID field in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability.

  - Separate Preamble Configuration in RACH Configuration Option 1

In RAN1#120bis meeting, RAN did not conclude separate preamble configuration in RACH Configuration Option 1 but left it up to RAN2, as follows:

Conclusion For RACH configuration Option 1, there is no consensus in RAN1 to support separate ssb-SharedRO-MaskIndex configurations for Additional-ROs and Legacy-ROs under the same FeatureCombinationPreambles configuration and it’s up to RAN2 to decide whether to support separate featureCombinationPreamblesList configurations for Additional-ROs and Legacy-ROs.

For RACH Configuration Option 1, feature combination preambles should also be separately configured for additional ROs and legacy ROs. The main reason is that additional ROs do not support some features e.g. RedCap. For example, if the network wants to operate with both RedCap and SBFD, it should configure feature combination preambles for RedCap only in Legacy ROs. Reserving feature combination preambles for RedCap UEs in additional ROs will not be used.

Feature combination preamble configurations are provided separately for Additional ROs and Legacy ROs in RACH configuration Option 1.

  - RA Resource Set Reselection after RO Type Switching

RAN2 agreed to support RO type change between legacy RO and additional RO. The issue is whether the RA resource set reselection should be performed after RO type change from one to the other. At least for RACH Configuration Option 2, separate sets can be configured. Considering the case that different configurations have different partitioning, RA resource set reselection seems to be essential.

RA resource set reselection is supported after RO type switching.

  - SBFD RA for LTM

In RAN2#129bis meeting, RAN2 agreed to support all existing triggering events except SI request, and LTM remains as an FFS. In general, SBFD has a benefit on latency reduction which is related to mobility. Thus, we see that it would be great to support SBFD in mobility procedure unless there is a critical problem. For early UL synchronization with LTM candidate cell, the expected specification change would be RRC signaling support e.g. EarlyUL-SyncConfig and potential inter-node signaling. But this change may not be so controversial.

Early UL synchronization with an LTM candidate cell is supported in SBFD symbols.

For LTM Cell Switch Command MAC CE, the benefit of latency reduction can be achieved. The expected specification impact is 1-bit indication of RO type in LTM Cell Switch Command MAC CE. The current MAC CE has enough reserved bits for future usages. We can simply use one of them.

1-bit indication of RO type is introduced in LTM Cell Switch Command MAC CE.

  - 2-Step RACH

In SBFD WI, RAN2 has been focusing on 4-step RACH for SBFD, based on the following RAN2#127 agreement:

RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed.

Meanwhile, RAN1 discussed whether to support 2-step RACH in SBFD symbols, and their conclusion in RAN1#120 meeting was no consensus as follows:

Conclusion There is no consensus in RAN1 to support Type-2 random access procedure (2-step RACH) in SBFD symbols for SBFD-aware UEs.

The RAN1 conclusion with no consensus is usually interpreted as “no support”. Now we are approaching to the end of the release. New discussions should be minimized for the on-time completion of the WI. Moreover, the same issue was already discussed in RAN1. RAN2 does not need to repeat the same discussion that was concluded in RAN1.

RAN2 to confirm that 2-step RACH in SBFD symbols is not supported.

# Conclusions

Based on the discussion above, RAN2 is requested to agree the following proposals:

- Msg3-based early indication with using LCID field in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability.

- Feature combination preamble configurations are provided separately for Additional ROs and Legacy ROs in RACH configuration Option 1.

- RA resource set reselection is supported after RO type switching.

- Early UL synchronization with an LTM candidate cell is supported in SBFD symbols.

- 1-bit indication of RO type is introduced in LTM Cell Switch Command MAC CE.

- RAN2 to confirm that 2-step RACH in SBFD symbols is not supported.