3GPP TSG-RAN WG2 Meeting #130	R2-2503516

St. Julians, Malta, May 19th – May 23rd, 2025

Agenda item:	8.3.5

Source:	Qualcomm Incorporated

Title:	Performance monitoring of AI/ML for mobility

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the last RAN2 #129-bis meeting [1], the final set of conclusions were drawn from the simulation studies on AI/ML for mobility. And, RAN2 initiated the study of LCM aspects and specification impacts for AI/ML for mobility, with a view towards converting the SI into a WI and determining the scope of the WI in Rel-20. A set of basic agreements were reached on the LCM aspects and specification impacts for AI/ML for mobility [1].

In this contribution, we continue the discussion on LCM aspects and specification impacts for AI/ML for mobility, focusing on Performance Monitoring for RRM measurement prediction or for Measurement Event prediction, for UE-side model.

# Performance Monitoring of UE-side model

In the last RAN2 #129-bis meeting [1], there was the following agreement to consider network-side and UE-side performance monitoring for AI/ML for mobility:

7 For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case

Additionally, we agreed that

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

Below we discuss proposals on network-side and UE-side performance monitoring of UE-side model for RRM prediction and Event prediction, following the general LCM framework agreed for beam management.

## Performance Monitoring of UE-side model for RRM prediction

Network-side monitoring

Proposal 1: Network side monitoring for RRM prediction of an AI/ML mobility feature can be done by comparing legacy measurement reports with the UE provided predictions.

Proposal 2: If the computed KPIs are not satisfactory, the gNB can disable, deconfigure, or use a different configuration of the AI/ML for mobility feature in the associated location.

Reconfiguration of the inference configuration might result in the UE using a different model.

UE-side monitoring

For UE-side monitoring, the UE computes the performance KPIs, takes the LCM operation decision by itself, and executes it. UE informs the network about the executed LCM operation.

Proposal 3: UE-side monitoring for RRM prediction should be supported. For UE-side monitoring, the gNB provides the UE with the performance monitoring configuration to use which includes the KPIs to compute.

Proposal 4: UE computes the performance KPIs and decides on the continued applicability of a configuration associated with a model.

## Performance Monitoring of UE-side model for Event prediction

Network-side monitoring

The procedure is similar as for the RRM prediction case.

Proposal 5: For network-side monitoring, the gNB uses the legacy UE event-triggered measurement reports for ground truth information. The network can compare those with the event prediction reports that will be defined as part of this feature which may include the expected time of occurrence of event and probability of occurrence within an indicated time window around the expected time.

Based on the UE reports as described in the above proposal, the gNB can compute the performance KPIs for Event prediction. For example, based on the UE reports, the gNB can determine whether the predicted time of occurrence of event is close to the actual event, and hence calculate the KPIs that were agreed for event prediction during simulation evaluation:

- Counter values for true event prediction, false event detection, and missed event detection,

- F1 score,

- Precision,

- Recall.

Based on the UE reports, the gNB can also compute system level KPIs, e.g., success or failure rate of handovers triggered due to event A3 or A5 prediction reports.

Proposal 6: The gNB can then decide on the subsequent action, based on computed KPIs: transmit an LCM request for deactivating the UE-side model or reconfigure the UE.

UE-side monitoring

The proposals for UE-side monitoring for Event prediction are similar as those for RRM prediction.

# 4 Conclusions

Based on the discussion above, we recommend RAN2 to discuss the observations and proposals below.

Network-side performance monitoring of UE-side model for RRM prediction

Proposal 1: Network side monitoring for RRM prediction of an AI/ML mobility feature can be done by comparing legacy measurement reports with the UE provided predictions.

Proposal 2: If the computed KPIs are not satisfactory, the gNB can disable, deconfigure, or use a different configuration of the AI/ML for mobility feature in the associated location.

UE-side performance monitoring of UE-side model for RRM prediction

Proposal 3: UE-side monitoring for RRM prediction should be supported. For UE-side monitoring, the gNB provides the UE with the performance monitoring configuration to use which includes the KPIs to compute.

Proposal 4: UE computes the performance KPIs and decides on the continued applicability of a configuration associated with a model.

Network-side performance monitoring of UE-side model for Event prediction

Proposal 5: For network-side monitoring, the gNB uses the legacy UE event-triggered measurement reports for ground truth information. The network can compare those with the event prediction reports that will be defined as part of this feature which may include the expected time of occurrence of event and probability of occurrence within an indicated time window around the expected time.

Proposal 6: The gNB can then decide on the subsequent action, based on computed KPIs: transmit an LCM request for deactivating the UE-side model or reconfigure the UE.

UE-side performance monitoring of UE-side model for Event prediction

The proposals for UE-side monitoring for Event prediction are similar as those for RRM prediction.

# References

[1] Chair notes, RAN2 #129-bis meeting.