3GPP TSG RAN WG2 Meeting #130                                                            R2-2503521

# St Julian's, Malta, May 19th - 23rd, 2025

Agenda item:	8.7.3

Source:	Ofinno

Title:	RRM Measurement Gaps Enhancements for XR

Document for:	Discussion and decision

# Introduction

This contribution proposes targeted enhancements to improve RRM measurement gap cancellation handling in MAC procedures and implement the details of UAI for measurement gap cancellation.

In the first section, we identified inconsistencies in the current MAC running CR [1] regarding the handling of measurement gaps that have been cancelled. While the CR already clarified in three sections (e.g., §5.4.2.2 "HARQ process," §5.4.4 "Scheduling Request," and §5.14 "Handling of measurement gaps") that only non-cancelled measurement gaps are considered in the MAC procedures, two additional cases require similar clarification:

- Case 1: RA resource selection

- Case 2: DCP monitoring during DRX

In the second section, the discussion focuses on UAI. According to the RAN4 LS [2], RAN4 has agreed to define an UAI allowing UEs to indicate a preferred ratio of measurement gap occasions recommended for cancellation. RAN2 is tasked with implementation details on the UAI, and the document proposes the following:

- UE capability for UAI

- Triggering of UAI transmission

- Prohibit timer for UAI

# Discussion

## Missing measurement gap cancellation cases in MAC

In the current MAC running CR [1], to avoid potential ambiguity in the determination of measurement gap, the rapporteur clarified that an activated measurement gap occasion or a measurement gap occasion overlapping with other occasions (e.g., SR occasion, other measurement gap occasions) refers specifically to a measurement gap that has not been cancelled, as illustrated below. While some companies suggested that such clarifications are not necessary to be explicitly captured, we believe this kind of clarifications are essential to avoid confusions in the future.

From the MAC perspective, once the measurement gap configuration is provided to the UE, MAC would typically assume that measurement gap occasions occur even if the measurement gap occasions have been cancelled. From our point of view, a cancelled measurement gap implies that the UE will not utilize that occasion for measurement; it does not mean the measurement gap occasion itself no longer exists. Therefore, without explicitly clarifying that the measurement gap occasions refer solely to those not cancelled, implementations may interpret differently, potentially resulting in inconsistent MAC behaviours.

MAC running CR [1] 5.4.2.2 HARQ process To generate a transmission for a TB, the HARQ process shall: 1> if the MAC PDU was obtained from the Msg3 buffer; or 1> if the MAC PDU was obtained from the MSGA buffer; or - if there is no measurement gap or there is an overlapping measurement gap but it has been canceled (as specified in clause 10.6 in [6]) at the time of the transmission and, in case of retransmission, the retransmission does not collide with a transmission for a MAC PDU obtained from the Msg3 buffer or the MSGA buffer: 5.4.4 Scheduling Request As long as at least one SR is pending, the MAC entity shall for each pending SR: … 1> else, for the SR configuration corresponding to the pending SR: 2> when the MAC entity has an SR transmission occasion on the valid PUCCH resource for SR configured; and 2> if sr-ProhibitTimer is not running at the time of the SR transmission occasion; and 2> if the PUCCH resource for the SR transmission occasion does not overlap with a measurement gap or it overlaps with a measurement gap that has been canceled (as specified in clause 10.6 in [6]): 5.14 Handling of measurement gaps During an activated measurement gap that has not been cancelled (as specified in clause 10.6 in [6]), the MAC entity shall, on the Serving Cell(s) in the corresponding frequency range of the measurement gap configured by measGapConfig as specified in TS 38.331 [5]: …

In the MAC running CR [1], it has already been explicitly captured in three separate cases (e.g., §5.4.2.2 “HARQ process”, §5.4.4 “Scheduling Request”, and §5.14 “Handling of measurement gap”) that the measurement gaps refer to the measurement gaps which have not been cancelled. However, some other cases are missing to be captured.

Following the same rationale applied to other sections in the MAC running CR [1], we have identified two additional cases which have not yet been explicitly specified and should also be addressed.

### Case 1: Measurement gap during RA resource selection

The Case 1 relates to measurement gap considerations during RA resource selection. In the RA resource selection procedure, the MAC entity may consider the potential occurrence of measurement gaps when determining the next available PRACH occasion. However, the current MAC running CR [1] has not explicitly specified that the measurement gaps considered in this context should specifically refer to measurement gaps that have not been cancelled.

Therefore, to maintain alignment and consistency with the rationale already applied in §5.4.2.2 “HARQ process”, §5.4.4 “Scheduling Request”, and §5.14 “Handling of measurement gap”, we propose explicitly clarifying this point as well in §5.1.2 “Random Access Resource selection” and §5.1.2a “Random Access Resource selection for 2-step RA type.”

The corresponding TP is provided below and in Appendix 1.

5.1.2 Random Access Resource selection … 1> else if an SSB is selected above: 2> if the set of Random Access resources associated with Msg1 repetition is selected for this Random Access procedure: 3> determine the next available set of PRACH occasions (as specified in TS 38.213 [6]) for the Msg1 repetition number applicable for this Random Access procedure corresponding to the selected SSB (the MAC entity shall select a set of PRACH occasions randomly with equal probability amongst sets of PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB and selected Msg1 repetition number for this Random Access procedure; the MAC entity may take into account the possible occurrence of measurement gaps that have not been cancelled (as specified in clause 10.6 in [6]) and MUSIM gaps when determining the next available set of PRACH occasions corresponding to the selected SSB). 2> else: 3> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured, or indicated by PDCCH, or indicated by the LTM Cell Switch Command MAC CE (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps that have not been cancelled (as specified in clause 10.6 in [6]) and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB). else if a CSI-RS is selected above: 2> if there is no contention-free Random Access Resource associated with the selected CSI-RS: 3> determine the next available PRACH occasion from the PRACH occasions, permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, corresponding to the SSB in candidateBeamRSList which is quasi-colocated with the selected CSI-RS as specified in TS 38.214 [7] (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the SSB which is quasi-colocated with the selected CSI-RS; the MAC entity may take into account the possible occurrence of measurement gaps that have not been cancelled (as specified in clause 10.6 in [6]) and MUSIM gaps when determining the next available PRACH occasion corresponding to the SSB which is quasi-colocated with the selected CSI-RS). 2> else: 3> determine the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the PRACH occasions occurring simultaneously but on different subcarriers regardless the FR2 UL gap, corresponding to the selected CSI-RS; the MAC entity may take into account the possible occurrence of measurement gaps that have not been cancelled (as specified in clause 10.6 in [6]) and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected CSI-RS). - perform the Random Access Preamble transmission procedure (see clause 5.1.3). 5.1.2a Random Access Resource selection for 2-step RA type … 1> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the msgA-SSB-SharedRO-MaskIndex if configured, or ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability among the consecutive PRACH occasions allocated for 2-step RA type according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps that have not been cancelled (as specified in clause 10.6 in [6]) and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB);

To update legacy references of “possible occurrence of measurement gaps” in TS 38.321 §5.1.2 and §5.1.2a on “Random Access Resource selection” by explicitly capturing that measurement gaps should be the measurement gaps that have not been cancelled.

If the Proposal 1 is agreed, adopt the TP in Appendix 1.

### Case 2: Measurement gap during DCP monitoring in DRX

The Case 2 relates to DCP monitoring during DRX operations. In the DRX procedure, the MAC entity determines whether to start the drx-onDurationTimer based on whether the configured DCP occasion(s) overlap in time domain with a measurement gap. Specifically, if all DCP occasion(s) overlap with a measurement gap, the MAC entity directly starts the drx-onDurationTimer without performing DCP monitoring. However, the current MAC running CR [1] does not explicitly clarify that the measurement gap considered in this scenario should exclusively refer to the measurement gap that have not been cancelled.

Therefore, consistent with the rationale already applied in §5.4.2.2 “HARQ process”, §5.4.4 “Scheduling Request”, and §5.14 “Handling of measurement gap”, we propose clearly specifying that the measurement gap considered in the context of DCP monitoring shall be the measurement gap that has not been cancelled.

The corresponding TP is provided below and in Appendix 2.

5.7 Discontinuous Reception (DRX) … 1> if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is not configured, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset; or 1> if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset: 2> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3: 3> if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap that has not been cancelled (as specified in clause 10.6 in [6]), or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or 3> if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers: 4> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. 2> else: 3> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.

To update legacy references of “during a measurement gap” in TS 38.321 §5.7 on “Discontinuous Reception (DRX)” by explicitly capturing that measurement gap should be the measurement gap that has not been cancelled.

If the Proposal 3 is agreed, adopt the TP in Appendix 2.

## UAI for measurement gap cancellation

Based on the RAN4 LS [2], RAN4 has agreed to define a UAI that can indicate a preferred ratio of measurement gap occasions recommended for cancellation during a time period of 1 second. The implementation details of this UAI will be handled by RAN2. In this section, we highlight certain issues that arise when RAN2 implements the UAI for measurement gap cancellation.

RAN4#114 Agreements - Agree to define UAI based on the following assumptions, and further discuss the content of UAI - RAN4 will not specify any explicit or implicit requirement or expectation on gNB behaviour in response to any XR UAI (from RAN4 perspective) - Not define UE behaviour and the corresponding requirements based on UAI - Supporting of UAI is optional for UE with optional UE capability. RAN4#114bis Agreements - The information in UAI: a ratio (R) of gap occasions that is recommended for cancellation during a time period of 1 s. - R ∈ {0, 20, 40, 60}%

RAN2 should further discuss and detail the implementation of the UAI for measurement gap cancellation, in line with the agreements reached within RAN4.

In RAN4#114, it was agreed that UAI support is optional for the UE and shall be associated with an explicit optional UE capability. Following this RAN4 guidance, RAN2 should introduce a suitable UE capability to declare the support of UAI for indicating the UE’s preference on the ratio of measurement gap occasions recommended for cancellation. Additionally, since this UAI specifically indicates a recommended ratio for measurement gap cancellation and given that current measurement gap cancellation procedure is based on DCI signalling (i.e., DCI-based measurement gap cancellation), it follows logically that without support of DCI-based measurement gap cancellation, the indicated ratio would be ineffective. In other words, the usability of this UAI feature directly relies upon support of the DCI-based measurement gap cancellation feature in the UE.

[UE Capability] Introduce an optional UE capability to indicate UE’s support of the UAI used to provide UE’s preference on the ratio of measurement gap occasions for cancellation. A UE supporting this feature shall also indicate the support of DCI-based measurement gap cancellation.

Additionally, RAN2 needs to discuss the initiating or triggering conditions for UE’s transmission of this new UAI. As RAN4 explicitly stated in RAN4#114 agreements, UE behaviour and the corresponding requirements derived from this UAI will not be defined. As such, we believe that RAN2 does not need to define complicated triggering conditions. Instead, it is sufficient to simply allow the UE to trigger the transmission of this new UAI when it has a preference on the ratio of measurement gap occasions before sending any preference or when its preference has changed compared to the most recently transmitted UAI information on the ratio of measurement gap occasions.

[RRC-5] If the UE is configured to provide UAI with its preference on the ratio of measurement gap occasions for cancellation, the UE initiates an UAI transmission to provide its preference on the ratio when:

  - (1) The UE has a preference on the ratio and the UE did not transmit any preference on the ratio since it was configured to provide its preference on the ratio; or

  - (2) The UE's preference on the ratio changed from the last time UE transmits the UAI to provide its preference on the ratio.

Considering our proposal not to impose specific triggering conditions for UAI transmissions related to measurement gap cancellation, the UE would have flexibility to initiate UAI transmission at its discretion. Therefore, a prohibit timer seems preferable to prevent overly frequent transmissions of this UAI (e.g., in cases where the UE changes its preference frequently), to mitigate potential signaling overhead. The values of such a prohibit timer can be FFS, e.g., to align with the prohibit timer values already defined for other UAI features.

[RRC-6] Introduce a prohibit timer for the UAI used to provide UE’s preference on the ratio of measurement gap occasions for cancellation.  FFS on the values of this prohibit timer.

# Conclusion

Based on the analysis above, we have made the following observations and proposals:

- In the MAC running CR [1], it has already been explicitly captured in three separate cases (e.g., 5.4.2.2 “HARQ process”, 5.4.4 “Scheduling Request”, and 5.14 “Handling of measurement gap”) that the measurement gaps refer to the measurement gaps which have not been cancelled. However, some other cases are missing to be captured.

Proposal 1	To update legacy references of “possible occurrence of measurement gaps” in TS 38.321 §5.1.2 and §5.1.2a on “Random Access Resource selection” by explicitly capturing that measurement gaps should be the measurement gaps that have not been cancelled.

Proposal 2	If the Proposal 1 is agreed, adopt the TP in Appendix 1.

Proposal 3	To update legacy references of “during a measurement gap” in TS 38.321 §5.7 on “Discontinuous Reception (DRX)” by explicitly capturing that measurement gap should be the measurement gap that has not been cancelled.

Proposal 4	If the Proposal 3 is agreed, adopt the TP in Appendix 2.

Observation 2	RAN2 should further discuss and detail the implementation of the UAI for measurement gap cancellation, in line with the agreements reached within RAN4.

Proposal 5	[UE Capability] Introduce an optional UE capability to indicate UE’s support of the UAI used to provide UE’s preference on the ratio of measurement gap occasions for cancellation. A UE supporting this feature shall also indicate the support of DCI-based measurement gap cancellation.

Proposal 6	[RRC-5] If the UE is configured to provide UAI with its preference on the ratio of measurement gap occasions for cancellation, the UE initiates an UAI transmission to provide its preference on the ratio when:

- (1) The UE has a preference on the ratio and the UE did not transmit any preference on the ratio since it was configured to provide its preference on the ratio; or

- (2) The UE's preference on the ratio changed from the last time UE transmits the UAI to provide its preference on the ratio.

Proposal 7	[RRC-6] Introduce a prohibit timer for the UAI used to provide UE’s preference on the ratio of measurement gap occasions for cancellation.  FFS on the values of this prohibit timer.

# Reference

[POST129bis][502][XR] MAC running CR and open issues (Qualcomm)

R4-2504972 Reply LS on UE assistance information

# Appendix

## Appendix 1 – TP for RA Resource selection

5.1.2 Random Access Resource selection … 1> else if an SSB is selected above: 2> if the set of Random Access resources associated with Msg1 repetition is selected for this Random Access procedure: 3> determine the next available set of PRACH occasions (as specified in TS 38.213 [6]) for the Msg1 repetition number applicable for this Random Access procedure corresponding to the selected SSB (the MAC entity shall select a set of PRACH occasions randomly with equal probability amongst sets of PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB and selected Msg1 repetition number for this Random Access procedure; the MAC entity may take into account the possible occurrence of measurement gaps that have not been cancelled (as specified in clause 10.6 in [6]) and MUSIM gaps when determining the next available set of PRACH occasions corresponding to the selected SSB). 2> else: 3> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured, or indicated by PDCCH, or indicated by the LTM Cell Switch Command MAC CE (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps that have not been cancelled (as specified in clause 10.6 in [6]) and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB). else if a CSI-RS is selected above: 2> if there is no contention-free Random Access Resource associated with the selected CSI-RS: 3> determine the next available PRACH occasion from the PRACH occasions, permitted by the restrictions given by the ra-ssb-OccasionMaskIndex if configured, corresponding to the SSB in candidateBeamRSList which is quasi-colocated with the selected CSI-RS as specified in TS 38.214 [7] (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the consecutive PRACH occasions according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the SSB which is quasi-colocated with the selected CSI-RS; the MAC entity may take into account the possible occurrence of measurement gaps that have not been cancelled (as specified in clause 10.6 in [6]) and MUSIM gaps when determining the next available PRACH occasion corresponding to the SSB which is quasi-colocated with the selected CSI-RS). 2> else: 3> determine the next available PRACH occasion from the PRACH occasions in ra-OccasionList corresponding to the selected CSI-RS (the MAC entity shall select a PRACH occasion randomly with equal probability amongst the PRACH occasions occurring simultaneously but on different subcarriers regardless the FR2 UL gap, corresponding to the selected CSI-RS; the MAC entity may take into account the possible occurrence of measurement gaps that have not been cancelled (as specified in clause 10.6 in [6]) and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected CSI-RS). - perform the Random Access Preamble transmission procedure (see clause 5.1.3). 5.1.2a Random Access Resource selection for 2-step RA type … 1> determine the next available PRACH occasion from the PRACH occasions corresponding to the selected SSB permitted by the restrictions given by the msgA-SSB-SharedRO-MaskIndex if configured, or ra-ssb-OccasionMaskIndex if configured, or ssb-SharedRO-MaskIndex if configured (the MAC entity shall select a PRACH occasion randomly with equal probability among the consecutive PRACH occasions allocated for 2-step RA type according to clause 8.1 of TS 38.213 [6] regardless the FR2 UL gap, corresponding to the selected SSB; the MAC entity may take into account the possible occurrence of measurement gaps that have not been cancelled (as specified in clause 10.6 in [6]) and MUSIM gaps when determining the next available PRACH occasion corresponding to the selected SSB);

## Appendix 2 – TP for DCP in DRX

5.7 Discontinuous Reception (DRX) … 1> if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is not configured, and [(SFN × 10) + subframe number] modulo (drx-LongCycle) = drx-StartOffset; or 1> if the Long DRX cycle is used for a DRX group and the drx-NonIntegerLongCycleStartOffset is configured, and floor([(DRX_SFN_COUNTER × 10240) + (SFN × 10) + subframe number] modulo (drx-NonIntegerLongCycle)) = drx-StartOffset: 2> if DCP monitoring is configured for the active DL BWP as specified in TS 38.213 [6], clause 10.3: 3> if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or 3> if all DCP occasion(s) in time domain, as specified in TS 38.213 [6], associated with the current DRX cycle occurred in Active Time considering grants/assignments/DRX Command MAC CE/Long DRX Command MAC CE received and Scheduling Request sent until 4 ms prior to start of the last DCP occasion, or during a measurement gap that has not been cancelled (as specified in clause 10.6 in [6]), or when the MAC entity monitors for a PDCCH transmission on the search space indicated by recoverySearchSpaceId of the SpCell identified by the C-RNTI while the ra-ResponseWindow is running (as specified in clause 5.1.4); or 3> if ps-Wakeup is configured with value true and DCP indication associated with the current DRX cycle has not been received from lower layers: 4> start drx-onDurationTimer after drx-SlotOffset from the beginning of the subframe. 2> else: 3> start drx-onDurationTimer for this DRX group after drx-SlotOffset from the beginning of the subframe.