3GPP TSG RAN WG2 Meeting #130                                                            R2-2503522

# St Julian's, Malta, May 19th - 23rd, 2025

Agenda item:	8.7.4.1

Source:	Ofinno

Title:	LCP Enhancements for XR

Document for:	Discussion and decision

# Introduction

According to RAN2 agreements, the additional priority adjustment feature has already been introduced. Currently, RAN2 is also discussing whether further Bj enhancements are necessary on top of this feature. Additionally, in the MAC running CR email discussion [1], an open issue concerning the impact of UL congestion scenario (e.g., when PSI-based SDU discard is configured) on priority adjustment has been identified for further discussion.

RAN2#127bis Agreements - As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. RAN2#129 Agreements - Only one additional priority is configured to an LCH for LCP enhancement. - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold. RAN2#129bis Agreements - Working assumption (to be revisited next meeting): No Bj enhancement is introduced. - It will only be considered provided that gains are proven and that we have a common understanding on the change that is needed in specifications - All UEs falls back to default priority in the 2nd round of the LCP procedure if there is no LCH-priority adjusted data left, i.e. no capability is needed (this reverts previous RAN2 agreement on this).

In this contribution, we would like to discuss the following two issues:

- Issue 1: whether Bj enhancements are necessary on top of the priority adjustment feature.

- Issue 2: whether the introduction of the additional priority adjustment feature has any impact on the UL congestion scenario (e.g., when PSI-based SDU discard is configured).

# Discussion

## Impact on Bj enhancement

The legacy LCP procedure typically allocates resources during the first round only to LCHs having a positive Bj value. The Bj parameter essentially operates as a token-bucket mechanism, limiting each LCH’s allocated data volume over time and thus ensuring fairness among different LCHs. RAN2 has previously agreed that the UE may apply, for an LCH, additional/higher-priority adjustment when the LCH’s buffered data becomes priority-adjusted (i.e., when the remaining delay budget of data falls below a configurable threshold).

However, under the existing Bj-based resource allocation mechanism, a LCH with an upgraded priority may still fail to obtain sufficient resources if its Bj is negative or insufficient to accommodate the required priority-adjusted data volume. Consequently, despite its higher priority, this data may incur additional delays or be discarded.

To solve this issue, different approaches have been proposed in RAN2, seeking to ensure that priority-adjusted data is transmitted ahead of lower-priority one. After substantial discussions and convergence, RAN2#129 agreed not to introduce an additional PBR but decided to further study whether to allow serving a LCH with a negative Bj value during the first round of the LCP procedure, provided their remaining delay budget falls below a configurable threshold and subject to network configuration.

Subsequently, at RAN2#129bis, a working assumption of “No Bj enhancement is introduced” was made. The guidance is that it will only be considered if gains are proven and majority of companies have a common understanding on the required change in the specifications.

From our perspective, we recognize that a Bj enhancement might ensure timely delivery for delay-sensitive data and prevent potential data discarding or obsolescence. However, introducing such an enhancement would inevitably pose complexity and raise various concerns requiring careful analysis and further discussion. For example, allowing transmission of priority-adjusted data regardless of negative Bj may lead to excessive allocation and depletion of available resources. Furthermore, removing Bj constraints is effectively equivalent to significantly increasing or removing the upper bound for the PBR. This may lead to fairness issues, potentially starving other LCHs, contradicting the original fairness objectives of the current LCP approach.

In our view, existing LCP procedures have already allowed LCHs with negative Bj to be allocated resources during the second round of LCP and applied additional priority adjustments in the second round if the LCH still has priority-adjusted data, according to previous agreements. Proper network configuration of LCP parameters such as PBR, default priority, and adjusted priority for LCHs should also already address most scheduling-related issues effectively. Additionally, preserving fairness (preventing other LCHs from starvation) is a fundamental goal of the original token-bucket algorithm embedded within the LCP procedure.

Considering the unclear gains, potential complexity, and issues associated with Bj enhancement and given that current mechanisms complemented with appropriate network configuration should sufficiently satisfy requirements, we propose to confirm the working assumption as a formal agreement, i.e. no Bj enhancement is performed in Rel-19 XR.

Confirm the working assumption “No Bj enhancement is introduced” as an agreement.

## Impact of PSI-based SDU discard on priority adjustment

In Rel-18 XR, the PDU Set Importance (PSI) based SDU discard mechanism was introduced. This mechanism is configured by a new discard timer with a shorter value, discardTimerForLowImportance, which replaces the legacy discardTimer for SDUs belonging to low importance PDU sets. When the PSI-based SDU discard is configured, its operation can be dynamically activated or deactivated via a MAC CE. Upon the PSI-based SDU discard is activated, PDCP starts the discardTimerForLowImportance timer, with a shorter timer value, for PDCP SDUs belonging to low importance PDU sets. Consequently, low importance PDCP SDUs are discarded earlier during UL congestion scenarios. As a result, this mechanism would allow for high importance data, such as key video frames, to use sufficient uplink resources and be delivered with lower delay, thus enhancing performance in high-demand scenarios.

In addition to discarding low importance SDUs earlier, prioritizing high importance SDUs over low-importance SDUs in the uplink resource allocation process can also achieve the same goal. Based on the RAN2 agreements in Rel-19, it has been specified that LCHs with priority-adjusted data can apply an additional/higher priority in LCP procedure. However, the importance of the data was not taken into consideration when applying this enhanced priority adjustment feature.

As illustrated in Figure 1, there may be cases where this lack of consideration leads to suboptimal outcomes. For example, assume there are two logical channels, where the original priority of the first LCH is lower than that of the second LCH. If the first LCH has priority-adjusted data that is classified as being of low importance, while the second LCH has high importance data that does not yet qualify as priority-adjusted, the first LCH would nonetheless apply a higher priority for resource allocation. Consequently, low importance data from the first LCH would be transmitted earlier than high importance data from the second LCH, which undermines efficiency in UL congestion scenarios. Specifically, when the PSI-based SDU discard has already been activated due to UL congestion, the low importance data are generally expected to be discarded more quickly to mitigate congestion. However, due to the additional priority adjustment feature without properly accounting for the data importance, the low importance data might instead be prioritized over the high importance data for the resource allocation, allowing it to be transmitted unexpectedly early and delaying the high importance data. This ultimately results in prolonged or unresolved UL congestion.

Figure 1

Due to the additional priority adjustment feature without considering the data importance, the low importance data might instead be prioritized over the high importance data for the resource allocation, allowing it to be transmitted unexpectedly early and delaying the high importance data, which results in prolonged or unresolved congestion in UL.

To address this issue, we propose a solution for identifying priority-adjusted data by considering the data importance.

As illustrated in Figure 2. Specifically, during the LCP procedure, when determining whether a LCH has priority-adjusted data, the UE shall not consider low importance data as priority-adjusted data, regardless of whether such low importance data currently has a remaining time below the configured threshold. In other words, when the UE determines, for a LCH, the smallest remaining value of the running PDCP discardTimers among its PDCP SDUs buffered for the LCH available for this transmission, these PDCP SDUs shall not belong to a low importance PDU Set.

Moreover, since the resource allocation should remain under network control, we believe that the decision to enable or disable this feature could be left to network configuration (i.e., configurable). This can be further study.

Figure 2

For the evaluation of the remaining time on determining whether to apply the additional priority for a LCH, the UE should not consider low importance data in the LCH, instead should only consider the high importance data (i.e., non-low importance data) in the LCH.

# Conclusion

Based on the analysis above, we have made the following observations and proposals:

Proposal 1	Confirm the working assumption “No Bj enhancement is introduced” as an agreement.

- Due to the additional priority adjustment feature without considering the data importance, the low importance data might instead be prioritized over the high importance data for the resource allocation, allowing it to be transmitted unexpectedly early and delaying the high importance data, which results in prolonged or unresolved congestion in UL.

Proposal 2	For the evaluation of the remaining time on determining whether to apply the additional priority for a LCH, the UE should not consider low importance data in the LCH, instead should only consider the high importance data (i.e., non-low importance data) in the LCH.

# Reference

[POST129bis][502][XR] MAC running CR and open issues (Qualcomm)