3GPP TSG-RAN WG2 Meeting #130	R2-2503529

St.Julians, Malta, May. 19th – 23rd, 2025

Agenda Item:	8.9.3

Source: 	OPPO

Title:  	Discussion on CB-msg3 EDT and msg4 enhancement

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, the following agreements were made for uplink capacity enhancement.

Agreements 1. Both SA and DSA are mandatorily supported by UEs supporting CB-msg3-EDT 2. We will specify one single procedure to support both DSA and SA, i.e. SA is a special setting (k=1) of the overall procedure 3. For CB-msg3-EDT, the transmission window can be configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). 4. We don’t introduce support for eMTC CE mode B case (it will not be possible to signal resources to be used for this case) 5. We specify support for NB-IoT with 15kHz with no specific enhancements, leaving to NW implementation whether to implement this or not, accepting potential performance degradation. 6. If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission. 7. The start of CB-msg3 EDT transmission window is aligned with the start of time domain (N)PUSCH resource. 8. The CB-msg3 EDT transmission window length and periodicity may be different. FFS on possible signalling optimization in case the length and periodicity are the same. 9. RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply (CB-Msg3-EDT configuration for eMTC) 10. For eMTC, introduce a new IE (e.g. CB-Msg3-ConfigSIB-r19) for shared resources configuration of CB-Msg3 in SIB2. 11. For eMTC, introduce MPDCCH configuration in shared resources configuration. The fields in IE PUR-MPDCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters (e.g. whether additional narrow band is needed). 12. We will not support TDD related parameters. 13. For eMTC, introduce PUSCH configuration in shared resources configuration. The fields in IE PUR-PUSCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (e.g. whether pusch-CyclicShift-r16, pusch-NB-MaxTBS-r16 are needed, whether prb-AllocationInfo should be defined as a “set” format with intention to provide a set of shared frequency-domain resources). 14. For eMTC, check with RAN1 if anything is needed for PDSCH configuration in shared resources configuration 15. For eMTC, introduce PUCCH configuration in shared resources configuration. The fields in IE PUR-PUCCH-Config-r16 could be reused as baseline. Confirm with RAN1 on the detail parameters. (CB-Msg3 configuration for NB-IoT) 16. For NB-IoT, introduce a new IE (e.g. CB-Msg3-ConfigSIB-NB-r19) for shared resources configuration of CB-Msg3 in SIB2-NB and SIB22-NB for non-anchor carrier. 17. For NB-IoT, introduce below physical layer parameters in shared resources configuration as below: - Number of resource units for NPUSCH (as in npusch-NumRUsIndex-r16) - Number of repetitions for NPUSCH (as in npusch-NumRepetitionsIndex-r16) - Set of subcarriers (similar to npusch-SubCarrierSetIndex but change it to a “set”), FFS whether subcarriers are provided as a contiguous set. - MCS configuration for NPUSCH (as in npusch-MCS-r16). - PDCCH parameters (as in NPDCCH-ConfigDedicated-NB-r13) - The non-anchor carrier index for monitoring Msg4. If this field is absent, anchor carrier is assumed to be used. NOTE: confirm with RAN1 is needed 18. For NB-IoT, FFS whether periodicity of CB-Msg3 resource may be larger than H-SFN duration Agreements (part 2): 1. For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window). 2. The lengths of the Msg3 transmission and Msg4 monitoring windows are configured by the network (in case of k=1 it will be possible to configure the parameters in a way to have the same behaviour as for normal Random Access procedure) 3. The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not). 4. FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring Agreements (part3): 1. The CB-msg3-EDT configuration (e.g., number of replicas, number of time resources and number of frequency resources) is CE level specific. 2. The Msg4 monitoring window configuration (e.g. length) is CE level specific 3. RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized. 4. RAN2 confirms that existing UP-EDT and CP-EDT RRC message procedures shall be applicable for CB-Msg3-EDT - For UP solution, after RRCConnectionResumeRequest, network may reply with RRCConnectionResume, RRCConnectionSetup, RRCConnectionRelease and RRCConnectionReject - For CP solution, after RRCEarlyDataRequest, the network can respond with RRCEarlyDataComplete, RRCConnectionSetup or RRCConnectionReject. 5. RAN2 also intends to support CB-msg3-EDT for MT cases 6. The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI) 7. Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1 8. The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4. 9. Parameter for maximum re-attempt number per CE level is introduced and UE can re-attempt in the same CE level due to contention resolution failure until the max re-attempt number has been reached. 10. Backoff information could be included in CB-Msg4. 11. L1 ACK as the Msg4 for the CB-Msg3-EDT is not supported. 12. HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

In this contribution, we would like to discuss the remaining open issues based on the outcome of [Post129bis][311].

Discussion

CB-RNTI calculation

In last RAN2 meeting, it was agreed to introduce a new RNTI (i.e., CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling,

- For CB-msg3-EDT we adopt a Single Msg4 monitoring window and Single RNTI (the RNTI is derived on the transmit resource for the transmission window). 7. Introduce a new RNTI (i.e. CB-RNTI) for CB-Msg4 monitoring and CB-Msg3 scrambling. We include this agreement in the LS to RAN1

The newly introduced CB-RNTI is derived based on the resource information for the transmission window while the details are unclear. Since a single RNTI will be adopted for CB-msg4 monitoring, the time or frequency information of a dedicated PUSCH occasion within the transmission window can not be considered in CB-RNTI calculation, i.e., in DSA case, the same CB-RNTI should be calculated even if different PUSCH occasions are selected for replica transmission. The simplest solution is to introduce a preconfigured/predefined CB-RNTI. For potential overlapping between the CB-Msg4 monitoring windows started for different CB-Msg3 transmissions, it can be avoided by NW configuration, e.g., defining the response window shorter than transmission window.

The CB-RNTI is preconfigured or predefined.

The length of CB-Msg3 response window is configured shorter than the length of CB-Msg3 transmission window.

Need of NW/UE processing time

In RAN2#129bis, it was agreed that the Msg4 monitoring window starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT. Further discussion is needed on whether or not to consider NW/UE processing time.

3. The Msg4 monitoring starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not).

For PUR, 4 subframes processing delay was considered when starting the DL response reception window, i.e., pur-ResponseWindowTimer.

After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer: - if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT. - else: - the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes.

As agreed, CB-Msg3 response window starts at the end of CB-Msg3 transmission window. It is possible that the processing time has been covered by CB-Msg3 transmission window in case that the UE has finished all replica transmissions at a relative early point within the window. But for other cases such as SA or the replica transmissions finish at the last PUSCH occasion in the window, NW/UE processing time is still required. Since we aim to have a unified solution, it is proposed that processing delay should be considered when starting the CB-Msg3 response window for saving UE power consumption. Different from PUR, the eNB may face greater challenges in processing the CB-msg3 because of contention, thus it is questionable whether 4 subframes is enough and we think the proper value should be decided by RAN1.

The processing time is considered for the start of CB-Msg3 response window. Up to RAN1 to determine the value.

Start of CB-Msg3 response window

In last meeting, it was also discussed whether it is possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT.

4. FFS it will also be possible for the NW to configure that the Msg4 monitoring window starts in the subframe containing the last (N)PUSCH repetition of the first replica plus UE-eNB RTT (FFS NW/UE processing time). To possibly resolve the FFS it needs to be clarified what happens if the Msg4 monitoring window is overlapping with replica, i.e. whether the UE prioritize the replica transmission or monitoring

The solution is helpful for reducing latency of CB-Msg4 reception but there will be DL/UL conflict issue in half-duplex system. When conflict occurs in this case, further discussion is needed whether the UE prioritize transmission or reception. Since the NW has no idea whether the UE is preparing for replica transmission when it schedules CB-Msg4, DL reception should be prioritized to avoid any missing. But if the UE operates in this way, the advantage of DSA will be eliminated by dropping the opportunity to transmit replica. For simplicity, it is suggested we do not consider the solution that the Msg4 monitoring window starts at the end of the first replica.

Do not consider the solution that the Msg4 monitoring window starts at the end of the first replica.

Support of L2 ACK

In last meeting, it was discussed whether a CB-Msg4 without RRC message is allowed as the complete response to the CB-Msg3 in CP solution. This discussion was postponed to the next meeting.

For CB-Msg3 EDT in CP solution, RRC message is not mandatory if there is no follow-up DL data. In PUR, DL response optimization has been specified for CP solution, i.e., eNB may terminate the PUR procedure by sending a L1/L2 ACK.

As agreed, C-RNTI will be included in CB-Msg4 if the UE is expected to receive additional RRC message or data from NW after CB-Msg4. For the FFS, the UE should further check whether there is C-RNTI included to determine whether the CB-Msg3 in CP solution can be terminated, i.e., in case that C-RNTI is not included, Contention resolution ID is enough to terminate the procedure.

For CB-Msg3-EDT in CP solution, the procedure can be considered as terminated if only contention resolution ID is included in CB-Msg4.

In case that the C-RNTI is included and there is no RRC message (i.e., RRCConnectionSetup) included in CB-Msg4, the UE should keep monitoring the follow-up CB-Msg3 responses after CB-Msg4. Once a further RRC message or data is received, the procedure is terminated.

If C-RNTI is included in a CB-Msg4 without RRC message, the CB-Msg3-EDT is considered as terminated once the additional RRC message or data is received after CB-Msg4.

CB-Msg3 retransmission

It is unclear whether HARQ operation is applicable to transmit CB-Msg3. In EDT/RACH, msg3 re-transmission scheduled by TC-RNTI is supported and it aids in faster contention resolution. While for CB-Msg3, NW-sheduled retransmission would be complicated and unnecessary for the following reasons:

- The use of CB-Msg3 resources is randomly selected by the UE, the NW can not determine whether the decoding failure is cause by false alarm, collision or radio problem. In case of false alarm, retransmission scheduling will cause waste of source.

- In case of DSA, the NW will receive replicas from the same UE. If the decoding fails for all replicas, only one retransmission is needed. However, since the NW can not identify which PUSCHs are selected by the UE for replica transmissions, it may schedule retransmission for the same UE multiple times, which is also resource wasting.

Based on above discussion, it is proposed that the NW-scheduled CB-msg3 retransmission is not supported.

NW-scheduled CB-Msg3 re-transmission is not supported.

HARQ feedback indicator

We had agreement that HARQ feedback is adopted to acknowledge Msg4.

12. HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

Further discussion is needed on how to provide the PUCCH resources for HARQ feedback when there is multiplexing in CB-Msg4. The case is same as NR 2-step RACH where the MsgB can include the feedbacks to multiple UEs. For UE with contention resolution, suceessRAR will be included in MsgB and the format is shown as follows.

In successRAR, the resource information for HARQ feedback is included, i.e., TPC, HARQ Feedback Timing Indicator, PUCCH Resource Indicator. The format of MsgB enables the NW to respond to multiple UEs in the same DL and provide HARQ feedback resources for each UE with contention resolution. With the same requirement, the HARQ feedback resources should also be indicated in CB-Msg4.

The resource information for HARQ feedback is indicated in CB-Msg4 for each UE with contention resolution.

CB-Msg4 details

As per agreements, the information that can be sent to the UE in response to CB-Msg3 at least includes Contention resolution ID, C-RNTI, RRC message, TAC, Backoff indication and HARQ feedback indicator.

Agreement-RAN2#128 9. The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4) Agreement-RAN2#129 8. The timing alignment information (FFS reusing TAC MAC-CE) can be included in the CB-Msg4. Agreement-RAN2#129b 6. The C-RNTI is included in CB-Msg4 if the UE is expected to receive additional RRC messages or data from the network after CB-Msg4 (FFS how to include the C-RNTI) 10. Backoff information could be included in CB-Msg4. 12. HARQ feedback is adopted to acknowledge Msg4. FFS for the detail (e.g., how the HARQ feedback is used for each response in Msg4 when there is multiplexing in Msg4.).

In last RAN2 meeting, it was confirmed that one CB-Msg4 can target multiple UEs simultaneously.

3. RAN2 confirms the working assumption that one CB-Msg4 can target multiple UEs simultaneously. FFS how the multiplexing is organized.

For multiplexing CB-Msg4, the following information should be included in the MAC PDU.

- A MAC subheader with Backoff indicator.

- MAC subheaders with time/frequency info of PUSCH occasion. Since only one RNTI will be used for CB-Msg4 monitoring, time/frequency info of the PUSCH occasion can be further indicated in CB-msg4 payload to distinguish the UEs selecting different POs within the transmission window. When there is no matched time/frequency info in MAC subheaders, the UE can save the effort to process the left part of the MAC PDU. Each MAC subheader corresponds to a successRAR

- successRAR for the UE with contention resolution. In successRAR, Contention resolution ID, C-RNTI, RRC message, TAC, Resource information for HARQ feedback can be included. Except for Contention resolution ID and resource information for HARQ feedback, other elements can be optionally present.

In multiplexed CB-msg4, the following information should be included in MAC PDU:

  - A MAC subheader with Backoff indicator.

  - MAC subheaders with time/frequency info of PUSCH occasion.

  - successRAR including Contention resolution ID, C-RNTI, RRC message, TAC, Resource Information for HARQ feedback.

Fallback

Whether the UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails was discussed in the RAN2#129bis but no consensus was made. In PUR, when fallback indication is received or the PUR procedure is considered as failure in MAC, RRC will be notified and the follow-up actions is up to UE implementation, e.g., trigger EDT or legacy RACH.

#### 5.3.3.3d UE actions upon receiving PUR indications from lower layers The UE shall: 1> if repetition adjustment is indicated by lower layers: 2> update numRepetitions (npusch-NumRepetitionsIndex in NB-IoT) in previously stored pur-Config in accordance with the received indication; 1> if pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is configured and timing advance adjustment is indicated by lower layers: 2> replace the serving cell reference (N)RSRP value with the current serving cell (N)RSRP value (see 5.3.3.19); For CP transmission using PUR, upon indication from lower layers that transmission using PUR is successfully completed, the UE shall perform the actions as specified in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received. Upon reception of PUR fallback or PUR failure indication from lower layers, the procedure ends. NOTE: For transmission using PUR, further UE actions upon reception of PUR fallback or PUR failure indication from lower layers (see TS 36.321 [6]) is left up to implementation.

For CB-Msg3-EDT, the same behaviour can be followed, i.e., when the maximum attempt time of CB-Msg3 is reached, MAC indicates CB-Msg3-EDT failure to upper layer. In RRC, the follow-up UE actions is left up to implementation when CB-Msg3-EDT failure indication is received from lower layers.

MAC indicates CB-Msg3-EDT failure to upper layer when the maximum attempt time of CB-Msg3 is reached.

In RRC, further UE actions upon reception of CB-Msg3-EDT failure indication from lower layers is left up to implementation.

Conclusion

Based on the discussion in section2, we have following observation proposals:

Proposal 1	The CB-RNTI is preconfigured or predefined.

Proposal 2	The length of CB-Msg3 response window is configured shorter than the length of CB-Msg3 transmission window.

Proposal 3	The processing time is considered for the start of CB-Msg3 response window. Up to RAN1 to determine the value.

Proposal 4	Do not consider the solution that the Msg4 monitoring window starts at the end of the first replica.

Proposal 5	For CB-Msg3-EDT in CP solution, the procedure can be considered as terminated if only contention resolution ID is included in CB-Msg4.

Proposal 6	If C-RNTI is included in a CB-Msg4 without RRC message, the CB-Msg3-EDT is considered as terminated once the additional RRC message or data is received after CB-Msg4.

Proposal 7	NW-scheduled CB-Msg3 re-transmission is not supported.

Proposal 8	The resource information for HARQ feedback is indicated in CB-Msg4 for each UE with contention resolution.

Proposal 9	In multiplexed CB-msg4, the following information should be included in MAC PDU:

a.	A MAC subheader with Backoff indicator.

b.	MAC subheaders with time/frequency info of PUSCH occasion.

c.	successRAR including Contention resolution ID, C-RNTI, RRC message, TAC, Resource Information for HARQ feedback.

Proposal 10	MAC indicates CB-Msg3-EDT failure to upper layer when the maximum attempt time of CB-Msg3 is reached.

Proposal 11	In RRC, further UE actions upon reception of CB-Msg3-EDT failure indication from lower layers is left up to implementation.