3GPP TSG-RAN WG2 Meeting #130	R2-2503532

St.Julians, Malta, May 19th – 23rd , 2025

Source: 	Xiaomi

Title:  	Remaining issues of inter-CU LTM

Agenda Item:	8.6.2

Document for:	Discussion and Decision

# Introduction

[1] provided the list of remaining FFSs according to the RRC running CR in [2] for LTM and CLTM that need to be solved, as shown below:

(RRC-1) Editor’s Note: FFS whether additional text for security key change in LTM (at the MCG and SCG) is needed. Rapporteur comment: This issue can be solved in the discussions for the running CR and no contribution from companies is needed. (RRC-2) Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message. Rapporteur comment: Whether this is an issue need to be clarified, but probably good to discuss this based on companies’ contributions (if companies think that current text is not okay). (RRC-3) Editor’s Note: FFS how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution. Rapporteur comment: Current procedural text leaves this case to the UE implementation (similar to CHO), so the proposal is to keep current text as it is and delete the FFS. (RRC-4) Editor’s Note: How to release ltm-ConfigNRDC is FFS. Rapporteur comment: This is a purely stage3 issue and the plan is for the rapporteur to propose a solution on how to address this. No contribution should be needed from companies. (RRC-5) Editor’s Note: FFS how report configuraiton for CSI acqusition should be configured. Pending with further R1 progress Rapporteur comment: This issue is related to how RAN1 decides to design the CSI-RS measurements for LTM. We can wait for their RAN1 parameter list. (RRC-6) Editor’s Note: FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration. Rapporteur comment: It is still not crystal clear how the fast recovery procedure would work, and especially when the UE should revert back to the previous PCell configuration. We suggest to discuss this based on companies’ contribution. (RRC-7) Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM. Rapporteur comment: This is something that we never discussed. Current Running CR re-use the list of sk-counters also for the case of inter-MN LTM (to support subsequent LTM) so our suggestion is to keep current text. (RRC-8) FFS if fast failure recovery with different Rel-19 IDs is allowed. Rapporteur comment: This is an open issue from last RAN2 meeting and, even if we don’t support the case mentioned in the FFS, things should work fine anyway. However, our suggestion is to discuss this based on companies’ contribution.

[3] provided MAC open issues for inter-CU LTM, as shown below:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| MAC-18 | FFS whether CFRA resource can be associated with CSI-RS Editor’s NOTE: FFS whether CFRA resource can be associated with CSI-RS. [Rapp]: Based on comments from ZTE and Samsung. | Issue Type: Not essential but important How to address it: based on companies’ contribution and input from RAN1. |
| MAC-19 | FFS whether NCC is optional present in Enhanced LTM Cell Switch Command MAC CE. FFS for which cases should this Enhanced LTM Cell Switch Command MAC CE is used, e.g. whether only for inter-CU case with security key update. Editor’s NOTE: FFS whether NCC is optional present in Enhanced LTM Cell Switch Command MAC CE. FFS for which cases should this Enhanced LTM Cell Switch Command MAC CE is used, e.g. whether only for inter-CU case with security key update. [Rapp]: Based on comments from ZTE and Samsung. | Issue Type: Essential How to address it: based on companies’ contribution |

In this contribution, we will discuss remaining issues of inter-CU LTM.

# Discussion

According to rapporteur comments in [1][2], we will provide our views for the following issues and classify these issues as follows:
LTM based failure recovery:

  - (RRC-8) FFS if fast failure recovery with different Rel-19 IDs is allowed.

  - (RRC-6) Editor’s Note: FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration.

Security issues of inter-MN LTM:

  - (MAC-19) FFS whether NCC is optional present in Enhanced LTM Cell Switch Command MAC CE. FFS for which cases should this Enhanced LTM Cell Switch Command MAC CE is used, e.g. whether only for inter-CU case with security key update.

  - (RRC-7) Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM.

Other issue：

  - (RRC-2) Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message.

  - LTM based failure recovery

RAN2 has reached the following agreement for the LTM based failure recovery [4]:

- At RLF and reconfiguration with sync failure: - If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18). - if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

o	(RRC-8) FFS if fast failure recovery with different Rel-19 IDs is allowed.

For inter-CU LTM failure recovery, there are 3 cases:

- Case 1: the selected candidate cell has the same Rel-19 ID as source

- Case 2: the selected candidate cell has the same Rel-19 ID as target

- Case 3: the selected candidate cell has the different Rel-19 ID as target and source.

Case 2 has been supported. the open issue is whether to support Case 1 and Case 3 for fast failure recovery.

For Case 1:

In Rel-18 LTM, the LTM based recovery is not supported for T304 expiry after HO (L3 HO) and CHO, because HO/CHO execution may incur PDCP and RLC re-establishment, which prohibits UE from reverting back to configuration and L2 state used in the source cell before HO/CHO attempt [5].

Similar to L3 HO and CHO failure, the inter-CU LTM execution incurs PDCP and RLC re-establishment, and the UE cannot fully revert back to the source configuration and L2 state used in the source cell before inter-CU LTM attempt. Hence, if there is no AS Key update in the recovery procedure, the Case 1 is not feasible.

If the UE performs the AS key update during recovery, there is no need to worry about whether the UE can fully revert back to source configuration or not after failure and Case 1 is feasible, because the UE needs to perform PDCP and RLC re-establishment with AS Key update.

Observation 1: (RRC-8) If there is no AS Key update in the recovery procedure, the Case 1(the selected candidate cell has the same Rel-19 ID as source for recovery after inter-CU LTM failure) is not feasible. Because the UE cannot fully revert back to source configuration after inter-CU LTM execution.

The key point to support Case1 is the source gNB needs to generate the new KgNB*(s) for LTM candidate cells belonging to the source gNB before the LTM execution. And after inter-CU LTM failure, even if the selected cell has the same Rel-19 ID as source, the UE can perform LTM based failure recovery to the selected cell with AS Key update. These new KgNB*(s) can only be used for recovery procedure, since RAN2 has agreed: “NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.”

Observation 2: (RRC-8) To support the key update during recovery for Case 1, the source gNB needs to generate the new KgNB*(s) for LTM candidate cells belonging to the source before the LTM execution. And these new KgNB*(s) can only be used for recovery procedure.

Whether the source gNB can generate the new KgNB*(s) for LTM candidate cells belonging to the source gNB only for LTM based failure recovery shall be finally decided by SA3 and RAN3. To support Case 1, RAN2 needs to send an LS to SA3 and RAN3.

For Case 3:

Whether to support Case 3 is up to how and when to distribute KgNB* between the source gNB and the candidate gNBs.

RAN3 has discussed the issue, reached the following agreements [6] and sent an LS [7] to SA3 and CC RAN2:

RAN3#127bis - The new NCC value needs to be sent from gNB-CU to the gNB-DU via UE context modification procedure. - For the initial inter-CU LTM preparation, the source gNB computes the KgNB*(s) (per candidate cell) for the candidate gNB, and forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy. - The new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure. FFS on the procedure design either per cell or per gNB.

In the signalling procedure for inter-CU LTM [8], the LTM Configuration Update procedure is step 20, which is before the current source gNB determines to initiate LTM. The BL CR to 38.423 for inter-CU LTM [9] indicates the purpose of the LTM configuration update:

### 8.x.1 LTM Configuration Update …… The purpose of the LTM Configuration Update procedure is to update LTM configuration data, or to inform the change of serving NG-RAN node and setup the UE associated signalling between the new serving NG-RAN node and all other candidate NG-RAN nodes. The procedure uses UE associated signalling.

Based on RAN3 agreements and the description captured in RAN2 and RAN3 Draft CR [8][9], the source gNB distributes the new KgNB*(s) before receiving the L1 or L3 measurement report form UE:

- For initial LTM, the source gNB forwards the {KgNB*, NCC} pair(s) to the candidate gNB via a Handover Request message as legacy.

- For subsequent LTM, the new source gNB needs to provide the new KgNB*(s) to the corresponding candidate gNB via a LTM configuration update procedure.

At this time, the (new) source gNB cannot determine the target gNB for initial/subsequent LTM execution. So, the (new) source gNB needs to provide the new KgNB*(s) to all other candidate gNBs via multiple Handover Request messages /LTM CONFIGURATION UPDATE messages.

In TS 33.501 [10], the gNB acquires the NCC value in the path switch procedure. The AMF shall then send the newly computed {NH, NCC} pair to the target gNB/ng-eNB in the NGAP PATH SWITCH REQUEST ACKNOWLEDGE message. Hence, the network side has only one available NCC value. And the (new) source gNB generates the new KgNB*(s) to all other candidate gNBs based on the same NCC value.

After the LTM configuration preparation/update procedure, the source gNB decides the target cell and the gNB based on the L1 or L3 measurement to trigger inter-CU LTM and sends the LTM CSC with indicating the NCC value to UE. If the LTM cell switch to the target gNB failed, other candidate gNBs (except for the source gNB) also have the new KgNB*(s) for the UE. The new KgNB*(s) of other candidate gNBs can be used for failure recovery. As a consequence, Case 3 can be supported.

Observation 3: (RRC-8) After inter-CU LTM failure, other candidate gNBs (except for the source gNB) also have the new KgNB*(s) for the UE. The new KgNB*(s) of other candidate gNBs can be used for failure recovery.

In summary, for inter-CU LTM failure (the R19 set ID is different between the target cell and source cell), at least Case 2 and Case 3 can be supported. RAN2 can discuss whether to support Case 1. If RAN2 wants to support Case1, RAN2 needs to send an LS to SA3 and RAN3 to ask whether the source gNB can generate the new KgNB*(s) for LTM candidate cells belonging to the source gNB only for LTM based failure recovery. To sum up, we provide the following proposals.

Proposal 1a: (RRC-8) For inter-CU LTM failure (the R19 set ID is different between the target cell and source cell), if the selected candidate cell has the different Rel-19 ID of source, the UE performs fast failure recovery.

Proposal 1b: (RRC-8) For inter-CU LTM failure (the R19 set ID is different between the target cell and source cell), RAN2 can discuss whether to support the LTM based fast failure recovery with AS Key update, if the selected candidate cell has the same Rel-19 ID as source cell.

Proposal 1c: (RRC-8) If RAN2 supports the case in P1b, RAN2 needs to send an LS to SA3 and RAN3 to ask whether the source gNB can generate the new KgNB*(s) for LTM candidate cells belonging to the source only for LTM based failure recovery.

o	 (RRC-6) Editor’s Note: FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration.

For the issue, according to the RRC running CR in [2], there are two options can be considered:

- Option 1: Upon inter-CU LTM failure, the UE reverts back to the previous source PCell configuration.

- Option 2: Upon inter-CU LTM failure, the UE keeps the target configuration. Then if the recovery attempt fails, the UE reverts back to the previous source PCell configuration.

Which options to choose depends on whether the key of the target cell can be reused for the selected cell during recovery. To reuse the key of the target cell, option 2 shall be supported, otherwise, option 1 can be considered which is the legacy behaviours of reconfiguration with sync failure.

For the above-mentioned three cases, only Case 2 can reuse the key of the target cell. It is up to whether the KgNB*(s) computed by the source gNB is per candidate cell or per candidate gNB.

- If the KgNB*(s) computed by the source gNB is per candidate cell, for case 2, AS key update is needed, the UE performs AS key derivation to the selected candidate cell during recovery using NCC value in LTM CSC, and option 1 can be supported.

- If the KgNB*(s) computed by the source gNB is per candidate gNB, for case 2, the UE can reuse the key of the target cell, and option 2 can be supported.

Proposal 2a: (RRC-6) “which point in time, for fast recovery, the UE reverts back to the previous PCell configuration” depends on whether the KgNB*(s) computed by the source gNB is per candidate cell or per candidate gNB:

•	(If it is per candidate cell): Upon inter-CU LTM failure, the UE reverts back to the previous source PCell configuration.

•	(If it is per candidate gNB): Upon inter-CU LTM failure, the UE keeps the target configuration. Then if the recovery attempt fails, the UE reverts back to the previous source PCell configuration.

Based on RAN3 meeting report [6], for initial LTM preparation, the KgNB*(s) is per candidate cell, but for LTM configuration update, it is still FFS (“FFS on the procedure design either per cell or per gNB”). Hence, for the RRC open issue, we suggest to send an LS to ask RAN3 whether the KgNB*(s) computed by the source gNB is per candidate cell or per candidate gNB in the LTM configuration update procedure.

Proposal 2b: (RRC-6) RAN2 can send an LS to RAN3 to ask whether the KgNB*(s) computed by the source gNB is per candidate cell or per candidate gNB in the LTM configuration update procedure.

  - Security issues of inter-MN LTM

o	(MAC-19) FFS whether NCC is optional present in Enhanced LTM Cell Switch Command MAC CE. FFS for which cases should this Enhanced LTM Cell Switch Command MAC CE is used, e.g. whether only for inter-CU case with security key update.

For the issue, if the NCC is optional present in Enhanced LTM Cell Switch Command MAC CE, RAN2 needs to discuss how the UE knows whether NCC value is included or not. Based on comments in [11], there are two methods:

- Method 1: We need another bit to indicate this.

- Method 2: The UE can identify whether NCC value is included or not based on the R19 set ID.

For method 1, an extra indication is introduced. For method 2, the RRC layer needs to inform Rel-19 ID(s) of serving and candidate cell(s) to MAC layer. Either method 1 or method 2 results in extra UE complexity. A simple method is the NCC is mandatory present in Enhanced LTM Cell Switch Command MAC CE. And the new MAC CE is used only for LTM with security key update. If no security key update, the legacy MAC CE is used.

Proposal 3: (MAC-19) The NCC is mandatory present in Enhanced LTM Cell Switch Command MAC CE. And the Enhanced LTM Cell Switch Command MAC CE is used only for LTM with security key update.

o	(RRC-7) Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM.

For the SN key update in inter-MN LTM, RAN2 has agreed “NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.” This is aligned with the legacy configuration of L3 HO with SCG configuration and the sk-Counter is included in the RRCReconfiguration of target configuration, as shown below [12]:

RRCReconfiguration-v1560-IEs ::= SEQUENCE { mrdc-SecondaryCellGroupConfig SetupRelease { MRDC-SecondaryCellGroupConfig } OPTIONAL, -- Need M radioBearerConfig2 OCTET STRING (CONTAINING RadioBearerConfig) OPTIONAL, -- Need M sk-Counter SK-Counter OPTIONAL, -- Need N nonCriticalExtension RRCReconfiguration-v1610-IEs OPTIONAL }

The SN key update procedure for inter-MN LTM can be achieve by legacy UE behaviours [12], which is aligned with the above agreement, as shown below:

TS 38.331[12] First, the UE apply the RRCReconfiguration of LTM candidate configuration upon LTM execution, as shown below: In 5.3.5.18.6 LTM cell switch execution:. And then, if the RRCReconfiguration includes the sk-Counter, UE performs SN key update, as shown below: In 5.3.5.3 Reception of an RRCReconfiguration by the UE:.

Some companies may have concerns on the reuse of sk-Counter upon inter-CU LTM. For inter-MN LTM with SCG, the MN key is updated, and the UE derives the SN key based on the sk-Counter and the new MN key. Even if the same sk-Counter is reused, the SN key is also updated, there is no security issues.

Hence, the list of sk-counters can be used only for the case of inter-SN LTM. For inter-MN LTM, the UE does not use the list of sk-counters.

Proposal 4: (RRC-7) The list of sk-counters can be used only for the case of inter-SN LTM. For inter-MN LTM, the UE does not use the list of sk-counters.

  - Open issue: RRC-2

o	(RRC-2) Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message.

In the discussion of Rel-18 LTM handling [13], the appliedLTM-CandidateId is introduced to avoid the re-use of the same RRC transaction ID for RRCReconfiguration messages which are pre-configured at the UE. For both MCG LTM and SCG LTM, the appliedLTM-CandidateId is useful. In Rel-18, for both MCG LTM and SCG LTM, the appliedLTM-CandidateId needs to be included in the RRCReconfigurationComplete messge.

Specifically, in Rel-19 LTM, inter-SN SCG LTM is supported and there are two-layer RRCReconfigurationComplete message, one is MN RRCReconfigurationComplete and the other is SN RRCReconfigurationComplete. The LTM candidate Id is allocated by the MN, and in preparation procedure, each target SN can also know the candidate Id allocation provided by the MN. Hence, appliedLTM-CandidateId is useful for both MN and SN to differ different LTM candidate configuration.

Hence, the appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message

Proposal 5: (RRC-2) The appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message.

# Conclusions

According to the analysis given above, we have the following observations and proposals:

2.1	LTM based failure recovery

o	(RRC-8) FFS if fast failure recovery with different Rel-19 IDs is allowed.

Observation 1: (RRC-8) If there is no AS Key update in the recovery procedure, the Case 1(the selected candidate cell has the same Rel-19 ID as source for recovery after inter-CU LTM failure) is not feasible. Because the UE cannot fully revert back to source configuration after inter-CU LTM execution.

Observation 2: (RRC-8) To support the key update during recovery for Case 1, the source gNB needs to generate the new KgNB*(s) for LTM candidate cells belonging to the source before the LTM execution. And these new KgNB*(s) can only be used for recovery procedure.

Observation 3: (RRC-8) After inter-CU LTM failure, other candidate gNBs (except for the source gNB) also have the new KgNB*(s) for the UE. The new KgNB*(s) of other candidate gNBs can be used for failure recovery.

Proposal 1a: (RRC-8) For inter-CU LTM failure (the R19 set ID is different between the target cell and source cell), if the selected candidate cell has the different Rel-19 ID of source, the UE performs fast failure recovery.

Proposal 1b: (RRC-8) For inter-CU LTM failure (the R19 set ID is different between the target cell and source cell), RAN2 can discuss whether to support the LTM based fast failure recovery with AS Key update, if the selected candidate cell has the same Rel-19 ID as source cell.

Proposal 1c: (RRC-8) If RAN2 supports the case in P1b, RAN2 needs to send an LS to SA3 and RAN3 to ask whether the source gNB can generate the new KgNB*(s) for LTM candidate cells belonging to the source only for LTM based failure recovery.

o	 (RRC-6) Editor’s Note: FFS on which point in time, for fast recovery, the UE reverts back to the previous PCell configuration.

Proposal 2a: (RRC-6) “which point in time, for fast recovery, the UE reverts back to the previous PCell configuration” depends on whether the KgNB*(s) computed by the source gNB is per candidate cell or per candidate gNB:

•	(If it is per candidate cell): Upon inter-CU LTM failure, the UE reverts back to the previous source PCell configuration.

•	(If it is per candidate gNB): Upon inter-CU LTM failure, the UE keeps the target configuration. Then if the recovery attempt fails, the UE reverts back to the previous source PCell configuration.

Proposal 2b: (RRC-6) RAN2 can send an LS to RAN3 to ask whether the KgNB*(s) computed by the source gNB is per candidate cell or per candidate gNB in the LTM configuration update procedure.

2.2 Security issues of inter-MN LTM

o	(MAC-19) FFS whether NCC is optional present in Enhanced LTM Cell Switch Command MAC CE. FFS for which cases should this Enhanced LTM Cell Switch Command MAC CE is used, e.g. whether only for inter-CU case with security key update.

Proposal 3: (MAC-19) The NCC is mandatory present in Enhanced LTM Cell Switch Command MAC CE. And the Enhanced LTM Cell Switch Command MAC CE is used only for LTM with security key update.

o	(RRC-7) Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM.

Proposal 4: (RRC-7) The list of sk-counters can be used only for the case of inter-SN LTM. For inter-MN LTM, the UE does not use the list of sk-counters.

2.3	Open issue: RRC-2

o	(RRC-2) Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message.

Proposal 5: (RRC-2) The appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message.

# Reference

- R2-250xxxx, “List of RRC FFSs for mobility”, Ericsson

- R2-250xxxx, “Running RRC CR for inter-CU and conditional LTM”, Ericsson

- R2-250xxxx, “Discussion summary and list of MAC open issues for mobility”, vivo

- Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China

- R2-2402235, “Fast Recovery with LTM Candidates”, MediaTek Inc.

- R3-25xxxxx, “RAN3#127bis Meeting Report”

- R2-2503320, “Reply LS for Reply LS on security handling for inter-CU LTM in non-DC cases (R3-252449; contact: CATT)”

- R2-250xxxx, “Introduction of NR mobility enhancements Phase 4 in TS 38.300”, Apple Inc

- R3-252520, “(BL CR to 38.423) Xn support for inter-CU LTM”, Ericsson, Samsung, Nokia, China Telecom, CATT, Huawei, Google, Lenovo, NEC, ZTE, LG Electronics, Ofinno, Qualcomm

- 3GPP TS 33.501

- R2-250xxxx, “Running MAC CR for enhanced mobility Ph4”, vivo

- 3GPP TS 38.331

- R2-2401368, Discussion on extending transaction ID space [E074], Ericsson