3GPP TSG-RAN WG2 #130 meeting																		R2-2503537

St Julian's, Malta, 19th May – 23rd May, 2025

Agenda Item:	8.1.2.2 LCM for UE-sided model for Beam Management use case

Source:	NEC

Title:	Discussion on remaining issue of LCM for UE-sided model

Document for:	Discussion, Decision

Introduction

RAN2#129bis meeting reached following agreements on applicability reporting.

Agreements on option B

1	RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list).

Potential aspects to consider if RAN2 revisit:

-	To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig.

-	to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior

Agreements on applicability reporting

1	Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

2	Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

3	No prohibit timer is introduced

Agreements on data collection configuration

	The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following:

•	An indication on start/stop of data collection

•	Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end.

	Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

This contribution discusses further activation/deactivation indication after AI/ML model functionality configured, as well as model monitoring configuration.

Discussion

activation/deactivation of semi-persistent CSI-reporting configuration

It was agreed in RAN2#129 that “Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

Semi-persistent reporting, activated by MAC CE/DCI

Aperiodic CSI reporting, activated by DCI”

Semi-persistent and aperiodic CSI reporting of applicable functionality is activated following legacy CSI framework:

Semi-persistent reporting, activated by MAC CE/DCI

Aperiodic CSI reporting, activated by DC

Here we focus on semi-persistent CSI-ReportConfig since semi-persistent CSI reporting can be activated by MAC CE which is in RAN2 scope.

Since the NW-side additional conditions (represented by CSI-ReportConfig or a set of inference-related parameters) are provided in Step 3, it is reasonable for the NW to provide its supported semi-persistent CSI-ReportConfig to all UEs reporting their Rel-19 AI/ML-enabled features/FGs (e.g., AI/ML-enabled beam prediction features/FGs). Upon receiving the NW-side additional conditions in Step 3, the UE reports its applicability for all provided semi-persistent CSI-ReportConfig in Step 4. The applicability reported by the UE may be distributed among all provided semi-persistent CSI reports.

The applicability reported by the UE in Step 4 may be distributed among all provided semi-persistent CSI reports.

In order to reduce the heavy overhead caused by activating individual semi-persistent CSI reports for different UEs, the network may aim to concentrate AI/ML inference configurations into certain semi-persistent CSI-ReportConfigs that are applicable to the majority of UEs. In practice, a large proportion of UEs may share similar applicability parameters, making group-based activation both feasible and effective. In such a case, an activation/deactivation MAC CE addressed to a UE group containing applicable UEs is a much more efficient way.

RAN2 to decide whether an activation / deactivation MAC CE of semi-persistent CSI reporting configuration address to a UE group containing applicable UEs is supported.

Activation/deactivation of periodic CSI-reporting configuration

It was agreed in RAN2#129 that “If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).”. However, it is still unclear that whether UE can autonomously activate the applicable functionalities upon reporting applicable functionalities via UAI after step 4. This case could be as below:

- the UE reports “inapplicable” for the initial applicability reporting via RRCReconfigiration

- the UE reports “applicable” for the subsequent reporting via UAI upon applicability status changing (i.e., from “inapplicable” to “applicable”)

Since the network does not have any hint or knowledge about when a UE's applicability status may change, if autonomous activation by the UE is supported, then the network needs to continuously transmit inapplicable periodic CSI reference signals, resulting in significant resource wastage. Of course, the network can explicitly release the inapplicable periodic CSI reporting configuration and stop the reference signal transmission; however, in such a case, the UE may lose the opportunity to report its applicability status change. We think supporting activation/deactivation of periodic CSI reporting configuration by MAC CE is a good way forward.

RAN2 to decide whether an activation / deactivation of periodic CSI reporting configuration by MAC CE is supported.

Model monitoring

When the model monitoring is performed at UE side, UE should report the model monitoring performance result towards network. In order to improve the monitoring accuracy, UE should perform model monitoring procedures for certain times as a whole model monitoring procedure.          In details, every time UE get the inference output, UE should be based on the inference output to perform model monitoring. Therefore, RAN2 should define model monitoring cycle. For each model monitoring cycle, UE will generate one model monitoring result. UE should report multiple model monitoring results towards network. In details, there are two reporting principles as listed in below:

- Dynamic reporting: for each monitoring cycle, as long as UE get the monitoring performance, it should feedback towards network immediately

- One time reporting: after the model monitoring is accomplished, UE will collect the monitoring result for all monitoring cycles as an output set and report towards network.

The pros and cons of the two performance feedback approaches are obviously observed, which is listed in the following table:

|  | Dynamic reporting | One time reporting |
| --- | --- | --- |
| Pros | Network can be aware of the model performance in a real-time way so that to perform LCM management more accurate | Decrease the complex of signalling exchange between UE and network |
| Cons | Every time UE get the monitoring result, it needs to report towards network, which needs extra procedure for uplink resource request. | Network cannot be aware of the model performance on time. |

According to the pro/con analysis, RAN2 should discuss which reporting principle of monitoring performance should be adopted.

The model monitoring accuracy can be improved by repeating multiple times of model monitoring procedure.

If model monitor is performed at UE side, RAN2 should define model monitoring cycle with regard to each model inference output.

If model monitor is performed at UE side, UE should report the performance result towards network for each model monitoring cycle.

If model monitor is performed at UE side, RAN2 should consider two reporting principles for performance reporting:

- Dynamic reporting: for each monitoring cycle, as long as UE get the monitoring performance, it should feedback towards network immediately

- One time reporting: after the model monitoring is accomplished, UE will collect the monitoring result for all monitoring cycles as an ouput set and report towards network.

The pros and cons of dynamic reporting and one time reporting are listed in the below table:

|  | Dynamic reporting | One time reporting |
| --- | --- | --- |
| Pros | Network can be aware of the model performance in a real-time way so that to perform LCM management more accurate | Decrease the complex of signalling exchange between UE and network |
| Cons | Every time UE get the monitoring result, it needs to report towards network, which needs extra procedure for uplink resource request. | Network cannot be aware of the model performance on time. |

RAN2 should determine which reporting principle should be adopted.

Conclusion

In this contribution, we have discussed the remaining issues of LCM for UE-sided model, a brunch of observations and proposals have been listed:

Observation 1	The applicability reported by the UE in Step 4 may be distributed among all provided semi-persistent CSI reports.

Observation 2	The model monitoring accuracy can be improved by repeating multiple times of model monitoring procedure.

-	Dynamic reporting: for each monitoring cycle, as long as UE get the monitoring performance, it should feedback towards network immediately

-	One time reporting: after the model monitoring is accomplished, UE will collect the monitoring result for all monitoring cycles as an ouput set and report towards network.

Observation 3	The pros and cons of dynamic reporting and one time reporting are listed in the below table:

Proposal 1	RAN2 to decide whether an activation / deactivation MAC CE of semi-persistent CSI reporting configuration address to a UE group containing applicable UEs is supported.

Proposal 2	RAN2 to decide whether an activation / deactivation of periodic CSI reporting configuration by MAC CE is supported.

Proposal 3	If model monitor is performed at UE side, RAN2 should define model monitoring cycle with regard to each model inference output.

Proposal 4	If model monitor is performed at UE side, UE should report the performance result towards network for each model monitoring cycle.

Proposal 5	If model monitor is performed at UE side, RAN2 should consider two reporting principles for performance reporting:

Proposal 6	RAN2 should determine which reporting principle should be adopted.

References

[1]