3GPP TSG-RAN WG2 Meeting #130	R2-2503543

St. Julians, Malta, May. 19th– 23rd, 2025

Agenda Item:	8.3.2

Source:	OPPO

Title:	Discussion on functionality management of UE sided model

Document for:	Discussion, Decision

Introduction

RAN2 has made progress on the applicability procedure of UE sided model for BM use case. Due to similarity between BM use case and RRM measurement, many agreements can be inherited by AI Mobility SI with slight modification. In this paper the agreements on applicability procedure of BM use case are examined to see which part of them can agreed for AI mobility SI. On top of that there are also some AI mobility specific issues to be resolved.

Applicability procedure

General definition of functionality

At RAN2#129bis meeting, RAN2 agreed not to discuss detail functionality:

Start by discussing spec/ran2 impact of the agreed cases/subcases and then discuss functionality granularity

But it is still necessary to align the general terms to facilitate discussion. In RAN2#127 meeting, RAN2 agreed following terms:

Agreements on definitions

- Supported functionalities refer to functionalities that UE can indicate by using UE capability information (via RRC/LPP signalling)

- Applicable functionalities refers to functionalities that the UE is ready to apply for inference

- Activated functionalities refers to functionalities already enabled for performing inference

The definitions of these 3 terms are such general that they can be also applied for AI mobility SI (apart from yellow part). Hence, we have following proposal:

Proposal 1: To agree following general terms for AI mobility SI:

Supported functionalities: refer to functionalities that UE can indicate by using UE capability information via RRC signalling

Applicable functionalities: refer to functionalities that the UE is ready to apply for inference

Activated functionalities refers to functionalities already enabled for performing inference

Applicability procedure based on partial configuration

At RAN2#129bis meeting, option B i.e. partial configuration is agreed in principle with more details to be figured out. The idea behind option B is that the applicability is checked in 2 steps manner because the parameters really matter for applicability assessment are rather limited. The same reason stands also for AI mobility use cases. Taking temporal domain case B as example, the intension is to save measurement in temporal domain. The RAN2 evaluations show that the key element impacting prediction accuracy is MRRT and UE speed. RAN2’s further study on generalization on UE speeds suggests that there is minor or no generalization issue at all. It basically means if UE is configured with MRRT together with one measurement task, then UE basically can judge whether corresponding functionality is applicable or not. Here obviously the key inference parameter is MRRT since measurement task is likely configured like legacy based on UE’s radio capability. The situation is similar for temporal domain case A, inter-frequency prediction and spatial domain prediction, where the key inference parameter could be however different.

Observation 1: The key inference parameter for UE to judge applicability is limited for AI mobility use case/scenarios

One example is illustrated by Table 2.2-1. For temporal domain case B, UE may support such scenario in band A and band B. In addition, UE may support MRRT=50% and MRRT=67%.

| Model availability | Band A | Band B |
| --- | --- | --- |
| MRRT=50% | Available | Not available |
| MRRT=67% | Not available | Available |

Table 2.2-1 Example of applicable condition combinations

Assuming the information listed in Table 2.2-1 is already known, gNB can simply configure a RRM measurement configuration with specific MRRT which measObject is corresponding band A or B. But the information listed in Table 2.2-1 could be not available in first place. It is because UE capability signalling only reflects what UE can potentially do but corresponding model may not be available locally. So, if UE is configured with a RRM measurement configuration with MRRT, for which no model is available, the signalling in step3 is simply wasted. Instead, gNB can check in step 3 e.g. the information listed in Table 2.2-1 and then configure detail RRM measurement in step 5 based on the feedback information in step 4. Here is one example:

Figure 2.2-1 example of partial configuration

Note in Figure 2.2-1 UE only need check those key inference parameters and internal condition. So, in step 4 what UE can feedback is applicable key inference parameters. In addition, since the RRM measurement configuration is configured in step 5, AI mobility functionality is only activated after step 5 but not step 3.

Figure 2.2-1 is just for illustration purpose i.e. detailed key inference parameter(s) for different scenarios need be further study.

Proposal 2: Partial configuration in step 3 is supported for AI mobility use cases/scenarios

Proposal 3: UE responses applicable key inference parameter(s) in step 4 if they are configured in step 3

Applicability procedure with full configuration

Once the key inference parameters e.g. as listed in Table 2.2-1 is checked, gNB can always configure a RRM measurement configuration i.e. full configuration in step 3 directly. The basic applicability procedure with full configuration approach is as following:

Figure 2.3-1

Step 1/2: UE report its AI/ML related capability to network

Step 3: gNB configure UE with full inference configuration. UE starts to check applicability based on received configuration and internal additional conditions. If at least one full configuration is applicable, UE will activate it

Step 4: UE then report applicability and/or inapplicability information back to gNB by referring to full configuration in step 3.

We understand step 1/2 could be done in offline manner i.e. UE can report its capability in advance before gNB plan to check applicability. It means gNB can fetch such UE capability from CN in addition to UE radio capability already. At RAN2#129bis meeting, RAN2 agreed:

Agreements

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

Table 2.3-1

After knowing UE’s radio capability and capability of AI mobility and applicable key inference parameters, gNB is already able to configure RRM measurement configuration together with some key inference parameter for UE to enable inference. Take temporal domain case B as example, gNB could have known that the UE support measurement reduction in temporal domain. In this case the additional parameter could be MRRT parameter. The content of measConfig could be modified as such that UE know how to report actual/predicted measurement result(s).

Once UE receives a RRM measurement configuration with key inference parameters, UE can check whether such RRM measurement configuration is applicable or not. If yes, UE can start to do measurement and inference. For example, UE can answer that one configured measId is applicable.

Proposal 4: In step 3, a RRM measurement configuration with key inference parameters can be configured via RRCReconfiguration message to UE

Proposal 5: in step 4, UE report applicability information via RRCReconfigurationComplete message back to gNB

For BM use case, it is agreed that one more set of full inference configurations can be configured:

- Agreements on applicability reporting

- Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

- Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

- No prohibit timer is introduced

Table 2.3-2

It should be also allowed for AI mobility. Taking temporal domain case B as example, gNB may configure UE with two measurement tasks, which both enable UE to do inference with even same inference parameters e.g. MRRT. And same principle should be applied i.e. UE should keep all these configurations until released by network.

Proposal 6: 2	Upon receiving one or more RRM measurement configuration via RRCReconfiguration message, UE shall maintain all the RRM measurement configuration(s) regardless of applicability until released by network explicitly

RAN2 agreed #RAN2#129bis meeting how to apply inference configuration as following:

The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required.    FFS when the UE can perform inference

UE could be configured with more than one RRM measurement configuration targeting same functionality. Taking temporal domain case B as example, UE could be configured with two measurement tasks, for both the key inference parameter e.g. MRRT is the same or close while in the e.g. same band. In this case the functionality itself is the same for both measurement tasks. So technically the functionality is applicable and activated only if UE can apply the key inference parameters. On the other hand, if a measurement task corresponding to one RRM measurement configuration is applicable, UE can feedback such applicable measId back to gNB so that gNB can know UE can also apply the inference parameters. But if a RRM measurement configuration is not applicable and it is because of the key inference parameters, UE should feedback that the key inference parameters are not applicable. This could happen if gNB configure a RRM measurement configuration in step3 blindly without check those key inference parameter at first. By feedback inapplicable key inference parameters gNB can avoid such problem for other RRM measurement configurations.

Proposal 7: If full RRM measurement configuration together with key inference parameters is configured in step 3 and it is applicable, UE need feedback measId directly

Proposal 8: If full RRM measurement configuration together with key inference parameters is configured in step 3 and it is not applicable, UE could feedback inapplicable key inference parameters

Report of applicability information

UE will activate an applicable functionality. Once it becomes inapplicable there are two options:

- Option 1: Follow principle for BM use case i.e. UE doesn’t automatically deactivate it and report it to network

- Option 2: UE automatically deactivate it and fall back to legacy RRM measurement procedure and report it to network

The benefit for option 1 is that UE’s behaviour is aligned between two cases:

- Case 1: One functionality is in applicable from beginning

- Case 2: One functionality changed from applicable to be inapplicable

The benefit for option 2 is that RRM measurement procedure can carry on without impairment. For BM use case related agreements are:

Agreements (RAN2#128)

1.	When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

2.	When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Agreements (RAN2#129)

3.	Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

4.	Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.   FFS if we report explicit cause

Agreements (RAN2#129bis)

5.	Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec

Table 2.4-1

Our view is that RRM measurement procedure itself is delay tolerant procedure. Even one functionality becomes inapplicable, the handshake between UE and network demands few 10ms at most, which is most likely less than a sample period. To keep it simple option 1 is preferred.

Proposal 9: When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable

Applicability report during mobility

One more issue is applicability report during mobility. There is agreement for BM use case in RAN2#128 meeting:

Agreements (RAN2#128)

Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

Table 2.5-1

Our understanding is that target gNB may be interested in different functionality but with same knowledge of UE context after signalling exchanging over Xn interface. So logically target gNB can check with UE via handover command directly and UE need answer in RRCReconfigurationComplete (i.e. upon handover complete message). For AI mobility use case, the logic is the same from our perspective.

Proposal 10: During mobility, target gNB can check applicability via handover command message i.e. RRCReconfiguration and UE can response with RRCReconfigurationComplete.

Associated ID issue

This issue is briefly discussed at last meeting without conclusion on whether associated Id is necessary for AI mobility use cases. The generalization study so far shows that there is no such problem at least for UE speeds and cell configuration. There is problem for inter-frequency prediction, which can be resolved by methodology for GC#2 i.e. to train model with mixed datasets.

Observation 2: there is either no generalization issue or there is with easy solution for AI mobility so far based on simulation result

At last meeting some company argue that e.g. beam pattern could impact e.g. sub use case 1 because beam pattern matters for BM use case. At first this statement may not be true even in RAN1 since there is no consensus. Even it is true for BM use case, it could still not true for AI mobility use case. The result used for RRM purpose is either L3 cell/beam level measurement result or measurement event. For those results there are few filtering process as illustrated below:

Figure 2.5-1 Filtering chain of RRM measurement

The beam pattern may influence instant L1 beam level sample (in RAN1’s simulation majority company don’t have L1 filtering at all), but after L1/L3 filtering process, the impact could be filtered out. For measurement event prediction with indirect methodology RRM measurement result will be filtered further with TTT timer. In addition, the impact on measurement results due to network condition e.g. beam pattern is rather static. While what model learns is the trends in one or more specific domain i.e. the network condition itself doesn’t change the correlation in temporal or spatial or frequency domain.

Proposal 11: No associated Id is needed for AI mobility

Inference report

For UE sided model, RAN2 concluded briefly on what and how to report to gNB at RAN2#129bis meeting:

Agreements (RAN2#129bis)

5	For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.   FFS details

6	For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

7	Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

Table 3.1

For RRM measurement prediction on cell level result, UE should report only L3 measurement result. For sub-use case 1, although the outcome of the model is L1 beam level measurement result. But UE should still report L3 result to network since the L3 result instead of L1 result is wanted by network. Plus, reporting L1 beam level measurement results could be a big burden for Uu signalling since most likely there are multiple L1 beams need be reported.

For RRM measurement prediction on beam level result, only L3 beam level result should be reported due to similar reason.

Observation 3: For RRM measurement prediction, Reporting L1 beam level measurement is not necessary

Proposal 12: For RRM measurement prediction, only L3 cell/beam level measurement results are reported

At RAN2#129bis meeting RAN2 agreed to study L3 beam level prediction. As for reporting procedure we think legacy principle i.e. L3 beam measurement is always reported together with L3 cell measurement result should be still valid i.e. it is not necessary to introduce procedure to report L3 beam level measurement result alone.

Proposal 13: legacy principle i.e. L3 beam measurement is always reported together with L3 cell measurement result should be still valid i.e. it is not necessary to introduce procedure to report L3 beam level measurement result alone

As for what to report depends on detailed scenarios. For temporal domain case A, UE can report at least one actual measurement result since no measurement will be saved. Then the most interested part is predicted measurement result(s) in PW. For both sliding approach or non-sliding approach with PW>=measurement period, there are at least one measurement results in PW.

Proposal 14: For temporal domain case A, one or more instances of predicted measurement results in PW per one cell or beam are reported in one measurementReport message

Considering the intention is to report actual and/or predicted measurement result is to help network to predict a potential future measurement event most likely, it could be also beneficial to report more than one actual measurement results in OW. During evaluation phase it seems more actual measurement results in OW are used to predicted measurement event, the more accurate is the prediction.

Proposal 15: For temporal domain case A, one or more instances of measurement results in OW per one cell or beam are reported in one measurementReport message

For temporal domain case B, eventually UE will get both actual and predicted measurement results in OW and/or PW. Since the intention for this scenario is to save measurement in temporal domain, one measurement result for one cell or one beam should be sufficient. No doubt this could be a predicted measurement result. On the other hand, it is also possible there is actual measurement result available before the reporting opportunity regardless it is pure periodic, or event triggered periodic procedure. Because of this one indication should be introduced to indicate whether a measurement result is actual or predicted measurement.

Proposal 16: For temporal domain case B, one measurement result per one cell or beam is reported in one measurementReport message.

Proposal 17: For temporal domain case B, one indication marking actual or predicted measurement result should be also reported along with the measurement result

For frequency domain prediction, for frequency to be predicted, all the measurement results are predicted, and it should be sufficient to just follow legacy report procedure i.e. one measurement result per cell or per cell are sufficient. For spatial domain, the reported measurement result should be also a predicted measurement result because only partial L1 beams participate the L3 filtering to form L3 cell level measurement result.

Proposal 18: For frequency domain prediction and spatial domain prediction, one predicted measurement result per one cell or per beam is reported in one measurementReport message.

For L3 beam measurement prediction however an indication per beam or beam group is necessary to indicate whether the measurement result is actual or predicted, which depends on the beam act as input or output of the model. If reported beam is input of the model, its measurement result is measured by UE otherwise it is predicted.

Proposal 19: For L3 beam prediction, predicted measurement result(s) should be distinguished from actual measurement result(s).

For measurement event report, how to report measurement event is related to the captured HO model in TR even it is for evaluation purpose in 1st place. Among all the 3 options UE should report predicted measurement event at t0. For spatial domain and frequency domain prediction, the time instance when a measurement event is predicted should be the same time instance the event occurring i.e. when corresponding TTT timer expires. For temporal domain case B, technically it is possible that TTT timer expires upon a predicted measurement result. In this case the event occurs in future instead of current time instance. But we think such optimization is not necessary because the intention of temporal case B is to save measurement. And simulation also show that minor or even no system level performance degrades by following such principle. Since network supposes to know the measurement event is based on which domain prediction, nothing else other than measurement event itself should be reported.

Proposal 20: For measurement event prediction based on temporal domain case B, spatial domain and frequency domain prediction, UE need report predicted measurement event by following legacy procedure i.e. no spec impact is foreseen.

For temporal domain case A, however more information is needed. The most important information is when the event supposes to happen from current time instance i.e. the time point when TTT timer supposes to expire.

Proposal 21: For measurement event prediction based on temporal domain case A, apart from measurement event and relevant measurement result, UE should also provide the timing information when corresponding TTT timer expires in future.

For measurement event prediction based on temporal domain case B, spatial domain and frequency domain prediction, UE reports the predicted measurement event just by following legacy procedure. i.e. when the measurement event is trigged. For prediction based on temporal domain case A, measurementReport message should be transmitted early. For handover model option 1 in the TR, UE could report predicted measurement event @t0. For handover model option 2 in the TR, network supposes to transmit handover command message upon time instance where entering condition is met as indicated in Table 3.2..

Figure 3.1 HO model option 2

Option 2 is illustrated in Figure 5.5.1-2. At current time i.e. t0 measurement event e.g. A3 event is predicted @ future time t1. Network transmits handover command when entry condition of the predicted measurement event is met based on actual measurement result @ t2 unless t2 is earlier than handover preparation phase. In later case, network transmits handover command immediately after handover preparation phase. In this way, not only handover preparation could be saved but also handover can be executed earlier.

Table 3.2

The issue is whether 2nd measurement report message around t2 in Figure 3.1 is necessary?

When UE report a future measurement event @t1 as illustrated in Figure 3.1, as proposed in proposal 21, UE should also indicate the timing information of t1 because otherwise network has no idea when the event will occur in future. Because of such timing information network can know when is t2 considering t1-t2 = TTT.

Observation 4: After receiving a future measurement event with timing information of measurement event, network can know when entering condition could be met.

The t1 in Figure 3.1 shall be within PW i.e. t1-t0<=PW. And the PW should be set as such that predict accuracy, L3 RSRP difference and/or F1 score, should be good enough because otherwise e.g. false alarm could hurt handover performance on the contrary. Meeting entering condition can also be realized by configuring PW<=TTT by network’s implementation, if network really want to make sure it is met based on actual measurement result.

Observation 5: Whether handover command is transmitted upon entering condition could be left for network’s implementation

Based on analysis for both handover models, we propose:

Proposal 22: UE reports measurementReport message containing a future measurement event when UE predicted it for measurement event prediction based on temporal domain case A.

Conclusion

Here are our observations and proposals:

Observation 1: The key inference parameter for UE to judge applicability is limited for AI mobility use case/scenarios

Observation 2: there is either no generalization issue or there is with easy solution for AI mobility so far based on simulation result

Observation 3: For RRM measurement prediction, Reporting L1 beam level measurement is not necessary

Observation 4: After receiving a future measurement event with timing information of measurement event, network can know when entering condition could be met.

Observation 5: Whether handover command is transmitted upon entering condition could be left for network’s implementation

Proposal 1: To agree following general terms for AI mobility SI:

Supported functionalities: refer to functionalities that UE can indicate by using UE capability information via RRC signalling

Applicable functionalities: refer to functionalities that the UE is ready to apply for inference

Activated functionalities refers to functionalities already enabled for performing inference

Proposal 2: Partial configuration in step 3 is supported for AI mobility use cases/scenarios

Proposal 3: UE responses applicable key inference parameter(s) in step 4 if they are configured in step 3

Proposal 4: In step 3, a RRM measurement configuration with key inference parameters can be configured via RRCReconfiguration message to UE

Proposal 5: in step 4, UE report applicability information via RRCReconfigurationComplete message back to gNB

Proposal 6: 2	Upon receiving one or more RRM measurement configuration via RRCReconfiguration message, UE shall maintain all the RRM measurement configuration(s) regardless of applicability until released by network explicitly

Proposal 7: If full RRM measurement configuration together with key inference parameters is configured in step 3 and it is applicable, UE need feedback measId directly

Proposal 8: If full RRM measurement configuration together with key inference parameters is configured in step 3 and it is not applicable, UE could feedback inapplicable key inference parameters

Proposal 9: When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable

Proposal 10: During mobility, target gNB can check applicability via handover command message i.e. RRCReconfiguration and UE can response with RRCReconfigurationComplete.

Proposal 11: No associated Id is needed for AI mobility

Proposal 12: For RRM measurement prediction, only L3 cell/beam level measurement results are reported

Proposal 13: legacy principle i.e. L3 beam measurement is always reported together with L3 cell measurement result should be still valid i.e. it is not necessary to introduce procedure to report L3 beam level measurement result alone

Proposal 14: For temporal domain case A, one or more instances of predicted measurement results in PW per one cell or beam are reported in one measurementReport message

Proposal 15: For temporal domain case A, one or more instances of measurement results in OW per one cell or beam are reported in one measurementReport message

Proposal 16: For temporal domain case B, one measurement result per one cell or beam is reported in one measurementReport message.

Proposal 17: For temporal domain case B, one indication marking actual or predicted measurement result should be also reported along with the measurement result

Proposal 18: For frequency domain prediction and spatial domain prediction, one predicted measurement result per one cell or per beam is reported in one measurementReport message.

Proposal 19: For L3 beam prediction, predicted measurement result(s) should be distinguished from actual measurement result(s).

Proposal 20: For measurement event prediction based on temporal domain case B, spatial domain and frequency domain prediction, UE need report predicted measurement event by following legacy procedure i.e. no spec impact is foreseen.

Proposal 21: For measurement event prediction based on temporal domain case A, apart from measurement event and relevant measurement result, UE should also provide the timing information when corresponding TTT timer expires in future.

Proposal 22: UE reports measurementReport message containing a future measurement event when UE predicted it for measurement event prediction based on temporal domain case A.