3GPP TSG-RAN WG2 Meeting #130	R2-2503544

St. Julians, Malta, May. 19th– 23rd, 2025

Agenda Item:	8.3.5

Source:	OPPO

Title:	Discussion on performance monitoring

Document for:	Discussion, Decision

Introduction

At RAN2#129bis meeting, RAN2 agreed following study scope [1]:

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes , all sub cases | Yes |
| Frequency domain | Yes , all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes, | Yes |

Table 1

This paper will address performance monitoring of both UE sided model and network sided model.

UE sided model

Procedure

At RAN2#129bis meeting, RAN2 agreed that for UE sided model:

For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case

Table 2.1-1

RAN1 introduces 3 options for BM use case:

- Type 1- Option1: Network sided performance monitoring

- Type 1-Option 2: UE assisted performance monitoring

- Type 2: UE based performance monitoring

Here is our understanding of those 3 options.

| Performance monitoring for BM | Who make decision | Who calculate metric | Potential spec impact |
| --- | --- | --- | --- |
| Type 1- option 1 | gNB | gNB | UE need provide ground truth measurement results |
| Type 1- option 2 | gNB | UE | UE need provide calculated metric |
| Type 2 | UE | UE | UE may report its decision to network |

Table 2.1-2 performance monitoring options

Without touching detailed content of exchanged information, the 3 options are also valid for AI mobility SI in general. However, due to concern from network vendors, we learned that type 2 is most likely not supported for BM use case in RAN1. Our understanding is that the concern is also valid for AI mobility. The intention is that functionality management should be under control by network as much as possible. In RAN2#125bis RAN2 agreed that “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19”. We think this spirit of agreement should be also applied for performance monitoring.

Proposal 1: UE based performance monitoring is excluded for AI mobility use cases

For AI mobility use case, to support option 1 means UE should report ground truth measurement result(s) during monitoring duration apart from actual/predicted measurement results. While for option 2, the signalling overhead can be saved since only metric itself need be reported. On the other hand, obviously the metric will be only limited to specified one(s) for option 2. For example, the metric in section 2.2 should be defined very clearly to avoid any ambiguity. For option 1, there is no such issue. Meanwhile network can calculate any metrics it wants.

| Monitoring procedure | Pro | con |
| --- | --- | --- |
| Type 1-option 1 | Flexible metrics are feasible | High signalling overhead |
| Type 1-option 2 | Limited to specified metric | Low signalling overhead |

Table 2.1-3 pro and con of two options

Proposal 2: Both network sided performance monitoring and UE assisted performance monitoring are supported

For both options during performance monitoring, to get ground truth measurement result, UE need perform measurement by following legacy measurement procedure. For temporal domain case A, it doesn’t really matter how long UE is required to do it. But for other scenarios it matters because UE supposes to skip performing some of the measurement in frequency/temporal/spatial domain.

In addition, UE need provide either ground truth measurement results or calculated metric to gNB. So even for temporal domain case A, this limited duration is also applicable for the sake of signalling overhead. To make it flexible, gNB can configure such parameter.

Proposal 3: The performance monitoring duration for UE sided model should be limited and configurable

If RRM measurement/event prediction occurs within same frequency layer, UE can get ground truth measurement result by following legacy measurement procedure while running the model. For inter-frequency prediction, the story is bit different. UE should be configured with another legacy measurement task for the predicted frequency layer temporarily.

Observation 1: For inter-frequency prediction, a temporal measurement task for predicted frequency layer should be configured temporarily for monitoring purpose

Monitoring metric

For RRM measurement prediction, the metric used during evaluation is prediction accuracy. Here is the definition cited from TR 38.744:

Measurement prediction accuracy for cell-level RRM measurement prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and ground truth L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases

Table 2.2-1 prediction accuracy definition (L3 cell level result)

For RRM sub-use case 1, it is also possible to calculate L1 RSRP difference like L3 RSRP difference. However, for UE sided model what really matters for network is the final L3 cell level measurement result. In addition, the ground truth L3 filtered cell-level measurement result refers to a measurement result UE can get without any prediction i.e. by following legacy measurement procedure. it should not be mixed with any RAN4 concept, where the ground truth measurement result has some special meaning in the context of test environment.

Proposal 4: For UE sided model L3 cell level prediction, the metric to be monitored is L3 RSRP difference as defined in TR 38.744

RAN2 agreed some general rule for inference configuration and reporting during RAN2#129bis meeting as following:

1	The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

Table 2.1-2 agreement on inference configuration and report

It basically means measurement configuration and report framework is used for inference configuration and reporting. For same scenario e.g. temporal domain case A, UE should be allowed to configure more than one measurement task to do inference. The question is whether different measurement task associated with different measId should be monitored separately or not. During evaluation phase we can observe that prediction accuracy is quite different at least between FR1 and FR2. For frequency within same range, the difference could be smaller. But it will be difficult for gNB to differentiate the performance between two measurement tasks. In addition, measurement task could be the granularity of functionality management. The granularity for monitoring should be at least finer or equal to granularity of functionality management.

The next question is whether finer monitoring granularity is necessary e.g. among cells belonging to same frequency layer. Our view is that prediction accuracy among cells belonging to same frequency is quite similar. In addition, even there is slight difference observed among intra-frequency cells, it will be also difficult for gNB to configure UE perform inference only for partial cells.

Proposal 5: For UE side model, the metric is calculated per frequency for L3 cell level prediction i.e. intra-frequency cells should be calculated together.

For L3 beam level prediction, the difference from L3 cell level prediction is the granularity of predicted object i.e. per beam instead of per cell. So, we think all above proposals are applicable. In addition, the metric should be also averaged among beams.

Proposal 6: For L3 beam level prediction, L3 RSRP difference should be further averaged among beams.

For temporal domain case A, it is possible that there are more than one predicted measurement results are available in PW. During evaluation phase, 3 kinds of L3 RSRP difference are reported by company:

- Type 1: L3 RSRP difference of each predicted measurement result within PW

- Type 2: Average L3 RSRP difference of all predicted measurement results within PW

- Type 3: L3 RSRP difference of last predicted measurement result within PW

Obviously, the outcome among 3 types is different. RAN2 need decide which one should be used for performance monitoring. If more than one type is specified, then UE should be configured which one is required. Our view is that type 2 should be sufficient. The prediction accuracy usually decreases from the 1st predicted measurement result to the last one in the PW. On the other hand, the tolerance of prediction accuracy also increases because UE is getting close to candidate cell as illustrated in our contribution [2].

Figure 3.1-1 Illustration of impact of prediction accuracy to measurement event prediction

For temporal domain case B, the PW is usually much smaller than temporal domain case B. Our view is that average L3 RSRP within PW should be sufficient.

Proposal 7: For temporal domain case A and case B, average L3 RSRP difference within PW is monitoring metric

For all above cases, the calculated L3 RSRP difference should be further averaged within configured monitoring duration.

Proposal 8: L3 RSRP difference should be further averaged within configured monitoring duration

For measurement event prediction, the first issue is what metric need be monitored? For indirect prediction methodology, prediction accuracy can already reflect the performance of the model. For direct prediction methodology, intermediate KPI i.e. F1 score and/or its components can reflect performance of the model. But it is bit tricky for UE to calculate F1 score. During simulation the ground truth is gathered via legacy HO procedure in advance. In field deployment UE may not be able to know anything about ground truth. For example, for temporal domain case A, handover may be triggered based on predicted measurement event before a real measurement event is triggered. For measurement event prediction based on temporal domain case B, spatial domain prediction and frequency domain prediction, a good model is expected to predict a measurement event closely around a ground truth measurement event. On the other hand, the preparation phase between reporting measurementReport message and handover execution is such short (40~50ms) it is likely that handover command is received before a ground truth measurement event is observed.

Observation 2: For measurement event prediction, it is possible that ground truth measurement event will never happen during monitoring.

One alternative for monitoring F1 score is to monitor system level performance e.g. HO failure rate. It means UE need collect handover history across serving cells. The detail can be studied by RAN2.

Proposal 9: For measurement event monitoring, RAN2 need discuss whether intermediate metric e.g. F1 score or system level KPI e.g. HO failure rate should be monitored

Proposal 10: If system level KPI is monitored, monitoring KPI across serving cells need be studied

Network sided model

RAN2 achieved following agreement on performance monitoring for network sided model:

RAN2#125bis:

1, RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

2, RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

RAN2#126:

3, The gNB is responsible for monitoring its own performance.  RAN2 will work on RAN2 specifications enhancements associated to gNB-side model monitoring, only based on RAN1 inputs, if any

Table 3-1 Agreement on performance monitoring of network sided model for BM use case

These 3 agreements (apart from aspect specific to RAN1 use case) is general principle, which is also applicable for AI mobility use case.

Proposal 11: RAN2 confirms that UE will not be informed about any network-sided functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) for AI mobility use case except being configured to provide the required ground truth measurement result(s)

Proposal 12: The gNB is responsible for monitoring its own performance

For L3 cell level result prediction, ground truth measurement result means legacy L3 filtered cell level measurement result. For L3 beam level result prediction, ground truth measurement result means legacy L3 filtered beam level measurement result. Those ground truth measurement result is necessary for temporal domain case B, and spatial domain prediction, but not for temporal domain case A and frequency domain prediction since UE can anyway be configured to report it by legacy procedure.

Observation 3: For temporal domain case A and inter-frequency prediction, no spec impact is foreseen to monitor performance for network sided model

Proposal 13: UE could be configured to report actual L3 filtered cell or L3 beam level measurement result as ground truth measurement result in order to monitor performance for temporal domain case B and spatial domain prediction in network side.

Conclusion

Here are our observations and proposals:

For UE sided model:

Observation 1: For inter-frequency prediction, a temporal measurement task for predicted frequency layer should be configured temporarily for monitoring purpose

Observation 2: For measurement event prediction, it is possible that ground truth measurement event will never happen during monitoring.

Proposal 1: UE based performance monitoring is excluded for AI mobility use cases

Proposal 2: Both network sided performance monitoring and UE assisted performance monitoring are supported

Proposal 3: The performance monitoring duration for UE sided model should be limited and configurable

Proposal 4: For UE sided model L3 cell level prediction, the metric to be monitored is L3 RSRP difference as defined in TR 38.744

Proposal 5: For UE side model, the metric is calculated per frequency for L3 cell level prediction i.e. intra-frequency cells should be calculated together.

Proposal 6: For L3 beam level prediction, L3 RSRP difference should be further averaged among beams.

Proposal 7: For temporal domain case A and case B, average L3 RSRP difference within PW is monitoring metric

Proposal 8: L3 RSRP difference should be further averaged within configured monitoring duration

Proposal 9: For measurement event monitoring, RAN2 need discuss whether intermediate metric e.g. F1 score or system level KPI e.g. HO failure rate should be monitored

Proposal 10: If system level KPI is monitored, monitoring KPI across serving cells need be studied

For network sided model:

Observation 3: For temporal domain case A and inter-frequency prediction, no spec impact is foreseen to monitor performance for network sided model

Proposal 11: RAN2 confirms that UE will not be informed about any network-sided functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.) for AI mobility use case except being configured to provide the required ground truth measurement result(s)

Proposal 12: The gNB is responsible for monitoring its own performance

Proposal 13: UE could be configured to report actual L3 filtered cell or L3 beam level measurement result as ground truth measurement result in order to monitor performance for temporal domain case B and spatial domain prediction in network side.

Reference

[1] R2_129b_ChairNotes_25-04-11_16-45 final

[2] R2-2500291 Discussion on simulation results of measurement event prediction OPPO