3GPP TSG-RAN WG2 Meeting #130	R2-2503546

St. Julians, Malta,  May 19th – 23rd , 2025

Agenda Item:	8.1.2.2

Source: 	Fujitsu

Title: 	Discussion on LCM for UE-sided Model for Beam Management Use Case

Document for:	Discussion

Introduction

In previous meetings, RAN2 made several agreements on functionality identification [1] related issues and one LS [2] has been sent to RAN1 pending for response. In this contribution, we would like to give our views on the following aspects of LCM for UE-sided model for beam management use case：

- Functionality LCM.

- Performance Monitoring.

Discussion

Functionality LCM

In previous RAN2 #129 meeting [1], the following agreements were made for the option A/B of functionality-based LCM:

Agreements on option B 1 RAN2 assumes UE receives RRCReconfiguration message including one set or multiple sets of inference related parameters via OtherConfig for option B. This assumption can be confirmed (i.e., whether to reconsider CSI-ReportConfig) after receiving Option B inference related parameters (e.g., in RAN1 RRC parameters list). Potential aspects to consider if RAN2 revisit: - To reconsider CSI-ReportConfig for option B, for example, if the list of inference related parameters is fully contained within existing CSI-ReportConfig. - to take into accounts UE behaviour when confirming the assumption e.g., whether option A and option B result in different UE behavior.

We believe that the default signaling for configuring option B is OtherConfig and the reporting signaling should be UAI, this is the key difference between option A and B that enables option B to report the applicability more flexible.

For CSI-ReportConfig, even if the inference related parameters are fully contained in the configuration, we still prefer not to use it as the configuration signaling of option B, because we regard option A and B as complementary solutions to deal with different situations. We do not want the corner case of option B to be included in option A, which is not the intention of having option A/B at the beginning of the discussion.

Proposal 1 For option B configuration, OtherConfig should be used rather than CSI-ReportConfig.

As for non-applicable functionality reporting, the following agreements were given as follows:

Agreements on applicability reporting 1 Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability FFS how to define this simple cause related to model availability and how we capture it in the spec. 2 Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter if the full inference configuration is applicable or inapplicable until the network releases it explicitly. 3 No prohibit timer is introduced

For the simple cause design, we think it may be better to leave it for stage 3 discussion. However, one very basic cause must be taken into consideration is that the functionality/model cannot be used at this moment and need to be replaced immediately, because for UE-side model, UE needs to consider the performance of the AI/ML functionality and related applications, if UE is aware of the danger of continuing the usage of AI/ML functionality, it must report to the NW as soon as possible for the follow-up actions.

Proposal 2 The simple cause of inapplicability should carry at least the following information: the current functionality needs to be deactivated and replaced immediately.

Performance Monitoring

Performance monitoring of UE-side model for beam management has been continuously discussed in RAN1 with minor progress per meeting, the following agreements were made during Rel-19 [3][4]:

Agreement [RAN1 #117] For BM-Case1 and BM-Case2 with a UE-side AI/ML model: - Support Type 1 performance monitoring, including the following two options: - Option 1 (NW-side performance monitoring): - UE sends a report to NW (for the calculation of performance metric at NW) - Measurement results from a resource set for monitoring, e.g., L1-RSRP and/or RS index is supported as the content of the report. - FFS on other contents. - The report is at least configured/triggered by NW. - Note: this may or may not have an additional spec impact - Option 2 (UE-assisted performance monitoring): - UE calculates performance metric(s) - FFS how to report and what to report. - FFS whether to trigger the report based on event(s) for Option 1 and/or Option 2 - FFS Type 2 performance monitoring. Agreement [RAN1 #118] For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), further study at least the following alternatives, including: - Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/resource for monitoring. - Alt 2: The L1-RSRP difference information based on actual measurement of the L1-RSRP of one or more of Top K predicted beam, and L1-RSRP measurements from a resource set/resource for monitoring. - Alt 3: The RSRP difference information between the predicted RSRP and measured L1-RSRP of corresponding beam(s) of a resource set/resources for monitoring - Note: resources for Set B for monitoring are not precluded and can be study. - Note: this is only applicable when the model can predict RSRP - Alt 4: The probability of the predicted beam(s) to be the Top 1 or Top K beam. - Note: this is only applicable when the model can generate probability information - FFS: for Alt 1/2/3, on other details including how to configure the resource set/resources for monitoring, including - E.g., whether/how to use full set of Set A for measurement. If not, whether/how to obtain the measurement of the predicted Top 1 or Top K beam for calculating the prediction accuracy or the RSRP difference. - For all alternatives, study whether the performance information is calculated per sample (one-shot), or per set of samples (window)

It can be seen from the above agreement that RAN1 has not determined whether the down-selection between Type I/II monitoring or between option 1/2 of Type I monitoring should be made, nor the details of reporting from UE to NW for option 2 monitoring. RAN1 will continue discussing the remaining issues for the next several consecutive meetings.

Observation 1 The details of UE-side BM performance monitoring are still pending further RAN1 conclusions.

RAN2 needs to wait for further RAN1 input for further actions. However, no matter if NW or UE derives the metrics and what will be reported from UE to NW, RAN2 can have some discussions on at least the mechanism to trigger the performance monitoring, and the potential signaling to complete the monitoring procedure.

Proposal 3 RAN2 starts at least the following discussions while waiting for further RAN1 input.

- Mechanism to trigger the performance monitoring procedure.

- Potential signaling to complete the performance monitoring procedure.

From RAN2 point of view, the performance monitoring procedure can be triggered either periodically or event-based, e.g., for option 1, NW can configure UE to periodically report L1-RSRP or BM index to assist NW derives the monitoring metrics, or UE can trigger the procedure for certain events such as UE moving or poor performance.

Proposal 4 The performance monitoring procedure can be triggered either periodically or event based.

Conclusion

Proposal 1 For option B configuration, OtherConfig should be used rather than CSI-ReportConfig.

Proposal 2 The simple cause of inapplicability should carry at least the following information: the current functionality needs to be deactivated and replaced immediately.

Observation 1 The details of UE-side BM performance monitoring are still pending further RAN1 conclusions.

Proposal 3 RAN2 starts at least the following discussions while waiting for further RAN1 input.

- Mechanism to trigger the performance monitoring procedure.

- Potential signaling to complete the performance monitoring procedure.

Proposal 4 The performance monitoring procedure can be triggered either periodically or event based.

References

[1] RAN2 129bis Chair Notes.

[2] R2-2407848 LS on applicable functionality reporting for beam management UE-sided model, 3GPP TSG RAN WG2#127	, Maastricht, Netherlands, Aug 19th – 23rd, 2024.

[3] Xiaodong's Session Notes RAN1#117 (9.1 AI&ML).

[4] Xiaodong's Session Notes RAN1#118 (9.1 AI&ML).