3GPP TSG-RAN WG2 Meeting #130	R2-2503558

St.Julians, Malta, May 19th – 23th, 2025

Agenda item:	8.7.6

Source: 	Fujitsu

Title: 	Discussions on XR rate control

Document for:	Discussion and decision

Introduction

On XR rate control, the following agreements are made in RAN2#129bis meeting [1]:

Agreements on XR rate control - Bit rate in Rel-19 UL Rate Control MAC CE is a physical-layer bit rate. - We specify a single table. - As a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier. This can be subject to further agreements on MAC CE contents/design. Overhead will be considered. - The working assumption is changed to agreements on rate query MAC CE. - From RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. FFS how QFI is indicated, e.g. with DRB ID + QFI ID.

In this contribution, we would like to discuss the details of the bit rate table, rate control MAC CE format and the rate query MAC CE for XR.

Discussion

Bit rate table

In the SA4 LS [2], SA4 replies that the range of bit rate varies from 100kbps to 40Mbps and finer granularity, e.g., 1%, is generally preferred but coarser granularity may also be sufficient, e.g., 10%.

In the last RAN2 meeting, RAN2 has agreed that we specify a single table. In addition, as a starting point, we aim to have a table with 8 bits and exponential distribution and no multiplier.

To design the bit rate table with 100kbps to 40Mbps range and exponential distribution, the granularity of about 2.4% could be adopted and this granularity is acceptable by SA4.

Proposal 1: The bit rate table has 256 code points with exponential distribution of around 2.4% granularity.

Proposal 2: Multiplier is not needed.

Rate control MAC CE format

In RAN2#129bis meeting, it is agreed that from RAN2 point of view per flow indication in MAC CE is preferred, but there are concerns on F1 impact which needs to be verified by RAN3, so RAN2 will go with per flow approach unless R3 has issues with this. There is an FFS on how QFI is indicated, e.g. with DRB ID + QFI ID.

From our perspective, to save the signalling overhead of indicating rate control information for multiple QoS flows by multiple MAC CEs where each MAC CE indicates rate control information for a single QoS flow, the rate control MAC CE can indicate the rate control information of multiple QoS flows at a time. To be simple, a bitmap can be used at the beginning of the MAC CE, indicating the index(indices) of the QoS flows for which the rate control information is included in the MAC CE. For each indicated QoS flow, the bit rate is indicated. In this format, the index (indices) of the QoS flows can be configured based on the QoS flow ID and the PDU session ID, or the mapping between the index and the QoS flow ID and the PDU Session ID can be predefined, so the DRB ID, QoS flow ID or PDU session ID does not need to be indicated explicitly.

An example of the potential MAC CE format is shown in Figure 1 where the length of the bit rate field is 8 bits. The need for other fields in the MAC CE can be determined based on further RAN2 discussions.

Figure 1 Potential format of rate control MAC CE

Proposal 3: The rate control MAC CE indicates the rate control information of multiple DRBs/QoS flows.

Proposal 4: In the rate control MAC CE, the index (indices) of the QoS flows is included, e.g., via a bitmap.

Proposal 5: The index in the rate control MAC CE can be configured by network based on the QoS flow ID and the PDU session ID, or the mapping between the index in the rate control MAC CE and the QoS flow ID and the PDU session ID can be pre-defined.

UL rate query

In RAN2#129bis meeting, rate query MAC CE is agreed. Some details on the rate query MAC CE can be further discussed. For example, the trigger of rate query can be similar to the trigger of Recommended bit rate query, e.g., requested by upper layer. After a rate query is triggered, it can be considered as pending until it is cancelled. After the generation or transmission of the rate query MAC CE, the triggered (pending) rate query should be cancelled if the rate query MAC CE contains rate query information associated with this triggered rate query. In addition, we think that the upper layer can also cancel the triggered rate query if the rate query is pending, e.g., when the upper layer has knowledge that the UL congestion is relieved.

Proposal 6: After the generation or transmission of the rate query MAC CE, the triggered rate query is cancelled.

Proposal 7: The upper layer can cancel the pending rate query.

Conclusion

In this contribution, we have discussed the details of the rate control MAC CE from gNB to UE, the potential format of the rate control MAC CE and whether to support the rate query MAC CE. We have the following proposals:

Proposal 1: The bit rate table has 256 code points with exponential distribution of around 2.4% granularity.

Proposal 2: Multiplier is not needed.

Proposal 3: The rate control MAC CE indicates the rate control information of multiple DRBs/QoS flows.

Proposal 4: In the rate control MAC CE, the index (indices) of the QoS flows is included, e.g., via a bitmap.

Proposal 5: The index in the rate control MAC CE can be configured by network based on the QoS flow ID and the PDU session ID, or the mapping between the index in the rate control MAC CE and the QoS flow ID and the PDU session ID can be pre-defined.

Proposal 6: After the generation or transmission of the rate query MAC CE, the triggered rate query is cancelled.

Proposal 7: The upper layer can cancel the pending rate query.

References

- Report from session R19 XR,	Session chair (Huawei), RAN2#129bis meeting

- R2-2500056, Reply to LS on appropriate range and granularity of bit rate adaptation for XR applications, SA4, LS in, RAN2#129 meeting