3GPP TSG-RAN WG2 Meeting #130	 R2-2503572

St Julian’s, Malta, May 19-23, 2025

Agenda Item:	8.3.2

Source: 	NEC

Title:  	Discussion on AIML mobility functionality management

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#129b meeting, the following agreements are made:

=> The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects:

- Data collection for model training

- UE capability

- Applicability reporting

- Inference configuration and reporting

- Performance monitoring and management

= > Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM)

=> The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required.  FFS when the UE can perform inference

=> For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements.  FFS details

=> For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements.  FFS details

=> Baseline is that this applies to all A1-6 events, unless technical problems are identified.  Baseline quantity is RSRP.

=> The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

=> For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

FFS whether association ID is required and should provide motivation on per use case bases

=> As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

=> Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline. Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

=> Model transfer will not be discussed in this study item.

=> For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring. FFS metrics per use case

In this contribution, we further discuss measurement report related issue and functionality management.

- 2 Discussion

- 2.1 Enhanced measurement report

In last RAN2#129b meeting, RAN2 has agreed that the inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework. It is straightforward since the handover related procedure is configured by RRM measurement configuration so that this configuration framework can be enhanced to match AIML mobility inference configuration. Here we further discuss measurement report related issue.

In legacy, measurement report can be initiated by either periodical or event-triggered, but noted that no matter which type of report is configured in reportConfig, the UE can only include the latest available measurement results into measurement report without explicit time information. When it comes to AIML mobility, when the event is fulfilled, multiple measurement results in future time instance can be predicted by AIML model for temporal domain, therefore, when AIML based measurement report is triggered, the time information about event fulfilment and future measurements results should be included into measurement report so that NW can be better aware of what situation happened to the UE.

Proposal-1: enhance measurement report to include the following information:

- predicted time information of event fulfilment

- predicted time information of future measurement results

- 2.2 Inference configuration

In last RAN2#129b meeting, RAN2 has agreed the UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference.

Based on online discussion, we understand that dynamic signalling will not be supported to activate/deactivate inference configuration, instead the UE just follows the inference configuration. Here we discuss FFS part, i.e., when the UE can perform inference, in our view, the AIML mobility prediction is mainly used to serve handover, but in some cases, the UE does not have to always perform AIML mobility prediction, for example, if one UE is staying in not-at-cell-edge (e.g., signal quality is good enough) or the UE is in low mobility/stationary mode, probably there is no need to perform AIML mobility inference. Therefore, when NW configures UE with inference configuration, some conditions (e.g., not-at-cell-edge and/or low mobility) can be included in this inference configuration. And the UE applying the inference configuration means once the configured conditions within configuration are met, the UE can start to perform inference.

Proposal-2: The UE can be configured to perform inference only when it is in high mobility and/or at the cell edge.

In addition to mobility and signal condition, UE resource constraint like battery status can also impact running of the AI/ML inference efficiently. Performing inference under low battery can lead to further draining of power unnecessarily. Thus, it will be beneficial to allow UE to stop based on the battery state. Simple threshold based solutions can be configured through RRC along with inference configuration to guide the UE behaviour under resource constraint situation.

Proposal-3: The UE can stop the AI/ML based inference based on the low battery state with the help of thresholds set in the inference configuration.

- 3 Conclusions

Proposal-1: enhance measurement report to include the following information:

- predicted time information of event fulfilment

- predicted time information of future measurement results

Proposal-2: The UE can be configured to perform inference only when it is in high mobility and/or at the cell edge.

Proposal-3: The UE can stop the AI/ML based inference based on the low battery state with the help of thresholds set in the inference configuration.

- 4 References