3GPP TSG-RAN WG2 Meeting #130	R2-2503600

# Malta, St Julian’s, May 19th – 23rd, 2025

Source: KDDI Corporation

Title:　 Considerations on open issues for RLC enhancements

Agenda Item:	8.7.5	RLC enhancements

Document for:	Discussion

- Introduction

In this contribution, we will share our considerations on the following open issues from the RLC rapporteur’s summary.

[POST129bis][505][XR] RLC running CR and open issues (vivo)

Scope:

- Update and review the CR

- List open issues related to the CR

Intended outcome:

- Running CR for endorsement in the next meeting

- List of open issues for discussion at the next meeting

Deadline:  Long

- Avoid unnecessary RLC retransmissions:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RLC-6 | Editor’s Note: FFS whether there are any RLF detection impacts when avoiding unnecessary retransmissions is introduced. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| RLC-10 | Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard. - As raised by ZTE below | Issue Type: Not essential not important How to address it: based on companies’ contribution |
| RLC-11 | FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with” - As raised by Nokia and Apple below | Issue Type: Not essential but important? How to address it: based on companies’ contribution |

- Ensure timely RLC retransmissions:

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RLC-7 | Editor’s Note: It is still open how Autonomous Retransmission coexists with ARQ procedures, i.e. whether/how to increment the RETX_COUNT for Autonomous Retransmission. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| RLC-8 | Editor’s Note: It is still open on how to avoid excessive polling for the polling enhancement, e.g. only one polling or multiple. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| RLC-9 | Editor’s Note: FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission due to polling enhancement. | Issue Type: Not essential but important How to address it: based on companies’ contribution |

- Discussion

Avoid unnecessary RLC retransmissions:

    - RLC-6

There is a possibility that avoiding unnecessary retransmissions may result in the RETX_COUNT not increasing, which could potentially prevent RLF detection. However, in our view, the impact of this is marginal, because there are other triggers for RLF detection, such as T310 expiry, beam failure recovery, T312 expiry, and if the radio conditions are poor and RETX_COUNT is not incremented, other RLF detection mechanisms will be triggered.

Proposal 1: RAN2 confirm that avoiding unnecessary retransmissions may have a marginal impact on RLF detection.

    - RLC-10

Regarding the associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard, we believe that RAN2 should specify UE behaviors for the case where both are configured at least. However, from a standard impact perspective, we think that supporting only the case where both are configured is much simpler. It also means that other cases should not be supported by the specification.

Proposal 2: RAN2 agrees that the specification should only support the case where both stopReTxObsoleteSDU and DL t-RxDiscard are configured, and that other cases should not be supported.

    - RLC-11

We believe it is better to discard the polling in cases where there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with, because it’s the simplest to handle the case.

Proposal 3: RAN2 agrees that discard the polling trigger in the case where there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with.

Ensure timely RLC retransmissions:

    - RLC-7

There is a possibility that if autonomous retransmissions trigger an increment of the RETX_COUNT, it may lead to earlier detection of RLF in poor radio conditions. However, in our view, whether this is beneficial or not depends on the specific situation. Therefore, we believe that the decision should be based on which approach has the less standard impact. We propose that autonomous retransmissions should trigger an increment of the RETX_COUNT, since otherwise we have to develop a new procedure for this special case.

Proposal 4: RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT.

    - RLC-8

For the polling enhancement, we tend to agree that we should have a solution for avoiding excessive polling, since it may give negative impacts on radio resource utilization. We also notice that there might be multiple candidate solutions. One is Multiple polling with enhancements at Rx based solution, polling enhancement triggers shorter Status PDU Prohibit timer. The other is single pooling with enhancements at Tx based solution with poll prohibit timer, with this solution Tx side will prevent excessive polling using poll prohibit timer, which means that during the timer, Tx side cannot set a poll bit. We propose to study both options and make final decision in the next August meeting.

Proposal 5: RAN2 study the following options:

- Option1: Multiple polling with enhancements at Rx based solution, shorter Status PDU Prohibit timer.

- Option2: Single pooling with enhancements at Tx based solution with poll prohibit timer.

    - RLC-9

We believe that this open issue arises from the perception that polling enhancement may lead to early retransmissions, and we agree the point that we should confirm that it does not cause any drawbacks. In our view, addressing excessive polling as discussed above should mitigate concerns, and there are no other issues or drawbacks associated with this.

Proposal 6: RAN2 confirm that polling enhancement does not lead to early/unnecessary retransmissions.

- 3. Conclusion

The followings are proposed.

Proposal 1: RAN2 confirm that avoiding unnecessary retransmissions may have a marginal impact on RLF detection.

Proposal 2: RAN2 agrees that the specification should only support the case where both stopReTxObsoleteSDU and DL t-RxDiscard are configured, and that other cases should not be supported.

Proposal 3: RAN2 agrees that discard the polling trigger in the case where there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with.

Proposal 4: RAN2 agrees that autonomous retransmissions should trigger an increment of the RETX_COUNT.

Proposal 5: RAN2 study the following options:

- Option1: Multiple polling with enhancements at Rx based solution, shorter Status PDU Prohibit timer.

- Option2: Single pooling with enhancements at Tx based solution with poll prohibit timer.

Proposal 6: RAN2 confirm that polling enhancement does not lead to early/unnecessary retransmissions.

- 4. Reference

[1] R2-2503767	Discussion summary and list of RLC open issue for R19 XR　vivo