3GPP TSG RAN2 Meeting #130                                                                R2-2503605

19th May – 23rd May 2025, St.Julians, Malta

Agenda item:	8.4.4

Source:	Samsung

Title:	Procedures for LP-WUS in RRC Connected Mode

Document for:	Discussion & Decision

Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1].

During the previous meeting #129 ~ #129bis, with respect to Procedures for LP-WUS in RRC Connected Mode agenda, RAN2 agreed as follows:

#129 For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer. #129bis For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used. Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback. Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously. If configured, the UE can signal a preferred time offset via UAI signalling. Ask RAN1 for further information regarding their conclusions. FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells. For NR-DC, the LP-WUS in MCG and SCG can be configured independently. Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC.

In this contribution, we would like to discuss the detailed procedures and configurations to support LP-WUS in RRC IDLE/INACTIVE mode.

Discussion

- Option 1-1 and 1-2

Based on the discussions that have been carried out in RAN1 and RAN2, it has become apparent that the main and most significant use case for LP-WUS in the RRC Connected mode will revolve around  'PDCCH monitoring which is triggered by LP-WUS in conjunction with the C-DRX configuration'.

For a quick summary, we capture the each options examples from RAN1’s discussion [6]:

Option 1-1: Replacing DCP following legacy configurations

Option 1-2: UE performs LP-WUS monitoring outside at least legacy C-DRX active time, and PDCCH monitoring may be additionally triggered

Based on the discussions from RAN2#127bis, RAN2 progressed and established the precise PDCCH monitoring timing as follows:

Observation 1. RAN2 defines when the UE starts monitor PDCCH as follows:

- Option 1-1: If LP-WUS addressed to UE or UE's subgroup is detected and the legacy drx-onDurationTimer is started

- Option 1-2: If LP-WUS addressed to UE or UE's subgroup is detected and the new timer is started. FFS on drx-onDurationTimer.

The remaining question is whether this timer is newly configured or utilizes an existing traditional timer such as drx-onDurationTimer. It is noteworthy that if the abovementioned timer for Option 1-2 corresponds to drx-onDurationTimer and the LP-WUS periodicity is comparable to the DRX cycle, then Option 1-2 might be equivalent to Option 1-1.

Observation 2. If the above timer for Option 1-2 is the same as drx-onDurationTimer and the LP-WUS periodicity is similar as DRX cycle, the Option 1-2 could be the same as Option 1-1

In the meantime, when the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the UE operating with LP-WUS option 1-2 is not required to monitor the PDCCH during the legacy DRX active time.

Proposal 1. When the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the UE operating with LP-WUS option 1-2 is not required to monitor the PDCCH during the legacy DRX active time.

- Activation/ Deactivation

Previously, RAN1 had discussed and agreed that the PDCCH monitoring triggered by LP-WUS can be enabled/disabled by RRC signaling when the UE is in RRC Connected mode.

- RAN1#116 Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

With the above RAN1 agreements, RAN2 can discuss whether the LP-WUS monitoring based PDCCH monitoring when the UE is in RRC Connected mode can be enabled/disabled using RRC signalling.

For the sake of UE power saving and operational efficiency, we

Proposal 2. RAN2 confirms that the LP-WUS monitoring in RRC Connected mode can be enabled/disabled using RRC signalling.

And for further study, RAN2 may discuss whether the MAC signalling could be introduced or not.

Proposal 3. RAN2 needs to discuss whether the LP-WUS monitoring in RRC Connected mode can be enabled/disabled using MAC signalling.

- 3 Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Observation 1. RAN2 defines when the UE starts monitor PDCCH as follows:

- Option 1-1: If LP-WUS addressed to UE or UE's subgroup is detected and the legacy drx-onDurationTimer is started

- Option 1-2: If LP-WUS addressed to UE or UE's subgroup is detected and the new timer is started. FFS on drx-onDurationTimer.

Observation 2. If the above timer for Option 1-2 is the same as drx-onDurationTimer and the LP-WUS periodicity is similar as DRX cycle, the Option 1-2 could be the same as Option 1-1

Proposal 1. When the LP-WUS enabled UE is configured with both Option 1-2 and legacy DRX, then the UE operating with LP-WUS option 1-2 is not required to monitor the PDCCH during the legacy DRX active time.

Proposal 2. RAN2 confirms that the LP-WUS monitoring in RRC Connected mode can be enabled/disabled using RRC signalling.

Proposal 3. RAN2 needs to discuss whether the LP-WUS monitoring in RRC Connected mode can be enabled/disabled using MAC signalling.

- References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC