3GPP TSG RAN2 Meeting#130                                                                R2-2503606

St. Julians, Malta, 19th – 23rd May, 2025

Agenda item:	8.11.2

Source:	Samsung

Title:	Random access in SBFD

Document for:	Discussion & Decision

Introduction

During the previous meeting (RAN2#129bis), RAN2 agreed as follows:

NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

In this contribution, we would like to discuss further on the remaining open issues.

Discussion

Early Indication

RAN2 needs to discuss and decide on the necessity of an 'Early Indication' mechanism for Release 19 SBFD. Similar to previous releases of NR that supported an early indication mechanism for REDCAP, we believe Msg 3-based early indication mechanism could be considered for Release 19 SBFD, in addition to the obvious Msg 1-based early indication which is already agreed as a RAN2 understanding.

The reason behind considering an early indication mechanism is the same as for previous NR features; namely, the inability to promptly exchange or retrieve UE radio access capabilities until later during the initial access procedure once the UE identity has been verified and NAS and AS authentication have successfully completed.

The example of NR standalone mode operation for FR1 TDD 30kHz is considered in below Figure 2. Any UE in RRC_IDLE must connect to the network and first perform the NAS initial access procedure (“with stored context”) before DL/UL data scheduling.

Figure 2 NAS initial access procedure (“with stored context”) in NR SA mode

It should be considered that about 8-10 DL/UL RRC/NAS messages must be exchanged between the network and the UE during NAS initial access procedure. If an early indication type of mechanism becomes available for SBFD-aware UEs, then any PUSCH containing RRC and NAS messages during the transition from RRC_IDLE to RRC_CONNECTED mode can already be transmitted usint the SBFD UL subband following RACH Message 5 (RRC Setup Complete + NAS Registration Request) upon entering RRC_CONNECTED mode. Hence, an early indication could reduce the initial access delay significantly.

This is because SBFD UL subbands can only be configured for the UE by the RRC re-configuration msg which itself can occur only after the UE has been identified and authenticated by gNB and AMF.

As a candidate for early indication (EI) of SBFD-aware UEs, we consider both Msg1-based EI which could rely on RACH/preamble resource and Msg3-based EI that could rely on Msg.3 (or later) transmissions.

When RACH configuration option 1 is configured or indicated through SIB1, the choice of ROs used for random access attempts by SBFD-aware UEs depends on the RO selection and validation rules. If an SBFD-aware UE utilizes a legacy RO, Early Indication may require a Layer 3 (L3)-based approach.

It is also crucial to consider that feature independence must be achievable for deploying SBFD on the network side. Operators may opt to deploy SBFD solely for operations involving UEs in RRC_CONNECTED mode, meaning they may choose not to leverage the random access in SBFD resources feature. Consequently, Early Indication upon transitioning to RRC_CONNECTED mode would still necessitate reliance on an L3-based solution.

Proposal 1. RAN2 supports the Msg3-based Early Indication for Rel-19 SBFD-aware UEs.

RO Type Selection

For the RO type selection, RAN2 now have the below working assumption:

Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources.

With the introduction of separate RSRP thresholds for Msg1 repetitions in SBFD RO, the applicable Msg1 repetition numbers may differ between the two RO types. Consequently, the sets of RA resources associated with these repetition numbers may also be different. In order to determine applicable Msg1 repetition number, and the associated RA resource set, RO type should be selected before RA resource set selection.

We would like to confirm the WA as an official agreement.

Proposal 2. RAN2 confirms the WA: For SBFD-aware UE, the selection of RO type is performed before the selection of the set of Random Access resources.

RA Resource Set Change

Considering that a complete random access procedure is typically executed within a single RA resource set, it is essential for RAN2 to determine whether RA resource set change should be introduced for RO type switching. However, since the change of an RA resource set implies the change of many essential RACH parameters during a single random access procedure, the change of an RA resource set may introduce many undesirable and unexpected issues, e.g., whether to compensate power ramping if new powerRampingStep is applied, what if the newly applied preambleTransMax has already been passed. Moreover, since the benefit brought by the RA resource set change is marginal, avoiding the potential issues by switching the RO type within the selected RA resource set is more preferable.

Proposal 3. RA resource set change is not supported for RO type switching.

Fallback of Msg1 Repetition Number

Considering RAN1 has introduced separate RSRP thresholds for Msg1 repetition in SBFD RO, and the RSRP thresholds are introduced for all the three Msg1 repetition numbers, the same mechanism for the Msg1 repetition should be applied for SBFD RO. That is, the same fallback mechanism of Msg1 repetition number should also be applied for SBFD RO. And, no further optimization is required.

Proposal 4. Fallback of Msg1 repetition number is supported for SBFD RO without further optimization.

Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1. RAN2 supports the Msg3-based Early Indication for Rel-19 SBFD-aware UEs.

Proposal 2. RAN2 confirms the WA: For SBFD-aware UE, the selection of RO type is performed before the selection of the set of Random Access resources.

Proposal 3. RA resource set change is not supported for RO type switching.

Proposal 4. Fallback of Msg1 repetition number is supported for SBFD RO without further optimization.