3GPP TSG-RAN WG2#130											         R2-2503610

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	8.2.2

Source: 	NEC

Title:	        Ambient-IoT Paging

Document for:	 Discussion

- Introduction

RAN2#129bis meeting achieved following agreements:

Agreements - FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation. - RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. - Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message. - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. - The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. - Send an LS to SA2 to tak this into account for their design. - In case of CBRA, only 16 bits random ID is included in Msg1. FFS can be revisited if message type will be needed for other D2R messages purposes - RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario. - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number. - IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices. - Same Msg2 format is used for initial transmission and retransmission of Msg2.

This contribution further discusses remaining issues of A-IoT paging.

- Discussion

  - Open issue: Device behavior of respond a paging for re-access purpose

In study phase we already agreed that the reader can send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN and the duplicated response from devices for the same service request should be avoided [2].

For paging indicating CBRA, we had following agreements:

Agreements on CBRA For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer.

For a service identified by a transaction ID, the device may store status information about whether it has been successfully responded, or whether re-access is needed:

a) If device has transmitted msg3 and not receiving a following "NACK" indication, device consider the re-access status of the service is “not needed”;

b) If device has not transmitted msg3 (this also includes cases that device selected resource but did not transmit msg1, or did not receive random ID response message echo its random ID), or device received "NACK" indication following the msg3 transmission, device considers the re-access status of the service is “needed”

From the device perspective, if a received paging is associated with a “successfully responded service” or re-access status of the service is “not needed”, device should avoid duplicated response.

(MAC 2-4) For a service identified by a transaction ID, the device may store status information about whether the service has been successfully responded, or whether re-access is needed; If device considers it had successfully responded the received service before or re-access status of the service is “not needed”, device should avoid duplicated response:

(MAC 2-4) If device has transmitted msg3 and not receiving a following "NACK" indication, device considers the re-access status of the service is “not needed”;

(MAC 2-4) If device has not transmitted msg3 (this also includes cases that device selected resource but did not transmit msg1, or did not receive random ID response message echo its random ID), or device received "NACK" indication following the msg3 transmission, device considers the re-access status of the service is “needed”.

And we also agreed:

Agreements on CFRA: For CFRA, NACK feedback and re-access is not supported. FFS how to achieve

Regarding how to achieve the last meeting agreement that “For CFRA, NACK feedback and re-access is not supported.”, our understanding is that the expected device behavior is to always respond to a CFRA without considering whether it has responded before or not.

(MAC 2-8) Device always respond to a CFRA if it is identified by the CFRA without considering whether it has responded before or not.

  - Open issue: Device behavior of respond a paging with a new transaction ID

Regarding device behaviour when receiving new service requests during an ongoing procedure: As discussed in the previous meeting, we need to select between Option A and Option B:

- Option A: ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure

- Option B: terminate the ongoing procedure and respond to the latest request

From our perspective, Option B presents a more straightforward implementation since R19 do not consider multi-reader scenarios. This approach requires the Core Network (CN) to ensure no parallel services from different readers that might cause unintended interruptions.

(MAC 1-1) For device behavior if it gets a new service request, support option B: terminate the ongoing procedure and respond to the latest request.

  - Open issue: Multiple device identifiers in single CFRA paging

If multiple device identifiers in single paging is supported, assuming a device ID list is used to indicate multiple device IDs, the list can also implicitly indicate the resource allocation, including the time-frequency location of the access occasion for random access, and the assigned AS ID. For example, each access occasion or assigned AS ID may be mapped to a device according to the ranking of the device in the device ID list. By using the mapping relationship, A-IoT devices can be scattering to different access occasions and achieve contention free random access.

(MAC 2-9) From RAN2 perspective, CFRA for multiple devices can be supported.

(MAC 2-9) For CFRA configuration when paging message contains multiple device identifiers, access occasions or AS IDs indicated in paging message are mapped to devices according to the ranking of the device identifier in the list.

  - Configurations for lean inventory

Currently, following assistance information are supported:

The following assistance information may be provided: - AIoT service type (e.g. Inventory, Command); - As part of the correlation ID, status for inventory and command. Where the status can indicate pending or complete - Optionally, approximate number of AIoT devices based on AF request; - Optionally, approximate D2R message size based on AF request. Editor's note: Other assistance information may be added later if necessary. Editor's note: How the AIOTF determines the assistance information (e.g. approximate number of AIoT Devices, approximate D2R message size) without input from the AF is FFS.

Given that the "approximate number of AIoT devices" is an optional configuration field, the RAN node might lack prior knowledge of device density, making it challenging to determine optimal resource allocation for inventory procedures—particularly contention-based resources for random ID transmissions.

The A-IoT RAN node might lack prior knowledge of “approximate number of AIoT devices”.

To address this, a RAN-initiated lean inventory mechanism could be implemented using a CFRA configuration. This configuration reserves a single dedicated Msg1 resource with paging identifier set to "all" devices. As specified in Vodafone's lean inventory proposal in SI stage (R2-2410416), devices receiving this contention-free paging signal will respond immediately via the allocated Msg1 resource without awaiting subsequent random ID echo procedures. RAN node can then estimate the number of in-coverage devices by analyzing the received signal strength from these responses. For subsequent Core Network (CN)-triggered inventory requests, the RAN node can allocate contention-based Msg1 resources based on the lean inventory results, thereby optimizing resource utilization

Support lean inventory using CFRA configurations: reserves a single dedicated Msg1 resource with paging identifier set to "all" devices. The A-IoT RAN node can then estimate the number of in-coverage devices by analyzing the received signal strength from devices’ responses.

- Conclusion

Observation 1:	The A-IoT RAN node might lack prior knowledge of “approximate number of AIoT devices”.

Proposal 1:	(MAC 2-4) For a service identified by a transaction ID, the device may store status information about whether the service has been successfully responded, or whether re-access is needed; If device considers it had successfully responded the received service before or re-access status of the service is “not needed”, device should avoid duplicated response:

Proposal 2:	(MAC 2-4) If device has transmitted msg3 and not receiving a following "NACK" indication, device considers the re-access status of the service is “not needed”;

Proposal 3:	(MAC 2-4) If device has not transmitted msg3 (this also includes cases that device selected resource but did not transmit msg1, or did not receive random ID response message echo its random ID), or device received "NACK" indication following the msg3 transmission, device considers the re-access status of the service is “needed”.

Proposal 4:	(MAC 2-8) Device always respond to a CFRA if it is identified by the CFRA without considering whether it has responded before or not.

Proposal 5:	(MAC 1-1) For device behavior if it gets a new service request, support option B: terminate the ongoing procedure and respond to the latest request.

Proposal 6:	(MAC 2-9) From RAN2 perspective, CFRA for multiple devices can be supported.

Proposal 7:	(MAC 2-9) For CFRA configuration when paging message contains multiple device identifiers, access occasions or AS IDs indicated in paging message are mapped to devices according to the ranking of the device identifier in the list.

Proposal 8:	Support lean inventory using CFRA configurations: reserves a single dedicated Msg1 resource with paging identifier set to "all" devices. The A-IoT RAN node can then estimate the number of in-coverage devices by analyzing the received signal strength from devices’ responses.

- References

[1] RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Huawei (moderator, RAN1 Vice-Chair).

[2] TR 38.769, Study on solutions for ambient IoT (Internet of Things)