3GPP TSG-RAN WG2 Meeting #130		R2-2503614

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Discussion on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we will further discuss the remaining issues on RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE for the following open issue list from post meeting email discussion in [1] and [2].

| Index | Issue description | Rapporteur suggestion |
| --- | --- | --- |
| RRC-6 | the value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold for serving cell relaxation/offloading and neighboring cell relaxation Editor’s NOTE: FFS on the value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold for serving cell relaxation/offloading and neighboring cell relaxation. | Issue Type: essential How to address it: can be addressed based on RAN1/RAN4 inputs or companies’ contribution |
| RRC-7 | FFS on exit condition for serving cell RRM relaxation Editor’s NOTE: FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| RRC-8 | FFS on whether/how to reduce the threshold number for LP-WUS/WUR Editor’s NOTE: FFS on the relationship between the thresholds for serving cell relaxation and offloading. FFS on the relationship between the thresholds for serving cell relaxation/offloading, neighboring cell relaxation and s-IntraSearchP/s-NonIntraSearchP. Editor’s NOTE: FFS on the relationship between the thresholds for serving cell relaxation/offloading, neighboring cell relaxation and entry/exit condition of using LP-WUS, [and potential pre-condition between RRM relaxation/offloading criteria and entry/exit condition of using LP-WUS]. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| RRC-9 | FFS on whether/how RRM relaxation is applicable for high priority frequency Editor’s NOTE: FFS on whether/how RRM relaxation is applicable for high priority frequency. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| RRC-10 | FFS on low mobility criteria Editor’s NOTE: FFS on “low mobility” criteria. | Issue Type: not essential not important How to address it: can be discussed based on companies’ contribution |

RRM relaxation/offloading: Open issue 38304-1: FFS on the terminology LP-WUS UE. Open issue 38304-2: FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. Open issue 38304-3: FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. (Same as the open issue in RRC, i.e., FFS on exit condition for serving cell RRM relaxation) Open issue 38304-4: FFS relaxed measurement criteria/RRM offloading criteria is different from LP-WUS monitoring criteria. (Same as the open issue in RRC, i.e., FFS on whether/how to reduce the threshold number for LP-WUS/WUR) Open issue 38304-6: Whether UE low mobility criterion or stationary criterion should be considered for RRM relaxation/offloading. (Same as the open issue in RRC, i.e., FFS on low mobility criteria) Open issue 38304-7: Whether Relaxed measurement and offloading measurement can be performed when there is NR inter-frequency and/or NR inter-RAT frequency with reselection priority higher than that of the camped frequency. (Same as the open issue in RRC, i.e., FFS on whether/how RRM relaxation is applicable for high priority frequency)

Besides, we will also discuss the other remaining issues on whether/how to reduce the threshold for LP-WUS/WUR WI, e.g. whether to remove the LR thresholds for entry condition.

- Discussion

  - Value range of configured threshold (RRC-6)

Regarding the open issue in RRC as below:

the value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold for serving cell relaxation/offloading and neighboring cell relaxation Editor’s NOTE: FFS on the value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold for serving cell relaxation/offloading and neighboring cell relaxation.

In current RRC running CR for LP-WUS, it is captured as follows:

which is the value range of ThresholdPLP and ThresholdQLP for LR measurement based threshold for serving cell RRM relaxation/offloading and neighboring cell RRM relaxation. Based on our observation, the receiver sensitivities of LR and MR are roughly the same. Although the receiver hardware is degraded for LR, the number of information bits carried is smaller and the data rate is lower, which results in a similar overall sensitivity compared to MR. Thus, the value range for LR measurement thresholds should reuse the range corresponding to the absolute measured values for MR. The detailed aspects can be discussed in the running CR.

Proposal 1: (RRC-6) The value range for the threshold on the measured RSRP/RSRP via LR could refer to the absolute measured values for MR. Details could be discussed in the running CR.

  - Exit condition for RRM measurement relaxation (RRC-7/38304-3)

Regarding the open issue:

FFS on the exit condition for serving cell RRM relaxation Editor’s NOTE: FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements.

In R16/17 RRM relaxation, no separate exit condition is defined. That is, if the entry condition for neighboring cell RRM relaxation is not fulfilled, neighboring cell RRM relaxation is exited and UE performs the normal neighboring cell RRM measurement. We think the same principle can be applied to R19 serving/neighboring cell RRM relaxation, i.e. no need to define separate exit condition other than “not fulfilling the entry condition”, in case the R19 serving/neighboring cell RRM relaxation criteria is not merged with the LP-WUS monitoring condition, as we discussed in section 2.3. On the other hand, if the exit conditions of R19 RRM relaxation are merged with LP-WUS monitoring exit condition, then the exit conditions for R19 RRM relaxation are separate from the entry condition which is the same as the condition for LP-WUS monitoring.

Proposal 2: (RRC-7/38304-3) If LP-WUS monitoring condition is not merged with Rel-19 serving/neighboring cell RRM relaxation criteria, no need to define a separate exit condition other than ‘not fulfilling the entry condition’ for serving/neighboring cell RRM relaxation.

  - Merge LP-WUS monitoring condition and RRM relaxation condition/criteria (38304-4)

After the study in Rel-18, the following conclusion was made in TR 38.869 [3]:

UE power saving gain cannot be observed if the existing Rel-18 MR RRM measurement periodicity for serving and neighbour cells are applied and UE MR enters in ultra-deep sleep during LP LP-WUS monitoring, therefore MR serving and neighbour cell measurement with further time domain relaxation than that is allowed in Rel-18 specification for IDLE/INACTIVE and/or at least serving cell RRM offloaded from MR to LR are beneficial.

It could be observed that there is no power saving gain if UE starts to monitor LP-WUS while the serving cell RRM relaxation/offloading or neighboring cell RRM relaxation has not been performed. Thus, the corresponding scenario should be avoided. One potential solution is to merge the entry/exit conditions for serving cell RRM offloading with LP-WUS monitoring condition or merge the entry/exit conditions for serving/neighboring cell RRM measurement relaxation with LP-WUS monitoring condition, i.e., when entry condition(s) for LP-WUS monitoring is satisfied, UE could relax the serving/neighboring cell RRM measurement or offloading the serving cell RRM measurement.

In RAN2#129bis meeting, we discussed how to reduce the configured threshold number for LP-WUS/WUR, and the corresponding solution direction was discussed in the offline discussion [4]. However, we didn’t reach a consensus on whether the condition for RRM relaxation or offloading should be combined with LP-WUS monitoring condition, as below.

Chair observation: only one company have strong concern on the following discussion point - Can further discuss Direction 3: - Direction 3a. merge LPWUS monitoring and serving cell offloading - Direction 3b. merge LPWUS monitoring and Rel-19 RRM relaxation

In our understanding, both Direction 3a and Direction 3b could get noticeable power saving gain based on the study in Rel-18, thus we could support both. In details:

- For the scenario that if a cell only allows the serving cell RRM offloading or UE only supports the serving cell RRM offloading, merge the conditions for LP-WUS monitoring and serving cell RRM offloading.

- For the scenario that if a cell only allows the serving/neighboring cell RRM relaxation or UE only supports the serving/neighboring cell RRM relaxation, merge the conditions for LP-WUS monitoring and Rel-19 serving/neighboring cell RRM relaxation.

- For the scenario that if a cell allows the serving/neighboring cell RRM relaxation and serving cell RRM offloading, and UE also supports both, it is up to network configuration on whether to merge which one with the LP-WUS monitoring.

Proposal 3-1: (38304-4) Merge LP-WUS monitoring entry/exit condition and serving cell RRM offloading if only serving cell RRM offloading is supported in the camped cell or UE only supports serving cell RRM offloading.

Proposal 3-2: (38304-4) Merge LP-WUS monitoring entry/exit condition and Rel-19 serving/neighboring cell RRM relaxation if only Rel-19 serving/neighboring cell RRM relaxation is supported in the camped cell or UE only supports Rel-19 serving/neighboring cell RRM relaxation.

Proposal 3-3: (38304-4) It is up to network configuration to merge LP-WUS monitoring entry/exit condition with serving cell RRM offloading or Rel-19 serving/neighboring cell RRM relaxation if both are supported in the camped cell and UE supports both.

  - Analysis of the relationship between thresholds for LP-WUR(RRC-8)

Open issue#1: Relationship between the thresholds for serving cell RRM relaxation and offloading

In this part, we will discuss the relationship of RRM relaxation/offloading and other thresholds which related to the following open issue:

Editor’s NOTE: FFS on the relationship between the thresholds for serving cell relaxation and offloading. FFS on the relationship between the thresholds for serving cell relaxation/offloading, neighboring cell relaxation and s-IntraSearchP/s-NonIntraSearchP.

Figure 1: Example of relationship between thresholds of RRM relaxation/offloading with legacy RRM relaxation/ SIntraSearchP/SnonIntraSearchP

The relationship between the threshold of serving cell RRM relaxation and RRM offloading

If both serving cell RRM relaxation and RRM offloading are supported in a cell, it is obvious that the threshold(s) of serving cell RRM offloading should be configured to be stricter than the threshold(s) of serving cell RRM relaxation, i.e., the thresholds of entry condition for RRM offloading is higher than that of serving cell RRM relaxation to achieve the purpose that when the measured serving cell quality becomes better, e.g. UE moves from cell edge to cell center, UE performs RRM relaxation firstly and later performs RRM offloading shown as the figure 1.

Proposal 4: (RRC-8) The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation.

The relationship between the threshold of serving cell RRM offloading with SIntraSearchP/SnonIntraSearchP

It is reasonable that MR does not need to perform neighboring cell RRM measurement when serving cell RRM measurement via MR is offloaded since serving cell RRM measurement is more important than neighboring cell RRM measurement. Thus if UE determines serving cell RRM measurement could be offloaded, it proves the current serving cell quality is good enough, which is most like higher than the threshold to stop neighboring cell measurement, i.e. SIntraSearchP/SnonIntraSearchP. In other words, the threshold of the entry condition for serving cell RRM offloading should be higher than or equal to the maximum of {SIntraSearchP, SnonIntraSearchP}.

Proposal 5-1: (RRC-8) The threshold of the enter condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, i.e. maximum of {SIntraSearchP, SnonIntraSearchP}.

Relationship between the threshold of R19 RRM relaxation and R17/R16 neighboring cell RRM relaxation

RAN2 has the following agreements in RAN2#129bis meeting：

Merge the entry/exit condition for Serving Cell RRM measurement relaxation and Rel-19 Neighboring Cell RRM measurement relaxation (higher priority frequency is separate discussion).

As we discussed before, the serving cell RRM measurement is more important than neighboring cell RRM measurement, and it is agreed to merge the entry/exit condition for serving cell RRM relaxation and Rel-19 neighboring cell RRM relaxation, while when R16/R17 neighboring cell RRM relaxation criteria is fulfilled, UE still performs normal serving cell RRM measurement which illustrates the criteria for R19 RRM relaxation should be stricter than the legacy R16/R17 neighboring cell RRM relaxation criteria.

Proposal 5-2: (RRC-8) The corresponding threshold of R19 neighboring cell RRM relaxation should be stricter than that of R16/17 neighboring cell RRM relaxation.

Open issue#2: Relationship between thresholds of RRM relaxation/offloading and LP-WUS monitoring

In this part, we will discuss the relationship of RRM relaxation/offloading and LP-WUS monitoring in case the proposal in section 2.2 is not agreed, i.e., we don’t agree to merge the entry/exit condition of LP-WUS monitoring with RRM relaxation or RRM offloading:

Editor’s NOTE: FFS on the relationship between the thresholds for serving cell relaxation/offloading, neighboring cell relaxation and entry/exit condition of using LP-WUS, [and potential pre-condition between RRM relaxation/offloading criteria and entry/exit condition of using LP-WUS].

In this case, to achieve the power saving gain, i.e., to guarantee serving/neighboring cell RRM measurement is relaxed when UE monitors LP-WUS, thresholds of the criteria for the serving/neighboring cell RRM relaxation should be lower than or equal to the threshold of the entry condition for LP-WUS monitoring.

Proposal 6: (RRC-8) If merging LP-WUS monitoring and Rel-19 RRM relaxation is not agreed, the threshold of the entry condition for R19 RRM relaxation should be lower than or equal to the threshold of the entry condition for LP-WUS monitoring.

And in case LP-WUS monitoring is not merged with serving cell RRM offloading, considering the entry conditions of LP-WUS monitoring include at least serving cell quality via MR, serving cell measurement via MR can not be offloaded to LR before UE starts the LP-WUS monitoring. Thus, thresholds of the criteria for the serving cell measurement offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

Proposal 7: (RRC-8) If merging LPWUS monitoring and serving cell RRM offloading is not agreed, the threshold of the entry condition for serving cell RRM offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

Based on the analysis above, we provide two examples to illustrate the thresholds and UE behaviours for the serving cell RRM relaxation/offloading and neighboring cell RRM relaxation compared to the threshold of entry condition on LP-WUS monitoring in the following figures.

For simplicity, the serving cell RRM offloading and serving cell RRM relaxation are provided in Figure 2 and Figure 3, respectively. All the criteria for serving cell RRM relaxation/offloading and neighboring cell RRM relaxation and the entry condition for LP-WUS monitoring are based on the serving cell quality via MR.

Figure 2: Thresholds and UE behaviors for serving cell RRM offloading, serving/neighboring cell RRM relaxation and LP-WUS monitoring

Figure 3: Thresholds and UE behaviours for serving/neighboring cell RRM relaxation and LP-WUS monitoring

  - High priority frequency RRM relaxation (RRC-9/38304-7)

RAN4 reached the following agreement in RAN4#114bis meeting and agreed to relaxed higher priority frequency layer search at case 1 (RRM measurement fully offloading):

Agreement: - Support relaxed higher priority frequency layer search at case 1 Tentative agreement: - For case 1, when the serving cell is above threshold for inter-frequency or inter-RAT neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search. - Note: K2 is 60

Regarding the following FFS:

FFS on whether/how RRM relaxation is applicable for high priority frequency Editor’s NOTE: FFS on whether/how RRM relaxation is applicable for high priority frequency.

Since RAN4 agreed to relaxed the search on higher priority frequency layer in case the entry condition of serving cell RRM offloading is satisfied for a UE, and in this case, considering there is no serving cell RRM measurement on MR, it is reasonable to also stop the RRM measurement on neighbor cell with higher priority frequency via MR.

And for the case the entry condition of serving/neighboring cell RRM relaxation is fulfilled, we think the measurement on neighboring cell with higher priority frequency could also be relaxed. Anyway, final decision could be up to RAN4.

Proposal 8: (RRC-9/38304-7) RAN2 assumes:

- When the entry condition of serving cell RRM offloading is fulfilled, the measurement on neighboring cells with higher priority frequency could be stopped;

- When the entry condition of serving cell RRM relaxation is fulfilled, the measurement on neighboring cells with higher priority frequency could be relaxed.

In Rel-16, the parameter highPriorityMeasRelax is introduced to indicate whether RRM measurements can be relaxed on high priority frequencies in case only low mobility criterion is satisfied, similarly here RAN2 also needs to discuss whether to introduce an indication to indicate whether the high priority frequencies measurement could be relaxed/stopped in case entry condition of serving cell RRM relaxation/offloading is satisfied. In our view, there is no need to introduce such indication, since in case serving cell RRM measurement is relaxed/offloaded, the RRM measurement on higher priority frequencies should be relaxed/stopped naturally, using network control is over-engineering. Anyway, final decision could be up to RAN4.

Proposal 9: (RRC-9/38304-7) RAN2 assumes no network indication is needed to control whether to relax/stop the RRM measurement on neighboring cells with higher priority frequency. It can be revisited based on RAN4 progress.

  - Neighboring cell measurement relaxation criteria

Low mobility (RRC-10/38304-6)

For the following FFS:

FFS on low mobility criteria Editor’s NOTE: FFS on “low mobility” criteria.

Since the current criteria for serving cell RRM offloading/relaxation doesn’t consider the low mobility, and introducing the low mobility condition will result long time discussion, e.g. when only one condition is configured, or when two conditions are configured but only one condition is fulfilled, etc., we prefer not to consider the low mobility criteria in Rel-19. On the other hand, the exit condition for RRM offloading is already introduced, thus the current mechanism is rigorous, considering the low mobility won’t bring much benefit or have much motivation.

Proposal 10: (RRC-10/38304-6) Low mobility condition is not needed for R19 RRM relaxation or offloading

FFS on the enhancements on R16 criteria (38304-2)

For the following FFS:

FFS on the enhancements based on R16 criteria FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

The case MR serving cell RRM measurement results are not available, is when the entry condition of serving cell RRM offloading is satisfied. As we discussed above, the threshold of the criteria for serving cell RRM offloading should be higher than or equal to the maximum of {SIntraSearchP, SnonIntraSearchP}, which means the measurement on equal/lower priority frequency/RAT has already been stopped when MR serving cell RRM measurement results are not available, and in this case the measurement on neighboring cell with higher priority frequency has also been stopped as discussed above. In summary, there is not any neighbor cell RRM measurement when MR serving cell RRM measurement results are not available. Thus, there is no need to work on the enhancement on R16 neighboring cell RRM relaxation in this case.

Proposal 11: (38304-2) There is no impact on R16 criterion of neighboring cell RRM relaxation for the case when MR serving cell RRM measurement results are not available.

  - LR thresholds in entry condition

In RAN2#129bis meeting, we discussed whether to remove the LR based thresholds for all entry conditions in the RRM relaxation/offloading and LP-WUS monitoring procedures to reduce the total numbers in LP-WUR, however, we didn’t reach any conclusion [4].

In our understanding, since we only agreed with the intra-band case in Rel-19. In this case, the serving cell RRM measurement performance via MR band could represent the LR band. Thus, the motivation to introduce the LR based threshold for entry conditions becomes meaningless. Besides, according to the following agreed RAN4 requirements:

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case) Agreement: At legacy state, when and how to turn on LR for serving cell measurement is up to UE implementation.

It seems that the LR may not perform the RRM measurement at the initial phase when UE accesses a cell since the LR is turned on based on UE implementation, for example, when the MR measurement is better than a threshold. In this case, if the LR thresholds are configured for entry conditions, the time UE enters for the RRM relaxation or LP-WUS monitoring procedure may be delayed intentionally and the power saving gain will be impacted since even if the entry condition based on MR measurement is satisfied, UE has to wait for the entry condition based on LR measurement considering the time UE to turn on the LR measurement may be late and the LR measurement is not obtained at this time.

Thus, we think removing the LR based thresholds for all entry conditions is a reasonable direction to reduce the total number of thresholds in LP-WUR WI without the impact on power saving gain and will not bring any other issues.

Proposal 12: Removing the LR based thresholds for all entry conditions for R19 RRM relaxation, serving cell RRM offloading and LP-WUS monitoring.

- Conclusion

In this contribution, we discuss the open issues for RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE. Based on the discussion, we have the following observations and proposals.

Value range of configured threshold (RRC-6)

Proposal 1: (RRC-6) The value range for the threshold on the measured RSRP/RSRP via LR could refer to the absolute measured values for MR. Details could be discussed in the running CR.

Exit condition for RRM measurement relaxation (RRC-7/38304-3)

Proposal 2: (RRC-7/38304-3) If LP-WUS monitoring condition is not merged with Rel-19 serving/neighboring cell RRM relaxation criteria, no need to define a separate exit condition other than ‘not fulfilling the entry condition’ for serving/neighboring cell RRM relaxation.

Merge LP-WUS monitoring condition and RRM relaxation condition/criteria (38304-4)

Proposal 3-1: (38304-4) Merge LP-WUS monitoring entry/exit condition and serving cell RRM offloading if only serving cell RRM offloading is supported in the camped cell or UE only supports serving cell RRM offloading.

Proposal 3-2: (38304-4) Merge LP-WUS monitoring entry/exit condition and Rel-19 serving/neighboring cell RRM relaxation if only Rel-19 serving/neighboring cell RRM relaxation is supported in the camped cell or UE only supports Rel-19 serving/neighboring cell RRM relaxation.

Proposal 3-3: (38304-4) It is up to network configuration to merge LP-WUS monitoring entry/exit condition with serving cell RRM offloading or Rel-19 serving/neighboring cell RRM relaxation if both are supported in the camped cell and UE supports both.

Analysis of the relationship between thresholds for LP-WUR(RRC-8)

Proposal 4: (RRC-8) The corresponding threshold(s) of entry condition for serving cell RRM offloading should be higher than the threshold(s) of entry condition for R19 RRM relaxation.

Proposal 5-1: (RRC-8) The threshold of the enter condition for serving cell RRM offloading should be higher than or equal to the threshold to stop neighboring cell RRM measurement, i.e. maximum of {SIntraSearchP, SnonIntraSearchP}.

Proposal 5-2: (RRC-8) The corresponding threshold of R19 neighboring cell RRM relaxation should be stricter than that of R16/17 neighboring cell RRM relaxation.

Proposal 6: (RRC-8) If merging LP-WUS monitoring and Rel-19 RRM relaxation is not agreed, the threshold of the entry condition for R19 RRM relaxation should be lower than or equal to the threshold of the entry condition for LP-WUS monitoring.

Proposal 7: (RRC-8) If merging LPWUS monitoring and serving cell RRM offloading is not agreed, the threshold of the entry condition for serving cell RRM offloading should be higher than or equal to the threshold of the entry condition for LP-WUS monitoring.

High priority frequency RRM relaxation (RRC-9/38304-7)

Proposal 8: (RRC-9/38304-7) RAN2 assumes:

- When the entry condition of serving cell RRM offloading is fulfilled, the measurement on neighboring cells with higher priority frequency could be stopped;

- When the entry condition of serving cell RRM relaxation is fulfilled, the measurement on neighboring cells with higher priority frequency could be relaxed.

Proposal 9: (RRC-9/38304-7) RAN2 assumes no network indication is needed to control whether to relax/stop the RRM measurement on neighboring cells with higher priority frequency. It can be revisited based on RAN4 progress.

Neighboring cell measurement relaxation criteria

Proposal 10: (RRC-10/38304-6) Low mobility condition is not needed for R19 RRM relaxation or offloading

Proposal 11: (38304-2) There is no impact on R16 criterion of neighboring cell RRM relaxation for the case when MR serving cell RRM measurement results are not available.

LR thresholds in entry condition

Proposal 12: Removing the LR based thresholds for all entry conditions for R19 RRM relaxation, serving cell RRM offloading and LP-WUS monitoring.

- References

- R2-2503765, Discussion summary and list of RRC open issue for LP-WUS WUR

- R2-2503658, Summary of [Post129bis][208][LPWUS] Running CR for 38.304 (CATT)

- 3GPP TR 38.869 v18.0.0

- R2-2503032 [AT129bis][202][LPWUS] Proposals for RRM measurement relaxation (vivo)