3GPP TSG-RAN WG2 Meeting #130		R2-2503617

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Discussion on inter-CU LTM

Agenda Item:	8.6.2

Document for:	Discussion and Decision

- Introduction

In the last meeting, the following agreements were made for inter-CU LTM [1]:

Agreements on inter-CU LTM:

- For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

- NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

- From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

- MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

- An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

- A list of sk-counters is linked to a Rel-19 ID configured by the SN.

- At RLF and reconfiguration with sync failure:

- If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

- if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

- How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration).

- The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message. We include this agreement in the LS to RAN3.

- Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

- Introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. Details will be further discussed during CR phase.

In this paper, we will discuss the remaining open issues for inter-CU LTM.

- Discussion

  - LTM fast recovery (RRC-8)

In [4], there is a open issue about if fast failure recovery with different Rel-19 IDs is allowed:

RRC-8

FFS if fast failure recovery with different Rel-19 IDs is allowed.

As for the LTM fast recovery, we have summarized all cases related to LTM fast recovery below:

- Case 1: If RLF: if the selected candidate cell has the same Rel-19 ID as the source cell;

- Case 2: If RLF: if the selected candidate cell has a different Rel-19 ID as the source cell;

- Case 3: If reconfiguration failure (intra-CU LTM): if the selected candidate cell has the same Rel-19 ID as the source cell;

- Case 4: If reconfiguration failure (intra-CU LTM): if the selected candidate cell has a different Rel-19 ID as the source cell;

- Case 5: If reconfiguration failure (inter-CU LTM): if the selected candidate cell has the same Rel-19 ID as the source cell;

- Case 6: If reconfiguration failure (inter-CU LTM): if the selected candidate cell has a different Rel-19 ID as the source cell;

- Case 7: If reconfiguration failure (L3 HO or CHO): if the selected candidate cell has the same Rel-19 ID as the source cell;

- Case 8: If reconfiguration failure (L3 HO or CHO without security key update): if the selected candidate cell has a different Rel-19 ID as source cell;

- Case 9: If reconfiguration failure (L3 HO or CHO with security key update): if the selected candidate cell has a different Rel-19 ID as source cell.

In the last meeting, the following agreements were made regarding LTM fast recovery for case 1 and case 6 above.

- At RLF and reconfiguration with sync failure:

- If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

- if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

RAN2 should further discuss the remaining cases to clarify UE behavior regarding LTM fast recovery.

Case 2 & case 4: inter-CU LTM fast recovery after RLF or intra-CU LTM failure

As agreed in the last meeting, NCC will only be included in the LTM cell switch command MAC CE if the R19 set ID is different between the target and source cell. Consequently, after RLF or intra-CU LTM failure, no NCC is available for key derivation at the UE, and recovery to a candidate cell that needs security key change cannot be supported.

Proposal 1: (RRC-8) After RLF or intra-CU LTM failure, if the selected candidate cell has a Rel-19 ID differs from the source cell, the UE cannot perform LTM fast recovery.

Case 3: intra-CU LTM fast recovery after intra-CU LTM failure

This case is the same as Rel-18 LTM and can be easily agreed.

Proposal 2: (RRC-8) After intra-CU LTM failure, if the selected candidate cell has the same Rel-19 ID as the source cell, the UE performs LTM fast recovery (same as in Rel-18).

Case 5: intra-CU LTM fast recovery after inter-CU LTM failure

In Rel-18, it was agreed that LTM fast recovery is not supported for T304 expiry after L3 HO and CHO:

RAN2#125bis Agreement

- LTM recovery is not supported for T304 expiry after HO (L3 HO) and CHO, but supported for T304 expiry after LTM cell switch (and RLF as earlier agreed). This agreement supersedes earlier agreements.

The motivation of the agreement is to avoid performing an intra-CU LTM fast recovery after an inter-CU handover. This is because, following an inter-CU handover, PDCP re-establishment and RLC re-establishment will be performed, making it difficult for a UE to revert back to the source cell configuration once buffers and state variables have been reset. Therefore, we believe that Rel-19 can follow Rel-18’s principle, i.e., LTM fast recovery targeting an intra-CU LTM candidate cell should not be supported for T304 expiry after inter-CU LTM cell switch.

Proposal 3: (RRC-8) After inter-CU LTM failure, if the selected candidate cell has the same Rel-19 ID as the source cell, the UE cannot perform LTM fast recovery.

Case 6: inter-CU LTM fast recovery after inter-CU LTM failure

In the last meeting, since key distribution approaches for gNBs were still under discussion in RAN3. Based on the fundamental assumption that the security key will be at least delivered to the target CU, RAN2 agreed that after inter-CU LTM failure, if the selected candidate cell has the same Rel-19 ID as the target cell, UE can perform LTM fast recovery. Whether UE can perform LTM fast recovery to a candidate cell with different Rel-19 ID as the target cell remained FFS.

According to the LS from RAN3 [2], security keys will be delivered to all the LTM candidate gNBs via the LTM Configuration Update Procedure, which guarantees that all the candidate gNBs maintain available security keys during the LTM fast recovery. Therefore, UE can also perform LTM fast recovery to a candidate cell with different Rel-19 ID with the target cell.

Based on case 5/6 analysis, we propose:

Proposal 4: (RRC-8) After inter-CU LTM failure, if the selected candidate cell has a Rel-19 ID differs from the source cell, the UE performs LTM fast recovery.

Case 7: intra-CU LTM fast recovery after L3 HO or CHO failure

Theoretically, there is no technical issues for supporting intra-CU LTM fast recovery after an L3 HO or CHO failure without security key update. However, since Rel-18 did not support intra-CU LTM fast recovery after any L3 HO/CHO failure (regardless of security key update status). we recommend applying the same principle to Case 7 for consistency.

Proposal 5: (RRC-8) After L3 HO or CHO failure, if the selected candidate cell has the same Rel-19 ID as the source cell, the UE cannot perform LTM fast recovery (same as in Rel-18).

Case 8: inter-CU LTM fast recovery after L3 HO or CHO failure  (without security key update)

Similar to case 4, there is no available NCC for key derivation at the UE after an L3 HO or CHO without security key update. Therefore, performing recovery to a candidate cell that needs security key change cannot be supported.

Proposal 6: (RRC-8) After L3 HO or CHO failure (without security key update), if the selected candidate cell has a different Rel-19 ID as the source cell, the UE cannot perform LTM fast recovery (same as the intra-CU LTM cell switch failure case).

Case 9: inter-CU LTM fast recovery after L3 HO or CHO failure  (with security key update)

This case is similar to case 6 for LTM fast recovery after inter-CU LTM cell switch failure, therefore, a similar agreement can be made.

Proposal 7: (RRC-8) After L3 HO or CHO failure (with security key update), if the selected candidate cell has a different Rel-19 ID as the source cell, the UE performs LTM fast recovery (same as the inter-CU LTM cell switch failure case).

  - appliedLTM-CandidateId included in both MCG and SCG RRCReconfigurationComplete message (RRC-2)

In [4], there is a left open issue about whether appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message for inter-CU SCG LTM.

RRC-2

Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message.

In our view, both the MCG and SCG RRCReconfigurationComplete message should include the appliedLTM-CandidateId.

The appliedLTM-CandidateId in the MN’s RRCReconfigurationComplete message allows the MN to identify the target SN so that the MN can forward the SN’s RRCReconfigurationComplete message to the target SN.

The appliedLTM-CandidateId in the SN’s RRCReconfigurationComplete message helps the SN determine which RRC configuration to apply when multiple RRC configurations are provided for the same candidate PSCell (e.g., when multiple LTM configuration exists for the same PSCell, or when both LTM and CPAC are configured for the same PSCell). This principle aligns with why we included the appliedLTM-CandidateId in the RRCReconfigurationComplete message in the standalone scenario.

Proposal 8: (RRC-2) appliedLTM-CandidateId is included in both MCG and SCG RRCReconfigurationComplete message.

  - CFRA resource in LTM Cell Switch Command MAC CE

To support the CFRA resource configuration included in the LTM cell switch command MAC CE for inter-CU LTM, two approaches can be considered:

Approach 1: Allocated by Target DU

- The CFRA resources are allocated by T-DU (Target Distributed Unit);

- Before sending an LTM Cell Switch Command MAC CE, S-DU (Source Distributed Unit) retrieves dedicated CFRA resources per UE from T-DU.

Approach 2: Allocated by Source DU

- During the handover preparation phase, S-DU obtains a shared CFRA resource pool from T-DU for multiple UEs within a single DU unit;

- S-DU then allocates specific CFRA resources to UEs requiring an LTM Cell Switch.

For approach 1 (CFRA resource allocated by Target DU), specific CFRA resources need to be reserved for each UE. This could lead to significant resource wastage when subsequent LTMs are supported since CFRAs would have to remain reserved over extended periods of time. Additionally, if CFRA resources are allocated by Target DU, there would be no practical difference between including them in an LTM Cell Switch Command MAC CE versus including them in preconfigured candidate RRC configurations. This goes against our original motivation behind including CFRA resources within an LTM Cell Switch Command MAC CE.

For approach 2 (CFRA resource allocated by source DU), CFRA resources are shared among UEs within a single DU, which is similar to the early-RACH resources sharing mechanism supported in Rel-18 LTM. The S-DU is responsible for allocating specific CFRA resources for each UE to perform RACH-based LTM cell switch while simultaneously avoiding the CFRA resource conflicts. As a result, the resource wastage can be largely reduced. Therefore, we prefer using Approach 2.

Proposal 9: For inter-CU LTM, a target cell specific CFRA resource pool could be provided by target gNB-DU to source gNB-DU,  and the CFRA resource are assigned by S-DU in the LTM cell switch command MAC CE.

Since the CFRA resources for RACH-based LTM are assigned to UE by source DU, the source DU needs to inform the target DU about the mapping relationship between the CFRA resource information (e.g., preamble index and RA-RNTI) and the UE to perform LTM. This ensures that the target cell knows which UE is performing RACH-based LTM via the CFRA resource used.

Observation 1: For inter-CU LTM, the target cell needs to identify which UE is performing CFRA-based RACH and which CFRA resources are being used when the CFRA resources are shared among multiple UEs.

Proposal 10: Send an LS to inform RAN3 to support the CFRA resources included in the LTM Cell Switch Command MAC CE assigned by S-DU for inter-CU LTM.

  - Coexistence of inter-CU LTM with MIMO 2TA

- Coexistence of inter-CU LTM with Rel-18 MIMO 2TA

In the last meeting, the following agreements were made for the co-existence of inter-CU LTM with Rel-18 MIMO 2TA,

- MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

However, whether RACH-based inter-CU LTM can coexist with MIMO 2TA has not been discussed.

As for RACH-based LTM and MIMO two TA, the same TAG mismatch issue exists as we discussed in Rel-18 [3]. Specifically, if a TCI state ID included in the LTM Cell Switch Command MAC CE is associated with TRP 1, while the TAG of TA in the RAR for RACH-based LTM is associated with TRP 2, after completing the LTM cell switch, UE shall use the TCI state in the LTM Cell Switch Command MAC CE for data transmission without having any available TA value associated to the TCI state.

Observation 2: Mismatch issue will occur when the TCI state ID indicated in the LTM Cell Switch Command MAC CE is associated with one TAG while the TA value in the RACH RAR is associated with another TAG, assuming the co-existence between RACH-based LTM and R19 MIMO two TA is supported.

In [3], four options were proposed to resolve the problem:

- Option 1: After the completion of LTM, the target DU initiates a PDCCH order triggered RACH procedure via the indicated TCI state in LTM Cell Switch Command. The RACH procedure is performed to obtain the TA value associated with the indicated TCI state.

- Option 2: The target DU sends a new TCI state in RACH Msg4. The new TCI state is associated with the TAG indicated in the RACH RAR.

- Option 3: If the mismatch issue occurs, after the completion of RACH procedure, UE follows the TCI state associated to the RACH procedure, i.e. the DL TCI state is associated to the SSB for RACH procedure and the UL TCI state is associated with the TCI state for Msg3 transmission. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

- Option 4: UE selects the SSB associated with the TAG ID same as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM.

Due to time constraints, it was agreed that network implementation (e.g., option 1) would handle this issue in Rel-18.

RAN2#127 discussion for RACH based LTM and 2 TAs

- RAN2 understand NW can handle this situation, e.g. by PDCCH order. No need of new option.

Actually, initiating a PDCCH order-triggered RACH procedure via the indicated TCI state after LTM cell switch completion would result in additional latency. To ensure the UE can perform data transmission in the target cell immediately, other options need to be considered.

Proposal 11: RAN2 selects from the following three options to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH-based LTM procedure are associated with different TAGs:

- Option 2: Target DU sends a new TCI state in RACH Msg 4.

- Option 3: If the mismatch issue occurs, UE follows the TCI state associated with the RACH-based LTM procedure. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

- Option 4: UE selects the SSB associated with the same TAG ID as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM procedure.

Proposal 12: If Option 3 in Proposal 11 is agreed, send an LS to RAN1 to inform the mismatch issue between the TCI state in LTM MAC CE and TA value in RAR and provide the corresponding solution.

- Coexistence of inter-CU LTM with Rel-19 MIMO 2TA

In Rel-19 MIMO 2TA, UL-only TRP is introduced. When a UL-only TRP is configured, the pathlossOffset for it should be indicated so that UE can perform uplink transmission via this TRP. In Rel-19 MIMO, this pathlossOffset for the UL-only TRP is indicated in the TCI-state of the serving cell and the network will indicate the TCI-state info for acquiring the pathlossOffset in the PDCCH-order.

However, if a UL-only TRP is configured for an LTM candidate cell, it remains unclear how the UE acquires the pathlossOffset to perform early RACH procedures for this TRP. To support PDCCH-order based TA measurement for such UL-only TRPs in LTM candidate cells, potential enhancements may be required.

Therefore, we propose to send an LS to ask RAN1 whether Rel-19 MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

Proposal 13: Send an LS to ask RAN1 whether Rel-19 MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- Conclusion

In this paper, we discuss the remaining open issues for inter-CU LTM. We have the following observations and proposals:

LTM fast recovery (RRC-8)

Proposal 1: (RRC-8) After RLF or intra-CU LTM failure, if the selected candidate cell has a Rel-19 ID differs from the source cell, the UE cannot perform LTM fast recovery.

Proposal 2: (RRC-8) After intra-CU LTM failure, if the selected candidate cell has the same Rel-19 ID as the source cell, the UE performs LTM fast recovery (same as in Rel-18).

Proposal 3: (RRC-8) After inter-CU LTM failure, if the selected candidate cell has the same Rel-19 ID as the source cell, the UE cannot perform LTM fast recovery.

Proposal 4: (RRC-8) After inter-CU LTM failure, if the selected candidate cell has a Rel-19 ID differs from the source cell, the UE performs LTM fast recovery.

Proposal 5: (RRC-8) After L3 HO or CHO failure, if the selected candidate cell has the same Rel-19 ID as the source cell, the UE cannot perform LTM fast recovery (same as in Rel-18).

Proposal 6: (RRC-8) After L3 HO or CHO failure (without security key update), if the selected candidate cell has a different Rel-19 ID as the source cell, the UE cannot perform LTM fast recovery (same as the intra-CU LTM cell switch failure case).

Proposal 7: (RRC-8) After L3 HO or CHO failure (with security key update), if the selected candidate cell has a different Rel-19 ID as the source cell, the UE performs LTM fast recovery (same as the inter-CU LTM cell switch failure case).

appliedLTM-CandidateId included in both MCG and SCG RRCReconfigurationComplete message (RRC-2)

Proposal 8: (RRC-2) appliedLTM-CandidateId is included in both MCG and SCG RRCReconfigurationComplete message.

CFRA resource in LTM Cell Switch Command MAC CE

Observation 1: For inter-CU LTM, the target cell needs to identify which UE is performing CFRA-based RACH and which CFRA resources are being used when the CFRA resources are shared among multiple UEs.

Proposal 9: For inter-CU LTM, a target cell specific CFRA resource pool could be provided by target gNB-DU to source gNB-DU,  and the CFRA resource are assigned by S-DU in the LTM cell switch command MAC CE.

Proposal 10: Send an LS to inform RAN3 to support the CFRA resources included in the LTM Cell Switch Command MAC CE assigned by S-DU for inter-CU LTM.

Coexistence of inter-CU LTM with MIMO 2TA

Observation 2: Mismatch issue will occur when the TCI state ID indicated in the LTM Cell Switch Command MAC CE is associated with one TAG while the TA value in the RACH RAR is associated with another TAG, assuming the co-existence between RACH-based LTM and R19 MIMO two TA is supported.

Proposal 11: RAN2 selects from the following three options to address the mismatch issue that the TCI state ID indicated in the LTM Cell Switch Command MAC CE and TA value in the RAR within the RACH-based LTM procedure are associated with different TAGs:

- Option 2: Target DU sends a new TCI state in RACH Msg 4.

- Option 3: If the mismatch issue occurs, UE follows the TCI state associated with the RACH-based LTM procedure. Otherwise, UE follows the indicated TCI state in the LTM cell switch command.

- Option 4: UE selects the SSB associated with the same TAG ID as the TAG ID associated with the indicated TCI state in LTM Cell Switch Command MAC CE during the RACH-based LTM procedure.

Proposal 12: If Option 3 in Proposal 11 is agreed, send an LS to RAN1 to inform the mismatch issue between the TCI state in LTM MAC CE and TA value in RAR and provide the corresponding solution.

Proposal 13: Send an LS to ask RAN1 whether Rel-19 MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- References

- RAN2#129bis_ChairNotes.

- R2-2503320/R3-252449, Reply LS for Reply LS on security handling for inter-CU LTM in non-DC cases, CATT.

- R2-2406439, Discussion on MAC open issue for LTM. vivo.

- R2-2504126, RRC open issues list for mobility, Ericsson.