3GPP TSG-RAN WG2 Meeting #130		R2-2503618

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:		Discussion on LTM measurement event evaluation and reporting

Agenda Item:		8.6.3

Document for:	Discussion and Decision

- Introduction

In RAN2 #129bis meeting[1], L1 event triggered measurement reporting was discussed and some agreements have been reached on many aspects.

In this contribution, we would like to discuss some remaining open issues, including:

- Open issue MAC-5: L1 MR MAC CE content for LTM2;

- Open issue MAC-2/15: how to cancel the triggering status of truncated L1 MR MAC CE;

- Open issue MAC-6: how to place the beams in truncated L1 MR MAC CE, e.g., in which order;

- Semi-persistent CSI-RS resource activation/deactivation and the MAC CE reuse;

- mTRP: suggest to have a simple agreement first, to select one beam from the two;

- Padding L1 MR MAC CE.

- Discussion

  - Open issue MAC-5 (L1 MR MAC CE for LTM2)

In [2], the following open issue is identified:

| MAC-5 | The content of L1 MR MAC CE triggered by LTM2 Editor’s NOTE: FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

According to RAN2 #129 bis meeting agreement:

- FFS For MR triggered by LTM2, whether only include the current beam information in the MR MAC CE or the MR can include measurements for LTM candidates.

It is not clear whether the candidate beams should be included in the LTM2 reporting. In fact, RAN1 has agreed in RAN1 #120bis that:

RAN1 #120bis agreement Agreement For the RS type determination for event triggered reporting with event LTM2, - Alt 1-2: At least one candidate RS shall be configured

But it is only for ‘RS type determination’, i.e., the candidate RS configuration is used to determine the RS type of serving beam.

Observation 1: RAN1 agreed at least one candidate RS shall be configured for LTM2, but it is for RS type determination.

Then let’s consider whether it is preferable to include candidate beams in LTM2 reporting. Basically, it is up to the network implementation whether to configure event LTM3/4/5 to have the UE reporting candidate beams, so it is not necessary to include measurements for LTM candidates in MR MAC CE triggered by LTM2.

Observation 2: Network can always configure LTM3/4/5 for the UE to report candidate beams.

On the other hand, if the L1 MR MAC CE can include candidate beams for LTM2, similar issues as we discussed for other events need to be discussed, e.g., whether to use truncated MAC CE if the intended candidate beams are not all included? Or, what candidate beams should be included in the L1 MR MAC CE triggered by LTM2, should the UE include the beams in triggered list of other events, or the UE should only include beams that are not reported by other LTM3/4/5 events? There would be a lot more questions to answer.

Observation 3: If we agree to include measurements for LTM candidates in L1 MR triggered by LTM2, some more questions need to be considered, e.g., whether to use truncated MAC CE if the intended candidate beams are not all included, what is the criteria to select candidate beams to be included in the L1 MR MAC CE etc.

Therefore, we propose to simplify the design that only the current beam information should be included in L1 MR triggered by LTM2.

Proposal 1: For L1 MR triggered by LTM2, only the current beam information is included in the L1 MR MAC CE.

However, according to Observation 1, the candidate RS will be configured. Therefore, we think it is better to have one more proposal, to clarify that the UE should ignore the candidate beam in the associated LTM CSI resource set.

Proposal 2: For reporting configuration of event LTM2, the UE ignores the candidate beam in the associated LTM CSI resource set (i.e., UE does not measure those candidate beams).

  - Open issue MAC-2/15 (Triggering status cancelling for Truncated L1 MR MAC CE)

In [2], the following open issues are identified:

| MAC-2 | Open issue MAC-2 (essential): How to ensure UE be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant |  |
| --- | --- | --- |
| MAC-15 | Whether the triggered MR should be cancelled in the below case: if all the triggered beam(s), i.e. Type#00 and Type#01, have been reported, no matter whether truncated MR MAC CE or regular MR MAC CE is used before, the trigger MR should be cancelled. [Rapp]: Raised by Apple during the discussion. | Issue Type: Essential How to address it: based on companies’ contribution |

These open issues are related to the triggering status cancelling of Truncated L1 MR MAC CE.

In RAN2 #129bis meeting, there is a general agreement:

- Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR.

In current MAC running CR[3], the cancelling condition for L1 MR is as follows:

2> if UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as befined in clause 6.1.3.xx; 3> increment the MR_SENT_COUNTER associated with the ltm-CSI-ReportConfigId for the RS triggered the report by 1; 3> stop the periodical reporting timer, if running; 3> if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount: 4> start the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig; 3> cancel the the triggered L1 measurement report; <omitted…> NOTE X: After sending a Truncated L1 measurement report MAC CE, if the subsequent UL grant is still not big enough to fit all the remaining beams, it is up to UE implementation to select the beam(s) which were not included in the previous MAC CE.

Observation 4: According to current MAC running CR, after sending only part of event-triggered beam(s), the triggering status of the corresponding event is not cancelled until the full L1 measurement report MAC CE is reported in following grant.

In our understanding, the current MAC running CR, with the newly added NOTE X, can cover all cases for truncated MAC CE. The UE’s behaviour is as follows:

Step 1: In the first truncated MAC CE, UE reports part of event-triggered beam(s);

Step 2: In the second following UL grant:

  - If it is large enough, the UE accommodates the normal L1 measurement report MAC CE plus its subheader, and cancel the report;

  - If it is not large enough, the UE selects the unreported triggered beams based on implementation to be included in the second truncated L1 MR MAC CE;

    - Step 3: In the third following UL grant, follow same behaviour as step 2a and step 2b.

Therefore, if the second UL grant is big enough, there is no problem, so we could rely on network implementation to provide a bigger grant after receiving truncated L1 MR MAC CE. The triggering status cancelling based on whether all triggered beams have been reported is kind of optimization and needs not to be introduced. Therefore, we would like to rely on the current running CR text.

Proposal 3: RAN2 agrees to only adopt the simple cancelling condition as in current MAC running CR, i.e.  the triggering status of the truncated L1 MR MAC CE is cancelled when the normal (not-truncated) L1 measurement report MAC CE is reported in following grant(s).

  - Open issue MAC-6 (Content of Truncated L1 MR MAC CE)

In [2], the following open issue is identified:

| MAC-6 | Truncated L1 MR MAC CE details. E.g. Except the triggered beam, whether any beam should be prioritised to be included in the truncated L1 MR MAC CE? In case the UL grant could include at least 2 beams in truncated L1 MR MAC CE, which beam should be included as the first beam in the truncated L1 MR MAC CE? Based on following agreement: - For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE. | Issue Type: Not essential How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

We would like to further discuss the left issues related to truncated MAC CE.

In RAN2 #129bis meeting, the following agreement related to truncated MAC CE for L1 MR was reached[1]:

- When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

Based on further discussions, the concept of ‘triggered beam’ was further clarified, as the beams are now categorized into four types according to agreements[1]:

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

Combining these agreements, we think it is better to further clarify and have a whole picture of how all the 4 types of beams should be placed in the truncated MAC CE for L1 MR. Meanwhile, as the serving beam should also be included when network configures the UE to do so according to the following agreement[1], it is also not clear whether/how to carry it in truncated MAC CE:

- The current beam information of SpCell (if configured to be included) is always placed at the end of the normal (not truncated) MAC CE.

Observation 5: It is not clear how the 4 types of candidates beams and the serving beam should be placed in the L1 MR truncated MAC CE (e.g., in which order).

In our understanding, the type-1 and type-2 beams are the most important ‘triggered beam’ which should be firstly reported to network, according to the previous agreement. After that, the beams in the triggered list, which belong to the type-3 beams, should be reported to network because it is better for the network to know all newest candidate beam information for decision to trigger LTM. The next should be the serving beam, because the serving beam which indicates the current beam quality also plays an important role in the decision of triggering LTM, while the type-4 beams are only unsatisfied beams which may not be used by the network at all. Therefore, we propose that the priority to place different types of beams should be clarified:

Proposal 4: RAN2 to agree the priority to include beams in L1 MR truncated MAC CE is as follows (highest priority listed first, FFS whether to capture it in spec):

- Type 1 or Type 2 beam;

- Type 3 beam;

- Current serving beam (if configured to be included);

- Type 4 beam (if configured to be included).

On top of Proposal 1, there is another case which should be considered, i.e., when there is only one byte left in the UL grant after accommodating some high priority beams. According to the previous agreement, only RSRP of current beam is needed in the L1 MR MAC CE[1]:

- When current beam is configured to be included in the L1 MR MAC CE, 7 bits of RSRP is included and no RSRI is needed.

While for the other type 1/2/3 beam, two bytes are needed to carry both RSRI and RSRP. To make the most use of the UL grant in truncated MAC CE case, we prefer to carry the current beam in the L1 MR truncated MAC CE when there is only one byte left in the UL grant after including e.g., triggered beams.

Proposal 5: When truncated L1 MR MAC CE is used, if only one octet is left in the UL grant and the network configures the UE to report current beam, the UE places the current beam at the end of truncated L1 MR MAC CE.

As for which beam should be included as the first beam in the truncated L1 MR MAC CE, we understand the same principle of non-truncated MAC CE should be reused, i.e., the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. In most cases, there may only be triggered beams (type-1/2 beams) in the truncated MAC CE, and the best triggered beam should be placed at the very beginning.

Proposal 6: For truncated L1 MR MAC CE, the best quality beam among the candidate beams included in truncated L1 MR MAC CE is taken as the reference beam as the first one.

  - Semi-persistent CSI-RS resource activation/deactivation

In RAN2 #129bis meeting, the following issue was discussed but without consensus. It was suggested to be revisited this meeting.

R2-2502799 Remaining Issues of L1 Event Triggered Measurement Report Samsung discussion Rel-19 NR_Mob_Ph4-Core Proposal 2: RAN2 decide How to handle the activated semi-persistent CSI-RS resources after LTM cell switch (i.e. support the Subsequent LTM) between following options. • Option 1: UE stop to measure the activated semi-persistent CSI-RS resources after cell switch • Option 2: UE continues to measure the activated semi-persistent CSI-RS resources after cell switch • Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e. other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch [LG, Nokia, OPPO]: Think option1 is sufficient. [CATT, MediaTek, Apple, IDC, Honor]: Think option2 is more reasonable. [ZTE]: We need to consider RAN3 impact. For option2, source should indicate to the target regarding the activated SP CSI-RS information. [Samsung]: Even with option1, the target may need to indicate the deactivation to all corresponding candidates. Probably similar impact on RAN3. In the functional point of view, option2 will be helpful and make decision this meeting will also help RAN3. [Honor]: With option1, it will also bring additional signalling overhead since it needs to send activation again for the same SP CSI-RS resources. - Will be revisited next meeting.

It seems that companies have different understanding for the options, regarding e.g., whether there is any RAN3 impact.

Basically, our understanding is that the semi-persistent CSI-RS resources are activated by the source serving cell, so as long as LTM cell switch is performed and the UE is moved to another new target cell, the measurement of activated semi-persistent CSI-RS resources should be stopped, otherwise, the source serving cell should inform new target cell which semi-persistent CSI-RS resources has been activated.

Observation 6: If the UE continues to measure the activated semi-persistent CSI-RS resources after cell switch, the source serving cell should inform target cell which semi-persistent CSI-RS resources of other candidate cells has been activated and it introduces RAN3 impact.

On the other hand, there are also arguments that if we go for option-1, i.e., UE stops to measure the activated semi-persistent CSI-RS resources after cell switch, the target cell may need to indicate the deactivation to all corresponding candidates. However, in current LTM signaling procedure, the target cell already needs to inform other candidate gNB(s) after LTM completion[4].

- 20. The new source gNB (i.e., the target gNB) sends the LTM CONFIGURATION UPDATE message to the candidate gNBs.

Therefore, once the candidate gNB(s) receives LTM CONFIGURATION UPDATE message, it will know that all semi-persistent CSI-RS resources of other candidate cells has been deactivated if we agree option-1. There is no new signaling introduced.

Observation 7: If the UE stops to measure the activated semi-persistent CSI-RS resources after cell switch, candidate gNB(s) will know that all semi-persistent CSI-RS resources of other candidate cells has been deactivated after receiving LTM CONFIGURATION UPDATE message.

Moreover, according to the reply LS from RAN3 to RAN1[5], RAN3 confirms it is feasible to specify the signaling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s):

RAN3 confirmed it is feasible to specify the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s), and RAN3 will continue to work on the details.

Based on the above observations, we understand option-1 or option-3 can work. The main difference is whether the UE should continue to measure the activated semi-persistent CSI-RS resources of the target cell.

According to RAN1 LS[6]:

Similarly to the semi-persistent CSI-RS resource supported for candidate cell L1-RSRP measurement for gNB scheduled reporting, RAN1 has also agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition.

The semi-persistent CSI-RS resource can be used for the purpose of CSI acquisition. Therefore, the measurement of the activated semi-persistent CSI-RS resources of the target cell should be continued to be performed.

Observation 8: RAN1 agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition.

Therefore, we think option-3 can be agreed to be supported. Upon LTM cell switch, the UE should stop the measurement of semi-persistent CSI-RS resources of those candidates. Meanwhile, the measurement of semi-persistent CSI-RS resources of target cell is going to be continued and it is used for early CSI acquisition.

Proposal 7: RAN2 to support option-3 to handle the activated semi-persistent CSI-RS resources after LTM cell switch:

- Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e., other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch.

Moreover, the RAN1 also indicated in the LS that[6]:

Agreement For candidate cell CSI acquisition - In addition to periodic CSI-RS resource, semi-persistent CSI-RS resource is supported - Support of semi-persistent CSI-RS resource is subject to separate UE capability. - MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2 As captured in the agreement, RAN1 sees the spec impact in RAN2 to define the MAC CE. Also, RAN1 foresees the RAN3 spec impact to enable this functionality for inter-DU/inter-CU case.

In current MAC running CR[3], the MAC CE to activate/deactivate the semi-persistent CSI-RS resource for gNB scheduled reporting (per description in RAN1’s LS) is as follows:

Figure 6.1.3.12a-1: SP CSI-RS Resource Set Activation/Deactivation for Candidate Cell MAC CE

To activate/deactivate the semi-persistent CSI-RS resource for CSI acquisition, we understand the same MAC CE can be reused. After all, no matter the semi-persistent CSI-RS resource is for gNB scheduled reporting or for CSI acquisition, the activation/deactivation behaviour are the same. Whether it is for which purpose, it can be further based on the reporting configuration, e.g., in [7], a new ltm-eCSI-ReportConfig is assumed to be defined to have a reportQuantity set to ‘cri-RI-PMI-CQI’.

Observation 9: Whether the semi-persistent CSI-RS resource is activated/deactivated for beam measurement reporting or for CSI acquisition, can be based on different reporting configuration.

Proposal 8: Reuse the MAC CE (SP CSI-RS Resource Set Activation/Deactivation for Candidate Cell MAC CE) to activate/deactivate the semi-persistent CSI-RS resource for CSI acquisition.

  - mTRP impact on L1 MR

in RAN2 #129bis meeting, the mTRP was discussed and the following agreement was reached:

- Will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

Although it is suggested to be revisited in August, we think it would be good to reach some initial/simple agreements to move one step further. When the source cell is configured with mTRP, the number of the indicated TCI states will be 2, regardless of sDCI-based mTRP and mDCI-based mTRP. Thus, the simplest approach is to select one of them as the serving cell beam. Based on this assumption, the spec impact is limited, and we could reuse most agreement for sTRP to be applied in mTRP.

Observation 10: The main difference between mTRP and sTRP is the number of the indicated TCI states of serving beam will be 2, and if we agree to select one of the two beams, most agreements for sTRP can be reused for mTRP.

There are different methods to select one from the two, but we can just make clear/simple rule to select 1 TCI state as the current beam, which is the first TCI state. The definition of first TCI state can just refer RAN1 specification to simplify the text in RAN2.

Observation 11: Definition of first indicated TCI can refer to TS 38.214.

Proposal 9: To support mTRP, RAN2 agrees to select one TCI state from the two (e.g., the first indicated TCI state as defined in TS 38.214), when serving cell is configured with mTRP.

  - Padding L1 measurement report MAC CE

Considering truncated measurement report MAC CE is already agreed to be supported, we could take one step further to consider whether padding measurement report MAC CE could also be supported.

In legacy BSR and SL-BSR discussion, the introduction of Padding BSR is driven by the following key considerations:

1/ Resource Utilization Optimization

When the uplink resources (UL Grant) allocated by the gNB are not fully utilized by the UE’s pending data, the remaining space would otherwise be filled with padding bits. Padding BSR enables the UE to utilize these padding bits to report buffer status, thereby minimizing resource waste and improving uplink efficiency.

2/ Reduced Signalling Overhead and Latency

Unlike Regular BSR, which requires explicit scheduling requests (SR) or dedicated grants, Padding BSR can be "piggybacked" on existing resource allocations. This eliminates additional signalling interactions, reducing latency and overhead, especially for scenarios with frequent buffer updates.

3/ Dynamic Buffer State Updates

While Regular and Periodic BSRs rely on specific triggers or timers, Padding BSR leverages padding space to dynamically report buffer status during any resource allocation. This allows the gNB to capture real-time data volume changes and optimize subsequent scheduling decisions.

4/ Support for Multi-Priority Logical Channels

When padding space is limited, Padding BSR employs Truncated BSR to prioritize reporting for the highest-priority LCG.

When it comes to L1 measurement reporting, similar reason could be considered, e.g., the L1 measurement results can update frequently, or the UL grant may not be able to accommodate all beam measurement results. Therefore, padding bits can be used to report beams such as:

- The updated beam measurement results that are already reported and do not satisfy the leaving condition

- The unsatisfied beam measurement results that has not been reported yet because of no triggering event is satisfied.

Therefore, we suggest that RAN2 to discuss whether padding L1 measurement report MAC CE should be supported. We prefer to support it considering the benefits it brings and the spec impact seems not much.

Proposal 10: Introduce padding L1 measurement report MAC CE

- Conclusion

In this contribution, we discussed a lot of MAC open issues for L1 LTM measurement event evaluation and reporting.

We have the following Observations and Proposals:

For LTM2:

Observation 1: RAN1 agreed at least one candidate RS shall be configured for LTM2, but it is for RS type determination.

Observation 2: Network can always configure LTM3/4/5 for the UE to report candidate beams.

Observation 3: If we agree to include measurements for LTM candidates in L1 MR triggered by LTM2, some more questions need to be considered, e.g., whether to use truncated MAC CE if the intended candidate beams are not all included, what is the criteria to select candidate beams to be included in the L1 MR MAC CE etc.

Proposal 1: For L1 MR triggered by LTM2, only the current beam information is included in the L1 MR MAC CE.

Proposal 2: For reporting configuration of event LTM2, the UE ignores the candidate beam in the associated LTM CSI resource set (i.e., UE does not measure those candidate beams).

For Triggering status cancelling for Truncated L1 MR MAC CE

Observation 4: According to current MAC running CR, after sending only part of event-triggered beam(s), the triggering status of the corresponding event is not cancelled until the full L1 measurement report MAC CE is reported in following grant.

Proposal 3: RAN2 agrees to only adopt the simple cancelling condition as in current MAC running CR, i.e.  the triggering status of the truncated L1 MR MAC CE is cancelled when the normal (not-truncated) L1 measurement report MAC CE is reported in following grant(s).

For Content of Truncated L1 MR MAC CE

Observation 5: It is not clear how the 4 types of candidates beams and the serving beam should be placed in the L1 MR truncated MAC CE (e.g., in which order).

Proposal 4: RAN2 to agree the priority to include beams in L1 MR truncated MAC CE is as follows (highest priority listed first, FFS whether to capture it in spec):

- Type 1 or Type 2 beam;

- Type 3 beam;

- Current serving beam (if configured to be included);

- Type 4 beam (if configured to be included).

Proposal 5: When truncated L1 MR MAC CE is used, if only one octet is left in the UL grant and the network configures the UE to report current beam, the UE places the current beam at the end of truncated L1 MR MAC CE.

Proposal 6: For truncated L1 MR MAC CE, the best quality beam among the candidate beams included in truncated L1 MR MAC CE is taken as the reference beam as the first one.

For Semi-persistent CSI-RS resource activation/deactivation

Observation 6: If the UE continues to measure the activated semi-persistent CSI-RS resources after cell switch, the source serving cell should inform target cell which semi-persistent CSI-RS resources of other candidate cells has been activated and it introduces RAN3 impact.

Observation 7: If the UE stops to measure the activated semi-persistent CSI-RS resources after cell switch, candidate gNB(s) will know that all semi-persistent CSI-RS resources of other candidate cells has been deactivated after receiving LTM CONFIGURATION UPDATE message.

Observation 8: RAN1 agreed to introduce semi-persistent CSI-RS resource for the purpose of CSI acquisition.

Observation 9: Whether the semi-persistent CSI-RS resource is activated/deactivated for beam measurement reporting or for CSI acquisition, can be based on different reporting configuration.

Proposal 7: RAN2 to support option-3 to handle the activated semi-persistent CSI-RS resources after LTM cell switch:

- Option 3: UE stop to measure the activated semi-persistent CSI-RS resources of the candidate cells (i.e., other than target cell) after cell switch. UE continues to measure the activated semi-persistent CSI-RS resources of the target cell after cell switch.

Proposal 8: Reuse the MAC CE (SP CSI-RS Resource Set Activation/Deactivation for Candidate Cell MAC CE) to activate/deactivate the semi-persistent CSI-RS resource for CSI acquisition.

For mTRP

Observation 10: The main difference between mTRP and sTRP is the number of the indicated TCI states of serving beam will be 2, and if we agree to select one of the two beams, most agreements for sTRP can be reused for mTRP.

Observation 11: Definition of first indicated TCI can refer to TS 38.214.

Proposal 9: To support mTRP, RAN2 agrees to select one TCI state from the two (e.g., the first indicated TCI state as defined in TS 38.214), when serving cell is configured with mTRP.

For Padding L1 measurement report MAC CE

Proposal 10: Introduce padding L1 measurement report MAC CE

- References

- RAN2 #129bis meeting chair notes;

- R2-253766, Discussion summary and list of MAC open issues for mobility Enhancements, vivo(rapporteur);

- R2-2503616_Running MAC CR for Mob Ph4_v19_Rapp2;

- R2-250xxxx_draftCR_running_38300-R19-Mob_v08_final_consolidated_rapp;

- R3-247911, Reply LS for LS on the support of semi-persistent CSI-RS resource for LTM candidate cells;

- R1-2503079, LS on the support of semi-persistent CSI-RS resource for LTM CSI acquisition for candidate cells;

- R1-2503492;