3GPP TSG-RAN WG2 Meeting #130	R2-2503623

St. Julian's, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Remaining issues on DSR enhancements for XR

Agenda Item:	8.7.4.2

Document for:	Discussion and Decision

- Introduction

In the email discussion of [POST129bis][504][XR] PDCP running CR and open issues (LGE) [1] and [POST129bis][505][XR] RLC running CR and open issues (vivo) [2], there is one related open issue respectively for delay-reporting data volume calculation:

PDCP-1: It is FFS which delay-reporting PDCP data volume shall consider PDCP Control PDUs, the PDCP SDUs to be retransmitted, and the PDCP Data PDUs to be retransmitted.

| RLC-5 | Editor’s Note: Same as PDCP open issue: it is FFS which delay-reporting RLC data volume shall consider RLC data PDUs to be retransmitted. Editor’s Note: It is FFS which delay-reporting RLC data volume shall consider STATUS PDU to be transmitted. | Issue Type: Essential How to address it: based on companies’ contribution |
| --- | --- | --- |

In the email discussion of [POST129bis][502][XR] MAC running CR and open issues(Qualcomm)[3], there is the following open issue:

[MAC-03] DSR cancelation in DC configuration

In the email discussion of [POST129bis][506][XR] UE capabilities CRs (Xiaomi)[4], there is the following open issue:

Open issue 1: the pre-requisite of enhanced delay status report

In the following, the above open issues will be discussed respectively.

- Discussion

  - Data volume of PDCP/RLC control PDU and retransmission data (PDCP-1/RLC-5)

In the chair notes of RAN2#129[6], there were the following issues on the data volume calculation with respect to the PDCP/RLC control PDUs and the retransmission data:

R2-2501348 Report of [AT129][504][XR] Data volume calculation for DSR LG Electronics discussion Rel-19 NR_XR_Ph3-Core - Noted Proposal: Choose one of the followings. Option 1. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about the issue of LCH only having the control PDU and retransmitted data at the next meeting. Option 2. Both PDCP and RLC consider Control PDU and retransmitted data into the shortest configured reporting threshold. Option 3. PDCP considers Control PDU and retransmitted data into the shortest reporting threshold included in the DSR, and discuss further about how RLC considers Control PDU and retransmitted data at the next meeting.

In RAN2#129bis, there were the following agreements regarding the remaining time determination and the data volume calculation of retransmission data:

- During DSR data volume calculation the remaining time of retransmitted is not considered, i.e. it is always put either in the smallest configured or smallest reported threshold. FFS which one.

For any LCH, the PDCP/RLC control PDU and retransmission data is prioritized over the other data for radio resource allocation in case of LCP procedure, the data volume calculation scheme of PDCP/RLC control PDU and retransmission data should ensure that with the reported data volume the gNB can allocate sufficient radio resources for the UE so that there is enough resource left for delay-reporting data transmission after radio resource allocation for PDCP/RLC control PDU and retransmission data. According to the above agreements, there are the following two alternatives:

- Alt.1. include PDCP/RLC control PDU and retransmission data into the data volume of the smallest configured threshold.

- Alt.2. include PDCP/RLC control PDU and retransmission data into the data volume of the smallest reported threshold.

The delay information generation comparison between the two alternatives are provided in the following table:

| Conditions | Comparison between Alt.1 and Alt.2 | comments |
| --- | --- | --- |
| Case 1. When there is other delay-reporting data for the smallest configured threshold in addition to PDCP/RLC control PDU and retransmission data for an LCG. | Completely the same DSR content between Alt.1 and Alt.2 | No difference between Alt.1 and Alt.2. |
| Case 2. When there is no other delay-reporting data for the smallest configured threshold for an LCG. | For Alt.1 there is one pair of remaining time and data volume for PDCP/RLC control PDU and retransmission data corresponding to the smallest configured threshold in addition to the delay information of the other thresholds for the delay-reporting data. RT0, DV0; %smallest configured threshold. RT1, DV1; %first reported threshold. …, RTN, DVN. %last reported threshold. For Alt.2, there are only pairs of remaining time and data volume in relation to reported thresholds RT1, DV1‘; %first reported threshold. …, RTN, DVN. %last reported threshold. While DV1‘ = DV1 + DV0. | 1.The DSR size for Alt.1 is 2 bytes which is larger than that for Alt.2. 2. With either Alt.1 or Alt.2, the gNB can allocate sufficient radio resource for the data correspondin to the smallest reported threshold based on the received DSR. 3. For Alt.1, there is additional standardization effort for remaining time determination of the first configured threshold for Case 2. |

Based on the above discussion, no matter whether the PDCP/RLC control PDU and retransmission data is counted into the smallest configured threshold or the smallest reported threshold, based on the DSR the gNB can allocate sufficient resources for the delay-reporting data. When the PDCP/RLC control PDU and retransmission data is counted into the smallest configured threshold, the DSR size may be two-byte larger and there is an extra issue to determine the remaining time for the first configured threshold when there is no delay-reporting data associated with the first configured threshold. From the perspective of reducing DSR size and the standardization effort, we propose:

- (PDCP-1/RLC-5) Both PDCP/RLC control PDUs and the retransmission data should be counted into the delay-reporting data volume of the smallest reported threshold included in the DSR.

Regarding the LCH with only PDCP/RLC control PDUs and retransmission data (i.e. without other delay-reporting data), this could happen when the LCH is out of data. But the LCG to which the LCH belongs should have at least one pair of remaining time and non-zero buffer size to be reported. With this assumption, it is preferred that these PDCP/RLC control PDUs and retransmission data are included in the data volume of the smallest report threshold of the LCG to which the LCH belongs.

- (PDCP-1/RLC-5) The PDCP/RLC control PDUs and retransmission data of an LCH without other delay-reporting data should be counted into the data volume of the shortest reported threshold of the corresponding LCG.

  - DSR cancelation in dual connectivity (MAC-03)

The related necessity and feasibility are discussed based on the following figure:

In case of DC, the UE has 3 TX buffers above MAC for one split DRB:

- PDCP buffer, which can be regarded as a shared buffer between two legs;

- RLC buffer of the first leg, which can be regarded as a dedicated upper layer buffer for the first leg;

- RLC buffer of the second leg, which can be regarded as a dedicated upper layer buffer for the second leg;

DSR of the first leg should be triggered and generated based on the data in the PDCP buffer and the RLC buffer of the first leg, while DSR of the second leg should be triggered and generated based on the data in the PDCP buffer and the RLC buffer of the second leg. If preprocessing is enabled, PDCP generates PDCP PDUs via preprocessing procedure for the two legs and deliver the PDCP PDUs to the two legs respectively as quick as possible.

- (MAC-03) For a LCG corresponding to split DRB in case of DC, the DSRs of both legs are generated based on the data in the common PDCP buffer and the data in the respective dedicated RLC buffers.

Considering the radio transmission charateristics, there can be respective instantaneous uplink data rate variations between the two legs due to different fading/interference variation and radio resource availability between the two legs, the RLC buffer size of the two legs varies can be very different. And for the same reason, the remaining time of the data in the RLC buffers of the two legs can be very different as well.

- (MAC-03) In case of DC, the RLC buffer size and the remaining time of the data in dedicated RLC buffer can be different between two legs due to differentiated channel quality variation /uplink resource allocation between the two legs.

Based on the above discussion and observations, it can be easily concluded that the content of DSR between two legs are different. The delay status of the first leg can not indicate the delay status of the second leg. Hence, we propose:

- (MAC-03) In case of DC, DSR cancellation of one leg does not trigger DSR cancellation of the other leg.

- (MAC-03) In case of DC, each MAC entity determines DSR cancellation independently based on the existing DSR cancellation conditions as defined in R18.

  - [Capability Open issue 1] DSR capability

The answer depends on whether the function of R18 DSR can be completely taken over by R19 DSR. In RAN2#129, there are the following agreements regarding enhanced DSR:

- Different LCGs may be configured with different number of reporting thresholds. - If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold.

According to the above agreements, a UE with enhanced DSR capability can be configured with respective number of reporting thresholds for different LCGs. For instance, one LCG of a UE with enhanced DSR capability, can be configured with single reporting threshold while another LCG of the same UE can be configured with multiple reporting thresholds. This means that the legacy DSR of single pair of remaining time/buffer size per LCG can well be covered by the enhanced DSR by configuring single configured reporting threshold for an LCG. In such sense, a UE with enhanced DSR capability has not to also support legacy DSR capability and whether a UE with enhanced DSR capability also supports legacy DSR capability should be left to UE implementation.

- (Capability Open issue 1) It is left to UE implementation whether a UE supporting enhanced DSR also supports legacy DSR.

- Conclusion

In this contribution, we have discussed remaining issues regarding DSR. Based on the discussions, there are the following observations:

Observation 1	(MAC-03) For a LCG corresponding to split DRB in case of DC, the DSRs of both legs are generated based on the data in the common PDCP buffer and the data in the respective dedicated RLC buffers.

Observation 2	(MAC-03) In case of DC, the RLC buffer size and the remaining time of the data in dedicated RLC buffer can be different between two legs due to differentiated channel quality variation /uplink resource allocation between the two legs.

Based on the above discussions and the observations, we have the following proposals:

Proposal 1	(PDCP-1/RLC-5) Both PDCP/RLC control PDUs and the retransmission data should be counted into the delay-reporting data volume of the smallest reported threshold included in the DSR.

Proposal 2	(PDCP-1/RLC-5) The PDCP/RLC control PDUs and retransmission data of an LCH without other delay-reporting data should be counted into the data volume of the shortest reported threshold of the corresponding LCG.

Proposal 3	(MAC-03) In case of DC, DSR cancellation of one leg does not trigger DSR cancellation of the other leg.

Proposal 4	(MAC-03) In case of DC, each MAC entity determines DSR cancellation independently based on the existing DSR cancellation conditions as defined in R18.

Proposal 5	(Capability Open issue 1) It is left to UE implementation whether a UE supporting enhanced DSR also supports legacy DSR.

- References

- [POST129bis][504][XR] PDCP running CR and open issues (LGE)

- [POST129bis][505][XR] RLC running CR and open issues (vivo)

- [POST129bis][502][XR] MAC running CR and open issues (Qualcomm)

- [POST129bis][506][XR] UE capabilities CRs (Xiaomi)

- RAN2-129bis, Chair notes

- RAN2-129, Chair notes