3GPP TSG-RAN WG2 Meeting #130		R2-2503624

St. Julian’s, Malta, 19th – 23rd May 2025

Source:	vivo

Title:	Discussion on RLC remaining issues for XR

Agenda Item:	8.7.5

Document for:	Discussion and Decision

- Introduction

In RAN2#129bis, the following agreements were made for RLC enhancement [1]:

- When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review) - For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission - Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers. - Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

In this paper, we will discuss the remaining open issues for the RLC enhancement of XR.

- Discussion

  - Enhancements to ensure timely RLC retransmissions

    - Behaviour when pdu-SetDiscard  is configured (PDCP-2)

In [2], there is an open issue about which PDCP SDU the transmitting PDCP entity shall trigger RLC autonomous retransmission and polling, if pdu-SetDiscard is configured.

Issue 2: It is FFS for which PDCP SDU the UE shall trigger remaining-time-based RLC retransmission and polling, if pdu-SetDiscard is configured.

In our view, the transmitting RLC entity should trigger autonomous retransmission and polling independently for each PDCP SDU based on its configured threshold, regardless of whether pdu-SetDiscard is configured.

If all PDUs in the PDU set simultaneously trigger autonomous retransmission just because the remaining time of one PDU of them falls below the configured threshold, it could lead to lots of unnecessary retransmissions for the left PDUs in the same PDU set. This may lead to huge radio resource wastage since the left PDUs whose remaining time has not reached the threshold might be successfully transmitted before their remaining time falls below the threshold.

Moreover, since each PDCP SDU with remaining time below the threshold will trigger autonomous retransmission and polling, every PDU in the PDU set already has an opportunity to do so. Therefore, there is no motivation to early trigger autonomous retransmission and polling for all PDUs in the PDU set.

Proposal 1: (PDCP-2) The transmitting RLC entity shall trigger RLC autonomous retransmission and polling for each PDCP SDU independently based on the configured threshold, regardless of whether PDU-setDiscard is configured.

    - Coexistence of autonomous retransmission and ARQ (RLC-7)

In [3], the following open issue is identified for autonomous retransmission:

| RLC-7 | Editor’s Note: It is still open how Autonomous Retransmission coexists with ARQ procedures, i.e. whether/how to increment the RETX_COUNT for Autonomous Retransmission. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |

As for the REXT_COUNT, since only one instance of autonomous retransmission may be triggered per RLC SDU, increasing RETX_COUNT based on autonomous retransmission is not a big problem for the RLF detection. Therefore, we propose to increase the RETX_COUNT by 1 when performing autonomous retransmission.

Proposal 2: (RLC-7) Autonomous retransmission will increase RETX_COUNT by 1, the same as legacy ARQ procedures.

    - Polling enhancement (RLC-8/RLC-9)

In [3], the following open issue is identified for polling enhancement:

| RLC-8 | Editor’s Note: It is still open on how to avoid excessive polling for the polling enhancement, e.g. only one polling or multiple. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |
| RLC-9 | Editor’s Note: FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission due to polling enhancement. | Issue Type: Not essential but important How to address it: based on companies’ contribution |

Discussion on RLC-8

In RAN2#129, it was agreed that a single autonomous retransmission would be triggered per RLC SDU. Similar to autonomous retransmission, polling should also be limited to one trigger per RLC SDU. On one hand, there may not be sufficient time left to trigger multiple pollings for a delay-critical RLC SDU. On the other hand, if multiple polling were allowed, mechanisms such as a prohibit timer would need to be introduced to avoid excessive triggering of polls, which would add unnecessary complexity without providing significant benefits.

Proposal 3: (RLC-8) Similar to autonomous retransmission, polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold.

Discussion on RLC-9

In the last meeting, it was agreed that autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers.

- Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers.

Whether additional conditions are needed for triggering polling enhancement still needs to be discussed. According to the current RLC specification, when polling is received, the Rx side of the AM RLC entity will trigger a STATUS report under the following conditions:

Triggers to initiate STATUS reporting include:

-	Polling from its peer AM RLC entity:

-	When an AMD PDU with SN = x and the P field set to "1" is received from lower layer, the receiving side of an AM RLC entity shall:

-	if the AMD PDU is to be discarded as specified in clause 5.2.3.2.2; or

-	if x < RX_Highest_Status or x >= RX_Next + AM_Window_Size:

-	trigger a STATUS report.

-	else:

-	delay triggering the STATUS report until x < RX_Highest_Status or x >= RX_Next + AM_Window_Size.

This means that the STATUA report will only be triggered when the Rx side knows the reception status of the AMD PDU. Additionally, a triggered STATUS report will only be submitted to lower layer if t-StatusProhibit is not running. This also ensures that the STATUS report is not sent to the peer AM RLC entity immediately after receiving a polling indication.

Therefore, with these mechanisms, too early and/or unnecessary retransmission will not happen due to polling enhancement. Additional conditions for polling enhancement are not needed.

Proposal 4: (RLC-9) No additional conditions are needed for the polling enhancement.

    - Coexistence of autonomous retransmission and polling enhancement

When both polling enhancement and autonomous retransmission are configured for the same Tx RLC entity, under reasonable network configuration, the threshold for enabling autonomous retransmission is expected to be set lower than that for enabling polling enhancement. Otherwise, the configuration for enabling polling enhancement will become useless since autonomous retransmission would have already been triggered before the UE polls the Rx RLC entity to request the STATUS report.

Additionally, unnecessary autonomous retransmission can reduce overall system capacity. Therefore, it should only be utilized when the RLC SDU(s) are particularly delay-critical. For RLC SDU(s) that are not delay-critical, polling enhancement should be preferred to avoid resource wastage caused by unnecessary autonomous retransmission.

Consequently, when both autonomous retransmission and polling enhancement are configured by the network, the threshold for enabling autonomous retransmission should be lower than the threshold for enabling polling enhancement (refer to Figure 4 below).

Figure 4. Different thresholds for enabling polling enhancement and autonomous retransmission

Proposal 5: The threshold for enabling autonomous retransmission is expected to be lower than the threshold for enabling polling enhancement when both are configured to the UE for the same RLC entity.

    - Priority of autonomous retransmission

According to the current RLC specification, AMD PDUs for retransmission are prioritized over AMD PDUs for initial transmission.

TS 38.322

5.2.3.1	Transmit operations

5.2.3.1.1	General

The transmitting side of an AM RLC entity shall prioritize transmission of RLC control PDUs over AMD PDUs. The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of AMD PDUs containing not previously transmitted RLC SDUs or RLC SDU segments.

When autonomous retransmission is introduced, the priority between AMD PDU(s) for autonomous retransmission and AMD PDU(s) for ARQ-based retransmission (triggered by STATUS report) needs to be discussed. Autonomous retransmission is triggered when the remaining time is below a threshold, indicating that AMD PDU(s) for autonomous retransmission are more delay-critical than those for ARQ-based retransmission. Therefore, AMD PDU(s) for autonomous retransmission are expected to have higher priority and should be transmitted first to ensure timely retransmission of delay-critical data.

Proposal 6: The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs for autonomous retransmission over AMD PDUs for ARQ-based retransmission.

  - Enhancements to avoid unnecessary retransmissions

    - UE capability for avoiding unnecessary retransmissions (Capability Open issue 2)

In [4], an open issue about whether to have separate or joint UE capability for UE Tx and Rx operations for unnecessary retransmission avoidance is proposed:

Company contributions can focus on following open issues:

- Open issue 1: the pre-requisite of enhanced delay status report

- Open issue 2: whether to have separate or joint UE capability for UE Tx and Rx operations for unnecessary retransmission avoidance.

- Open issue 3: whether RAN2 to define the UE capability for UAI for a ratio of gap occasions.

In our view, separate UE capabilities should be introduced for transmitter and receiver enhancements. For DL, UE only needs to support the receiver enhancements. For UL, UE only needs to support the transmitter enhancements.

Proposal 7: (Capability Open issue 2) Define separate UE capabilities for transmitter and receiver enhancements to avoid unnecessary retransmissions.

    - RLC-6/RLC-10/RLC-11

In [3], the following open issue is identified for avoiding unnecessary RLC retransmission:

| RLC-6 | Editor’s Note: FFS whether there are any RLF detection impacts when avoiding unnecessary retransmissions is introduced. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |
| RLC-10 | Whether/How to introduce any limitations or associations for the configuration of stopReTxObsoleteSDU and DL t-RxDiscard. - As raised by ZTE below | Issue Type: Not essential not important How to address it: based on companies’ contribution |
| RLC-11 | FFS how to handle the case “ if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with” - As raised by Nokia and Apple below | Issue Type: Not essential but important? How to address it: based on companies’ contribution |

Discussion on RLC-6

According to the current RLC and RRC specification, RLF will be triggered if the retransmission count of an RLC SDU reaches maxRetxThreshold. The purpose of this mechanism is to enable the UE to trigger RLF and initiate RRC re-establishment when it determines that uplink transmission is unavailable/unreliable.

However, when stopReTxObsoleteSDU (the feature of avoiding unnecessary retransmission) is configured to the Tx RLC entity, maxRetxThreshold might never be reached. This is because the UE will stop retransmitting the corresponding RLC SDU based on the discard indication from PDCP before reaching the maxRetxThreshold. To ensure that RLF can still be triggered in cases where uplink transmission becomes unavailable/unreliable, a new approach for triggering RLF needs to be introduced. For example, RLF could be triggered when:

- The RLC entity receives N consecutive SDU discard indications from the PDCP entity; or

- The RLC entity receives a total of N SDU discard indications from the PDCP entity within a configured time window.

Proposal 8: (RLC-6) RAN2 should discuss the following approaches for triggering RLF when avoiding unnecessary retransmission is enabled:

- Approach 1: Triggering RLF when the RLC entity receives consecutive N SDU discard indications from the PDCP entity.

- Approach 2: Triggering RLF when the RLC entity receives a total of N SDU discard indications from the PDCP entity within the configured time window.

Discussion on RLC-10

In the RLC open issue discussion, ZTE has proposed that “if DL stop ReTx Obsolete SDU is supported, the DL t-RxDiscard should be configured; and if UL stopReTxObsoleteSDU is configured, UL timer based Rx discard should be activated in NW”, this is technoically right, but we think this could be ensured by the network implementation and no spec impact is needed.

Proposal 9: (RLC-10) The network implementation should ensure:

- If DL stop retransmission obsolete SDU is supported, the DL t-RxDiscard should be configured;

- If UL stopReTxObsoleteSDU is configured, timer based Rx discard should be activated in the network .

Discussion on RLC-11

According to the current RLC running CR [X], when t-PollRetransmit expires, if there are only SDUs buffered whose transmissions have been stopped due to discard indication from PDCP, there is no SDU to retransmit the poll with:

TS 38.322

5.3.3.4	Expiry of t-PollRetransmit

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1); or

-	if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling):

-	consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission (excluding the RLC SDUs for which discard indication has been received from upper layers when stopReTxObsoleteSDU is configured); or

-	consider any RLC SDU which has not been positively acknowledged for retransmission (excluding the RLC SDUs for which discard indication has been received from upper layers when stopReTxObsoleteSDU is configured).

-	include a poll in an AMD PDU as described in clause 5.3.3.2.

For this case, an easy solution is to ensure that polling will only be triggered if there is an AMD PDU that can be transmitted when t-PollRetransmit expires.

Proposal 10: (RLC-11) polling will only be triggered if there is an AMD PDU that can be transmitted when t-PollRetransmit expires.

The corresponding TP is provided in Annex.

Proposal 11: (RLC-11) If P8 is agreeable, RAN2 to adopt the TP provided in Annex.

    - Other remaining issues

Indication is sent from Tx to Rx when the Tx decides the outdated SDU(s)

In RAN2#128, the Tx to Rx indicator for the combined approach was discussed, and the following agreements were made.

RAN2#128 Agreement

- There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer

- Unless critical issue is identified, no Tx to Rx indication will be introduced

In our view, the Tx to Rx indication is critical for keeping the timely synchronization between the Tx window and the Rx window. Without the indication, stalling of the Tx and Rx window may occur in some situations.

For example, when PDU set discard is enabled, all PDUs in the entire PDU set will be discarded if any one of the PDUs expires. At this time, the Tx RLC entity may have already stopped (re)transmission for amounts of obsolete RLC SDUs but has not received the STATUS report from the Rx side for a long time. This is because the local timer on the Rx side has not started due to no gap detected (refer to Figure 3 below). In this scenario, both the Tx window and the Rx window may stall and delay the transmission of the subsequent packets. With the indication from Tx to Rx, we can avoid this window stalling.

Figure 3. Window stalling due to without Tx to Rx indicator

Observation 1: Without Tx to Rx indication, stalling of the Tx and Rx window may occur, particularly in cases where PDU set discard is enabled.

Additionally, when the Tx side decides to stop (re)transmission of the RLC SDUs, the Rx side may still send STATUS reports with NACK for the corresponding RLC SDUs before the local timer on the Rx side expires. In practice, the Tx side will ignore these NACK indications for those RLC SDUs and will not perform (re)transmission. Therefore, these STATUS reports waste radio resources and should be avoided. With the Tx to Rx indication, the Rx can promptly identify the outdated SDUs based on the indication and send STATUS report with positive ACK without waiting for the local timer on the Rx side to expire.

Observation 2: Without the Tx to Rx indication, unnecessary STATUS reports with NACK indication for obsolete RLC SDUs will be sent to the Tx side, leading to resource wastage.

Proposal 12: Indication about obsolete SDUs from Tx side to Rx side is needed for the combined approach.

As for the format of the indication from the Tx to the Rx side, we believe a simple RLC control PDU carrying the sets of SNs or SN range of the outdated RLC SDUs is enough.

Proposal 13: The indication from the Tx side to the Rx side should include the sets of SNs or the SN range associated with the outdated RLC SDUs.

Rx RLC indicates Rx PDCP when t-RxDiscard expires

When t-RxDiscard (the new RLC timer introduced to determine obsolete RLC SDUs) expires, the RLC entity stops waiting to receive the obsolete RLC SDUs and the corresponding RLC SDUs will be positively acknowledged in the STATUS report. However, since this is a fake ACK, the transmitting PDCP entity will consider these RLC SDUs as successfully received based on the STATUS report, while the receiving PDCP entity will still be waiting to receive the RLC SDUs from the RLC entity until the t-reordering timer expires. In this case, HFN asynchronization between the transmitting PDCP entity and the receiving PDCP entity may occur. For example, the transmitting PDCP entity may consider the current HFN for a PDCP SDU to be X +1 based on the fake ACK from the STATUS report, while the receiving PDCP entity may consider it to be X. This problem has also been discussed in Nokia’s contribution [2].

Observation 3: HFN asynchronization between the transmitting PDCP entity and receiving PDCP entity may occur when the feature of avoiding unnecessary retransmission is configured.

To solve this problem, a simple approach is to make the receiving RLC entity indicate the SN of obsolete RLC SDUs when the t-RxDiscard expires, so that the receiving PDCP entity can update its state variable to maintain HFN synchronization with the transmitting PDCP entity.

Proposal 14: The receiving RLC entity should indicate to the receiving PDCP entity the SN of obsolete RLC SDUs when t-RxDiscard (the new Rx RLC timer to determine obsolete RLC SDUs) expires.

- Conclusion

In this contribution, we discuss the two directions of RLC AM enhancements, and we have the following observations and proposals:

Enhancements to ensure timely RLC retransmissions

Proposal 1: (PDCP-2) The transmitting RLC entity shall trigger RLC autonomous retransmission and polling for each PDCP SDU independently based on the configured threshold, regardless of whether PDU-setDiscard is configured.

Proposal 2: (RLC-7) Autonomous retransmission will increase RETX_COUNT by 1, the same as legacy ARQ procedures.

Proposal 3: (RLC-8) Similar to autonomous retransmission, polling shall only be triggered once per RLC SDU when its remaining time falls below a specified threshold.

Proposal 4: (RLC-9) No additional conditions are needed for the polling enhancement.

Proposal 5: The threshold for enabling autonomous retransmission is expected to be lower than the threshold for enabling polling enhancement when both are configured to the UE for the same RLC entity.

Proposal 6: The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs for autonomous retransmission over AMD PDUs for ARQ-based retransmission.

Enhancements to avoid unnecessary retransmissions

Observation 1: Without Tx to Rx indication, stalling of the Tx and Rx window may occur, particularly in cases where PDU set discard is enabled.

Observation 2: Without the Tx to Rx indication, unnecessary STATUS reports with NACK indication for obsolete RLC SDUs will be sent to the Tx side, leading to resource wastage.

Observation 3: HFN asynchronization between the transmitting PDCP entity and receiving PDCP entity may occur when the feature of avoiding unnecessary retransmission is configured.

Proposal 7: (Capability Open issue 2) Define separate UE capabilities for transmitter and receiver enhancements to avoid unnecessary retransmissions.

Proposal 8: (RLC-6) RAN2 should discuss the following approaches for triggering RLF when avoiding unnecessary retransmission is enabled:

- Approach 1: Triggering RLF when the RLC entity receives consecutive N SDU discard indications from the PDCP entity.

- Approach 2: Triggering RLF when the RLC entity receives a total of N SDU discard indications from the PDCP entity within the configured time window.

Proposal 9: (RLC-10) The network implementation should ensure:

- If DL stop retransmission obsolete SDU is supported, the DL t-RxDiscard should be configured;

- If UL stopReTxObsoleteSDU is configured, timer based Rx discard should be activated in the network .

Proposal 10: (RLC-11) polling will only be triggered if there is an AMD PDU that can be transmitted when t-PollRetransmit expires.

Proposal 11: (RLC-11) If P8 is agreeable, RAN2 to adopt the TP provided in Annex.

Proposal 12: Indication about obsolete SDUs from Tx side to Rx side is needed for the combined approach.

Proposal 13: The indication from the Tx side to the Rx side should include the sets of SNs or the SN range associated with the outdated RLC SDUs.

Proposal 14: The receiving RLC entity should indicate to the receiving PDCP entity the SN of obsolete RLC SDUs when t-RxDiscard (the new Rx RLC timer to determine obsolete RLC SDUs) expires.

- References

- R2_129bis_ChairNotes.

- R2-2503697, Discussion of [POST129bis][504][XR] PDCP running CR, LG Electronics.

- R2-2503767, Discussion summary and list of RLC open issue for R19 XR, vivo.

- R2-2503438, Open issues of Rel-19 XR UE capabilities, Xiaomi

Annex: TP for TS 38.322

5.3.3.4	Expiry of t-PollRetransmit

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements and excluding RLC SDUs or RLC SDU segments for which the transmission and retransmission are stopped as specified in clause 5.2.3.1.1); or

-	if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling):

-	consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission (excluding the RLC SDUs for which discard indication has been received from upper layers when stopReTxObsoleteSDU is configured); or

-	consider any RLC SDU which has not been positively acknowledged for retransmission (excluding the RLC SDUs for which discard indication has been received from upper layers when stopReTxObsoleteSDU is configured).

-	include a poll in an AMD PDU, if any, as described in clause 5.3.3.2.