3GPP TSG-RAN WG2 Meeting #130	R2-2503631

# St Julian’s, Malta, 19 – 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.320 | CR | 0138 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Immediate MDT measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Immediate MDT measurements and reporting triggers are specified for RRC Connected UE state in three subclauses: 5.2.1, 5.3.1. 5.4.1 for E-UTRAN, UTRAN and NR, respectively. Each of these sections, introduces the list of measurements: M1-Mx that “shall be supported for Immediate MDT performance”. The mandatory requirement (indicated by “shall”) remains in contradiction with optional support of the metrics. For some of the UE measurements (e.g. UL PDCP), the mandatory support is conflicting with their optional support and UE Capabilities sub-clause (5.1.4). For RAN, the performance of measurements is determined by MDT Configuration (TS36.413, 9.2.1.81 and TS38.413, 9.3.1.169), which can be also optionally utilized as subsequent step following Trace Activation procedure. The discrepancy and activation for UTRAN can be considered as reliant on stage 3 specification. However, for E-UTRAN and NR, it is important to note that ongoing RAN3 WI NR_AIML_NGRAN_enh-Core disscuss solutions that depend on the outlined sections. Thus, it is essential to clarify the conflicting and requirements to prevent ambiguities concerning ongoing 3GPP Work Items. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Immediate MDT measurements performance in E-UTRAN and NR is changed from “shall” to “may” Impact analysis Impacted 5G architecture option: NR SA, (NG)EN-DC, NE-DC. Impacted functionality: MDT Inter-operability: - Implementation of this CR by a Release 17 UE or RAN will not cause compatibility issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned requirments on Immediate MDT measurements performance remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.1, 5.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

#### 5.2.1.1	Measurements and reporting triggers for Immediate MDT

Measurements to be performed for Immediate MDT purposes involve reporting triggers and criteria utilized for RRM. An MDT specific UE-based measurement for UL PDCP delay is applied for QoS verification purpose. In addition, there are measurements performed in eNB.

In particular, the following measurements may be supported for Immediate MDT performance:

Measurements:

-	M1: RSRP, RSRQ and SINR measurement by UE, see TS 36.214 [9].

-	M2: Power Headroom measurement by UE, see TS 36.213 [11].

-	M3: Received Interference Power measurement by eNB, see TS 36.214 [9]. This is a cell measurement. One sample is logged each measurement collection period, where one sample corresponds to a measurement period as specified in TS 36.133 [3].

-	M4: Data Volume measurement separately for DL and UL, per QCI per UE, by eNB, see TS 36.314 [13].

-	M5: Scheduled IP Throughput for MDT measurement separately for DL and UL, per RAB per UE and per UE for the DL, per UE for the UL, by eNB, see TS 36.314 [13]. QCI values of the RABs that have contributed to a measurement value are logged with the measurement values.

-	M6: Packet Delay measurement, separately for DL and UL, per QCI per UE, see UL PDCP Delay, by the UE, and Packet Delay in the DL per QCI, by the eNB, TS 36.314 [13].

NOTE 1:	If the UE does not detect any UL PDCP delay based on the delay threshold and delay report interval configured by the network, the UE does not report any UL PDCP delay measurement within that period.

NOTE 2:	A UE in EN-DC mode of operation can be configured with UL PDCP Packet Average Delay (ul-DelayValueConfig), if UE is capable of performing the UL average PDCP queueing delay.

-	M7: Packet Loss rate measurement, separately for DL and UL per QCI per UE, by the eNB, see Packet Loss rate in the UL and Packet Uu Loss rate in the DL TS 36.314 [13].

-	M8: RSSI measurement by UE, see TS 36.331 [5].

-	M9: RTT measurement by UE, see TS 36.331 [5].

Measurement collection triggers:

-	For M1:

-	Event-triggered measurement reports according to existing RRM configuration for events A1, A2, A3, A4, A5 A6, B1 or B2

-	Periodic, A2 event-triggered, or A2 event triggered periodic measurement report according to MDT specific measurement configuration.

-	For M2:

-	Reception of Power Headroom Report (PHR) according to existing RRM configuration.

NOTE 3:	PHR is carried by MAC signalling. Thus, the existing mechanism of PHR transmission applies, see TS 36.321 [10].

-	For M3:

-	End of measurement collection period

-	For M4:

-	End of measurement collection period.

-	For M5:

-	End of measurement collection period.

-	For M6:

-	End of measurement collection period.

-	For M7:

-	End of measurement collection period.

-	For M8:

-	Associated to M1 and/or M6 related measurement reporting triggers.

-	For M9:

-	Associated to M1 and/or M6 related measurement reporting triggers.

Next Modified Subclause

#### 5.4.1.1	Measurements and reporting triggers for Immediate MDT

Measurements to be performed for Immediate MDT purposes involve reporting triggers and criteria utilized for RRM. In addition, there are associated network performance measurements performed in the gNB.

In particular, the following measurements may be supported for Immediate MDT performance:

Measurements:

Measurements:

⁻	M1: DL signal quantities measurement results for the serving cell and for intra-frequency/Inter-frequency/inter-RAT neighbour cells, including cell/beam level measurement for NR cells only, TS 38.215 [19].

⁻	M2: Power Headroom measurement by UE, TS 38.213 [20].

⁻	M3: Void.

⁻	M4: PDCP SDU Data Volume measurement separately for DL and UL, per DRB per UE, see TS 28.552 [17].

⁻	M5: Average UE throughput measurement separately for DL and UL, per DRB per UE and per UE for the DL, per DRB per UE and per UE for the UL, by gNB, see TS 28.552 [17].

⁻	M6: Packet Delay measurement separately for DL and UL, per DRB per UE, TS 28.552 [17] and TS 38.314 [18].

NOTE 0:	UL PDCP Excess Packet Delay measurement can be configured with a threshold as specified in TS 38.331 [15].

⁻	M7: Packet loss rate measurement separately for DL and UL, per DRB per UE, TS 28.552 [17] and TS 38.314 [18].

⁻	M8: RSSI measurement by UE (for WLAN/Bluetooth measurement) see TS 38.331 [15].

⁻	M9: RTT Measurement by UE (for WLAN measurement) see TS 38.331 [15].

NOTE 1:	Void

NOTE 1a:	M5 ~ M7 can apply to MR-DC and EN-DC SN terminated MCG/split bearers and MN terminated SCG/split bearers.

Measurement collection triggers:

-	For M1:

-	Event-triggered measurement reports according to existing RRM configuration for events A1, A2, A3, A4, A5, A6, B1 or B2.

-	Periodic, A2 event-triggered, or A2 event triggered periodic measurement report according to MDT specific measurement configuration.

-	For M2:

-	Reception of Power Headroom Report (PHR) according to existing RRM configuration.

NOTE 2:	PHR is carried by MAC signalling. Thus, the existing mechanism of PHR transmission applies, see TS 38.321 [21].

-	For M3:

-	Void.

-	For M4:

-	End of measurement collection period.

-	For M5:

-	End of measurement collection period.

NOTE 3:	If transmission of a data burst is ongoing at the boundary of the measurement collection period, T1 and T2 in throughput evaluations are set to the end and the start of the measurement period, respectively.

-	For M6:

-	End of measurement collection period.

-	For M7:

-	End of measurement collection period.

-	For M8:

-	Associated to M1 and/or M6 related measurement reporting triggers.

-	For M9:

-	Associated to M1 and/or M6 related UE measurement reporting triggers.

End of Changes