3GPP TSG-RAN WG2 Meeting #130	R2-2503634

St Julian’s, Malta, 19 – 23 May 2025

Agenda item:	8.10.3

Source:	Nokia

Title:	SON/MDT enhancements for network slicing, MHI and NTN

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core – Rel-19

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses three topics for Rel-19 SON/MDT enhancements: enhancements for network slicing, enhancements for MHI, and enhancements for NTN.

Regarding to SON/MDT enhancements in LS R3-250914 RAN3 asked RAN2 to consider if any enhancement is needed to address the following scenario:

•	A UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, thereby UE have to camp on cells belonging to the lower ranked NSAG or eventually use legacy cell reselection procedure to find a suitable cell.

This is issue was briefly discussed at RAN2#129bis without a conclusion. This paper proposes a way forward.

At RAN2#129bis it was agreed that “UE reports the absolute time it has spent in PSCell with SCG activated in MHI”. Despite this agreement, we still believe that the solution is not yet complete, and we discuss the need for additional MHI/UHI enhancements with SCG Activation/de-activation time in this Tdoc.

This paper also contains proposals to enhance SHR, and RLF report for RACH-less access for NTN, which has not been addressed earlier.

# 2	Discussion

## 2.1	SON MDT enhancements for network slicing

Rel-17 introduced network slice-based cell reselection, where the UE is provided with one or more network slice groups and associated frequency priorities. Based on network slice(s) the UE is interested in and on the configured prioritisation, it performs frequency selection and tries to camp on the frequency that the network prioritizes for the prioritized slice group. The goal is to avoid long delay and extensive signalling during the connection setup that may occur when redirecting the UE to the right frequency, if the one it camps on is not appropriate for the requested slice.

The short overview of the slice-based cell reselection is the following:

- The UE receives Network Slice AS Group (NSAG) and their priorities via NAS from the core network.

- The UE receives the cell reselection parameters for the NSAGs either in SIB16 or in dedicated RRC signalling.

- The UE tries to find a new suitable cell supporting the highest ranked NSAG(s) using the cell reselection parameters of the highest ranked NSAG(s).

- If step 3 fails, then the next highest ranked NSAG(s) are considered as the highest ranked NSAG(s), and step 3 is repeated with them.

- If there is no more NSAG to be used for cell reselection then the UE performs the normal cell reselection without considering any slice configuration.

However, if these cell re-selection configurations (the cell reselection parameters received in step 2) are not correctly defined and optimised, this goal may not be achieved: it could happen that the UE is not able to connect to its required slice in the cell where it is camping based on the slice-specific frequency priorities provided by the network. The problems that the UE may encounter are, for example:

- Despite following the NSAG priorities, the UE selects a cell where the needed slice is not supported or not allowed.

- Despite following the NSAG priorities, the UE still must be redirected at the connection setup.

- After following the frequency priorities of the highest priority NSAG of the UE, the UE is forced to revert to lower-priority NSAG configuration and repeat cell reselection process.

Therefore, it is important that the network gets some feedback from the UE about how well the slice-based cell reselection configurations are working and if the UE is experiencing some slice connectivity problems. More specifically, RAN3 requested RAN2 to support the scenario when “A UE supporting slice-based cell reselection may not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, thereby UE have to camp on cells belonging to the lower ranked NSAG or eventually use legacy cell reselection procedure to find a suitable cell.”

The logged MDT measurements are collected when the UE is in IDLE/INACTIVE mode when the cell reselection is performed. The current logged MDT reports are good to recognize coverage issues, but they do not provide enough information to recognize slice-based cell reselection issues, as they do not contain any information how the UE selected the frequencies to be measured, and how the UE selected a new suitable cell. It is very important to notice that the RAN is not aware of the UE specific NSAG priorities.

A simple example is when there are two NSAGs: NSAG1 for which the preferred frequency is F1, and NSAG2 for which the preferred frequency is F2. If a UE selects a cell on F2 in an area where no cell for F1 is available, the RAN cannot differentiate the following cases based on legacy logged measurements:

- the highest priority NSAG of the UE is NSAG1 and the UE was not able to select the suitable cell for the highest priority NSAG; or

- the highest priority NSAG of the UE is NSAG2 and the UE was able to select the suitable cell for the highest priority NSAG.

More complex cases with more NSAGs and more frequencies can easily happen in practice. Therefore, new enhancements in logged MDT are needed on how the UE used the sliced based cell reselection parameters to address the scenario that RAN3 requested.

Observation 1.1: New enhancements in logged MDT are needed on how the UE used the sliced based cell reselection parameters.

Simply adding a flag that the highest rank NSAG ID was not used for cell reselection is not very meaningful as the NG-RAN has no information about the NSAGs and their priorities that are used by the UE. E.g., if there are two NSAGs for which the most preferred frequency is the same and this frequency is not available in the location of the UE, then a simple flag does not provide enough information for the network which NSAG ID was not considered successfully for cell reselection.

Observation 1.2: A flag that the highest rank NSAG ID was not used for cell reselection does not help the NG-RAN to optimize the slice-based cell reselection parameters, as NG-RAN cannot know the NSAG IDs and their priorities in the UE.

Considering the above depicted procedure of slice-based cell reselection and the RAN is not aware of the NSAGs and their priorities in the UEs, to enable the optimization of the NSAG related cell reselection parameters the RAN should know

(i)	which NSAGs lead to cell reselection failure and

(ii)	which NSAG was the one that lead to a successful cell reselection (or that cell reselection happened without considering any NSAG).

Therefore, we propose to add these parameters in the logged MDT report for slice-based cell reselection.

Proposal 1: Enhance the logged MDT report with all the NSAG IDs that did not lead to successful cell reselection and the NSAG ID (or the lack of any NSAG ID) that was used to select a new cell.

## 2.2	MHI Enhancement for SCG Deactivation/Activation

The mechanism for SCG activation and deactivation was introduced in Rel-17 to enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured. While the SCG is deactivated, there is no transmission via SCG RLC bearers. Only the NR SCG can be deactivated, and all SCG SCell(s) are in deactivated state while the SCG is deactivated.

SCG activation/deactivation can be triggered by UL or DL traffic on Data Radio Bearers (DRBs) with an SCG Radio Link Control (RLC) bearer (split or SCG bearer) by MN, SN or UE based on some activation/deactivation condition that is not clearly specified in 37.340. The following figure shows an example of how this could work assuming the activation/deactivation condition is based on a traffic threshold.

The concept of SCG activation and deactivation is most efficient for traffic where bursts of data large enough to benefit from SCG transmission are separated by periods of lower activity, during which the SCG can be deactivated. As the energy consumption in SCG active state is high, the criterion deciding upon SCG activation and deactivation shall be carefully selected. The simplest way is a UE traffic threshold where periods with data traffic above the threshold correspond to SCG activated state, while periods with data traffic below the threshold correspond to SCG deactivated state which may be configured for each UE or group of UEs with similar traffic profile.

In case the threshold is set as too low, the SCG activated period may contains also bursts of data which are not large enough to benefit from SCG transmission, hence leading to unnecessary power consumption. In case the threshold is set as too aggressively high, it could happen that not all bursts of data which are large enough benefit from SCG transmission are detected, which may lead to QoS degradation from end user perspective, though energy consumption may be reduced in this case.

Currently, the UHI and MHI for SCG include no information about SCG activation and deactivation. Despite majority of companies consider the information about the time of SCG activation and/or deactivation could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity in our understanding, due to possible different setting of traffic thresholds between the PSCells of different nodes this information may be insufficient. The detailed information about relative time in SCG activated state may therefore need to be extended with some more information. For instance, if the relative time in SCG activated state is quite low, it means that DC is possibly not so necessary for the UE, and the network might not need configure DC for the UE, to reduce the UE’s battery consumption and the signaling overhead of DC configuration at network side. However, such a decision may not be optimal if the traffic threshold for the PSCells(s) reported within the MHI is set as too aggressively high. On the other hand, if the relative time in SCG activated state is quite high, it possibly means that the Dual Connectivity is highly demanded, and the MN could configure and/or activate the SCG more efficiently, which again may not be optimal if traffic threshold is set as too low.

Observation 2: The information of SCG activation and/or deactivation, e.g., the relative time in SCG activated state together with additional assistance information related to the condition(s) for SCG activation/deactivation, could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

At RAN2#129bis it was agreed that “UE reports the absolute time it has spent in PSCell with SCG activated in MHI the following was agreed”.

However, in relation to MHI/UHI enhancement with the information about the time of SCG activation and/or deactivation to help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity, this information is insufficient as described above.

Proposal 2: It is proposed to include additional assistance information related to the condition(s) for SCG activation/deactivation, in MHI such as DL and/or UL volume per second which was transmitted or received by UE during the SCG activation time for the given PSCell.

## 2.3	SON/MDT for NTN

As part of Rel-18, RAN2 specified different features to reduce the signalling overhead in NR NTN and reduce mobility latency. One of these features was the RACH-less Handover (HO) that aims at skipping Msg1 and Msg2 of the RACH procedure during the access of the target cell.

We look at the optimization of RACH-less access in NTN deployments and propose SHR & RLF Report enhancements for this purpose.

NTN deployments, despite being challenging due to the high-speed mobility of the network nodes (i.e., satellites), position of satellites is deterministic because the satellites follow fixed trajectories – this allows for some of the network parameters to be very much predictable. In NTN, parameters such as TA and doppler shift are pre-compensated at the UE side based on satellite ephemeris that the UE obtains from system information (i.e., SIB19). RACH-less HO in NTN is supported as part of normal HO, Conditional HO (CHO) and also when using time-based CHO. The schematic of RACH-less normal HO in NTN is given in Figure 1.

Figure 1: Handover Procedure with RACH-less Access of T-Cell

TA Alignment

The HO command (RRCReconfiguration message) will include an indication (NTA) whether the TA offset is zero or if it is assumed to be the same as in the serving cell. The UE then uses this to compute the overall TA (including additional information broadcast in SIB19, using the UE location and satellite ephemeris). This TA is then used to do UL synchronization of the target cell (without using the RA procedure)

UL Grant for Target Cell

Two approaches were defined Configured Grant (CG) and Dynamic Grant (DG). Both are shown in Figure 1.

The UE receives a Type-1 CG (Configured Grant) as part of the RRCReconfiguration message – it contains parameters to indicate SSB index, time/frequency resource allocations, antenna port, MCS, power, and RSRP threshold among others. Once the UE has detached from the source cell, it attempts to transmit in the time and frequency resources contained in the CG only if the RSRP of the indicated SSB index is above a certain threshold.

Alternatively, the UE can be also configured to access via a DG. In this case, the UE receives in the HO command only the SSB/beam information of the target cell. Once the UE has detached from the source cell, detect the target cell, and acquired DL synchronization it starts monitoring the PDCCH channel waiting for a PUSCH transmission assignment (e.g., DCI format 0_0 or 0_1). In this case, the UE is not required to check for an RSRP threshold before transmitting.

In this paper we are considering failures and proposing enhancements to the DG case.

Observation 3.1: Dynamic Grant based RACH-less access for NTN mobility involves optimization in multiple areas. It requires a lot of information to be transferred to the UE prior to or along with the HO Command which are processed by the UE to realize RACH-less access of the target Cell.

To investigate the potential errors that could occur in the RACH-less access mechanism step 3 of Figure 1 is further discussed. This step consists of two sub-steps:

Sub-step #1: After detecting the target cell and acquiring DL synchronization, UE uses quasi co-location (QCL) (SSB given in source as part of the RRC Reconfiguration vs DMRS of target) to monitor PDCCH of target.

Sub-step #2: The UE monitors PDCCH waiting for a DCI assignment for PUSCH transmission. This UL grant also contains additional information to perform the transmission such as modulation and coding formats, time and frequency resource allocations, HARQ configuration or power parameters.

The key thing to note here is that the UE does these actions on its own and the network is not aware of the time elapsed between successful reception of the HO command and the UE initiating the monitoring of the PDCCH. For this reason, the NW must consider the need to complete the Sub-step #1 when the time of transmitting the DCI with PUSCH assignment in Sub-step #2 of step 3 is selected.

Near failure scenario: Even though the RACH-less HO procedure may succeed, there could be issues like the UE having to monitor the PDCCH for a longer time for it to acquire PUSCH related configuration (i.e, NW estimates more delay in when the UE will start monitoring PDCCH) or the NW may have to transmit the UL grant for repeated times over a given search space until the UE acquires it (i.e., due to NW underestimating the time when the UE will start monitoring and transmitting the UL grant before the UE is ready). This issue may result in longer time for successful HO with the following consequences:

- UE’s energy consumption increases if it must monitor the PDCCH longer

- Sub-optimal utilization of allocated resources: resources in the NW are allocated for longer duration in case of the NW has to send the UL grant repeatedly - these resources could be used for other UEs purposes)

Observation 3.2: The DG based RACH-less access defined for NTN mobility could lead to sub-optimal scenarios (near failure scenarios) which would require constant measurement, monitoring and performance optimization.

We have covered the near-failures scenario above. The following are the possible NTN-specific failures scenarios that could happen in RACH-less access.

- Outdated/drifted SIB19 information for estimation of the target cell parameters received in the HO command. UE’s TA/doppler estimation may drift over time, if the estimation is off the optimal value, it may force the NW to allocate more resources to each PUSCH assignment to protect those resources.

- Power control parameters were not adjusted for latest channel impairments (a UE may receive sub-optimal PC parameters which may cause an unsuccessful PUSCH transmission).

- The ssb_index in HO Command could be changed/optimized. If a UE is located on the cell edge between two cells, it may result that the UE experiences better signal conditions on the adjacent cell (e.g., signals from both cells are fully uncorrelated since those belong to different satellites).

- Step 3 Sub-step 1 could fail because when the UE monitors for PDCCH, it is unable to decode the same due to bad radio conditions

- Step 3 Sub-step 2 could fail because the UE is unable to acquire the UL grant in time and timer T304 expires.

Observation 3.3: The DG based RACH-less access defined for NTN mobility can lead to many failure scenarios related to NTN specific parameters that would require some MRO enhancements.

Based on the above observations we think that enhancement of RLF report and SHR are needed to enable network optimization for RACH-less access for NTN.

Proposal 3.1: RAN2 to consider enhancing RLF Report and SHR for DG based RACH-less access for NTN with the following additional parameters:

- PDCCH Monitoring Time Until Success

- No of PUSCH Attempts Until Success

- Full TA, Doppler estimation and PC control parameters used during PUSCH transmissions

- Target NTN-config in SIB19

- Target NTN-config in RRC Reconfiguration message

- New Cause value specific to NTN RACH-less access failure for RLF Report

- New Cause value specific to NTN RACH-less HO (for SHR)

We also think that defining a new SHR trigger to capture the newly defined measurements upon successful completion of NTN RACH-less HO as below would also be beneficial.

•	Time the UE spent monitoring the PDCCH channel is higher than the configured Threshold_1 AND/OR

•	Number of PUSCH Attempts is larger than Threshold_2

The Threshold-1 & Threshold-2 values can be configured via RRCReconfiguration message.

Proposal 3.2: Define a new SHR trigger (based on PDCCH monitoring time and/or number of PUSCH attempts) for the UE to log the enhancements measurements specific to NTN RACH-less access.

# 3	Conclusion

This document has made the following observations and proposal for SON/MDT enhancements for slicing:

Observation 1.1: New enhancements in logged MDT are needed on how the UE used the sliced based cell reselection parameters.

Observation 1.2: A flag that the highest rank NSAG ID was not used for cell reselection does not help the NG-RAN to optimize the slice-based cell reselection parameters, as NG-RAN cannot know the NSAG IDs and their priorities in the UE.

Proposal 1: Enhance the logged MDT report with all the NSAG IDs that did not lead to successful cell reselection and the NSAG ID (or the lack of any NSAG ID) that was used to select a new cell.

This document has made the following observation and proposal on MHI Enhancement for SCG Deactivation/Activation:

Observation 2: The information of SCG activation and/or deactivation, e.g., the relative time in SCG activated state together with additional assistance information related to the condition(s) for SCG activation/deactivation, could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

Proposal 2: It is proposed to include additional assistance information related to the condition(s) for SCG activation/deactivation, in MHI such as DL and/or UL volume per second which was transmitted or received by UE during the SCG activation time for the given PSCell.

This document has made the following observations and proposals on SON/MDT for NTN:

Observation 3.1: Dynamic Grant based RACH-less access for NTN mobility involves optimization in multiple areas. It requires a lot of information to be transferred to the UE prior to or along with the HO Command which are processed by the UE to realize RACH-less access of the target Cell.

Observation 3.2: The DG based RACH-less access defined for NTN mobility could lead to sub-optimal scenarios (near failure scenarios) which would require constant measurement, monitoring and performance optimization.

Observation 3.3: The DG based RACH-less access defined for NTN mobility can lead to many failure scenarios related to NTN specific parameters that would require some MRO enhancements.

Proposal 3.1: RAN2 to consider enhancing RLF Report and SHR for DG based RACH-less access for NTN with the following additional parameters:

- PDCCH Monitoring Time Until Success

- No of PUSCH Attempts Until Success

- Full TA, Doppler estimation and PC control parameters used during PUSCH transmissions

- Target NTN-config in SIB19

- Target NTN-config in RRC Reconfiguration message

- New Cause value specific to NTN RACH-less access failure for RLF Report

- New Cause value specific to NTN RACH-less HO (for SHR)

Proposal 3.2: Define a new SHR trigger (based on PDCCH monitoring time and/or number of PUSCH attempts) for the UE to log the enhancements measurements specific to NTN RACH-less access.