3GPP TSG-RAN WG2 Meeting #130	                                          R2-2503639

Malta, 19 – 23, May, 2025

Source: 	Xiaomi

Title:  	Discussion on configuration and report for NW sided model

Agenda Item:	8.3.3

Document for:	Discussion and Decision

Introduction

In this discussion, we discuss the scope of NW sided model and the signalling to report the model input to NW.

Discussion

In last meeting, following scope is agreed for further study,

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes, all sub cases | Yes |
| Frequency domain | Yes, all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes, | Yes |

Three sub use cases are supported for RRM prediction according to TR[1] as following,

Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

For L3 beam level measurement prediction, following three sub-cases are supported according to TR [1],

Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated

Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

The inference input of sub use case 1, 3, 4 and 6 are L1 beam measurement results. Note the L1 beam for RRM is configured by CSI-RS-ResourceConfigMobility, which is different from the beam for beam management. Current CSI report framework doesn’t support reporting L1 beam measurement result for RRM purpose. Furthermore, RRM prediction may require neighbor cell’s L1 beam measurement result as model input. Current CSI report framework only support reporting serving cell’s L1 beam measurement result.

It’s up to UE how to measure the RRM L1 beam within the required latency, i.e. by what periodicity filtering. NW side model may require L1 beam measurement with fixed periodicity as input. It’s new requirement for UE measurement, which is impossible to apply from UE point of view. Furthermore, it’s up to UE how to perform the L1 filtering. UEs may apply different L1 filtering method. NW is not aware which L1 filtering is applied by UE. The inference accuracy may be poor without such information.

As discussed above, new report signalling is required to report L1 beam measurement result. New requirement on UE measurement is needed to align with the model input interval. UE may also need to report how L1 filtering is done. The spec impact is huge to support L1 beam as model input, i.e. sub-case 1 or 3, for NW sided model.

Observation 1: Huge spec impact is foreseen to support L1 beam as model input for NW sided model, i.e. sub case 1, 3, 4 and 6, including

- new signalling to report L1 beam measurement result of serving and neighbour cell for RRM, as current L1 beam for RRM is configured by CSI-RS-ResourceConfigMobility, rather than using CSI framework

- new requirement on UE measurement with potential RAN4 impact

- new signalling to report how L1 filtering is done, which is now completely up to UE implementation

Therefore, we propose to only support sub-case 2, i.e. L3 cell level measurement results as model input, is supported for NW sided model in RRM prediction.

Proposal 1: L1 beam measurement result as model input, i.e. sub case 1 and 3, is not supported for NW sided model in RRM prediction and L3 beam prediction.

In NW sided model, the inference is done at NW. The model input is real measurement results, which has to be reported by UE. Current spec has already supported periodical and event triggered measurement report of L3 cell level measurement result and L3 level beam measurement result. NW can configure the report periodicity, which matches the interval of model inputs. Therefore, legacy measurement report can be reused to report model input of NW sided model.

Proposal 2: Legacy measurement report procedure can be reused for UE to report L3 cell level or L3 beam level measurement result as model input for NW sided model.

It’s up to NW how to perform inference and use the inference results, which has no impact on the spec.

Proposal 3: There is no specification impact associated to NW-side model inference. It’s up to NW how to use the inference results.

During the study of AI air, it’s agreed UE is not involved to the NW sided model management, except being configured to provide required measurement/data. It’s up to NW implementation how to manage the NW side additional condition. We believe these agreements can also be applied for AI mobility.

Proposal 4: RAN2 confirms following agreements from AI air is also applicable to AI mobility

- RAN2 confirms that UE will not be informed about any gNB -sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

- RAN2 confirms that UE will not be involved in any gNB -sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

- For the network-side model, required network side additional condition is left to the network implementation

Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation 1: Huge spec impact is foreseen to support L1 beam as model input for NW sided model, i.e. sub case 1, 3, 4 and 6, including

- new signalling to report L1 beam measurement result of serving and neighbour cell for RRM, as current L1 beam for RRM is configured by CSI-RS-ResourceConfigMobility, rather than using CSI framework

- new requirement on UE measurement with potential RAN4 impact

- new signalling to report how L1 filtering is done, which is now completely up to UE implementation

Proposal 1: L1 beam measurement result as model input, i.e. sub case 1 and 3, is not supported for NW sided model in RRM prediction and L3 beam prediction.

Proposal 2: Legacy measurement report procedure can be reused for UE to report L3 cell level or L3 beam level measurement result as model input for NW sided model.

Proposal 3: There is no specification impact associated to NW-side model inference. It’s up to NW how to use the inference results.

Proposal 4: RAN2 confirms following agreements from AI air is also applicable to AI mobility

- RAN2 confirms that UE will not be informed about any gNB -sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

- RAN2 confirms that UE will not be involved in any gNB -sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

- For the network-side model, required network side additional condition is left to the network implementation

Reference

[1] 38.744