3GPP TSG-RAN WG2 Meeting #130	                                          R2-2503640

Malta, 19 – 23, May, 2025

Source: 	Xiaomi

Title:  	Discussion on data collection

Agenda Item:	8.3.4

Document for:	Discussion and Decision

Introduction

In this discussion, we discuss potential configuration and procedure of data collection for NW sided and UE sided model in AI mobility.

Discussion

NW side data collection

In legacy, UE doesn’t log data and report only the latest measurement result. NW sided temporal prediction model training may require multiple consecutive measurement results, which requires frequent UE report. Therefore, logging is introduced to avoid frequent UE report of the measurement results. In AI mobility, similar issue exists. Logging is also needed for NW sided data collection in AI mobility.

Proposal 1: Data logging is supported for NW side data collection in AI mobility.

In AI air, the data logging can be triggered in following ways,

- Radio Event, RSRP of serving cell becomes higher or lower (A1/A2)

- Periodic

These triggers can be reused. In addition, new radio event shall can be introduced. In RRM prediction 3 and 4, the target goal is to improve mobility performance. Handover is expected to be occur at cell edge rather than cell centre. Therefore, the data logged in cell centre may not be useful for NW model in RRM prediction 3 and 4. To reduce the unnecessary data logging, new radio event, e.g. A3, A5 alike event, can be introduced.

Proposal 2: New radio event, e.g. A3, A5 alike event, can be introduced to control data logging in AI mobility.

RAN2 need to discuss the data content for NW side data collection. RRM and L3 beam prediction is supported by NW sided model.

For RRM temporal domain prediction, model input are the historic measurement results. The measurement results can include historic cell level measurement results. In cluster prediction, the measurement results may include multiple cells. If logging event is configured, logged data may be inconsecutive. NW needs to know whether there is gap in the logged data. Similar issue exists in AI air BM, timing information may be used to indicate the gap.

For L3 beam prediction, as discuss in [1], we propose to only support temporal domain prediction. The data content is similar as RRM temporal domain prediction. The only difference is the measurement is per beam, not per cell.

Proposal 3: For RRM and L3 beam temporal domain prediction, the data content may include,

- Cell level measurement results

- L3 beam level measurement results

- Timing information

For RRM frequency domain prediction, model input is the measurement result of cells on one or multiple frequencies. Some model may also require historic measurement results to improve the accuracy. If historic measurement results are required, timing information is also needed.

Proposal 3: For RRM frequency domain prediction, the data content may include,

- Cell level measurement results on one or multiple frequencies

- Timing information, if needed

In AI air, a minimum AS layer memory size is assumed for data logging. And the memory is shared across all use cases of three use cases. It’s not clear whether additional/separate memory is needed for AI mobility data logging. We understand the AI mobility data can also share the same memory. Because it’s more efficient and can avoid the fragmentation. However, in future, more AI use cases are expected. Larger memory size may be required to log more data from many AI use cases. For forward compatibility, UE may support larger memory size and shall be allowed to report the supported memory size.

Proposal 4: UE stores logged data in AS layer memory, which is shared across all AI use cases, including AI mobility, AI air and future AI use cases.

Proposal 5: UE can indicate the supported AS layer memory size for data logging.

In AI air, it’s agreed the NW can indicate whether logged data is retained during HO. UE would release logged data upon RLF or leaving RRC_CONNECTED. We understand same solution can be reused in AI mobility. Because the inference is only in RRC_CONNECTED. Also, unexpected data exposure to other gNBs can be avoided.

Proposal 6: Confirm following agreements are applied in AI mobility for logged data handling,

- NW indicate whether logged data is retained after HO

- UE discards logged data upon RLF or leaving RRC_CONNECTED

In Ai air, it’s agreed the UE can trigger availability indication due to full buffer, buffer threshold or low power. We understand similar solution can also be used in AI mobility.

Proposal 7: Confirm following agreements are applied in AI mobility for data availability report,

- Availability indication can be triggered due to:Full buffer being reached (if configured), Buffer threshold being reached (if configured), Low power (if configured)

UE side data collection

In AI air, following agreements are agreed,

- The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: - • An indication on start/stop of data collection - • Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end.

We understand these agreements can also be applied in AI mobility.

Proposal 7: Confirm following agreements are applied in AI mobility for UE request data collection configuration,

- The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

• An indication on start/stop of data collection

• Preferred configuration from a list of candidate configurations provided by NW. It is up to network what it configures at the end.

The detail of candidate configuration is up to RAN1 in BM. In AI mobility, RAN2 can discuss the content of the candidate configuration. In AI mobility, the RS for RRM is per cell configured, which not UE specific. If NW decides to not transmit RRM RS due to NES purpose, UE shall not request RS only for data collection purpose. So, we understand there is no need for UE to request RS for data collection in AI mobility.

Proposal 8: UE doesn’t request RRM RS configuration in data collection configuration.

However, UE may require measurement gap to collect inter-frequency measurement results. The measurement gap is configured by NW. Therefore, UE shall be able to request measurement gap in data collection configuration. Furthermore, UE may indicate which cell/frequency is to be measured to NW. NW can provide the required configuration to UE to assist the data collection on the corresponding cell/frequency.

Proposal 9: RAN2 discuss UE can request following in data collection configuration,

- measurement gap configuration to collect data on inter frequency cell

- Cell/frequency to be measured

Conclusions

According to the analysis given above, we have the following observations and proposals:

NW side data collection

Proposal 1: Data logging is supported for NW side data collection in AI mobility.

Proposal 2: New radio event, e.g. A3, A5 alike event, can be introduced to control data logging in AI mobility.

Proposal 3: For RRM and L3 beam temporal domain prediction, the data content may include,

- Cell level measurement results

- L3 beam level measurement results

- Timing information

Proposal 3: For RRM frequency domain prediction, the data content may include,

- Cell level measurement results on one or multiple frequencies

- Timing information, if needed

Proposal 4: UE stores logged data in AS layer memory, which is shared across all AI use cases, including AI mobility, AI air and future AI use cases.

Proposal 5: UE can indicate the supported AS layer memory size for data logging.

Proposal 6: Confirm following agreements are applied in AI mobility for logged data handling,

- NW indicate whether logged data is retained after HO

- UE discards logged data upon RLF or leaving RRC_CONNECTED

Proposal 7: Confirm following agreements are applied in AI mobility for data availability report,

- Availability indication can be triggered due to:Full buffer being reached (if configured), Buffer threshold being reached (if configured), Low power (if configured)

UE side data collection

Proposal 7: Confirm following agreements are applied in AI mobility for UE request data collection configuration,

- The UE can request measurement configuration for data collection of AI/ML based beam management.   The request can contain one or more of the following:

• An indication on start/stop of data collection

• Preferred configuration from a list of candidate configurations provided by NW. It is up to network what it configures at the end.

Proposal 8: UE doesn’t request RRM RS configuration in data collection configuration.

Proposal 9: RAN2 discuss UE can request following in data collection configuration,

- measurement gap configuration to collect data on inter frequency cell

- Cell/frequency to be measured

Reference

[1] R2-250XXX Discussion on inference solution and functionality management for UE sided model, Xiaomi

[2] RP-242393 SID of AI/ML mobility