3GPP TSG-RAN WG2 #130	R2-2503642

Malta, 19th – 23rd May 2025

Source:	NTT DOCOMO, INC.

Title:	On remaining issues on A-IoT Random Access

Document for:	Discussion

Agenda Item:	8.2.3

- Introduction

This contribution is to provide our views on some of remaining issues collected in the offline discussion after RAN2#129-bis [1] and to raise one issue which has not been listed.

- Discussion

Issue 2-1: Msg1 resource selection

| Issue number, brief title | Issue description | Issue classification |
| --- | --- | --- |
| Issue 2-1: Msg1 resource selection | Whether/how to specify the device detailed behaviour of randomly selecting the Msg1 resource based on the R2D trigger message. - Relevant agreements: - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number. - Status in running CR: captured as Editor’s Note in 5.3.3.1. | To be discussed by company contributions |

We do not have any concrete suggestions on this issue.

Issue 2-2: Paging&first R2D trigger message

| Issue number, brief title | Issue description | Issue classification |
| --- | --- | --- |
| Issue 2-2: Paging&first R2D trigger message | Whether the R2D trigger message is needed in CFRA, and whether the first R2D trigger message will be merged into paging message in CBRA. - The Rapp understands the discussion of the R2D trigger message focused on CBRA, and it’s not crystal clear whether the R2D trigger message is also needed in CFRA, and whether it can be merged to paging message if it’s the first R2D trigger message in CBRA. - Status in running CR: not captured yet. | To be discussed by company contributions |

This issue can be rephrased as follows: Regardless of whether it is CFRA or CBRA, should the Paging message include information about the first set of access occasions after Paging, or should the indication of that information be left to the first R2D trigger message after Paging?

Since we will specify the R2D trigger message, which is the message that notifies the information on access occasion, we could consider clearly separating the roles of the messages and notifying all access occasion information through the R2D trigger messages. However, in that case, we need to clarify the following point.

Figure 1. Example where R2D trigger message indicates first set of AOs.

Assuming that the paging message does not include information about the first set of access occasions, as shown in Figure 1, the first R2D trigger will be the first to notify the device of the access occasion information. From the perspective of system efficiency, since the device has nothing to do between receiving the paging message and receiving the first R2D trigger message, it is desirable for the reader to send the R2D trigger message as soon as possible after sending the paging message. The best option in this case would be to multiplex the two messages into one PRDCH and send them together.

Our understanding is that the assumption of multiplexing multiple R2D messages has not been raised in past RAN2 discussions. If this scenario is to be introduced, RAN1 involvement would be necessary. Considering that RAN1's WI discussions will conclude with the May meeting, involving RAN1 is not realistic.

However, on the other hand, if the paging message includes information about the first set of access occasions, it should be noted that this is as efficient as multiplexing the paging and R2D trigger messages together as described above. We therefore think that this is a better idea which can achieve high efficiency without RAN1 involvement.

Proposal 1.	(Issue 2-2) Paging message includes information of access occasions with the same contents as R2D trigger message.

Issue 2-3: R2D trigger message byte alignment

| Issue number, brief title | Issue description | Issue classification |
| --- | --- | --- |
| Issue 2-3: R2D trigger message byte alignment | The R2D trigger message should be byte aligned or not. - Relevant agreements: - The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message. - Status in running CR: not captured yet. | To be discussed by company contributions |

We do not have any strong preference on this issue.

Issue 2-4: CBRA failure detection

| Issue number, brief title | Issue description | Issue classification |
| --- | --- | --- |
| Issue 2-4: CBRA failure detection | How to determine CBRA failure/contention resolution failure. - In SI, RAN2 agreed to support re-access in case of contention resolution failure as capture in TR 38.796. In WI phase, RAN2 has agreed to re-use the subsequent paging message to trigger re-access, but has not discussed how to determine CBRA failure/ contention resolution failure. - Status in running CR: captured as Editor’s Note in 5.5. | To be discussed by company contributions |

We think the following definitions for contention resolution failure and Msg3 failure are well exclusive and sufficient.

Contention resolution failure: The reader receives a random ID from a device during an access occasion and, if it does not collide with other Msg1, sends Msg2 containing that random ID. Therefore, the device should consider contention resolution as failed unless it receives Msg2 containing the random ID it sent. When the device receives Msg2, it should be considered that it has exited the contention resolution failure status.

Msg3 failure: The reader notifies Msg3 NACK to the AS ID that failed to receive Msg3. Therefore, the device should consider that it has entered the Msg3 failure status when it receives Msg3 NACK.

Proposal 2.	(Issue 2-4) The device considers contention resolution as failed from the time it sends Msg1 until it receives Msg2 including random ID that the device sent.

Issue 2-5: random ID differentiation in Msg2

| Issue number, brief title | Issue description | Issue classification |
| --- | --- | --- |
| Issue 2-5: random ID differentiation in Msg2 | Whether/how to address random ID collision in Msg2, i.e. multiples devices generate same random ID using different Msg1 resources. - In previous meetings, RAN2 discussed whether Msg2 need to include more information on top of the random ID to avoid random ID collision, but there was no consensus. - Status in running CR: not captured yet. | To be discussed by company contributions |

Considering that the maximum TB size of messages in Ambient IoT is around 1000 bits, a single Msg2 can indicate a maximum of approximately 60 random IDs of 16 bits each. Furthermore, since Msg2 also includes indications for Msg3 transmission occasion for each device, the actual number of random IDs that can be indicated is even fewer. In this case, the possibility of 16-bit random IDs colliding within the same Msg2 is negligible, and therefore, it is not necessary to address this in the specification.

Proposal 3.	(Issue 2-5) RAN2 does not specify anything specific for resolution of random ID collision within a Msg2.

Issue 2-6: number indication of echoed random IDs in Msg2

| Issue number, brief title | Issue description | Issue classification |
| --- | --- | --- |
| Issue 2-6: number indication of echoed random IDs in Msg2 | Whether/how to indicate the number of echoed random IDs included in Msg2. - RAN2 agreed that A-IoT Msg2 contains one or multiple echoed random ID(s) from A-IoT Msg1 of different A-IoT devices, but there is no discussion on whether/how to indicate the number of echoed random IDs. The Rapp understands this can be considered as signaling design/stage3 issue which should be quite straightforward. Companies can check the proposal in 2.2. - Status in running CR: not captured yet. | To be discussed by company contributions Straightforward |

Figure 2 illustrates an example where the reader generates a paging message using a TB with a size of L octets, with the header length being N octets and the paging content (e.g., random IDs, AS IDs) length being M octets. In this case, L-(N+M) bytes become padding bits, which are usually set to 0 in our understanding.

Figure 2. Paging message.

If the paging message does not include a field indicating the number of echoed random IDs, there needs to be a way for the device to determine where the padding bits start. Therefore, if we assume that a random ID with all 16 bits set to 0 does not exist, i.e., the device generates random IDs between 1 and 65536, the device can determine that the padding starts from the point where the random ID field is all zero.

Proposal 4.	(Issue 2-6) Field for number of random IDs is not needed in paging message.

Proposal 5.	(Issue 2-6) Random ID is generated between 1 and 65536.

Issue 2-7: present/absent indication of assigned AS ID in Msg2

| Issue number, brief title | Issue description | Issue classification |
| --- | --- | --- |
| Issue 2-7: present/absent indication of assigned AS ID in Msg2 | How to indicate the AS ID presence in Msg2. - Based on the following agreement, the Rapp understands the assigned AS ID is an optional field in Msg2, but how to indicate the presence/absence has not been discussed yet. This can be considered as signaling design/stage3 issue which should be quite straightforward. Companies can check the proposal in 2.2. - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - For CBRA, Msg 2 is used for AS ID assignment . - Status in running CR: captured as Editor’s Note in 6.2.1.3. | Straightforward |

We understand this issue is not to be treated based on company contributions.

Issue 2-8: re-access for CFRA

| Issue number, brief title | Issue description | Issue classification |
| --- | --- | --- |
| Issue 2-8: no re-access for CFRA | How to achieve “no re-access” for CFRA - Relevant agreements: - For CFRA, NACK feedback and re-access is not supported. FFS how to achieve. - Status in running CR: captured as Editor’s Note in 5.2. | To be discussed by company contributions |

We do not have any strong preference on this issue.

Issue 2-9: ID assignment in multi-device CFRA

| Issue number, brief title | Issue description | Issue classification |
| --- | --- | --- |
| Issue 2-9: AS ID assignment in multi-device CFRA | Whether to consider multiple device scenario as to the AS ID in CFRA. - Relevant agreements: - ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario. - Status in running CR: not captured. | To be discussed by company contributions |

In our view there is no big demand for extending CFRA applying to multiple devices at once. In other words, in future releases, if the network wants to page multiple specific devices it is sufficient to rely on CBRA.

Proposal 6.	(Issue 2-9) Do not consider scenario to use CFRA to target multiple specific devices.

Issue 2-10: NACK before paging or R2D trigger message

| Issue number, brief title | Issue description | Issue classification |
| --- | --- | --- |
| Issue 2-10: NACK before paging or R2D trigger message | For the re-access due to reception of NACK indication before subsequent R2D message, whether the subsequent R2D message is the R2D trigger message or paging message. - Relevant agreements: - For msg3, we rely on whether the device receives NACK indication before subsequent R2D message to determine re-access. No need for a timer. FFS whether subsequent R2D message is trigger message or paging - Status in running CR: captured as Editor’s Note in 5.5. | To be discussed by company contributions |

We do not have any strong preference on this issue.

Issue 2-11: explicit message for NACK

| Issue number, brief title | Issue description | Issue classification |
| --- | --- | --- |
| Issue 2-11: explicit message for NACK | Whether to use a new/explicit R2D message for NACK feedback. - There is an FFS in the following agreement, but according to the online discussion in last meeting, the Rapp feels companies already consider this NACK message is a separate message, thus marks this as straightforward issue, and companies can check the proposal in 2.2. - NACK based mechanism is supported for D2R messages to determine re-access for at least msg3. FFS details including whether we need a timer or explicit message and when reader sends feedback. - Status in running CR: captured as Editor’s Note in 5.5. | Straightforward |

We understand this issue is not to be treated based on company contributions.

Issue 2-12: multiplexing for NACK indication

| Issue number, brief title | Issue description | Issue classification |
| --- | --- | --- |
| Issue 2-12: multiplexing for NACK indication | Whether to support multiplexing of information for multiple devices in NACK feedback. - Relevant agreements: - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages. - Status in running CR: not captured yet. | To be discussed by company contributions |

We do not have any strong preference on this issue.

Issue not listed: RA types supported by devices

The comparison of the 3-step CBRA and CFRA procedures is shown in Figure 3.

Figure 3. 3-step CBRA and CFRA.

However, we are wondering whether all Ambient IoT devices can support both 3-step CBRA and CFRA. We particularly want to clarify this question regarding device type 1, which is considered to have a very simple design and is within the scope of Release 19. If some devices only support either 3-step CBRA or CFRA, the reader needs to indicate the RA type supported by the target device when sending the A-IoT paging message.

The simplest solution to this question is to assume that all devices support both 3-step CBRA and CFRA. If this is the case, the reader can freely indicate the RA type according to the situation.

Proposal 7.	All devices are assumed to support both 3-step CBRA and CFRA.

- Summary and proposal

Proposal 1.	(Issue 2-2) Paging message includes information of access occasions with the same contents as R2D trigger message.

Proposal 2.	(Issue 2-4) The device considers contention resolution as failed from the time it sends Msg1 until it receives Msg2 including random ID that the device sent.

Proposal 3.	(Issue 2-5) RAN2 does not specify anything specific for resolution of random ID collision within a Msg2.

Proposal 4.	(Issue 2-6) Field for number of random IDs is not needed in paging message.

Proposal 5.	(Issue 2-6) Random ID is generated between 1 and 65536.

Proposal 6.	(Issue 2-9) Do not consider scenario to use CFRA to target multiple specific devices.

Proposal 7.	All devices are assumed to support both 3-step CBRA and CFRA.

- References

[1]	[POST129bis][017][AIoT] 38.391 Running CR (Huawei).