3GPP TSG-RAN WG2 Meeting #130	                	                  R2-2503645

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.12.3

Source: 	OPPO

Title:  	Clarification on the UL skipping for event-triggered beam reporting

Document for:	Discussion and Decision

# Introduction

In the RAN2#129bis meeting, RAN2 made the following agreements on how the uplink grant is used for event-triggered beam reporting:

RAN2#129bis meeting agreements: Confirm the following RAN2 understandings: - The CG type-1 PUSCH carrying the beam report of Mode-B does not carry MAC PDU (i.e. UL-SCH). - The DG PUSCH carrying the beam report of Mode-A carries MAC PDU (i.e. UL-SCH) as legacy. FFS if any other MAC impact for UL skipping

In this contribution, we discuss the potential MAC specification impacts due to the uplink skipping function.

# Discussion

## Uplink skipping

RAN1#116bis meeting agreements: Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2.

TS 38.321: The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity.

According to the RAN1#116bis meeting agreements as quoted above, the uplink grants used for the transmission of the UCI of the UE initiated beam report include the following two types:

- Mode-A DG

- Mode-B CG

According to the TS 38.321 [1] as quoted above, the uplink skipping functions include the following 4 types:

- Rel-15 CG UL skipping (mandatory)

- Rel-15 DG UL skipping (configurable via skipUplinkTxDynamic)

- Rel-16 CG UL skipping (configurable via enhancedSkipUplinkTxConfigured)

- Rel-16 DG UL skipping (configurable via enhancedSkipUplinkTxDynamic)

If Rel-16 CG/DG UL skipping is configured, the UE will use the Rel-16 CG/DG UL skipping, instead of the Rel-15 CG/DG UL skipping. Depending on whether/which UL skipping function is configured/enabled, the MAC behaviours are as follows:

- If UL skipping is not configured/enabled, the MAC entity always generates MAC PDU for PHY (e.g. even though there is no MAC SDU, the MAC generates MAC PDU with padding).

- If the Rel-15 UL skipping is configured/enabled, the MAC does not generate MAC PDU if MAC SDU or aperiodic CSI or MAC CE (except for periodic BSR/padding BSR) is not present, and generates MAC PDU for other cases.

- If the Rel-16 UL skipping is configured/enabled, the MAC does not generate MAC PDU if MAC SDU or UCI or aperiodic CSI or MAC CE (except for periodic BSR/padding BSR) is not present, and generates MAC PDU for other cases.

Observation 1: If the (Rel-15/Rel-16) UL skipping is not enabled, the legacy MAC always generate MAC PDU.

Observation 2: The Rel-15 CG UL skipping is mandatorily enabled without configuration.

The reason of introducing Rel-16 UL skipping is to fix the issue of UCI multiplexing in PUSCH, as not every UCI (e.g. HARQ feedback) multiplexed in PUSCH is aperiodic CSI. If the gNB configures UCI multiplexing in PUSCH and Rel-15 UL skipping for the UE, when a UCI (not aperiodic CSI) is to be multiplexed in PUSCH, the gNB may need to blindly decode the following two different PUSCH formats, as the generation of MAC PDU is random due to the random arrival of the UL MAC SDU. This will increase the gNB implementation complexity.

- PUSCH format 1 with MAC PDU and UCI: Some RE(s) are used for UCI, and some RE(s) are used for MAC PDU.

- PUSCH format 2 with only UCI (due to the Rel-15 UL skipping): All RE(s) are used for UCI.

Observation 3: The introduction of Rel-16 UL skipping is to fix the UCI multiplexing issue of Rel-15 UL skipping. If the gNB is to multiplex UCI (e.g. HARQ) in PUSCH, the recommended gNB implementation is to configured Rel-16 UL skipping.

Then, we could have the following tables to clarify the UE behaviours for Mode-A DG and Mode-B CG:

|  | UL skipping | Expected UE behaviours | Change of UE behaviours (compared with the legacy uplink grant) |
| --- | --- | --- | --- |
| Mode-A DG | No UL skipping | For both Mode-A DG and the legacy DG, MAC always generates MAC PDU. PHY multiplexes MAC PDU with UCI (if available). | No |
|  | Rel-15 UL skipping | For both Mode-A DG and the legacy DG, MAC does not generate MAC PDU if MAC SDU or aperiodic CSI or MAC CE (except for periodic BSR/padding BSR) is not present, and generates MAC PDU for other cases. PHY multiplexes MAC PDU (if available) with UCI (if available). If MAC PDU is not available, the PHY sends UCI in PUSCH. If UCI other than aperiodic CSI is to be multiplexed in PUSCH, the recommended gNB implementation is to configured Rel-16 UL skipping. | No |
|  | Rel-16 UL skipping | For both Mode-A DG and the legacy DG, MAC does not generate MAC PDU if MAC SDU or UCI or aperiodic CSI or MAC CE (except for periodic BSR/padding BSR) is not present, and generates MAC PDU for other cases. PHY multiplexes MAC PDU (if available) with UCI (if available). If MAC PDU is not available, the PHY sends UCI in PUSCH. | No |
| Mode-B CG | No UL skipping | For the legacy CG, MAC always generates MAC PDU. PHY multiplexes MAC PDU with UCI (if available). For Mode-B CG, MAC does not generate MAC PDU. PHY sends UCI in PUSCH. | Yes |
|  | Rel-15 UL skipping | For the legacy CG, MAC does not generate MAC PDU if MAC SDU or aperiodic CSI or MAC CE (except for periodic BSR/padding BSR) is not present, and generates MAC PDU for other cases. PHY multiplexes MAC PDU (if available) with UCI (if available). If MAC PDU is not available, the PHY sends UCI in PUSCH. For Mode-B CG, MAC does not generate MAC PDU. PHY sends UCI in PUSCH. | Yes |
|  | Rel-16 UL skipping | For the legacy CG, MAC does not generate MAC PDU if MAC SDU or aperiodic CSI or MAC CE (except for periodic BSR/padding BSR) is not present, and generates MAC PDU for other cases. PHY multiplexes MAC PDU (if available) with UCI (if available). If MAC PDU is not available, the PHY sends UCI in PUSCH. For Mode-B CG, MAC does not generate MAC PDU. PHY sends UCI in PUSCH. | Yes |

According to the analysis given in the above table, there is no need to change/clarify the UE behaviours for the mode-A DG. Then the MAC specification does not need to differentiate the mode-A DG from the legacy DG. If UCI other than aperiodic CSI is to be multiplexed in PUSCH, the recommended gNB implementation is to configured Rel-16 UL skipping. However, it is still up to the gNB implementation to configure Rel-15 UL skipping for the case that UCI (e.g. HARQ) is multiplexed in PUSCH, if the gNB is able to blindly decode two different PUSCH formats as list above or if the UE does not support Rel-16 UL skipping.

For the mode-B CG, the MAC specification needs to differentiate the mode-B CG from the legacy CG, so that the MAC entity always skips the generation of the MAC PDU, regardless of whether (Rel-15/Rel-16) UL skipping is enabled or not.

Proposal 1: No matter whether Rel-15/Rel-16 uplink skipping is enabled, the MAC does not generate MAC PDU for mode-B CG.

Proposal 2: No matter whether Rel-15/Rel-16 uplink skipping is enabled, the MAC entity does not differentiate mode-A DG from other DG. No specification change is needed.

Proposal 3: If UCI other than aperiodic CSI is to be multiplexed in mode-A DG, it is up to gNB implementation to configure Rel-16 UL skipping or Rel-15 UL skipping or no skipping. No specification change is needed.

To capture the above UE behaviours, we have same text proposals as given in the Annex.

# Conclusion

According to the analysis provided above, we have the following observations and proposals:

Observation 1: If the (Rel-15/Rel-16) UL skipping is not enabled, the legacy MAC always generate MAC PDU.

Observation 2: The Rel-15 CG UL skipping is mandatorily enabled without configuration.

Observation 3: The introduction of Rel-16 UL skipping is to fix the UCI multiplexing issue of Rel-15 UL skipping. If the gNB is to multiplex UCI (e.g. HARQ) in PUSCH, the recommended gNB implementation is to configured Rel-16 UL skipping.

Proposal 1: No matter whether Rel-15/Rel-16 uplink skipping is enabled, the MAC does not generate MAC PDU for mode-B CG.

Proposal 2: No matter whether Rel-15/Rel-16 uplink skipping is enabled, the MAC entity does not differentiate mode-A DG from other DG. No specification change is needed.

Proposal 3: If UCI other than aperiodic CSI is to be multiplexed in mode-A DG, it is up to gNB implementation to configure Rel-16 UL skipping or Rel-15 UL skipping or no skipping. No specification change is needed.

# Reference

[1] 3GPP TS38.321, “Technical Specification Group Radio Access Network; NR; Medium Access Control (MAC) protocol specification”

# Annex: text proposals

The following specification text changes are examples on how to capture the above proposals in the MAC specification.

##### 5.4.3.1.3 Allocation of resources … The MAC entity shall: 1> if the grant indicated to the HARQ entity is a configured uplink grant configured for mode-B UE-initiated beam reporting: 2> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. …