3GPP TSG-RAN WG2 Meeting #130											R2-2503656

Saint Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	8.3.2

Source: 	Sharp

Title:  	Discussion on Inference Reporting for AI/ML-based Mobility

Document for: 	Discussion

# Introduction

In RAN2#129bis, the following agreements were made related to LCM aspects for AI/ML mobility.

Agreements 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2 Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model based LCM) => Start by discussing spec/ran2 impact of the agreed cases/subcases and then discuss functionality granularity = > The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference 5 For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details 6 For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements. FFS details 7 Baseline is that this applies to all A1-6 events, unless technical problems are identified. Baseline quantity is RSRP.

Agreements

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

- For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message.

- FFS whether association ID is required and should provide motivation on per use case bases

- As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

- Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.   Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

- Model transfer will not be discussed in this study item

7	For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring.

FFS metrics per use case

# Discussion

It was agreed during RAN2#129bis [1], that the general LCM framework for beam management can be the baseline for AI/ML for applicable functionality reporting, as shown in Fig. 1. After the given signalling exchange between the network and the UE, it can be assumed that AI/ML mobility functionality maybe activated for the UE-sided models for RRM measurement predictions and mobility event predictions. It was also agreed that the RRM measurement framework can be used as the baseline for inference configuration and reporting for AI/ML mobility, with all measurement events from A1 to A6.

Figure 1: Signalling flow for applicable functionality reporting

Observation 1: AI/ML mobility functionalities can be activated using the LCM framework developed for beam management as the framework.

Observation 2: The measurement events A1 to A6 can be considered as the candidate mobility events for AI/ML-based mobility functionality.

It is agreed in the SID [2] that any decisions on handovers are made by the network, in case UE-sided AI/ML mobility functionality is activated for a UE. Once the functionality is activated, it is essential that the UE is appropriately configured with the prediction-related measurement events for triggering measurement reports. Predicted measurements and associated events inherently differ from legacy measurement results/events. Therefore, further considerations during the study on whether the existing measurement reporting frameworks are sufficient for UE-sided AI/ML functionality, even though the predicted results may be in the same format (RSRP) as the legacy measurements. This is required to enable the network to differentiate clearly between actual measurements and predicted measurements. Further, the measurement events (e.g. A3) corresponding to actual measurements and predicted measurement events (e.g. predicted A3 event) based on predicted measurement results would also require similar differentiation when being reported by the UE.

Observation 3: Measurement result predictions and measurement event predictions are inherently different from the corresponding legacy (or actual) measurement results, even though the two entities might represent the same quantities, e.g. RSRP and predicted RSRP or legacy A3 event and predicted A3 event.

Proposal 1: RAN2 to discuss the relationship between legacy measurement results and events, and the corresponding predicted measurements and events.

For legacy measurement events (e.g. A3) and corresponding reporting procedures, the measurement results obtained by the UE for its current serving cell and/or any detected neighboring cells are compared against parameters configured for the UE by the network. For example, the A3 event is specified [3] using the following measurement reporting conditions, where Mn and Mp represent the measurement results for the neighboring cell and SpCell respectively. The associated offsets (e.g. Ofn, Ocn) and hysteresis parameters are configured by the network.

Legacy A3 event (Neighbour becomes offset better than SpCell)

Inequality A3-1 (Entering condition)

Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off

Inequality A3-2 (Leaving condition)

Mn + Ofn + Ocn + Hys < Mp + Ofp + Ocp + Off

With RRM measurement predictions (temporal and/or spatial domain), the UE may have predictions available for the SpCell and/or neighboring cell(s), which maybe utilized by the UE to evaluate the associated predicted measurement events. Therefore, considering legacy and predicted measurement results (and events) together for UE mobility requires precise definitions for UE measurement reporting.

Observation 4: RRM measurement predictions for SpCell and neighboring cells may be utilized by the UE to evaluate predicted measurement events.

Observation 5: Considering prediction results for UE mobility along with legacy measurement results requires clear definitions for the prediction-based measurement reporting criteria.

For example, the predicted A3 event may be formulated in relation to the legacy A3 event as follows.

Predicted A3 event (prediction of neighbour becomes offset better than prediction of SpCell), which utilizes predicted measurements, based on observed measurements.

Inequality A3-3 (Entering condition for predictions)

Pred(Mn) + Ofn + Ocn – Hys > Pred(Mp) + Ofp + Ocp + Off

Inequality A3-4 (Leaving condition for predictions)

Pred(Mn) + Ofn + Ocn + Hys < Pred(Mp) + Ofp + Ocp + Off

Similar definitions for measurement report triggering criteria related to other mobility events (A1, A2, etc.) may also be considered, whereby any prediction results available to the UE may be utilized appropriately for generating the measurement reports to be sent by the UE to the network.

Proposal 2: Predicted measurement events and respective conditions for measurement report triggering are defined in relation to legacy measurement events (e.g. A3 event and predicted A3 event).

As agreed previously, the UE can be configured by the network to predict mobility events within a certain prediction window, based on the observed measurement results in the corresponding observation window. It was also agreed that the predicted mobility events need to follow similar rules as legacy events, such as the relevant Time-to-Trigger (TTT) to be observed. Therefore, the legacy measurement reporting criteria can be extended to also consider prediction-based information, when the necessary AI/ML functionalities are available/configured/activated to the UE.

Proposal 3: The legacy measurement report events and the corresponding triggering criteria configurable to UEs can be extended to also consider prediction-based information when available, with similar parameters (TTT, hysteresis, offsets) as the legacy measurement events.

# Conclusions

In this contribution, we have the following observations:

Observation 1: AI/ML mobility functionalities can be activated using the LCM framework developed for beam management as the framework.

Observation 2: The measurement events A1 to A6 can be considered as the candidate mobility events for AI/ML-based mobility functionality.

Observation 3: Measurement result predictions and measurement event predictions are inherently different from the corresponding legacy (or actual) measurement results, even though the two entities might represent the same quantities, e.g. RSRP and predicted RSRP or legacy A3 event and predicted A3 event.

Observation 4: RRM measurement predictions for SpCell and neighboring cells may be utilized by the UE to evaluate predicted measurement events.

Observation 5: Considering prediction results for UE mobility along with legacy measurement results requires clear definitions for the prediction-based measurement reporting criteria.

Based on the observations, the following proposals are made.

Proposal 1: RAN2 to discuss the relationship between legacy measurement results and events, and the corresponding predicted measurements and events.

Proposal 2: Predicted measurement events and respective conditions for measurement report triggering are defined in relation to legacy measurement events (e.g. A3 event and predicted A3 event).

Proposal 3: The legacy measurement report events and the corresponding triggering criteria configurable to UEs can be extended to also consider prediction-based information when available, with similar parameters (TTT, hysteresis, offsets) as the legacy measurement events.

# References

[1] Chair Notes RAN2#129bis

[2] RP-240082 Revised SID on AIML for mobility in NR.

[3] TS 38.331