3GPP TSG-RAN WG2 Meeting #130	R2-2503661

St. Julian’s, Malta, May 19th – 23rd, 2025

Source:	CATT

Title:	Analysis on LP-WUS for RRC_CONNECTED

Agenda Item:	8.4.4

Document for:	Discussion and Decision

Introduction

In previous RAN1 meetings and RAN2 meetings, great progress was made for LP-WUS in RRC_CONNECTED mode. Based on the agreements, we will continue to analyze the LP-WUS monitoring procedure in RRC_CONNECTED mode. And our observations and proposals are provided.

Discussion

2.1	RRC-3/MAC-1: whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group

Based on the open issues listed in [1] and [2], whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group in this section.

RRC:

| RRC-3 | whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group Editor’s NOTE: FFS on whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group. | Issue Type: not essential but important How to address it: can be discussed based on companies’ contribution |
| --- | --- | --- |

MAC:

Open issue 1: Whether/How to support the LP-WUS (including Option 1-1 and 1-2) and dual DRX group.

In order to reduce the power consumption when both FR1 and FR2 cells are configured via Carrier Aggregation, a secondary DRX group was introduced with separate drx-InactivityTimer and drx-onDurationTimer configured for the FR2 cells enabling FR2 to go to sleep more quickly. However, it is FFS whether LP-WUS can be configured with dual DRX group.

Based on RAN1 progress, it can be seen that there were some basic conclusion for the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode for Option 1-1. Considering the LP-WUS configured with dual group is beneficial on power saving, it is proposed that:

Proposal 1A: (RRC-3/MAC-1) For Rel-19 LP-WUS, the dual DRX group and LP-WUS can be configured simultaneously for Option 1-1.

Regading Optition 1-2, it can follow Option 1-1 to support simultaneous configuration of dual DRX group and LP-WUS.

Proposal 1B: (RRC-3/MAC-1) For Rel-19 LP-WUS, the dual DRX group and LP-WUS can be configured simultaneously for for Option 1-2.

- 2.1.1	Option 1-1

In RAN1#120bis meeting[3], regarding the details of LP-WUS configuration with dual DRX group, RAN1 made the following conclusion for Option 1-1:

Conclusion For the case when UE is configured with CA with dual DRX groups in RRC CONNECTED mode, at least for LP-WUS procedure Option 1-1, there is no consensus in RAN1 which of the following options to support for this case in Rel-19. - Alt1 - Basic capability - LP-WUS can be configured only on PCell - LP-WUS indication is applicable to all serving cells of all DRX groups. - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - Advanced capability - LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has prerequisite of the above basic capability - Alt2 - LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group - LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored - Note: There is no impact to PDCCH monitoring behaviour related to SCell dormancy/activation/deactivation - This capability has candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}

In the following, the alternatives concluded in RAN1 are analyzed.

Basic capability in Alt1

Regarding basic capability in Alt 1, the LP -WUS can be configured only on PCell, but LP-WUS indication is applicable to all serving cells of all DRX groups. In this case, the length of drx-onDurationTimer and drx-inactivityTimer can be configured separately for different DRX groups.

In this option, the drx-DurationTimer of both DRX groups is triggered to be started when LP-WUS indicating the UE to wake up. It is less flexible on drx-onDurationTimer control but friendly to resource consumption. Eventhough both DRX groups need to be wakep by LP-WUS, but the power saving of both DRX group can still be achieved by LP-WUS.

Observation 1: Regarding basic capalility in Alt1 in Option 1-1, it is simple with less resource consumption but less flexible.

Figure 1: Basic capability in Alt1

Advanced capability in Alt1

For advanced capability in Alt1, the LP-WUS can be configured on PCell for primary DRX group and an activated SCell for secondary DRX group and LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored. In this case, the drx-onDurationTimer in each DRX group can be trggered by corresponding LP-WUS to start separately. I.e., when the UE receives LP-WUS which is configured on PCell, the UE starts the drx-onDurationTimer of the primary DRX group and when the UE recieves LP-WUS which is configured on SCell, the UE starts drx-onDurationTimer of the secondary DRX group. And even though the UE indicates that it supports advanced capability in Alt1, the network can also configures the LP-WUS on PCell to control both DRX groups considering the resoure consumptions.

Figure 2: Advanced capability in Alt1

It is friendly to the control flexibility but there are some other issues.

Firstly, it brings extra resource consumption on LP-WUS since 2 sets of LP-WUS resources need to be configured for each DRX groups.

Secondly, the power saving gain is questionable for advanced capability in Alt1. Considering the UE is allowed to monitor LP-WUS configured for the primary DRX group and secondary DRX group separately, it may happen that one DRX group is activated while the other is deactivated for sake of power saving. However, even only one cell group is activated, the MR is switched on, in which case, the power saving gain brought by LP-WUS is omitted.

Thirdly, it may bring complexities on SCell activation/deactivation procedure. For advanced capability in Alt1, it was concluded that the LP-WUS was configured on an activated SCell for secondary DRX group. But then the following issues are identified:

- Issue 1: Whether the SCell configured with LP-WUS can be deactivated;

- Issue 2: If the SCell configured with LP-WUS is allowed to be deactivated, what is the UE behavior.

For issue 1, it can be up to the network implementation to keep the SCell always activated, but it brings restriction on network implementation. Alternatively, the SCell configured with LP-WUS can be deactivated. One option is like PUCCH SCell, i.e., the network ensures that while SCell configured with LP-WUS is deactivated, SCells of secondary DRX group should not be activated. Or when the SCell configured with LP-WUS is deactivated, the LP-WUS monitoring procedure fallbacks to the basic capability in Alt1, i.e., the LP -WUS configured on PCell is applicable to all serving cells of all DRX groups. No matther which solution is chosen, additional complexities on SCell activation/deactivation are introduced.

Observation 2: Advanced capability in Alt1 shows the following properties:

- It is friendly to the control flexibility;

- It brings extra resource consumption on LP-WUS;

- The power saving gain brought by LP-WUS is questionable;

- There are some issues on activation/deactivation for the SCell configured LP-WUS, e.g., whether the SCell configured with LP-WUS can be deactivated and if the SCell configured with LP-WUS is allowed to be deactivated, what is the UE behavior.

Alt2

For Alt2, LP-WUS can be configured on PCell for primary DRX group and/or an activated SCell for secondary DRX group and LP-WUS indication is applicable to all serving cells in the DRX group where LP-WUS is monitored. Besides, the UE can indicate the capability with candidate values for the LP-WUS configuration with {primary DRX group, secondary DRX, both}. For example, when the UE indicates that it supports secondary DRX group, the LP-WUS can only be configured on SCell of the secondary DRX group. Its configuration is the more flexbile compared with Alt1. And the resource comsuption depends on the UE capability indication. But the power saving gain may not be as large as Alt1 especially when the UE only supports LP-WUS for primary DRX group or secondary DRX group.

Observation 3: Alt2 shows more flexiblity on configuration but less power saving gain especially when the UE only supports LP-WUS for primary DRX group or secondary DRX group.

According to Observation 1-3, we think the power saving gain of LP-WUS is more critical and basic capability in Alt1 shows better performance on power saving. Hence, it is proposed that:

Proposal 2: (RRC-3/MAC-1) For Option 1-1, support basic capability of Alt1, i.e., LP-WUS can be configured only on PCell and LP-WUS indication is applicable to all serving cells of all DRX groups when dual DRX group is configured simultaneously with LP-WUS.

- 2.1.2	Option 1-2

For option 1-2, the UE starts the new timer when it receives LP-WUS. It is more flexible on PDCCH monitoring control. For simplicity, we think the same mechanism of Option 1-1 can be reused for Option 1-2 for dual DRX group.

Proposal 3: (RRC-3/MAC-1) For Option 1-2, support LP-WUS can be configured only on PCell and LP-WUS indication is applicable to all serving cells of all DRX groups when dual DRX group is configured simultaneously with LP-WUS.

According to the open issues listed in [2], it is still FFS on whether whether the maintenance of lpwus_PDCCHMonitoringTimer is per DRX group or per MAC entity and whether lpwus_PDCCHMonitoringTimer is configured per DRX group or common to DRX groups which is highlighted in yellow.

Editor’s NOTE: FFS whether the maintenance of lpwus_PDCCHMonitoringTimer is per DRX group or per MAC entity. Editor’s NOTE: FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group. Editor’s NOTE: FFS whether lpwus_PDCCHMonitoringTimer is configured per DRX group or common to DRX groups

If Proposal 4 is agreed, different values of the new timer, i.e., lpwus_PDCCHMonitoringTimer can be configured per DRX group. And lpwus_PDCCHMonitoringTimer of each DRX group is triggered by LP-WUS on the PCell but with different values, it should be maintained per DRX group individually.

Proposal 4: For Option 1-2, lpwus_PDCCHMonitoringTimer can be configured per DRX group when dual DRX group is configured simultaneously with LP-WUS.

Proposal 5: For Option 1-2, lpwus_PDCCHMonitoringTimer can be maintained per DRX group when dual DRX group is configured simultaneously with LP-WUS.

2.2	UE behavior for the case of potential collision of LP-WUS

- 2.2.1	Option 1-1 with dual DRX group

For basic capability in Alt1, since the drx-onDurationTimer and drx-inactivityTimer of the primary DRX group is configured with larger values comparing with the timers of the secondary DRX group, it may happen that the primary DRX group is in DRX Active time while the secondary DRX group isn’t in DRX Active Time. In this case the UE does not monitor LP-WUS. Hence, the cell(s) belonging to secondary DRX group can’t be waked up by LP-WUS when the primary DRX group is in DRX Active time.

Acccording to TS 38.331[4], if drx-ConfigSecondaryGroup is configured in the MAC-CellGroupConfig associated with this serving cell, the scheduling cell and the scheduled cell belong to the same Frequency Range. I.e., the network can’t schedule the cell(s) belonging to FR2 using the PDCCH in FR1 when drx-ConfigSecondaryGroup is configured. Hence, the cell(s) belonging to secondary DRX group can’t be waked up by cross carrier scheduling either.

And the solution can follow the WA agreed in RAN2#129bis meeting:

Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2.

I.e., when the LP-WUS configured on PCell can’t be monitored, the UE starts the drx-onDurationTimer of both primary and secondary DRX group for Alt1 Basic capability of Option 1-1.

Proposal 6: For Basic capability of Alt1 in Option 1-1, the UE starts the drx-onDurationTimer of both primary DRX group and the secondary DRX group, when LP-WUS configured on PCell can’t be monitored.

- 2.2.1	Option 1-2

It was still FFS on the UE behavior for the case of potential collision in Option 1-2. Generally, the following options are provided:

- Option 1: the UE starts the new timer when the UE is not able to monitor the LP-WUS occasion(s);

- Option 2: no new UE behavior, i.e., the UE monitors LP-WUS after the potential collision and starts the new timer when LP-WUS indicates;

- Option 3: based on RRC configuration to start the new timer, i.e., if it is configured to start the new timer, the UE goes Option 1; otherwise, the UE goes Option 2.

Regarding which option is selected greatly depends on the periodicity of LP-WUS in Option 1-2. If the candidate values of the periodicity are comparable with DRX cycle, Option 1 is preferred. However, if the candidate values of LP-WUS periodicity are much less than that of the DRX cycle, Option 2 is better. Considering the discussion on candidate value of the LP-WUS is in RAN1 scope and RAN2 has sent LS to RAN1. RAN2 can wait for RAN1 progress.

Like Option 1-1, it may happen that when dual DRX group is configured and the LP-WUS configured on PCell can’t be monitored, whether/how the UE starts the new timer of both DRX groups. We think we can wait for RAN1 progess.

Proposal 7: In Option 1-2, RAN2 waits RAN1 to decide what is the UE behavior with/without dual DRX group configuration, when the UE is not able to monitor the LP-WUS occasion(s).

2.3	LP-WUS enabling/disabling and activation/deactivation

- 2.3.1	Enabling/disabling

In RAN1#115 meeting, RAN1 it was agreed that:

RAN1#115 Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance.

For RRC CONNECTED mode, from RAN1 perspective, PDCCH monitoring triggered by LP-WUS is enabled or disabled by gNB RRC signaling. But it is FFS whether to support UE assistance to enable/disable PDCCH monitoring triggered by LP-WUS. In our understanding, the gNB can determine when/whether to enable/disable PDCCH monitoring triggered by LP-WUS based on UE capability and the network load. Hence, no UE assistance information is needed.

Proposal 8: UE assistance is not needed to enable/disable PDCCH monitoring triggered by LP-WUS.

- 2.3.2	Activation/deactivation

In RAN1 previous meetings, it was agreed that:

RAN1#118 meeting Agreement - In RRC CONNECTED mode, LP-WUS monitoring can be activated/deactivated by at least one or more of - by gNB RRC signaling, with or without UE assistance. - by gNB L1/L2 LP-WUS activation/deactivation signaling, with or without UE assistance. - based on pre-configured condition(s), such as timer. - LP-WUS monitoring by UE is known to gNB, study whether it could be transparent to gNB. - other options are not precluded.

RAN1#119 meeting [Wed]Proposal 4-1: When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE fallback to PDCCH monitoring when UE monitors LP-WUS - FFS the condition for the fallback - For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior - For LP-WUS operation option 1-2, - FFS what UE fallbacks to - FFS how to avoid misalignment between gNB and UE on the UE LP-WUS/PDCCH monitoring

According to RAN1 progress, there is no further progress regarding LP-WUS Activation/deactivation from RAN1. Considering this late stage and the complexity of activation/deactivation procedure. It is proposed that

Proposal 9: LP-WUS activation, deactivation, or fallback to PDCCH monitoring is not supported in Rel-19.

Conclusion

In this document, we analyze the remaining issues in RRC_CONNECTED for LP-WUS, and the following and proposals are provided:

Dual DRX group

Proposal 1A: (RRC-3/MAC-1) For Rel-19 LP-WUS, the dual DRX group and LP-WUS can be configured simultaneously for Option 1-1.

Proposal 1B: (RRC-3/MAC-1) For Rel-19 LP-WUS, the dual DRX group and LP-WUS can be configured simultaneously for for Option 1-2.

Observation 1: Regarding basic capalility in Alt1 in Option 1-1, it is simple with less resource consumption but less flexible.

Observation 2: Advanced capability in Alt1 shows the following properties:

- It is friendly to the control flexibility;

- It brings extra resource consumption on LP-WUS;

- The power saving gain brought by LP-WUS is questionable;

- There are some issues on activation/deactivation for the SCell configured LP-WUS, e.g., whether the SCell configured with LP-WUS can be deactivated and if the SCell configured with LP-WUS is allowed to be deactivated, what is the UE behavior.

Observation 3: Alt2 shows more flexiblity on configuration but less power saving gain especially when the UE only supports LP-WUS for primary DRX group or secondary DRX group.

Proposal 2: (RRC-3/MAC-1) For Option 1-1, support basic capability of Alt1, i.e., LP-WUS can be configured only on PCell and LP-WUS indication is applicable to all serving cells of all DRX groups when dual DRX group is configured simultaneously with LP-WUS.

Proposal 3: (RRC-3/MAC-1) For Option 1-2, support LP-WUS can be configured only on PCell and LP-WUS indication is applicable to all serving cells of all DRX groups when dual DRX group is configured simultaneously with LP-WUS.

Proposal 4: For Option 1-2, lpwus_PDCCHMonitoringTimer can be configured per DRX group when dual DRX group is configured simultaneously with LP-WUS.

Proposal 5: For Option 1-2, lpwus_PDCCHMonitoringTimer can be maintained per DRX group when dual DRX group is configured simultaneously with LP-WUS.

UE behavior for the case of potential collision of LP-WUS

Proposal 6: For Basic capability of Alt1 in Option 1-1, the UE starts the drx-onDurationTimer of both primary DRX group and the secondary DRX group, when LP-WUS configured on PCell can’t be monitored.

Proposal 7: In Option 1-2, RAN2 waits RAN1 to decide what is the UE behavior with/without dual DRX group configuration, when the UE is not able to monitor the LP-WUS occasion(s).

LP-WUS enable/disable and activation/deactivation

Proposal 8: UE assistance is not needed to enable/disable PDCCH monitoring triggered by LP-WUS.

Proposal 9: LP-WUS activation, deactivation, or fallback to PDCCH monitoring is not supported in Rel-19.

Reference

- R2-2503765, Discussion summary and list of RRC open issue for LP-WUS WUR;

- R2-2503808, [Post129bis][210][LPWUS] Running CR for TS 38.321 (Apple)

- R1-2503201, Report of RAN1#120bis meeting;

- TS 38.331, Radio Resource Control (RRC) protocol specification.