3GPP TSG-RAN WG2 Meeting #130	           R2-2503668

St.Julians, Malta, May 19th – May 23rd, 2025

Agenda item:	8.13.2

Source:	China Telecom

Title:	Discussion on multi-hop relay reselection

Document for:	Discussion

- Introduction

In this paper, we would like to discuss the left issues of multi-hop relay reselection.

- Discussion

RAN2 has the following agreements on multi-hop Relay UE (re)selection:

RAN2#129 For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE. For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition: - PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold. - When U2N Remote UE receives a PC5-S link release message from current first relay UE. - When U2N Remote UE detects PC5 RLF with the current first relay UE. - Indicated by upper layer. L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design): - cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network. - release of RRC connection between an intermediate relay UE and the network. - PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s). - FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE). - FFS on the signalling contents of the PC5-RRC indication from the first relay UE. The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. FFS upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE. Upon intermediate relay UE performing relay (re)selection, it can notify its child node. If the cause of (re)selection already causes a notification, there is no double-trigger of the notification. FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger) RAN2#129bis Notification for Link quality degradation, as distinct from existing notification causes, is not supported (no spec impact expected). When the intermediate relay UE receives a notification message from the last relay UE indicating a failure on Uu, the intermediate relay UE may transmit a notification message downstream (towards the remote UE). FFS if the notification message is a forwarded copy of the original notification (same cause) or a regenerated message from the intermediate relay (cause might reflect its own reaction such as relay reselection or re-establishment). This does not change the agreement that it is up to relay implementation whether to release the downstream link, and if the link is released there is no downstream notification message. RAN2 will focus solely on the L2 Multi-hop Relay design within the scope of this work item. This does not exclude that some work (e.g., discovery conditions, transmission resources) could be reused in L3 without specific impact under our WI. When the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE. FFS detailed information in the indication and child UE handling. FFS if there are exceptional cases where the indication can be suppressed, e.g., reselection under the same serving cell without changing the hop count. FFS if the notification message is used or we rely on upper layer signalling (e.g., discovery).

According to RAN2 agreements, when the parent UE in idle/inactive performs reselection, RAN2 assume that an indication of the reselection is needed for the child UE. Regarding RRC_CONNECTED parent UE, when the parent UE performs path switch, an indication is also needed for the child UE.

Proposal 1: If the first U2N relay UE performs path switch, send Notification message to notify the R19 U2N remote UE.

There may be some new scenarios. In previous RAN2 discussion, we always talk about multi-hop U2N Remote UE (i.e. R19 U2N Remote UE). However, the multi-hop U2N Relay UE (i.e. R19 U2N Relay UE) may also serve single-hop U2N Remote UE (i.e. R17 U2N Remote UE) as described in figure 1, which is straightforward. The R19 U2N Relay UE may be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED. When the R19 U2N Relay UE A moves, it may perform relay selection or path switch from the serving gNB to the U2N Relay UE B.

Figure 1.  Relay UE select/switch from gNB to parent UE

Observation 1: It is possible that R17 U2N Remote UE communicate with the network via a single R19 U2N Relay UE. And the R19 U2N Relay UE may select/switch from gNB to another U2N Relay UE.

According to previous agreements, upon intermediate relay UE performing relay (re)selection, it can notify its child node. However, if the child UE is R17 Remote UE, it can’t understand new causes (such as path switch, relay selection) in notification message. In case of that, the R19 U2N Relay UE A should send PC5-S link release message to the R17 Remote UE instead of notification, which is understandable for the R17 Remote UE.

Proposal 2: If the first U2N relay UE performs path switch or relay selection, it should trigger PC5-S link release toward the R17 U2N remote UE.

3	Conclusions

Based on the discussion above, we have the following proposals.

Proposal 1: If the first U2N relay UE performs path switch, send Notification message to notify the R19 U2N remote UE.

Observation 1: It is possible that R17 U2N Remote UE communicate with the network via a single R19 U2N Relay UE. And the R19 U2N Relay UE may select/switch from gNB to another U2N Relay UE.

Proposal 2: If the first U2N relay UE performs path switch or relay selection, it should trigger PC5-S link release toward the R17 U2N remote UE.