3GPP TSG-RAN WG2 Meeting #130	R2-2503670

Malta, MT, 19th – 23rd May, 2025

Agenda item:	8.1.3

Source:	China Telecom

Title:	Discussion on NW side data collection

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

In last RAN2#129bis meeting, the following agreements were made with several FFSs:

- All agreements for NW side data collection - Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO. Source gNB decides whether the data should be kept. The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell). FFS signaling details. - Upon going to RRC_IDLE, RLF, or RRC_INACTIVE, UE discards any logged data - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined - The measurement configuration of AI/ML data collection can configure measurements for multiple sets of resources and use cases (e.g. BM, Mobility, etc) - For temporal domain, the network is made aware whether there is a gap between two consecutive samples. FFS amount of gap and whether this is implicit or explicit - New SRB can be configured for NW-side data collection (with lower priority)

In this contribution, we will discuss the remaining issues for NW side data collection.

# 2 Discussion

## 2.1 Data availability indication and cause value

In last RAN2#129bis meeting, the following agreements were made

Agreements on availability indication - Availability indication can be triggered due to: - Full buffer being reached (if configured) - Buffer threshold being reached (if configured). - Low power (if configured) - The UE send a UAI that indicates: - Data is available - Reason for trigger (full buffer, threshold) - Low power indication - The encoding of the data is available/UAI and the cause value is FFS NOTE: it is up to UE Implementation how buffer threshold reached and low power is determined

According to the agreements, the low power indication is not included in the reason for trigger. It is not yet clear should the low power indication, buffer full, and buffer threshold reached indications be separated or combined into a single bit. From our side, separate bits provide granularity, allowing the network to distinguish between different trigger conditions (e.g., low power, buffer full). In addition, it supports potential future extensions if additional trigger types are defined.

Proposal 1: The cause value for triggering the data availability, including low power, buffer full being reached and buffer threshold being reached, can be encoded as separate bits.

It is not yet clear whether, when and how the UE indicates that it is no longer in a low power state or with full buffer. In our opinion, indicating these states (e.g., exiting low power or full buffer state) could be useful for the network to resume normal data collection and resource allocation. Therefore, updating the low power indication and buffer full being reached indication can be supported. If implemented, the UE might send updated indication:

- Immediately upon exiting the low power or full buffer state.

- Periodically, if configured.

- On-demand, in response to a network query about the UE's state.

From our side, UE need proactively report the power and memory state. Therefore, UE might send updated indication immediately upon exiting the low power or full buffer state.

Proposal 2: Support updating the low power indication and buffer full being reached indication after UE exiting the low power or full buffer state.

Proposal 3: If data logging is stopped due to lower power state or full buffer, support restarting data logging after UE exiting the low power or full buffer state.

## 2.2 UE retains logged data during handover (HO)

In last RAN2#129bis meeting, the following agreements were made:

Introduce 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration during/before HO.  Source gNB decides whether the data should be kept.  The indication is provided in RRCReconfiguration (i.e. not in RRC Reconfiguration from target cell).   FFS signaling details.

It should be clarified whether/how the 1-bit indication can be sent during or before HO, taking into account that the source gNB decides whether the data should be kept. From our side, there are two possible solutions for this issue:

- Send indication before HO: The source gNB sends the 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration to UE before HO.

  - When the source gNB receives the data availability indication, it can indicate the UE to release or retain data before sending the handover command. If there is sufficient time, the source gNB may still try to fetch the data.

  - If the source gNB indicates UE to retain data, the UE might send the data availability indication included in the RRCReconfigurationComplete to target gNB.

  - After receiving the data availability indication from the UE, the target gNB might fetch the data from the UE.

  - After fetching the data from the UE, the target gNB might forward it to the source gNB or OAM.

- Send indication during HO: The source gNB decides if the 1-bit indication on whether to release or retain un-retrieved data is needed and, if so, sends it to the target gNB, which includes it in the RRCReconfiguration sent to the UE during HO.

  - When the source gNB decides that the 1-bit indication is needed, it can send the indication to the target gNB.

  - Then, the target gNB can include the indication in the RRCReconfiguration sent to the UE during HO to indicate the UE to release or retain data.

  - If the UE is indicated to retain the data, the target gNB might fetch the data from the UE.

  - After fetching the data from the UE, the target gNB might forward it to the source gNB or OAM.

Proposal 4: The source gNB sends the 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration to UE before HO.

Proposal 5: If the source gNB sends the 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration to UE before HO, the details are as follow:

- The source gNB receives the data availability indication, and indicates the UE to release or retain data before sending the handover command. If there is sufficient time, the source gNB may still try to fetch the data.

- If the source gNB indicates UE to retain data, the UE might send the data availability indication included in the RRCReconfigurationComplete to target gNB.

- After receiving the data availability indication from the UE, the target gNB might fetch the data from the UE.

- After fetching the data from the UE, the target gNB might forward it to the source gNB or OAM.

Proposal 6: The source gNB sends the 1-bit indication on whether to release or retain un-retrieved data to the target gNB during. The target gNB includes the indication in the RRCReconfiguration and send it to the UE during HO.

Proposal 7: If the source gNB decides and sends the 1-bit indication on whether to release or retain un-retrieved data to the target gNB, which includes it in the RRCReconfiguration sent to the UE during HO, the details are as follows:

- When the source gNB decides that the 1-bit indication is needed, it can send the indication to the target gNB.

- Then, the target gNB can include the indication in the RRCReconfiguration sent to the UE during HO to indicate the UE to release or retain data.

- If the UE is indicated to retain the data, the target gNB might fetch the data from the UE.

- After fetching the data from the UE, the target gNB might forward it to the source gNB or OAM.

# 3 Conclusion

In this contribution, we further discuss the remaining issues related to data availability for NW-side data collection. We have the following proposals:

Data availability indication and cause value

Proposal 1: The cause value for triggering the data availability, including low power, buffer full being reached and buffer threshold being reached, can be encoded as separate bits.

Proposal 2: Support updating the low power indication and buffer full being reached indication after UE exiting the low power or full buffer state.

Proposal 3: If data logging is stopped due to lower power state or full buffer, support restarting data logging after UE exiting the low power or full buffer state.

UE retains logged data during handover (HO)

Proposal 4: The source gNB sends the 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration to UE before HO.

Proposal 5: If the source gNB sends the 1-bit indication on whether to release or retain un-retrieved data in RRCReconfiguration to UE before HO, the details are as follow:

- The source gNB receives the data availability indication, and indicates the UE to release or retain data before sending the handover command. If there is sufficient time, the source gNB may still try to fetch the data.

- If the source gNB indicates UE to retain data, the UE might send the data availability indication included in the RRCReconfigurationComplete to target gNB.

- After receiving the data availability indication from the UE, the target gNB might fetch the data from the UE.

- After fetching the data from the UE, the target gNB might forward it to the source gNB or OAM.

Proposal 6: The source gNB sends the 1-bit indication on whether to release or retain un-retrieved data to the target gNB during. The target gNB includes the indication in the RRCReconfiguration and send it to the UE during HO.

Proposal 7: If the source gNB decides and sends the 1-bit indication on whether to release or retain un-retrieved data to the target gNB, which includes it in the RRCReconfiguration sent to the UE during HO, the details are as follows:

- When the source gNB decides that the 1-bit indication is needed, it can send the indication to the target gNB.

- Then, the target gNB can include the indication in the RRCReconfiguration sent to the UE during HO to indicate the UE to release or retain data.

- If the UE is indicated to retain the data, the target gNB might fetch the data from the UE.

- After fetching the data from the UE, the target gNB might forward it to the source gNB or OAM.