3GPP TSG-RAN WG2 Meeting #130	R2-2503681

St Julian, Malta, 19th – 23rd May 2025

Agenda item:	8.6.2

Source: 	China Telecom

Title:	Discussion on remaining issues of Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last few RAN2 meetings, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made. Regarding inter-CU LTM, the following agreements were reached in last RAN2 meeting.

Agreements on inter-CU LTM:

- For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

- NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

- From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

- MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

- An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

However, there are still some open issues on the details of inter-CU LTM in both non-DC and DC scenarios. In this contribution, we would like to share our opinions on the following remaining issues for inter-CU LTM.

- MCG LTM with SN addition;

- Fast recovery handling for inter-CU LTM.

- Discussion

MCG LTM with SN addition

In previous RAN2 meeting, the following scenarios were agreed to be supported for inter-CU MCG LTM.

Agreements in RAN2#127

SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

Agreements in RAN2#128

RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

As per the comments received in the post meeting discussion on 37.340 running CR, it seems that there are different understandings on the following agreement made in RAN2#128.

RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

Understanding 1: Only MCG LTM with the same SN addition is supported, which means the SN is released before execution of LTM and then added during the MCG LTM.

Understanding 2: MCG LTM with SN addition is supported with no restriction on the same SN before LTM.

In the latest 37.340 running CR, it is implemented with Understanding 2 and an Editor’s Note is added for further discussion.

Inter-MN or Intra-MN MCG LTM with SCG addition, SCG release or configuration change without SN change is supported. Inter-MN or intra-MN MCG LTM with SN changed to another SN is not supported. Editor’s Note: FFS on whether to have the restriction on only supporting MCG LTM with SN addition within the same SN.

According to the agreement made in RAN2#128, inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. In the current running CR, there is no restriction on whether to only support MCG LTM with SN addition within the same SN. As illustrated in Fig.1(copied from Fig.10.7.2-2 in the running CR R2-2503679), for an inter-CU MCG LTM without SN change, the source SN and the target SN shown in Fig.1 are the same node. For an inter-CU MCG LTM with SN addition, the source SN and steps involving the source SN in Fig.1 are ignored. However, if there is restriction on only supporting MCG LTM with SN addition within the same SN, it will be hard to distinguish which steps shall be ignored, since the target SN is the previous source SN. If we go for Understanding 1, it will bring extra spec impact and more complexity to gNB implementation to guarantee the same SN is involved. Thus, we suggest keep the current version and delete the Editor’s note.

Proposal 1: Regarding MCG LTM with SN addition, RAN2 to confirm that there is no restriction on only supporting MCG LTM with SN addition within the same SN. The related Editor’s note could be deleted.

Fig.1 An example flow chart for inter-MN MCG LTM with Secondary Node

Fast recovery handling for inter-CU LTM

In previous RAN2 meetings, the following agreements on fast recovery handling for inter-CU LTM were reached.

Agreements in RAN2#127bis

1. The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

Agreements in RAN2#128

1. RAN2 to postpone the discussion on fast recovery upon inter-CU LTM failure until inter-CU LTM security solution is finalized, if fast recovery is feasible.

Agreements in RAN2#129bis

At RLF and reconfiguration with sync failure:

- If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

- if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

Since the mix configuration of intra-CU and inter-CU candidate cell is supported in R19, the following fast recovery handling cases after reconfiguration failure could be considered.

- Case 1: After an intra-CU LTM failure, the UE selects an intra-CU LTM candidate cell;

- Case 2: After an intra-CU LTM failure, the UE selects an inter-CU LTM candidate cell;

- Case 3: After an inter-CU LTM failure, the UE selects an intra-CU LTM candidate cell;

- Case 4: After an inter-CU LTM failure, the UE selects an inter-CU LTM candidate cell.

Case 1 is actually what we have already agreed in RAN2#127bis, and the Rel-18 handling on failure recovery is reused. For Case 3, the selected candidate cell has the same Rel-19 set ID as the target, and it was agreed to be supported in last meeting. For Case 2 and 4, the selected candidate cell has a different Rel-19 set ID from the previous target cell. To support Case 2 and 4, certain enhancement is needed, which may lead to more RAN3 impact as well. However, considering the limited TU left, we suggest not to support fast recovery for Case 2 and 4, since the fast recovery is a best effort way for the radio link recovery. Even if we have perfect enhancement towards those cases, it is still possible to fail again. For those cases, we can just rely on the RRC re-establishment as usual.

Proposal 2: RAN2 confirms not to support the fast recovery with different Rel-19 set ID from the target.

- Conclusion

In this contribution, we discuss remaining issues on inter-CU LTM in Rel-19. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: Regarding MCG LTM with SN addition, RAN2 to confirm that there is no restriction on only supporting MCG LTM with SN addition within the same SN. The related Editor’s note could be deleted.

Proposal 2: RAN2 confirms not to support the fast recovery with different Rel-19 set ID from the target.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

R2-2503679	Introduction of NR mobility enhancements Phase 4 in TS 37.340