3GPP TSG-RAN WG2 Meeting #130	R2-2503693

Malta, May 19th – 23rd, 2025

Agenda Item:	8.3.2

Source:	InterDigital Inc.

Title:	Functionality management for UE sided model for RRM measurement prediction

Document for:	Discussion

# 1. Introduction

In RAN2-129bis, initial discussion on LCM and the spec impact for the AI/ML mobility use case were made [1]. In this contribution, we discuss some of the open issues regarding LCM and spec impact for UE sided model for RRM measurement prediction.

# 2. Discussion

### 2.1 Inference and reporting configuration

Regarding the inference configuration for the AIML mobility use case, the following was agreed in RAN2-129bis [1]:

- The inference configuration and reporting for AI/ML mobility can be based on RRM measurement framework

It was also agreed that all the RRM measurement prediction sub use cases will be considered. Additionally, both temporal and frequency domain cases are also agreed to be considered for the specification impact study, while the spatial case is pending.

The RRM measurement prediction sub-use cases listed below:

- Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

- Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

- Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Sub-use case 4: L1 filtered beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 beam-level measurement result is generated

- Sub-use case 5: L3 beam-level measurement result(s) is predicted based on actual L3 beam-level measurement result(s)

- Sub-use case 6: L3 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

Based on the required input for the model, the sub-use cases can be categorized as below:

  - L3 cell level measurements: sub-use case 2

  - L1 beam level measurements: sub-use cases 1, 3, 4 and 6

  - L3 beam level measurements: sub-use case 5

Based on the inference (or post processed inference) that is to be sent to the network, the sub-use cases can be categorized as below:

  - L3 cell level measurements: sub-use case 1, 2, 3

  - L3 beam level measurements: sub-use case 4,5,6

It can be seen that, except for sub-use case 2, the input required for the model is related to beam measurements. In RAN2-129bis, some companies have proposed for these cases, the CSI framework can be used for inference configuration ([3]). However, our understanding is that the set A/B configuration may be relevant only in the case of spatial domain prediction. For temporal and inter-frequency predictions, the exact beams from a cell(s) that the UE uses as an input for making the predictions can be left up to UE implementation (i.e., no specific set A/B configuration is needed). That is, the UE just needs to be configured with the cell(s) to use as an input and it can measure whatever beams of the cell(s) it can detect and use it to perform the predictions.

Observation 1: For temporal and frequency domain predictions, it is not necessary for the UE to be configured for set A/B configurations, even for the RRM sub-use cases where the input to the AI/ML model is L1 beam measurements.

For temporal domain Case A, the model predicts a prediction window duration of measurements in the future based on historical measurements (of observation window duration). Our understanding is that the observation window size is up to model implementation and only the prediction window length needs to be configured by the network (e.g., based on the prediction window length capability indicated by the UE).

Observation 2: For temporal domain case A, observation window duration is up to UE implementation.

For temporal domain Case B, the higher the MRRT, the lower the prediction accuracy is likely to be. So, the NW may need to have some control over the MRRT to be used by the UE (i.e., to ensure the HO performance will not degrade due to the UE using a high MRRT that leads to lower accuracy). The UE may realize the same MRRT using different skipping patterns (E.g., for 50% MRRT, the UE can skip every other measurement, or make two measurements and skip two measurements, etc.,). Even though the network may not necessarily need to know the skipping pattern, knowing the skipping pattern may enable more optimal operations (E.g., measurement off periods aligned with C-DRX, network not transmitting some UE specific reference signals when the UE is not performing measurements, etc.,).

Observation 3: For temporal domain case B, UE may use different skipping patterns to achieve the same level of MRRT. The network may not need to know the skipping patterns being used but knowing it will enable further UE/network energy saving.

Based on the discussion above, we propose the following regarding the required inference configurations for all RRM measurement prediction sub-use case and domains:

Proposal 1: For the temporal and frequency domain RRM measurement prediction, the inference input is provided at cell level for all sub-use cases (i.e., if the input to the model is beam measurements, it can be left to UE implementation to decide which beams of the cells to be used for the prediction.). FFS on how to configure inference input for the spatial domain.

Proposal 2: For temporal case A, the UE is configured with the prediction window length.

Proposal 3: For temporal case B, the UE is configured with the MRRT to use and may optionally be configured with a skipping pattern to apply to achieve the MRRT (e.g., among the skipping patterns supported by the UE).

Regarding inference output and reporting for the case of RRM measurement prediction for temporal case B and frequency domain predictions, the legacy RRM measurement reporting configuration can be reused as the aim is to reduce the measurements to be performed by the UE. However, for temporal case A, enhancements are needed to configure the UE regarding which predicted results the UE can report. For example, the UE may be configured to report the predicted samples within the whole predicted window, the last predicted sample, some statistical information of the samples within the predicted window, etc.

Proposal 4: For temporal domain case B and frequency domain RRM measurement prediction, legacy RRM measurement reporting configuration can be reused.

Proposal 5: For temporal domain case A RRM measurement prediction, enhancements are needed regarding the predicted measurements to be included in the measurement report. The details are FFS (e.g., number of samples or which samples within the prediction window to be reported).

### 2.2 Applicability determination and reporting

Regarding the inference configuration for the AIML mobility use case, the following was agreed in RAN2-129bis [1]:

- For AI/ML for mobility, UE can report the applicable functionalities to NW via UAI or RRCReconfigurationComplete message. FFS whether association ID is required and should provide motivation on per-use case bases

Simulations results regarding model generalization across different network configurations were discussed in RAN2-129bis and there was a consensus that the models generalized well across the considered configuration parameters [1]. However, only a handful of parameters were considered (e.g., antenna height, Tx power, ISD, Umi/Uma propagation models). Thus, it will be too optimistic that the RRM measurement prediction models will generalize across any network configuration parameters. Thus, we believe that associated ID, which implicitly represents a certain configuration/condition at the network when that the model was trained under, may still be needed for handling cases where model generalization may not work.

Observation 4: Though initial model generalization results across different network configurations have shown good results, only a handful of configuration parameters and parameter values were considered.

In the BM case, the associated ID corresponds to the serving cell where the beams are being predicted. In the case of AIML mobility, on the other hand, the UE may be configured to perform the simultaneous predictions of multiple cells (e.g., temporal prediction of the serving cell and multiple neighbor cells). As such, discussion is needed on how the UE can be configured with an associated ID(s) for the multiple cells that are being predicted.

Proposal 6: The UE may be configured with one or more associated IDs (corresponding to the serving and neighbor cells) along with the inference configuration for the RRM measurement prediction. The details of the associated ID configuration are FFS.

Like the BM use case, the UE can determine the applicability of the RRM measurement prediction functionality by considering model availability, network side additional conditions (if provided), UE-side additional conditions (internally known by UE) and inference configuration.

Proposal 7: The UE determines that an RRM measurement prediction configuration is applicable if the UE has at least one model that satisfies all the following:

- The model was trained under the indicated network side additional conditions (e.g., if associated IDs are provided).

- The model was trained under the current UE side additional conditions.

- The model can perform the predictions under the indicated inference configuration.

### 2.2.4 Performing the inference

Regarding when the UE applies the inference configuration and start performing the inference, the following was agreed in RAN2-129bis [1]:

- The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC.  As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference

In the case of BM, periodic CSI reporting is activated upon configuration, while semi-persistent reporting needs to be activated by MAC CE/DCI and aperiodic CSI reporting can be activated by DCI. However, for RRM measurements, the activation is always upon the reception of the configuration.

One approach for the case of the RRM measurement prediction is to apply legacy behavior. That is, the UE will start performing the predictions according to the received inference configuration if it has determined that it has an applicable RRM measurement prediction model as discussed above. A disadvantage of such an approach is that the UE may perform unnecessary inferences/predictions even when it is not needed (e.g., when the UE is near the serving cell center). Even in legacy RRM framework, there are optional configurations such as s-measure that prevent the UE from performing unnecessary measurements. Thus, it should be discussed whether such mechanisms are needed for controlling inferences as well.

Observation 5: Performing RRM measurement predictions all the time is highly inefficient (e.g., when the UE is under excellent serving cell coverage).

Proposal 8: The UE can be configured to start performing the RRM measurement predictions in one of the following ways:

- automatically upon the reception of the inference configuration and determining that it has an applicable model

- upon the fulfillment of further conditions. The details of the conditions are FFS (E.g., configuration like the s-measure for starting inference).

# 3. Conclusion

In this contribution, we discussed LCM and specification impact of the RRM measurement prediction functionality and the following observations and proposals were made:

Observation 1: For temporal and frequency domain predictions, it is not necessary for the UE to be configured for set A/B configurations, even for the RRM sub-use cases where the input to the AI/ML model is L1 beam measurements.

Observation 2: For temporal domain case A, observation window duration is up to UE implementation.

Observation 3: For temporal domain case B, UE may use different skipping patterns to achieve the same level of MRRT. The network may not need to know the skipping patterns being used but knowing it will enable further UE/network energy saving.

Observation 4: Though initial model generalization results across different network configurations have shown good results, only a handful of configuration parameters and parameter values were considered.

Observation 5: Performing RRM measurement predictions all the time is highly inefficient (e.g., when the UE is under excellent serving cell coverage).

Proposal 1: For the temporal and frequency domain RRM measurement prediction, the inference input is provided at cell level for all sub-use cases (i.e., if the input to the model is beam measurements, it can be left to UE implementation to decide which beams of the cells to be used for the prediction.). FFS on how to configure inference input for the spatial domain.

Proposal 2: For temporal case A, the UE is configured with the prediction window length.

Proposal 3: For temporal case B, the UE is configured with the MRRT to use and may optionally be configured with a skipping pattern to apply to achieve the MRRT (e.g., among the skipping patterns supported by the UE).

Proposal 4: For temporal domain case B and frequency domain RRM measurement prediction, legacy RRM measurement reporting configuration can be reused.

Proposal 5: For temporal domain case A RRM measurement prediction, enhancements are needed regarding the predicted measurements to be included in the measurement report. The details are FFS (e.g., number of samples or which samples within the prediction window to be reported).

Proposal 6: The UE may be configured with one or more associated IDs (corresponding to the serving and neighbor cells) along with the inference configuration for the RRM measurement prediction. The details of the associated ID configuration are FFS.

Proposal 7: The UE determines that an RRM measurement prediction configuration is applicable if the UE has at least one model that satisfies all the following:

- The model was trained under the indicated network side additional conditions (e.g., if associated IDs are provided).

- The model was trained under the current UE side additional conditions.

- The model can perform the predictions under the indicated inference configuration.

Proposal 8: The UE can be configured to start performing the RRM measurement predictions in one of the following ways:

- automatically upon the reception of the inference configuration and determining that it has an applicable model

- upon the fulfillment of further conditions. The details of the conditions are FFS (E.g., configuration like the s-measure for starting inference).

# 4. References

[1]	RAN2-129bis Chairman Notes, Wuhan, China, April 2025

[2]	TR 38.744, Study on AI/ML for mobility in NR

[3]	R2-2501928, Specification impact for AIML mobility, CATT, Turkcell, RAN2-129bis, Wuhan, China, April 2025.