3GPP TSG-RAN WG2 Meeting #130	R2-2503695

Malta, May 19th – 23rd, 2025

Agenda Item:	8.3.5

Source:	InterDigital Inc.

Title:	Performance Monitoring for AI/ML Mobility

Document for:	Discussion

# 1. Introduction

In RAN2-129bis, initial discussion on LCM and the spec impact for the AI/ML for mobility use case were made and the following was agreed [1]:

Agreements (RAN2#129bis) 1. The general LCM framework for beam management can be the baseline (where applicable) for AI mobility, such as the following aspects: - Data collection for model training - UE capability - Applicability reporting - Inference configuration and reporting - Performance monitoring and management 2. Only functionality-based LCM is considered for AI/ML mobility (i.e. we don’t support model-based LCM)

In this contribution, we discuss the performance monitoring aspects of AI/ML mobility.

# 2. Discussion

## 2.1 Background

As agreed in RAN2#129bis, the LCM for the AIML mobility feature is to be based on the general LCM framework of the beam management (BM) case.

Performance monitoring can be performed by either the network or the UE. Based on such performance monitoring, various management decisions can be taken by the management entity (the UE or the network), such as model/functionality activation/deactivation/switching or fallback. The following 5 management frameworks were identified during rel-18 study of AI/ML for NR air interface [2]:

- Decision by the network and network-initiated AI/ML management

- Decision by the network and UE-initiated AI/ML management

- Decision by the UE and event-triggered as configured by the network

- Decision (autonomous) by the UE and reported to the network

- Decision (autonomous) by the UE and not reported to the network

## 2.2 NW-side model

For the NW-side AIML model for the BM case, it has been agreed that [3]:

Agreements

1	RAN2 confirms that UE will not be informed about any gNB/LMF-sided model/functionality management decision (e.g., selection, (de)activation, switching, fallback, etc.)

2	RAN2 confirms that UE will not be involved in any gNB/LMF-sided model/functionality management decision making (e.g., selection, (de)activation, switching, fallback, etc.), except being configured to provide the required measurement/data.

3	RAN2 focuses on the data collection procedure from UE to NW (e.g., gNB, LMF, or OAM) for the sake of NW-sided model LCM (including training, inference, management).

Thus, it is up to the network to monitor its model’s accuracy/performance and the only aspect that may have specification impact is whether any additional information (and in what form) may need to be provided to the network by the UE for monitoring purposes.

Based on the above we propose:

Proposal 1: For AI/ML mobility, performance monitoring for NW-side models is up to gNB’s implementation. Assistance information from the UE for enabling the performance monitoring at the gNB (in addition to legacy measurement reports) is FFS.

## 2.3 UE-side model

For UE-sided model for the AI/ML for NR air interface use cases, the following was agreed in RAN2-125bis [3]:

Agreements: 1 For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). 2 “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19.

For the AI/ML mobility use case, the following was agreed in RAN2-129bis [1]:

Agreements: For UE sided model, the performance monitoring considered in AIML air interface can be studied as a baseline for AIML aided mobility, i.e. NW-side and UE-sided performance monitoring. FFS metrics per use case

For NW-side monitoring, the UE reports back information that allows for the computation of relevant performance metrics by the network. For UE-side performance monitoring, the UE calculates the performance metrics and either reports them to the network or reports when an event that is based on the performance metrics are fulfilled.  Both options align with the definition of management framework 1 (NW-decision/initiated), they only differ in what type of information the UE reports to the network.

An autonomous decision by the UE (management frameworks #4 and #5) for mobility aspects may significantly impact the efficient management of the network’s resources and infrastructure, even more so if it is not even reported to the network (i.e., framework #5). For example, if a UE was using inter-frequency domain RRM predictions decides to fallback to legacy RRM measurements, it must notify the network so that it can be provided with the measurement gaps for performing the inter-frequency measurement (or for the gaps to be activated if they were already configured).

Observation 1: An autonomous management decision by the UE may significantly impact the network’s/UEs’ performance.

Additionally, both management frameworks #3 and #4 are associated with UE-managed decisions and subsequent reporting of such decisions. The difference is on what criteria the decision is based on. In management framework #3, the decision is taken by the UE according to prior network configuration (e.g., based on events/thresholds), while in #4, the decision is taken autonomously by the UE. As such, in framework #4, even though the network is notified about the UE’s decision, it may still negatively impact the performance of the network (e.g., due to implementation differences across UE vendors).

Thus, we propose:

Proposal 2: For the performance monitoring of a UE sided model for AI/ML mobility, autonomous management decisions by the UE (with or without decision reporting to the network) are not considered.

Proposal 3: Consider the NW-decision/NW-initiated management case (framework 1) as the baseline.

Proposal 4: Further consider the NW-decision/UE-initiated (framework 2) and UE-decision/event-triggered as configured by the network (framework 3) management cases.

Proposals 1 to 4 are compactly summarized in the table below:

| Management Framework | Performance Monitoring Entity | Support |
| --- | --- | --- |
| - NW-decision/NW-Initiated | NW | Yes |
| - NW-decision/UE-initiated | UE | Yes |
| - UE-decision/event-triggered as configured by NW | UE | Yes |
| - UE-autonomous/with decision reporting | UE | No |
| - UE-autonomous/no decision reporting | UE | No |

Table 1: Proposals on which management/performance monitoring cases to support

For the first management case (NW-decision, NW-initiated), the UE needs to provide the necessary information that would enable the network to efficiently monitor the accuracy of the predictions of the AIML model and to timely perform the correct management decisions.

Additionally, performance information may be referring to KPIs that can be directly/indirectly utilized by the network for performance monitoring. For example, the UE may provide the necessary raw information for the network to compute the corresponding monitoring metrics (e.g., predicted RSRP values and the ground truth, i.e., measured RSRP values), or it may provide intermediate KPIs itself (e.g., RSRP difference values, i.e., difference between the predictions and ground truth (RRM measurements). This is applicable to all RRM measurement prediction sub use-cases (in temporal/frequency/spatial domain).

For indirect RRM event prediction, the same values (RSRP and/or RSRP difference) may also be provided, as there is strong correlation between the accuracy of RRM measurement predictions and the corresponding prediction of the associated event (e.g., A3), according to the results reported by various companies.

Based on the above, we propose:

Proposal 5: For performance monitoring of UE sided model for RRM measurement prediction and indirect measurement event prediction, the UE can be configured to provide RSRP difference between predicted and measured values.

# 3. Conclusion

In this contribution, we discussed performance monitoring and management of NW and UE sided models for AI/ML mobility and the following observations and proposals were made:

Observation 1: An autonomous management decision by the UE may significantly impact the network’s/UEs’ performance.

Proposal 1: For AI/ML mobility, performance monitoring for NW-side models is up to gNB’s implementation. Assistance information from the UE for enabling the performance monitoring at the gNB (in addition to legacy measurement reports) is FFS.

Proposal 2: For the performance monitoring of a UE sided model for AI/ML mobility, autonomous management decisions by the UE (with or without decision reporting to the network) are not considered.

Proposal 3: Consider the NW-decision/NW-initiated management case (framework 1) as the baseline.

Proposal 4: Further consider the NW-decision/UE-initiated (framework 2) and UE-decision/event-triggered as configured by the network (framework 3) management cases.

Proposal 5: For performance monitoring of UE sided model for RRM measurement prediction and indirect measurement event prediction, the UE can be configured to provide RSRP difference between predicted and measured values.

# 4. References

[1]	3GPP R2 #129bis Meeting, Chairman Notes.

[2]	TR 38.843, Study on AI/ML for NR air interface.

[3]	3GPP R2 #125bis Meeting, Chairman Notes.