3GPP TSG-RAN WG2 Meeting #130	R2-2503701

St. Julians, Malta, 19 – 23 May 2025

Agenda item:	8.7.5

Source:	Apple

Title:	Discussions on Fast RLC Retransmission

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

This paper aims to provide of our views on some of the remaining open issues identified for Fast RLC Retransmissions, including:

- Whether/how to increment the RETX_COUNT for Autonomous Retransmission.

- How to prevent too early and/or unnecessary autonomous retransmission.

- How to avoid excessive polling for the polling enhancement

# Discussions

## Issues with RETX_COUNT for Autonomous Retransmission

We think it is simpler to just model autonomous retransmission as one additional triggering condition of typical RLC retransmission. Hence, the RETX_COUNT for a RLC SDU should also be incremented when an autonomous retransmission is performed for this RLC SDU. This seems to be more complicated if we have a separate handling for RETX_COUNT associated to autonomous retransmission. Since we have also agreed that autonomous retransmission can only be performed once for a RLC SDU, unlikely it will lead to early RLF very often.

Proposal 1: RETX_COUNT associated to a RLC SDU is also incremented when autonomous retransmission for this RLC SDU is performed.

## Issues with Unnecessary Retransmissions

During RAN2 #129bis, we have reached the following agreement:

For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission

Consider a situation where autonomous retransmission is triggered for a RLC SDU. However, shortly after - even before the MAC layer obtains a UL grant and corresponding AMD PDU is submitted to the lower layer, the transmitter may receive a STATUS PDU that positively acknowledges this RLC SDU. Obviously, the autonomous retransmission is no longer needed if the RLC SDU is already successfully delivered at the receiver, and apparently the transmitter should immediately cancel the triggered autonomous retransmission. However, currently such behavior is not captured in the specifications.

It is worth noting that, in legacy the transmitter performs retransmission based on the status report, and hence it is almost impossible that the transmitter would receive positive acknowledgement shortly after the retransmission is considered based on a previous status report. Hence, no special handling has been specified in TS 38.322. With the introduction of autonomous retransmission, we think it represents a new situation that needs to be addressed in the specifications, to ensure the triggered autonomous retransmission will not be performed once it is positively acknowledged.

Proposal 2: To avoid unnecessary autonomous retransmission, a triggered autonomous retransmission for a RLC SDU should be cancelled, if this RLC SDU is positively acknowledged before the autonomous retransmission is performed.

## Issues with Excessive Polling

In addition to autonomous retransmission, RAN2 has agreed that a poll can be triggered more timely when the remaining time of a RLC SDU falls below a threshold. That is, the poll can be triggered by both legacy events and new events related to delay. As a result, the UE may trigger excessive polls. To make sure that more urgent poll can be retransmitted faster when there is a need, while minimizing excessive polls that can unnecessarily impact the capacity, we think RAN2 can consider an adaptive t-PollRetransmit timer value to strike a balance. More specifically, the transmitter may apply different t-PollRetransmit timer values, in accordance with whether an urgent packet is present. If there is a RLC SDU in the queue that needs to be delivered urgently (e.g. the remaining of which is small), a t-PollRetransmit timer with a shorter duration can be used in a hope to retransmit the poll more rapidly. Conversely, if none of the RLC SDUs in the queue (or the RLC SDUs that are waiting for acknowledgement) is considered urgent, the default t-PollRetransmit timer can be applied to minimize polling.

Proposal 3: To handle potential issues of excessive polling, different t-PollRetransmit timer can be applied based on the poll triggering events (e.g. if the poll is triggered by the presence of an urgent packet).

# Conclusions

This paper presents our views on some of the remaining issues related to Rel-19 RLC-AM enhancements to support faster retransmission. We have the following proposals:

Proposal 1: RETX_COUNT associated to a RLC SDU is also incremented when autonomous retransmission for this RLC SDU is performed.

Proposal 2: To avoid unnecessary autonomous retransmission, a triggered autonomous retransmission for a RLC SDU should be cancelled, if this RLC SDU is positively acknowledged before the autonomous retransmission is performed.

Proposal 3: To handle potential issues of excessive polling, different t-PollRetransmit timer can be applied based on the poll triggering events (e.g. if the poll is triggered by the presence of an urgent packet).

# References

- 3GPP RAN2 #129bis Chair’s Notes, Wuhan, China, April 2025.