3GPP TSG-RAN WG2 Meeting #130													     R2-2503707

St. Julians, Malta, May 19th – 23rd, 2025

Agenda item:	8.3.4

Source:	Apple

Title:	On data collection for AI/ML mobility

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we provide our views on data collection, including UE-sided and NW-sided models.

# 2   	Discussion

For reference, let’s recap the relevant agreements [4] and [5]:

FFS whether association ID is required and should provide motivation on per use case bases

As a baseline, use the AI ML PHY NW-side data collection procedures, configuration and reporting framework.

Study UE side data collection configuration, taking AI ML PHY relevant procedure as baseline.

Postpone UE side data collection transfer discussion until further progress is made in AI ML PHY in R20.

## 2.1   	Network-sided

As we’ve agreed to re-use the AI/ML PHY framework for its good to recap the procedure agreed in RAN2 for this, which goes as follows (for the purpose of this discussion we focus on the air interface signaling and UE-related aspects and therefore do not differentiate between the cases when the procedure is initiated by OAM or gNB):

- gNB configures UE to log measurements for NW-sided data collection using RRC signaling

- UE logs the measurements

- UE indicates data availability to network using IEs in RRCReconfigurationComplete

- Network retrieves measurements using UEInformationRequest

- UE provides measurements using UEInformationResponse

The most natural way to realize the first step (gNB configuring UE to log measurements for data collection) is to re-use (with some enhancements) the legacy measurement configuration framework. At the very least, an indication would need to be added for UE to know which of the measurements configured it should log. Alternatively, we could consider a completely new framework, but that’s likely to turn out counterproductive. Hence the first proposal to re-use (with enhancements) the legacy measurement configuration framework and to enhance the MeasConfig IE to support NW-sided data collection configuration.

Proposal 1: to enhance the MeasConfig IE to support NW-sided data collection configuration.

As we’ve mentioned, there should be at least an indication that the measurements in question are configured for logging (as opposed to reporting in the usual manner).

Observation 1: UE should know which measurements to log (for NW-sided data collection) and which to report in the usual/legacy manner.

Such indication can be added to e.g. MeasObjectNR, which is appealing as this would have the least amount of standards impacts, as then most of the remaining ASN.1 could be re-used. Of course, some of the configurations in MeasObjectNR are not applicable to data collection (as it is defined as of Rel-19) e.g. NTN-related configurations and others and naturally the Rel-20 AI/ML data collection capable UE should not be configured to log NTN measurements (unless we explicitly agree to support NTN AI/ML in Rel-20, which seems unlikely). This is likely to result in a somewhat complicated procedural text, but this can be handled. Furthermore, re-using MeasObjectNR is future proof for the eventuality where we add more features to AI/ML mobility. Hence the proposal to extend MeasObjectNR with an indication whether the UE should log the configured measurements for NW-sided data collection.

Proposal 2: to extend MeasObjectNR with an indication whether the UE should log the configured measurements for NW-sided data collection.

The question then becomes whether the same MO can be configured for both logging for data collection and immediate reporting. Obviously, this would be extremely wasteful and counterproductive. If the measurements for an MO are reporting immediately in the legacy manner, there is no motivation to also log them for future retrieval, not to mention that it would result in air interface overhead and unnecessary burden on UEs. Therefore, we proposed to add a new CHOICE to measObject of the same type as MeasObjectNR, which would be separate from measObjectNR, i.e. in the following fashion:

MeasObjectToAddModList information element

-- ASN1START

-- TAG-MEASOBJECTTOADDMODLIST-START

MeasObjectToAddModList ::=                  SEQUENCE (SIZE (1..maxNrofObjectId)) OF MeasObjectToAddMod

MeasObjectToAddMod ::=                      SEQUENCE {

measObjectId                                MeasObjectId,

measObject                                  CHOICE {

measObjectNR                                MeasObjectNR,

...,

measObjectEUTRA                             MeasObjectEUTRA,

measObjectUTRA-FDD-r16                      MeasObjectUTRA-FDD-r16,

measObjectNR-SL-r16                         MeasObjectNR-SL-r16,

measObjectCLI-r16                           MeasObjectCLI-r16,

measObjectRxTxDiff-r17                      MeasObjectRxTxDiff-r17,

measObjectRelay-r17                         SL-MeasObject-r16,

measObjectNR-SL-r18                         MeasObjectNR-SL-r18,

measObjectNR-dataCollection-r20             MeasObjectNR

}

}

-- TAG-MEASOBJECTTOADDMODLIST-STOP

-- ASN1STOP

Such design achieves all the goals in a simple manner: there is a clear implicit indication which MO is for data collection, there is full ASN.1 re-use of MeasObjectNR and it is easy to make sure that the same measurements are not configured for both logging and immediate reporting. Further details of the said new measurement object CHOICE can be discussed in the WI phase. The ASN.1 example provided above probably goes a bit too far for a SI, and therefore we are not making a more concrete proposals at this stage – the ASN.1 snippet is just for illustration.

One thing which perhaps follows naturally, but still important enough to discuss and make an explicit agreement on is that with the decision to use AI/ML data collection framework for NW-sided data collection, the network shouldn’t “abuse” the legacy measurement framework for the purpose of data collection. Legacy measurements should only be configured for mobility (and other features they have been defined for), not for data collection and NW-sided model training.

Proposal 3: network shouldn’t “abuse” legacy measurements for NW-sided data collection.

We are not sure whether this can be reflected in a normative specification, but such agreement should at the very least be captured in the meeting minutes.

Finally, it should be clear that NW-sided data collection is a significant burden on UEs – even more so, much more so in fact, than let’s say MDT. The amount of data needed for efficient NW-sided data collection is likely to far exceed the amount of data needed for MDT. As MDT was defined as a separate optional feature for these reasons, so must be NW-sided data collection. In other words, a UE can be AI/ML mobility capable but not support NW-sided data collection.

Observation 2: NW-sided data collection is a significant burden on UEs – much more than let’s say MDT (due to likely much larger data volumes).

Proposal 4: UE support for network-sided data collection is an optional feature (i.e. it should be a “dedicated feature” separate from AI/ML mobility) – in other words, a UE can be AI/ML mobility capable but not support NW-sided data collection.

Finally, but importantly nonetheless, there is the issue of user privacy of NW-sided data collection, which in our view requires user consent (as is the case for MDT in general, which is the framework that has been agreed for NW-sided data collection). This issue is somewhat bigger than AI/ML mobility and is also applicable to AI/ML PHY and so it should be discussed as part of the general AI/ML LCM framework. There is a paper submitted to that agenda in [7] we would like to draw your attention to.

Observation 4: the issue of user privacy in the context of NW-sided data collection is discussed in [7] R2-2503526.

## 2.2   	UE-sided

For UE-sided data collection, there are ongoing discussions in SA2 which will only produce agreements in Rel-20. Therefore, we may have some discussions for the time being, but it is very likely that most if not all the decisions we may take now will need to be revisited when SA2 concludes.

Observation 5: there are ongoing discussions in SA2 on UE-sided data collection that would only conclude in Rel-20.

Observation 6: it is very likely that most if not all the decisions we may take now on UE-sided data collection will need to be revisited when SA2 concludes

Finally, we should not discuss data transfer, only configuration, in accordance with other agreements.

Observation 7: if companies’ urge to discuss UE-sided data collection cannot be resisted, the discussion should be limited to configuration only (and not cover data transfer).

With these understandings in mind, we should first recognize that UEs may use their measurement configurations for data collection as well. This requires no enhancements, adds no overhead of the air interface and no additional burden on the network. This probably needs no explicit agreements, but it may still be worth capturing in the TR so that everybody is on the same page.

Proposal 5: UE can use the measurement configuration it received for mobility for data collection as well; this requires no enhancements, adds no overhead and no additional burden on network.

Then we can discuss additional enhancements we may want to define on top of the above. While it’s reasonable to expect that over time a UE should be able to collect enough measurements for AI/ML training based on just the measurement configurations for mobility (which are likely to be the most useful anyway), it is also possible that in some cases it would be beneficial for a UE to be able to collect measurements for additional cells/frequencies it is currently not configured with for legacy mobility. This is in fact what the evaluation conclusion demonstrating the added gains of the cluster approach illustrate.

Observation 8: the fact that cluster approach outperforms single cell approach in the evaluation indicates that at least in some cases it is beneficial for the UE to collect measurements for cells/frequencies it is not configured for otherwise.

To this end, we can re-use the UE-sided data collection framework defined in AI/ML PHY, specifically dataCollectionStart (true/false) and dataCollectionPreferredConfiguration.

Proposal 6: dataCollectionStart (true/false) and dataCollectionPreferredConfiguration defined for AI/ML PHY can be re-used for AI/ML mobility UE-sided data collection.

To this end, the dataCollectionPreferredConfiguration IE may need to be enhanced to allow a UE to request measurement configuration for the purpose of UE-sided data collection at least for additional cells and frequencies it is otherwise not configured with.

Proposal 7: dataCollectionPreferredConfiguration IE may need to be enhanced to allow a UE to request measurement configuration for the purpose of UE-sided data collection at least for additional cells and frequencies.

Finally, associated id would also be useful. The rationale for using associated id for data collection is the same as the rationale for using it in inference and it is discussed in more detail in [6].

Proposal 8: associated id is beneficial for UE-sided data collection.

# 3	Proposals

Proposal 1: to enhance the MeasConfig IE to support NW-sided data collection configuration.

Observation 1: UE should know which measurements to log (for NW-sided data collection) and which to report in the usual/legacy manner.

Proposal 2: to extend MeasObjectNR with an indication whether the UE should log the configured measurements for NW-sided data collection .

Proposal 3: network shouldn’t “abuse” legacy measurements for NW-sided data collection.

Observation 2: NW-sided data collection is a significant burden on UEs – much more than let’s say MDT (due to likely much larger data volumes).

Proposal 4: UE support for network-sided data collection is an optional feature (i.e. it should be a “dedicated feature” separate from AI/ML mobility) – in other words, a UE can be AI/ML mobility capable but not support NW-sided data collection.

Observation 4: the issue of user privacy in the context of NW-sided data collection is discussed in [7] R2-2503526.

Observation 5: there are ongoing discussions in SA2 on UE-sided data collection that would only conclude in Rel-20.

Observation 6: it is very likely that most if not all the decisions we may take now on UE-sided data collection will need to be revisited when SA2 concludes

Observation 7: if companies’ urge to discuss UE-sided data collection cannot be resisted, the discussion should be limited to configuration only (and not cover data transfer).

Proposal 5: UE can use the measurement configuration it received for mobility for data collection as well ; this requires no enhancements, adds no overhead and no additional burden on network.

Observation 8: the fact that cluster approach outperforms single cell approach in the evaluation indicates that at least in some cases it is beneficial for the UE to collect measurements for cells/ frequencies it is not configured for otherwise.

Proposal 6: dataCollectionStart (true/false) and dataCollectionPreferredConfiguration defined for AI/ML PHY can be re-used for AI/ML mobility UE-sided data collection .

Proposal 7: dataCollectionPreferredConfiguration IE may need to be enhanced to allow a UE to request measurement configuration for the purpose of UE-sided data collection at least for additional cells and frequencies.

Proposal 8: associated id is beneficial for UE-sided data collection.

# 5	References

[1]				TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[2]				RP-240082 Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR

[3]				RP-243244 Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[4]				RAN2#129 meeting minutes

[5]				RAN2#129bis meeting minutes

[6]				R2-2503705, On functionality of UE-sided AI/ML mobility, Apple

[7]				R2-2503526, User consent for NW-side data collection, NTT DOCOMO, Apple, vivo, Xiaomi