# 3GPP TSG RAN WG2 Meeting #130      	                           R2-2503715
St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	8.1.2.3

Source:	Apple

Title:	Remaining issues on LCM procedure of AI/ML based positioning

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The revised Rel-19 WID of AI/ML for air interface (WID RP-243244) was agreed in RAN#106 [1]. The WI objective on general LCM framework, including applicable functionalities, is led by RAN2 which is copied below:

By RAN2#129b [2], most of essential issues on LCM of AI/ML based positioning have been closed. However, there are still many open issues to resolve. For example, CR Rapporteur of TS 37.355 provide a list of open issues in summary report [3].

In this contribution, we discuss the following open issues:

- Open issue 1: Whether support on-demand request for NW-side additional condition

- Open issue 2: Whether support reporting UE-side additional condition

- Open issue 3: Order of inference config and applicability reporting

- Open issue 4: Applicability of NR-SelectedDL-PRS-IndexList

- Open issue 5: Applicability of "batch reporting" for AI/ML positioning

- Open issue 6: Location server error causes

- Open issue 7: Target device error causes

- Open issue 8: Applicability of Positioning Integrity to AI/ML positioning

- Open issue 9: Signalling of Monitoring Outcome

- Open issue 10: Signalling of "ground-truth label" information

# 2 Discussion

## 2.1 Open issue 1: On-demand request for NW-side additional condition

This is one issue discussed in post-meeting email discussion [11], but no conclusion is made.

First, on-demand request for DL-PRS assistance data is already supported from Rel-17. Please note that RAN1#119 agreed all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE [4]:

Agreement

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following:

- Alternative 1. Info #7 is provided implicitly via associated ID.

  - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

  - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 3.  Info #7 is not be provided from LMF to UE.

  - If info #7 is not provided, UE may assume info #7 is consistent between training and inference.

- Alternative 4. Info #7 is provided explicitly from LMF to UE.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

Observation 1: On-demand request for DL-PRS assistance data is already supported from Rel-17.

Observation 2: RAN1 agreed all assistance data of legacy UE-based DL-TDOA (other than info 7, i.e. geographical coordinates of TRPs) are reused.

We think RAN2 can reuse the legacy on-demand request for DL-PRS assistance data. Whether allowing the UE to on-demand request for TRP-specific assistance data depends on RAN1.

Proposal 1: Support the legacy on-demand request for DL-PRS assistance data. Whether to allow the UE to on-demand request for TRP-specific assistance data depends on RAN1.

## 2.2 Open issue 2: Whether support reporting UE-side additional condition

This is one issue discussed in post-meeting email discussion [11], but no conclusion is made.

In RAN2#127 [5], it was agreed that the UE determines applicable functionalities based on NW-side additional condition, UE-side additional condition and model availability in AI/ML based beam management:

Agreements on procedures - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

We understand that this agreement means the applicability reporting already takes UE-side additional condition into consideration, and UE-side additional condition doesn’t need to be explicitly reported to the NW. The logic behind this agreement is because the UE-side additional condition includes UE’s internal information (e.g. battery and memory), which can’t be exposed to the NW.

Observation 3: In AI/ML based beam management, the applicability reporting already takes UE-side additional condition into consideration, and UE-side additional condition is not explicitly reported to the NW because it includes UE’s internal information (e.g. battery and memory), which can’t be exposed to the NW.

We think the same principle can be reused to AI/ML based positioning. Thus, we propose:

Proposal 2: Same as AI/ML based BM, the UE decides the applicable functionalities based on NW-side additional conditions, UE-side additional conditions and model availability in device. UE-side additional condition is not explicitly reported to the NW.

## 2.3 Open issue 3: order of inference config and applicability reporting

This FFS is captured in RAN2#127 post-meeting email discussion summary [6]:

Proposal 7: For positioning Case 1, UE can report a change in applicable functionality by sending applicable functionalities upon change via LPP signaling (unsolicited information transfer). UE can also provide applicable functionality reporting as a response to LMF request (solicited information transfer). FFS on the order of inference configuration and applicable functionality reporting, FFS on functionality activation.

Please note that existing TS 38.305 has captured below principle for LPP procedures [7]:

7.1.2.5	Sequence of procedures

LPP procedures are not required to occur in any fixed order, in order to provide greater flexibility in positioning. Thus, a UE may request assistance data at any time in order to comply with a previous request for location measurements from the LMF; an LMF may instigate more than one request for location information (e.g., measurements or a location estimate) in case location results from a previous request were not adequate for the requested QoS; and the target device may transfer capability information to the server at any time if not already performed.

Despite the flexibility allowed by LPP, it is expected that procedures will normally occur in the following order:

1. Capability Transfer;

2. Assistance Data Transfer;

3. Location Information Transfer (measurements and/or location estimate).

We see no reason to challenge above principle of LPP in AI/ML based positioning. Thus, we propose:

Proposal 3: Following existing principle on sequence order of LPP procedures, no need to specify fixed step order between inference configuration and applicable functionality reporting.

## 2.4 Open issue 4: Applicability of NR-SelectedDL-PRS-IndexList

This issue is captured in running CR discussion summary report of [3].

NR-SelectedDL-PRS-IndexList is used by the location server to provide the selected DL-PRS Resource of nr-DL-PRS-AssistanceDataList to the target device. In the case of assistance data for multiple NR positioning methods are provided, it shall be present in only one of NR-Multi-RTT-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData, NR-DL-TDOA-ProvideAssistanceData.

As RAN1#119 agreed all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE [4], we think the assistance information of AI/ML positioning Case 1 can be shared with assistance information of other positioning methods. Thus, it should be applicable to AI/ML positioning Case 1.

Proposal 4: NR-SelectedDL-PRS-IndexList is applicable to AI/ML positioning Case 1.

## 2.5 Open issue 5: Applicability of "batch reporting" for AI/ML positioning

This issue is captured in running CR discussion summary report of [3].

As “batch reporting” was introduced in RAN1 in Rel-17, we think it is up to RAN1 to decide whether it is applicable to AI/ML positioning Case 1.

Proposal 5: As “batch reporting” was introduced in RAN1 in Rel-17, it is up to RAN1 to decide whether it is applicable to AI/ML positioning Case 1.

## 2.6 Open issue 6: Location server error causes

This issue is captured in running CR discussion summary report of [3].

In the current running CR of TS 37.355 [8], the IE NR-DL-AI-ML-LocationServerErrorCauses is captured as follows:

The IE NR-DL-AI-ML-LocationServerErrorCauses is used by the location server to provide NR AI/ML positioning error reasons to the target device.

-- ASN1START

NR-DL-AI-ML-LocationServerErrorCauses-r19 ::= SEQUENCE {

cause-r19		ENUMERATED	{	undefined,

assistanceDataNotSupportedByServer,

assistanceDataSupportedButCurrentlyNotAvailableByServer,

notProvidedAssistanceDataNotSupportedByServer,

on-demand-dl-prs-NotSupportedByServer,

on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer,

...

},

...

}

We think the current error cause list is sufficient.

Proposal 6: For NR-DL-AI-ML-LocationServerErrorCauses, no need to introduce other error cause beyond the existing list in running CR.

## 2.7 Open issue 7: Target device error causes

This issue is captured in running CR discussion summary report of [3].

In the current running CR of TS 37.355 [8], the IE NR-DL-AI-ML-LocationServerErrorCauses is captured as follows:

The IE NR-DL-AI-ML-TargetDeviceErrorCauses is used by the target device to provide NR AI/ML positioning error reasons to the location server.

-- ASN1START

NR-DL-AI-ML-TargetDeviceErrorCauses-r19 ::= SEQUENCE {

cause-r19						ENUMERATED {	undefined,

assistance-data-missing,

unableToMeasureAnyTRP,

attemptedButUnableToMeasureSomeNeighbourTRPs,

thereWereNotEnoughSignalsReceived,

locationCalculationAssistanceDataMissing,

...

},

remoteUE-Indication-r18			ENUMERATED {true}						OPTIONAL,	-- Cond NR

...

}

We have the following suggestions:

- As agreed in RAN2#128 [9], the UE may report AI/ML based positioning method is non-applicable. Thus, a new cause (e.g. AIML method NonApplicable) can be introduced.

Agreements 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

- On “thereWereNotEnoughSignalsReceived”, it is not clear whether it means signals for training or inference or monitoring. As NW reaction may be different, we think it can be further split into “Not Enough Training Signals Received”, “Not Enough Inference Signals Received” and “Not Enough Monitoring Signals Received”.

- Add one more cause “Performance monitoring Assistance Data Missing”

Thus, we propose:

Proposal 7: For NR-DL-AI-ML-TargetDeviceErrorCauses, introduce the following new error causes:

- A new cause “AIML method NonApplicable”.

- Split “thereWereNotEnoughSignalsReceived” into “Not Enough Training Signals Received”, “Not Enough Inference Signals Received” and “Not Enough Monitoring Signals Received”.

- A new cause “Performance monitoring Assistance Data Missing”.

## 2.8 Open issue 8: Applicability of Positioning Integrity to AI/ML positioning

This issue is captured in running CR discussion summary report of [3].

Please note that RAN1#119 agreed all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE [4], which include info#15 related to positioning integrity.

| 15 | Data facilitating the integrity results determination of the calculated location |
| --- | --- |

Thus, we think this means that RAN1 assume positioning integrity is applicable to AI/ML positioning Case 1.

Proposal 8: Positioning integrity is applicable to AI/ML positioning Case 1.

## 2.9 Open issue 9: Signalling of Monitoring Outcome

Issue description:

In terms of monitoring for AI/ML positioning, RAN1 agrees to support:

- Option A: The target UE side performs monitoring metric calculation. The target UE may signal the monitoring outcome to the LMF.

- the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method.

This includes all the sub-options A-1, A-2 and A-3. The details are upon RAN1 still, but there are expected RAN2 impact.

This issue is captured in running CR discussion summary report of [3]:

We understand RAN1 will continue to down-select among sub-options A-1, A-2 and A-3. Before RAN1 make conclusion, it is difficulty for RAN2 to determine whether to introduce a new LPP message or reuse existing LPP message (e.g., LPP ProvideLocationInformation).

Proposal 9a: On signaling of monitoring outcome, RAN2 discuss whether to introduce a new LPP message or reuse existing LPP message (e.g. LPP ProvideLocationInformation) after RAN1 down select sub-option of Option A.

On the other hand, the UE anyway needs the performance monitoring configuration from LMF. It is straight forward to reuse LPP ProvideAssistanceData to provide performance monitoring configuration.

Proposal 9b: On signaling of monitoring outcome, reuse LPP ProvideAssistanceData to provide performance monitoring configuration from LMF to the target UE.

## 2.10 Open issue 10: Signalling of "ground-truth label" information

Issue description:

Following up on sub-option A-1 (see #LPP-16), RAN1 agreed that:

Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE.

The details on whether a new message or existing message is used to provide the ground truth label to the target UE is also expected to have RAN2 impact.

This issue is captured in running CR discussion summary report of [3]:

We notice that RAN1#120 made the following agreement [10]:

Agreement (RAN1#120) For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. - From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

We think that the LPP ProvideAssistanceData can be reused to send the "ground-truth label" information from LMF to the target UE.

Proposal 10: LPP ProvideAssistanceData is reused to send the "ground-truth label" information from LMF to the target UE.

# 3 Conclusion

In this contribution, we discuss remaining issues on LCM procedure of AI/ML based positioning. Our observations are:

Observation 1: On-demand request for DL-PRS assistance data is already supported from Rel-17.

Observation 2: RAN1 agreed all assistance data of legacy UE-based DL-TDOA (other than info 7) are reused.

Observation 3: In AI/ML based beam management, the applicability reporting already takes UE-side additional condition into consideration, and UE-side additional condition is not explicitly reported to the NW because it includes UE’s internal information (e.g. battery and memory), which can’t be exposed to the NW.

Based on observations, our proposals can be found below:

Proposal 1: Support the legacy on-demand request for DL-PRS assistance data. Whether to allow the UE to on-demand request for TRP-specific assistance data depends on RAN1.

Proposal 2: Same as AI/ML based BM, the UE decides the applicable functionalities based on NW-side additional conditions, UE-side additional conditions and model availability in device. UE-side additional condition is not explicitly reported to the NW.

Proposal 3: Following existing principle on sequence order of LPP procedures, no need to specify fixed step order between inference configuration and applicable functionality reporting.

Proposal 4: NR-SelectedDL-PRS-IndexList is applicable to AI/ML positioning Case 1.

Proposal 5: As “batch reporting” was introduced in RAN1 in Rel-17, it is up to RAN1 to decide whether it is applicable to AI/ML positioning Case 1.

Proposal 6: For NR-DL-AI-ML-LocationServerErrorCauses, no need to introduce other error cause beyond the existing list in running CR.

Proposal 7: For NR-DL-AI-ML-TargetDeviceErrorCauses, introduce the following new error causes:

- A new cause “AIML method NonApplicable”.

- Split “thereWereNotEnoughSignalsReceived” into “Not Enough Training Signals Received”, “Not Enough Inference Signals Received” and “Not Enough Monitoring Signals Received”.

- A new cause “Performance monitoring Assistance Data Missing”.

Proposal 8: Positioning integrity is applicable to AI/ML positioning Case 1.

Proposal 9a: On signaling of monitoring outcome, RAN2 discuss whether to introduce a new LPP message or reuse existing LPP message (e.g. LPP ProvideLocationInformation) after RAN1 down select sub-option of Option A.

Proposal 9b: On signaling of monitoring outcome, reuse LPP ProvideAssistanceData to provide performance monitoring configuration from LMF to the target UE.

Proposal 10: LPP ProvideAssistanceData is reused to send the "ground-truth label" information from LMF to the target UE.

# 4 References

[1] RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#129b, Chair Notes.

[3] R2-250xxxx, LPP open issues for feature "AI/ML for NR air interface, Qualcomm Incorporated (Rapporteur).

[4] RAN1#119, Chair Notes.

[5] RAN2#127, Chair Notes.

[6] R2-2406381, Report of [POST126][032][AI/ML PHY] LCM (Intel/Samsung)_Phase 2, Intel Corporation.

[7] 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[8] 3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[9] RAN2#128, Chair Notes.

[10] RAN1#120, Chair Notes.

[11] R2-2500968, Report of [POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson), Ericsson.