3GPP TSG-RAN WG2 Meeting #130	R2-2503726

St. Julians, Malta, 19-23 May 2025

Agenda item:	8.13.3

Source:	Apple, FirstNet, Ericsson, AT&T, Kyocera, Philips, NEC

Title:	Fast & Parallel RRC Establishment for Multi-hop U2N relaying and TP

WID/SID:	NR_SL_relay_multihop – Release 19

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#129bis meeting[1], the proposals [2][3] to speed up the RRC setup for multi-hop U2N have been discussed and the following agreements have been reached:

Agreements:

Continue to consider forwarding of SRB0 messages by relay UEs not in RRC_CONNECTED with respect to control plane approach 1.

TPs showing the two approaches for fast forwarding of SRB0 (SRAP header and local ID assignment by RRC signalling) are invited for next meeting (co-sourcing strongly encouraged).

Other approaches are not precluded (contribution-driven) but should be shown at a mature stage considering the time left.

Strive to avoid additional RAN3 impact specific to fast forwarding.

FFS if applicable to DL.

FFS what level of gNB awareness of the path information would be needed.

FFS if fast forwarding is optional/mandatory for UEs to support.

In this paper, we provide the design details for the high-level proposal of fast & parallel RRC setup proposed in [2], especially on how to address the FFS issues above, and how to further reduce potential specification impact while keep the benefits of latency reduction. A TP is also provided to cover the necessary changes in TS 38.351 and TS 38.331.

# 2 Discussion

First, we explain how the fast forwarding of RRC SRB0 message works generally with the following signalling diagram:

Figure 1: Signalling Flow of Fast Forwarding

The signalling flow in Figure 1 includes the following steps:

Step 1: Remote UE triggers its first RRC message as same as legacy approach (SRB0 message w/o SRAP header on SL-RLC0)

Step 2: First Intermediate Relay UE not in RRC_CONNECTED generates the new SRAP header for the received SRB0 message and forward this in SL-RLCx channel. The L2 ID is obtained from SL MAC header of the received SL transmission

Step 3: The (first) Intermediate relay UE, as triggered to enter RRC_CONNECTED, acting as a remote UE to generate its own SRB0 message and send it to parent UE via SL-RLC0

Step 4: Th last relay UE receives the SRB0 message(s) from the downstream nodes (remote UE and/or intermediate relay UEs) and generate SUI message to solicit local ID allocation for its child UE and indirectly connected downstream nodes and corresponding configurations.

Step 5: gNB sends RRCReconfiguration message to the last relay UE provide local ID(s) for “Remote UE(s)” (which include the remote UE and intermediate relay UE acting as a remote) reported and corresponding SRAP mappings for SRBs, and respective Uu Relay RLC channel and PC5 relay RLC channel configuration.

Step 6: The last relay UE forwards SRB0 message (e.g., RRCSetupReq) by using the dedicated configuration provided by gNB with legacy Uu SRAP header in a configured Uu Relay RLC channel for SRB0.

Step 7: gNB sends Uu SRB0 (e.g., RRCSetup) for the respective “Remote UE(s)” to provide corresponding SRAP mappings for SRBs and related PC5 Relay RLC channel configuration to the last relay UE. The last relay UE replaces Uu SRAP header with PC5 SRAP header with L2 ID based on the L2-ID-local ID configuration provided in Step 5.

Step 8: To determine the egress PC5 link of the DL SRB0 message, the relay UE checks the L2 ID in PC5 SRAP header to determine whether this DL SRB0 message is for which downstream node or for itsef; and if it is for a downstream remote UE, it thend identify the egress PC5 link as the same PC5 link from which it received the first RRC message with the same L2 ID.

Step 9: The first intermediate relay UE removes the SRAP header and delivers DL SRB0 message (w/o SRAP header) on SL-RLC0 to the remote UE identified by the L2 ID in the SRAP header.

Step 10: Remote UE generates and sends its first SRB1 message (e.g., RRCSetupComplete) according to the dedicated configuration enclosed in RRCSetup, and then follow the baseline Approach 1 process for the remaining CP procedure.

As explained in the signalling diagram in Figure 1, remote UE and intermediate Relay UE can enter RRC_CONNECTED in parallel with significant latency reduction, while only introducing very limited changes in SRAP signalling.

Then, we discuss the FFS issues and key design considerations to enable the above scheme:

There was a proposal in RAN2#129bis meeting discussion [1] to only support fast forwarding of UL SRB0 message, but not DL SRB0 message. In our view, this does not realize the latency reduction objective of fast forwarding. This is because if the RRCSetup message(s) are still buffered in the gNB or U2N Relay UE, then the design still requires the upstream node to wait the downstream relay UEs to enter RRC_CONNECTED sequentially to solicit local ID allocation & configuration one by one. Moreover, with the support of DL SRB0 fast forwarding, the remote UE and relay UEs can start establishing PC5 Relay RLC channels for SRB1/2 traffic based on the received dedicated configuration in RRCSetup message much earlier, which helps to reduce the overall latency. Hence, we think it is quite important to allow SRB0 message in both directions to be forwarded in this expediated manner.

Proposal 1 	Fast forwarding is applicable to Both UL and DL SRB0 message.

To allow different remote UEs to be differentiated, we propose to have a new SRAP header attached to SRB0 RRC messages. In this SRAP header, the 24-bit Source L2 ID of the remote UE is indicated so that the receiving relay UE(s) can knows the source of the RRC message. The SRAP header format is designed as shown in Figure 2:

Figure 2: PC5 SRAP PDU format for SRB0 message

Note that when this new header is used for forwarding SRB0 message, the UE ID field in the header does not need to be used. In UL direction, the PC5 SRAP header is formed when local ID has not been allocated yet. For DL case, although local ID is known by the last relay UE, it does not need to include this information in the SRAP header because the same local ID allocation is indicated in the sl-L2RemoteUE-Config of RRCSetup message itself.

Observation 1	Remote UE get to know its Local ID allocation from DL RRC message (SRB0). UE ID indication need not to be set in the new SRAP header with L2 ID.

As each relay UE may have more than one child UE, it is important for relay UE to figure out which egress PC5 link is to be used to forward the DL SRB0 message. This is done by the relay UE remembering the ingress PC5 link associated with a L2 ID in SRAP header. And this ingress link will be used as the egress link for the DL SRB0 message associate with the same L2 ID in SRAP header.

So, to support L2 ID based forwarding, we have the following two proposals:

Proposal 2	L2 ID is included in the SRAP header between relay UEs when forwarding SRB0 message in both directions.

Proposal 3	Relay UE remembers which PC5 link is received the first UL RRC message from the downstream nodes and uses it for egress PC5 link determination for DL SRB0 message.

There is also a concern that if the Uu SRAP header is changed to include L2 ID in the SRAP header, the gNB-DU has to obtain the “L2 ID” information form gNB-CU, along with the local ID (UE ID). This may involve some RAN2 signalling change and have RAN3 impact. However, the fast-forwarding scheme can work w/o change Uu SRAP header. This is because the L2 ID information of the remote UE will be reported by the last relay UE to gNB via SUI message, and the gNB will then allocate local ID and configure the last Relay UE so it can forward the SRB0 message as same as legacy. For the DL SRB0 message sent by gNB, the last relay UE can process the legacy Uu SRAP header and regenerate the new PC5 SRAP header with L2 ID and forward it to its child UE.

Observation 2	With Uu SRAP header unchanged, there is no RAN3 impact in regards of support fast forwarding.

Also, the SRAP operation in the first PC5 hop remains as same as legacy L2 U2N design, which means the remote UE can simply transmit the same SRB0 message via SL-RLC0 regardless of whether the first relay UE is in IDLE/INACTIVE or CONNECTED state and whether the first relay UE supports the fast forwarding or not. The first relay UE, if not in RRC_CONNECTED, can generate the SRAP header with L2 ID obtained from the MAC address of the received packet from remote UE.

Proposal 4	The SRAP operation in the first PC5 hop (Remote-FirstRelay) and last hop (Uu hop) remain as same as Rel-17 legacy.

When the new SRAP header is introduced for fast forwarding, the legacy operation to generate SRAP PDU for SRB0 based on Rel-17 design is still valid.  This means there will be two SRAP PDU formats to be supported between the Relay UEs. Namely, the legacy SRAP PDU format w/o SRAP header will be sent by an intermediate relay UE acting as a remote UE, and the new SRAP PDU format with new SRAP header will be sent by an intermediate relay UE forwarding its downstream node’s request. If both of format are supported in SL-RLC0, then the SRAP entity in the receiving UE does not know which format an incoming SRAP PDU on SL-RLC0 uses. Hence, it is necessary to introduce a new pre-configured SL-RLC channel to the fast-forwarding SRAP operation on the hop between two relay UEs, as shown in Figure 3.

Figure 3: Self-generated SRB0 message in SL-RLC0 and forwarded SRB0 message in SL-RLCx

This new channel SL-RLCx (FFS exact name) is used by intermediate relay UEs in UL direction and by the last relay UE and intermediate relay UE (except first relay UE) in DL direction.

Proposal 5	A new default SL-RLCx is introduced between relay UEs to carry fast-forwarded SRB0 traffic.

Regarding “FFS what level of gNB awareness of the path information would be needed”, we think there is no need for gNB to be aware of the full topology to support fast-forwarding. In baseline design, when gNB receives the SUI message from the last relay UE, it is only aware of the last relay UE and the list of connected remote UE(s) reported by the last relay UE. When fast forwarding is introduced, the last relay UE will report some additional indirect connected downstream UEs to gNB, but gNB still does not know the relationship among those UEs. Basically, gNB only generates RRCSetup message including the local ID assignment for the remote UE and SRAP mapping needed for remote UE to further send its Uu SRB1/2 traffic. The gNB does not need to provide exact egress PC5 link information or PC5 Relay RLC channel for the DL forwarding of SRB0. This can be automatically done by the method explained in Proposal 3 and Proposal 5.

Note that when each UE enters CONNECTED and sends its respective SUI message to report its child UE (as remote UE) to gNB, gNB will know which L2 ID represents a child UE of the parent UE and be aware of the full path information. Also, as gNB has already allocated the local ID(s) based on reported L2 ID(s) in the SUI message from the Last Relay UE, the gNB will not need to assign new local ID based on those SUI messages. Then, gNB can provide further reconfiguration (e.g., SRAP configuration based on those local IDs already assigned), if needed, for DRB traffic, which is as similar as baseline procedure.

Observation 3	gNB awareness of full path information is not needed when handling SRB0 message.

Note that the DL SRB0 message (i.e., RRCSetup) need provide SRAP configuration for SRB1 when gNB is stil not aware of the full path information. For this, the gNB only contains SRAP mapping (local ID -> egress PC5 relay RLC channel in DL) for SRB1 w/o providing next-hop L2 ID, the reflective mapping can be used by each relay UE to determine the DL egress PC5 link. It is also worth noting that such a configuration actually conforms with the Legacy Rel-17 configuration for each relay UE, as explicit next-hop L2 ID is not provided by gNB, only the L2 address of (each) remote UE is indicated.

Proposal 6	SRAP configurations in DL SRB0 message for each Relay UE for SRB1 message forwarding can be signaled by the gNB reusing the legacy Rel-17 SRAP configuration structure without knowing the path information. The relay UE determines the egress link based on reflective mapping.

As explained above, the SUI message send by the last relay UE does not need to provide full path information. It is also true that the fulAs explained above, the SUI message send by the last relay UE does not need to provide full path information. It is also true that the full path information is not known by the last relay UE as it cannot tell this relationship based on the first RRC message(s) received. However, the last relay UE can report the indirectly linked L2 IDs associated with forwarded SRB0 message(s) in SUI to solicit the Local ID allocation for those UEs so that SRB0 message can be forwarded by Last Relay UE with dedicatedly configured Uu Relay RLC channel mapped to those local ID(s). The current SidelinkUEInformationNR message needs to be enhanced to add a list of those L2 IDs. Note that those Destination Layer-2 IDs cannot be reported by reusing the legacy R17 list structure because it does not have corresponding sl-TxInterestedFreqListL2U2N to report (which is a mandatory IE in SUI) and does not need to solicit Tx resource from gNB.

There is no any other need of ASN.1 change in TS 38.331 to support SRB0 fast forwarding scheme.

Proposal 7	Support SUI signalling to report the L2 ID(s) of the indirectly connected downstream nodes to solicit local ID allocation.

Fast forwarding scheme is only to be used by Intermediate Relay UE in IDLE/INACTIVE OOC state. If the Intermediate Relay UE is in RRC_CONNECTED state, then it would be triggered to send a SUI message and follow NW configuration, as similar to the baseline procedure. Note that in the SUI message, the relay UE may report L2 IDs for all SRB0 triggers (as described in P6), not only the L2 ID of its child UE. That will allow this relay UE to forward the received SRB0 messages with respective local IDs correctly set in SRAP header. Figure 4 below described the signalling flow sequence of this scenario

Figure 4:  Fast-Forwarding Case with CONNECTED intermediate relay

Observation 4 A relay UE in RRC_CONNECTED forwards SRB0 message(s) received in SL-RLC0/SL-RLCx as same as baseline procedure.

Assuming the fast-forwarding is an optional capability for Approach 1, not all U2N relay UEs along the path may support it. For the mixed scenario where some of relay UEs not supporting fast forwarding, it is evident that whether an adjacent relay UE support the fast forwarding can be learned by PC5-RRC capability exchange procedure, which occurs when PC5 link is established. So, before the remote UE RRC establishment procedure is triggered, each relay UE has already known whether its parent relay or child relay supports fast forwarding or not.

Observation 5	 A relay UE can detect whether its next hop relay supports fast-forwarding or not via PC5-RRC Capability Exchange procedure.

Then, we consider the following three cases:

- If the first relay UE does not support fast-forwarding, no fast forwarding is possible. And the First Relay UE act as same as baseline procedure.

- If the first relay UE supports fast forwarding, but the second intermediate relay UE does not support, then the first relay UE knows its parent relay could not understand the new SRAP format, so it will not initiate the fast-forward procedure. The baseline CP procedure will be initiated instead.

- If the first relay UE and the second intermediate relay UE supports fast forwarding, but the last relay UE does not support this feature, then the second intermediate relay UE knows its parent relay could not understand the new SRAP format, so it will not continue the fast forward procedure. In this case, it will trigger itself enter RRC_CONNECED state and then baseline CP procedure will be continued as similar to the sequence shown in Figure 4.

Based on the analysis above, we can see there is no technical problems for the mixed scenarios when fast forwarding is introduced as an optional feature.

Proposal 8 	Assume fast forwarding is optional for U2N relay UEs to support. A Relay UE which does not support SRB0 fast-forward or has detected its parent relay does not support fast-forward should enter RRC_CONNECTED first and then forward the SRB0 message(s) to gNB as same as baseline procedure.

In regards of specification impact, we provide the Text Proposals for TS 38.351 and TS 38.331 in Annex.

# 3 Conclusion

In this contribution, we discuss the design of fast forwarding of SRB0 for Multi-hop SL relay, and have the following observations:

Observation 1	Remote UE get to know its Local ID allocation from DL RRC message (SRB0). UE ID indication need not to be set in the new SRAP header with L2 ID.

Observation 2	With Uu SRAP header unchanged, there is no RAN3 impact in regards of support fast forwarding.

Observation 3	gNB awareness of full path information is not needed when handling SRB0 message.

Observation 4 	A relay UE in RRC_CONNECTED forwards SRB0 message(s) received in SL-RLC0/SL-RLCx as same as baseline procedure.

Observation 5	A relay UE can detect whether its next hop relay supports fast-forwarding or not via PC5-RRC Capability Exchange procedure.

Based on the discussion, we propose:

Proposal 1 	Fast forwarding is applicable to Both UL and DL SRB0 message.

Proposal 2	L2 ID is included in the SRAP header between relay UEs when forwarding SRB0 message in both directions.

Proposal 3	Relay UE remember which PC5 link is received the first UL RRC message from the downstream nodes and use it for egress PC5 link determination for DL SRB0 message.

Proposal 4	The SRAP operation in the first PC5 hop (Remote-FirstRelay) and last hop (Uu hop) remain as same as Rel-17 legacy.

Proposal 5	A new default SL-RLCx is introduced between relay UEs to carry fast-forwarded SRB0 traffic.

Proposal 6	SRAP configurations in DL SRB0 message for each Relay UE for SRB1 message forwarding can be signaled by the gNB reusing the legacy Rel-17 SRAP configuration structure without knowing the path information. The relay UE determines the egress link based on reflective mapping.

Proposal 7	Support SUI signalling to report the L2 ID(s) of the indirectly connected downstream nodes to solicit local ID allocation.

Proposal 8 	Assume fast forwarding is optional for U2N relay UEs to support. A Relay UE which does not support SRB0 fast-forward or has detected its parent relay does not support fast-forward should enter RRC_CONNECTED first and then forward the SRB0 message(s) to gNB as same as baseline procedure.

# 4 References

[1] 	Chairman’s Notes, RAN2#129bis

[2] 	R2-2502189	Fast & Parallel RRC Establishment/Configuration for Multi-hop U2N relaying	Apple, FirstNet, Ericsson, AT&T, Kyocera	discussion	Rel-19	NR_SL_relay_multihop

[3] 	R2-2502778	Consideration on CP issues for multi-hop SL relay	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

# Annex A: Text Proposal for TS 38.351

First change

## 5.2	DL Data transfer

<text omitted>

### 5.2.2	Transmitting operation of U2N Relay UE

#### 5.2.2.0	General

The transmitting part of the SRAP entity on the PC5 interface of U2N Relay UE receives SRAP data packets from the receiving part of the SRAP entity on the Uu interface of the same U2N Relay UE, and construct SRAP Data PDUs as needed (see clause 4.2.2).

When the transmitting part of the SRAP entity on the PC5 interface has an SRAP Data PDU to transmit, the transmitting part of the SRAP entity on the PC5 interface shall:

-	If the SRAP Data PDU is for SRB0 (the BEARER ID field is 0, and the bearer is identified as SRB based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelUu which matches the LCID of the Uu Relay RLC Channel from which the SRAP Data PDU is received or the SRAP Data PDU is received from SL-RLCx):

-	if the SRAP Data PDU is determined to be eligible for fast forwarding operation according to 5.2.2.x:

-	Determine egress link & RLC channel and prepare SRAP Data PDU according to 5.2.2.y;

-	else:

-	Remove the SRAP header from the SRAP Data PDU;

- 	else :

- 	Determine the egress link in accordance with clause 5.2.2.1;

-	Determine the egress RLC channel in accordance with clause 5.2.2.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel of the determined egress link.

Next change

#### 5.2.2.x	Determine the eligibility of SRAP Data PDU for DL fast forwarding

For a SRAP Data PDU to be transmitted, SRAP entity shall determine the eligibility of fast forwarding as follow:

-	if the SRAP Data PDU contains a L2 ID field:

-	the SRAP Data PDU is eligible for fast forwarding.

- 	else if there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU but whose sl-L2IdentityRemote does not match any Child Relay UE’s L2 ID:

#### -	the SRAP Data PDU is eligible for fast forwarding.

#### 5.2.2.y	Handling of DL fast forwarding

For U2N Relay UE capable of fast forwarding operation and fast forwarding is allowed between this UE and its Child Relay as specified in TS 38.331, the transmitting part of the SRAP entity on the PC5 interface shall:

-	if the SRAP Data PDU contains a L2 ID field:

-	if the UE is the First Relay UE:

- 	Determine the egress link on PC5 interface as the one corresponding to the L2 ID field;

- 	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLC0 as specified in TS 38.331 [3];

- 	Remove the SRAP header from the SRAP Data PDU;

-	else if L2 ID field matches a stored Remote UE L2 ID as specified in 5.3.2:

-	Determine the egress link on PC5 interface as the one corresponding to the ingress PC5 link associated with the stored Remote UE L2 ID;

-	Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLCx as specified in TS 38.331 [3];

-	else if there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay (as specified in TS 38.331 [3]) matches the UE ID field in SRAP Data PDU and sl-L2IdentityRemote matches a stored Remote UE L2 ID as specified in 5.3.2:

-  Determine the egress link on PC5 interface as the one corresponding to the ingress PC5 link associated with the stored Remote UE L2 ID;

-  Determine the egress PC5 Relay RLC channel in the determined egress link corresponding to logicalChannelIdentity for SL-RLCx as specified in TS 38.331 [3];

-	Construct an SRAP Data PDU with enhanced SRAP header as specified in clause 6.2.2, where the L2 ID field is set as sl-L2IdentityRemote.

Next change

## 5.3	UL Data transfer

<text omitted>

### 5.3.2	Receiving operation of U2N Relay UE

Upon receiving an SRAP Data PDU from lower layer, the receiving part of the SRAP entity on the PC5 interface shall:

-	Deliver the SRAP data packet to the transmitting part of the collocated SRAP entity on the Uu interface.

-	If L2 ID field is present and BEARER ID field is 0 in SRAP Data PDU:

-	Store the Remote UE Layer-2 ID included in L2 ID and associate it with the ingress PC5 link from which the SRAP Data PDU is receveid;

-	else if the U2N Relay UE is the last U2N relay UE and there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU and there is a stored Remote UE Layer-2 ID matches the sl-L2IdentityRemote with the concerned sl-LocalIdentity:

- 	Remove the corresponding stored Remote UE Layer-2 ID and its association to the PC5 link.

### 5.3.3	Transmitting operation of U2N Relay UE

The transmitting part of the SRAP entity on the Uu interface of U2N Relay UE can receive SRAP data packets from the receiving part of the SRAP entity on the PC5 interface of the same U2N Relay UE, and construct SRAP Data PDUs as needed (see clause 4.2.2).

When the transmitting part of the SRAP entity on the Uu interface has an SRAP Data PDU to transmit, the transmitting part of the SRAP entity on the Uu interface shall:

-	If the SRAP Data PDU is received from SL-RLC0 or SL-RLCx as specified in TS 38.331 [3] and the UE is last U2N relay UE:

-	Determine the UE ID field and BEARER ID field in accordance with clause 5.3.3.1;

-	Construct an SRAP Data PDU with SRAP header, where the UE ID field and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

When the transmitting part of the SRAP entity on the PC5 interface has an SRAP Data PDU to transmit, the transmitting part of the SRAP entity on the PC5 interface shall:

-	Determine the egress link as the PC5 link to its parent relay UE as specified in TS 38.331 [3];

-	If the UE is an Intermediate U2N relay UE in CONNECTED state or fast forwarding operation is disallowed between this Intermediate U2N relay UE and its Parent Relay according to TS 38.331 [3];

-	if the SRAP Data PDU is received from SL-RLC0 and the UE is the First Relay UE; or

-	if the SRAP Data PDU is received from SL-RLCx and the UE is not the First Relay UE;

-	Determine the UE ID field and BEARER ID field in accordance with clause 5.3.3.1;

-	Construct an SRAP Data PDU with SRAP header, where the UE ID field and BEARER ID field are set to the determined values, in accordance with clause 6.2.2;

-	else if the SRAP Data PDU is received from SL-RLC0 and the UE is the First Relay UE:

-	Construct an SRAP Data PDU with SRAP header, where the L2 ID field is set as the Source Layer-2 ID of the U2N remote UE, and BEAER ID field is set to 0;

-	Determine the egress RLC channel in accordance with clause 5.3.3.2;

-	Submit this SRAP Data PDU to the determined egress RLC channel.

#### 5.3.3.1	UE ID field and BEARER ID field determination

For an SRAP Data PDU received from SL-RLC0 or SL-RLCx as specified in TS 38.331 [3], the SRAP entity shall:

-	If there is an entry in sl-RemoteUE-ToAddModList, whose sl-L2IdentityRemote matches the Layer-2 ID of the remote UE from which the SRAP Data PDU is received or matches the L2 ID field in the SRAP header of the SRAP Data PDU received:

-	Determine the UE ID field corresponding to sl-LocalIdentity configured for the concerned sl-L2IdentityRemote as specified in TS 38.331 [3];

-	Determine the BEARER ID field as 0 (i.e., set BEARER ID field as 0).

#### 5.3.3.2	Egress RLC channel determination

For a SRAP Data PDU to be transmitted, the SRAP entity shall:

-	If SRAP Data PDU contains a L2 ID field:

-	Determine the egress PC5 Relay RLC channel to be SL-RLCx;

-	Else if there is an entry in sl-RemoteUE-ToAddModList, whose sl-LocalIdentity included in sl-SRAP-ConfigRelay matches the UE ID field in SRAP Data PDU:

-	If the SRAP Data PDU is for SRB0:

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for SRB0 for the concerned sl-LocalIdentity as specified in TS 38.331 [3].

-	Else if the SRAP Data PDU is received from SL-RLC1 as specified in TS 38.331 [3]:

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for SRB1 for the concerned sl-LocalIdentity as specified in TS 38.331 [3].

-	Else if there is an entry in sl-RemoteUE-ToAddModList which includes an sl-RemoteUE-RB-Identity matches SRB identity or DRB identity of the SRAP Data PDU determined by the BEARER ID field (For the BEARER ID shared by both SRB and DRB, SRB and DRB are differentiated based on sl-RemoteUE-RB-Identity associated with the entry containing the sl-EgressRLC-ChannelPC5 which matches LCID of the PC5 Relay RLC Channel from which the SRAP Data PDU is received, and for DRB, the DRB identity is BEARER ID plus 1):

-	Determine the egress Uu Relay RLC channel corresponding to sl-EgressRLC-ChannelUu configured for the concerned sl-LocalIdentity and concerned sl-RemoteUE-RB-Identity as specified in TS 38.331 [3].

Next change

### 6.2.2	Data PDU

<text omitted>

Figure 6.2.2-4 shows the format of the U2N SRAP Data PDU with enhanced SRAP header. This SRAP Data PDU format is applicable to multi-hop U2N relaying over both PC5 and Uu interface.

Figure 6.2.2-4: U2N SRAP Data PDU format with enhanced SRAP header

Next change

## 6.3	Parameters

.<text omitted>

### 6.3.x	F

Length: 1 bit

This field indicates SRAP PDU format (i.e., whether the corresponding SRAP header is of enhanced format or not). Bit set as "1" indicates the U2N SRAP Data PDU format with enhanced SRAP header as shown in Figure 6.2.2-4 is used. Bit set as "0" indicates U2N SRAP Data PDU format with SRAP header as shown in Figure 6.2.2.-1.

### 6.3.y	L2 ID

Length: 24 bits

This field indicates the Source Layer-2 ID used by the L2 Remote UE for L2 U2N Relay communication.

# Annex B: Text Proposal for TS 38.331

First change

#### 5.3.3.1a	Conditions for establishing RRC Connection for NR sidelink communication/discovery/V2X sidelink communication/MP operation

For NR sidelink communication/discovery, an RRC connection establishment is initiated only in the following cases:

1>	if configured by upper layers to transmit NR sidelink communication and related data is available for transmission:

2>	if the frequency on which the UE is configured to transmit NR sidelink communication is included in sl-FreqInfoList/sl-FreqInfoListSizeExt within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 does not include sl-TxPoolSelectedNormal for the concerned frequency;

1>	if configured by upper layers to transmit NR sidelink discovery and related data is available for transmission:

2>	if the UE is configured by upper layers to transmit NR sidelink L2 U2N relay discovery messages and sl-L2U2N-Relay is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink L3 U2N relay discovery messages and sl-L3U2N-RelayDiscovery is included in SIB12; or

2>	if the UE is configured by upper layers to transmit NR sidelink non-relay discovery messages and sl-NonRelayDiscovery is included in SIB12:

3>	if the frequency on which the UE is configured to transmit NR sidelink discovery is included in sl-FreqInfoList within SIB12 provided by the cell on which the UE camps; and if the valid version of SIB12 includes neither sl-DiscTxPoolSelected nor sl-TxPoolSelectedNormal for the concerned frequency;

For L2 U2N Relay UE in RRC_IDLE, an RRC connection establishment is initiated in the following cases:

1>	if any message is received from a L2 U2N Remote UE via SL-RLC0 as specified in 9.1.1.4 or SL-RLC1 as specified in 9.2.4;

1> if any message is received from an Intermediate Relay UE via SL-RLCx as specified in 9.1.1.4; or

1>	if RemoteUEInformationSidelink containing the connectionForMP is received from a L2 U2N Remote UE as specified in 5.8.9.8.3;

For V2X sidelink communication, an RRC connection is initiated only when the conditions specified for V2X sidelink communication in clause 5.3.3.1a of TS 36.331 [10] are met.

NOTE 1:	Upper layers initiate an RRC connection (except if the RRC connection is initiated at the L2 U2N Relay UE upon reception of a message from a L2 U2N Remote UE via SL-RLC0 or SL-RLC1, or upon reception of RemoteUEInformationSidelink message containing the connectionForMP). The interaction with NAS is left to UE implementation.

For N3C relay UE in RRC_IDLE, an RRC connection establishment is initiated when a N3C remote UE indicates it to enter RRC_CONNECTED state.

NOTE 2:	How/when the N3C remote UE to indicate N3C relay UE to enter RRC_CONNECTED state is left to UE implementation, e.g. before reporting relay UE information with non-3GPP connection(s).

Next change

#### 5.8.3.3	Actions related to transmission of SidelinkUEInformationNR message

<text omitted>

3>	if SIB12 includes sl-L2U2N-Relay or sl-L2U2N-MH-relay and if configured by upper layers to transmit NR sidelink L2 U2N relay communication and the UE is acting as L2 U2N Relay UE:

4>	include sl-TxResourceReqL2U2N-Relay in sl-TxResourceReqListCommRelay and set its fields (if needed) as follows for each destination for which it requests network to assign NR sidelink L2 U2N relay communication resource:

5>	set sl-DestinationIdentityL2U2N to the destination identity configured by upper layer for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TxInterestedFreqListL2U2N to indicate the frequency of the associated destination for NR sidelink L2 U2N relay communication transmission;

5>	set sl-TypeTxSyncListL2U2N to the current synchronization reference type used on the associated sl-TxInterestedFreqListL2U2N for NR sidelink L2 U2N relay communication transmission;

5>	set sl-LocalID-Request to request local ID for L2 U2N Remote UE transiting to RRC_CONNECTED or in RRC_CONNECTED state;

5>	set sl-PagingIdentityRemoteUE to the paging UE ID received from peer L2 U2N Remote UE, if it is not released as in 5.8.9.8.3;

5>	set sl-CapabilityInformationSidelink to include UECapabilityInformationSidelink message, if any, received from peer UE;

5>	if one or more fast-forwarded SRB0 message (see TS 38.351) forwarded by the Intermediate Relay UE corresponding to sl-DestinationIdentityL2U2N has been received by this L2 U2N relay UE;

6>	set sl-MultiHopLocalIDReqList to include the list of L2 ID(s) extracted from the SRAP Data PDU containing SRB0 message;

Next change

##### 5.8.9.2.5	Actions related to reception of the UECapabilityInformationSidelink by the UE

If the peer UE is a L2 U2N Relay UE which indicates not supporting the SRB0 fast forwarding capability, the UE shall regard the SRB0 fast forwarding operation is not allowed in the PC5 hop between this UE and the peer UE.

Next change

### 6.2.2	Message definitions

<text omitted>

#### –	SidelinkUEInformationNR

The SidelinkUEinformationNR message is used for the indication of NR sidelink UE information to the network.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

SidelinkUEInformationNR message

-- ASN1START

-- TAG-SIDELINKUEINFORMATIONNR-START

SidelinkUEInformationNR-r16::=         SEQUENCE {

criticalExtensions                  CHOICE {

sidelinkUEInformationNR-r16         SidelinkUEInformationNR-r16-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SidelinkUEInformationNR-r16-IEs ::=    SEQUENCE {

sl-RxInterestedFreqList-r16            SL-InterestedFreqList-r16           OPTIONAL,

sl-TxResourceReqList-r16               SL-TxResourceReqList-r16            OPTIONAL,

sl-FailureList-r16                     SL-FailureList-r16                  OPTIONAL,

lateNonCriticalExtension               OCTET STRING                        OPTIONAL,

nonCriticalExtension                   SidelinkUEInformationNR-v1700-IEs   OPTIONAL

}

SidelinkUEInformationNR-v1700-IEs ::=  SEQUENCE {

sl-TxResourceReqList-v1700             SL-TxResourceReqList-v1700                                                 OPTIONAL,

sl-RxDRX-ReportList-v1700              SL-RxDRX-ReportList-v1700                                                  OPTIONAL,

sl-RxInterestedGC-BC-DestList-r17      SL-RxInterestedGC-BC-DestList-r17                                          OPTIONAL,

sl-RxInterestedFreqListDisc-r17        SL-InterestedFreqList-r16                                                  OPTIONAL,

sl-TxResourceReqListDisc-r17           SL-TxResourceReqListDisc-r17                                               OPTIONAL,

sl-TxResourceReqListCommRelay-r17      SL-TxResourceReqListCommRelay-r17                                          OPTIONAL,

ue-Type-r17                            ENUMERATED {relayUE, remoteUE}                                             OPTIONAL,

sl-SourceIdentityRemoteUE-r17          SL-SourceIdentity-r17                                                      OPTIONAL,

nonCriticalExtension                   SidelinkUEInformationNR-v1800-IEs                                          OPTIONAL

}

SidelinkUEInformationNR-v1800-IEs ::=  SEQUENCE {

sl-CarrierFailureList-r18              SL-CarrierFailureList-r18                                                  OPTIONAL,

sl-TxResourceReqListL2-U2U-r18         SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReqL2-U2U-r18      OPTIONAL,

sl-PosRxInterestedFreqList-r18         SL-InterestedFreqList-r16                                                  OPTIONAL,

sl-PosTxResourceReqList-r18            SL-PosTxResourceReqList-r18                                                OPTIONAL,

nonCriticalExtension                   SidelinkUEInformationNR-v1840-IEs                                          OPTIONAL

}

SidelinkUEInformationNR-v1840-IEs ::=  SEQUENCE {

sl-PosRxInterestedFreqList2-r18        SL-InterestedFreqList-r16                                                  OPTIONAL,

nonCriticalExtension                   SEQUENCE {}                                                                OPTIONAL

}

SL-InterestedFreqList-r16 ::=          SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16)

SL-TxResourceReqList-r16 ::=           SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReq-r16

SL-PosTxResourceReqList-r18 ::=        SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-PosTxResourceReq-r18

SL-TxResourceReq-r16 ::=               SEQUENCE {

sl-DestinationIdentity-r16             SL-DestinationIdentity-r16,

sl-CastType-r16                        ENUMERATED {broadcast, groupcast, unicast, spare1},

sl-RLC-ModeIndicationList-r16          SEQUENCE (SIZE (1.. maxNrofSLRB-r16)) OF SL-RLC-ModeIndication-r16         OPTIONAL,

sl-QoS-InfoList-r16                    SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16          OPTIONAL,

sl-TypeTxSyncList-r16                  SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16                OPTIONAL,

sl-TxInterestedFreqList-r16            SL-TxInterestedFreqList-r16                                                OPTIONAL,

sl-CapabilityInformationSidelink-r16   OCTET STRING                                                               OPTIONAL

}

SL-TxResourceReqList-v1700 ::=         SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReq-v1700

SL-RxDRX-ReportList-v1700 ::=          SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-RxDRX-Report-v1700

SL-TxResourceReq-v1700 ::=             SEQUENCE {

sl-DRX-InfoFromRxList-r17              SEQUENCE (SIZE (1..maxNrofSL-RxInfoSet-r17)) OF SL-DRX-ConfigUC-SemiStatic-r17   OPTIONAL,

sl-DRX-Indication-r17                  ENUMERATED {on, off}                                                             OPTIONAL,

...,

[[

sl-QoS-InfoList-v1800                  SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-v1800              OPTIONAL

]]

}

SL-RxDRX-Report-v1700 ::=              SEQUENCE {

sl-DRX-ConfigFromTx-r17                SL-DRX-ConfigUC-SemiStatic-r17,

...

}

SL-RxInterestedGC-BC-DestList-r17 ::=  SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-RxInterestedGC-BC-Dest-r17

SL-RxInterestedGC-BC-Dest-r17 ::=      SEQUENCE {

sl-RxInterestedQoS-InfoList-r17        SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16,

sl-DestinationIdentity-r16             SL-DestinationIdentity-r16

}

SL-TxResourceReqListDisc-r17 ::=       SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReqDisc-r17

SL-TxResourceReqDisc-r17 ::=           SEQUENCE {

sl-DestinationIdentityDisc-r17         SL-DestinationIdentity-r16,

sl-SourceIdentityRelayUE-r17           SL-SourceIdentity-r17                                                      OPTIONAL,

sl-CastTypeDisc-r17                    ENUMERATED {broadcast, groupcast, unicast, spare1},

sl-TxInterestedFreqListDisc-r17        SL-TxInterestedFreqList-r16,

sl-TypeTxSyncListDisc-r17              SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16,

sl-DiscoveryType-r17                   ENUMERATED {relay, non-Relay},

...,

[[

ue-TypeU2U-r18                         ENUMERATED {relayUE, remoteUE}                                             OPTIONAL

]]

}

SL-TxResourceReqListCommRelay-r17 ::=  SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReqCommRelayInfo-r17

SL-TxResourceReqCommRelayInfo-r17 ::=  SEQUENCE {

sl-RelayDRXConfig-r17                 SL-TxResourceReq-v1700                                                       OPTIONAL,

sl-TxResourceReqCommRelay-r17         SL-TxResourceReqCommRelay-r17

}

SL-TxResourceReqCommRelay-r17 ::=      CHOICE {

sl-TxResourceReqL2U2N-Relay-r17        SL-TxResourceReqL2U2N-Relay-r17,

sl-TxResourceReqL3U2N-Relay-r17        SL-TxResourceReq-r16

}

SL-TxResourceReqL2U2N-Relay-r17 ::=    SEQUENCE {

sl-DestinationIdentityL2U2N-r17        SL-DestinationIdentity-r16                                                 OPTIONAL,

sl-TxInterestedFreqListL2U2N-r17       SL-TxInterestedFreqList-r16,

sl-TypeTxSyncListL2U2N-r17             SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16,

sl-LocalID-Request-r17                 ENUMERATED {true}                                                          OPTIONAL,

sl-PagingIdentityRemoteUE-r17          SL-PagingIdentityRemoteUE-r17                                              OPTIONAL,

sl-CapabilityInformationSidelink-r17   OCTET STRING                                                               OPTIONAL,

...

[[

sl-MultiHopLocalIDReqList-r19  SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) SL-DestinationIdentity-r16 OPTIONAL,

]]

}

<text omitted>

SL-TxResourceReqCommRelayInfo field descriptions

sl-RelayDRXConfig This field is used to indicate the applied sidelink DRX configuration for the relay related communication.

sl-DestinationIdentityL2U2N This field is used to indicate the destination L2 ID for which the TX resource request and allocation from the network are concerned for the established PC5 link for relay by L2 U2N Relay UE.

sl-LocalID-Request This field is used to request local UE ID for the corresponding destination by the L2 U2N Relay UE.

sl-TxInterestedFreqListL2U2N Each entry of this field indicates the index of frequency on which the UE is interested to transmit NR sidelink communication for established PC5 link for relay. The value 1 corresponds to the frequency of first entry in sl-FreqInfoList broadcast in SIB12, the value 2 corresponds to the frequency of second entry in sl-FreqInfoList broadcast in SIB12 and so on. In this release, only value 1 can be included in the interested frequency list. In this release, only one entry can be included in the list.

sl-PagingIdentityRemoteUE This field is used to indicate the paging UE ID(s) for the corresponding destination(s) by the L2 U2N Relay UE.

sl-MultiHopLocalIDReqList This field is used to indicate the L2 ID(s) of downstream UEs connected via the Intermediate L2 U2N Relay UE for which the Tx Resource is requested.

Next change

#### 9.1.1.4	SCCH configuration

<text omitted>

Parameters that are specified for NR sidelink L2 U2N Relay operations, which is used for the PC5 Relay RLC channel for Remote UE's SRB0 message transmission/reception. The PC5 Relay RLC channel using this configuration is named as SL-RLC0.

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC configuration |  | AM |  |
| >sn-FieldLength | 12 |  |  |
| >t-Reassembly | Undefined | Selected by the receving UE, up to UE implementation |  |
| >t-PollRetransmit | Undefined | Selected by the transmitting UE, up to UE implementation |  |
| >pollPDU | Undefined | Selected by the transmitting UE, up to UE implementation |  |
| >pollByte | Undefined | Selected by the transmitting UE, up to UE implementation |  |
| >maxRetxThreshold | Undefined | Selected by the transmitting UE, up to UE implementation |  |
| >t-StatusProhibit | Undefined | Selected by the receiving UE, up to UE implementation |  |
| >logicalChannelIdentity | 56 |  |  |
| MAC configuration |  |  |  |
| >priority | 1 |  |  |
| >prioritisedBitRate | Infinity |  |  |
| >logicalChannelGroup | 0 |  |  |
| >schedulingRequestId | 0 | The scheduling request configuration with this value is applicable for this SCCH if configured by the network. The scheduling request configuration is not applicable to L2 U2N Remote UE. |  |
| >sl-HARQ-FeedbackEnabled | Undefined | Selected by the transmitting UE, up to UE implementation |  |

Parameters that are specified for Multi-hop NR sidelink L2 U2N Relay operations, which is used for the PC5 Relay RLC channel for forwarding Remote UE's SRB0 message. The PC5 Relay RLC channel using this configuration is named as SL-RLCx.

| Name | Value | Semantics description | Ver |
| --- | --- | --- | --- |
| RLC configuration |  | AM |  |
| >sn-FieldLength | 12 |  |  |
| >t-Reassembly | Undefined | Selected by the receving UE, up to UE implementation |  |
| >t-PollRetransmit | Undefined | Selected by the transmitting UE, up to UE implementation |  |
| >pollPDU | Undefined | Selected by the transmitting UE, up to UE implementation |  |
| >pollByte | Undefined | Selected by the transmitting UE, up to UE implementation |  |
| >maxRetxThreshold | Undefined | Selected by the transmitting UE, up to UE implementation |  |
| >t-StatusProhibit | Undefined | Selected by the receiving UE, up to UE implementation |  |
| >logicalChannelIdentity | 52 |  |  |
| MAC configuration |  |  |  |
| >priority | 1 |  |  |
| >prioritisedBitRate | Infinity |  |  |
| >logicalChannelGroup | 0 |  |  |
| >schedulingRequestId | 0 | The scheduling request configuration with this value is applicable for this SCCH if configured by the network. The scheduling request configuration is not applicable to L2 U2N Remote UE. |  |
| >sl-HARQ-FeedbackEnabled | Undefined | Selected by the transmitting UE, up to UE implementation |  |