3GPP TSG-RAN WG2 Meeting #130	R2-2503727

St. Julians, Malta, 19-23 May 2025

Agenda item:	8.13.3

Source:	Apple

Title:	Discussion on SRAP and Control Plane for Multi-hop Layer-2 U2N Relay

WID/SID:	NR_SL_relay_multihop – Release 19

Document for:	Discussion and Decision

# 1 Introduction

There are a few basic agreements made in the last few RAN2 meetings [1][2] regarding the SRAP and RRC, but the detailed SRAP and RRC signaling design aspects are barely touched or not essentially discussed yet.

In this paper, we discuss the SRAP procedure and RRC signaling design issues for multi-hop U2N relaying.

# 2 Discussion

### 2.1 SRAP procedure and SRAP-related configuration for Relay UE

In Legacy Rel-17 U2N relay, each relay UE is configured with a SRAP mapping for a local ID with the remote UE L2 ID and the SRAP mapping per bearer information

SL-RemoteUE-ToAddMod-r17 ::= SEQUENCE {

sl-L2IdentityRemote-r17 SL-DestinationIdentity-r16,

sl-SRAP-ConfigRelay-r17 SL-SRAP-Config-r17 OPTIONAL, -- Need M

...

}

SL-SRAP-Config-r17 ::= SEQUENCE {

sl-LocalIdentity-r17 INTEGER (0..255) OPTIONAL, -- Need M

sl-MappingToAddModList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-MappingToAddMod-r17 OPTIONAL, -- Need N

sl-MappingToReleaseList-r17 SEQUENCE (SIZE (1..maxLC-ID)) OF SL-RemoteUE-RB-Identity-r17 OPTIONAL, -- Need N

...

}

SL-MappingToAddMod-r17 ::= SEQUENCE {

sl-RemoteUE-RB-Identity-r17 SL-RemoteUE-RB-Identity-r17,

sl-EgressRLC-ChannelUu-r17 Uu-RelayRLC-ChannelID-r17 OPTIONAL, -- Cond L2RelayUE

sl-EgressRLC-ChannelPC5-r17 SL-RLC-ChannelID-r17 OPTIONAL, -- Need N

...

}

From the signaling design above, the Rel-17 Relay only knows the remote UE’s L2 ID in the single-hop relay case. However, in multi-hop relay UE case, the relay UE need to know the next-hop L2 ID which is different from the remote UE’s L2 ID. The current signaling could be redesigned to introduce this L2 ID hierarchy where a direct child and its indirect children’s L2 ID are all provided to NW. That will make the SRAP configuration of RRC signaling rather complex. This can be avoided by support “reflective mapping”, which allows the relay UE to remember the L2 ID of the ingress link for a local ID, then use it for DL egress PC5 link determination. With this method, the SRAP configuration does not need to have a complex change and keep it largely as same as the legacy design. To be more specific, the new IE such as “sl-SRAP-ConfigRelayList “ in the RRC running CR is not needed.

Proposal 1 	Reflective mapping is used for SRAP entity to derive DL egress PC5 link as it does not require the change of legacy SRAP configuration.

Note that in R17 U2N Relay, Uu and PC5 Relay RLC mapping are both included for the (last) relay UE case. But for intermediate relay UE, this has to be two PC5 Relay RLC channels to be used in UL and DL directions respectively. Therefore, we would like to discuss the relay UE behavior when it receives one RLC channel configuration from sl-ConfigDedicatedNR, whether the relay UE need to establish the RLC channel in both directions. The problem can be explained in the following Figure 1.

Figure 1: Double-Side PC5 Relay RLC Channel Configuration Issue

As shown in Figure 1, UE2 (Intermediate Relay UE) first establish itself as a remote UE and get configuration of PC5 SL RLC channel Configuration in sl-ConfigDedicatedNR , which are to be used by UE2 to establish PC5 Relay RLC channels towards UE3 (Last Relay UE) for its UL traffic. Later, when remote UE1’s SRB0 message reaches the UE2, UE2 will trigger SidelinkUEInformationNR procedure to report this remote UE. As a result, NW may send new sl-ConfigDedicatedNR, which contain an additional PC5 SL RLC channel Configuration, which is needed to support the DL forwarding to the new UE1. However, according to the default UE behavior, this configuration will be used to establish PC5 Relay RLC channel towards every peer UE, which includes both UE1 and UE3. While it makes sense to establish this new PC5 relay RLC channel towards the remote UE, setting up one additional RLC channel towards UE1 does not make sense, and would not be useful. So, we think RAN2 need to discuss whether the PC5 Relay RLC channel configuration is directional agnostic, and if yes, whether the relay UE need to establish all of them in both directions.

Proposal 2 	RAN2 discuss whether SL-RLC Channel configuration for Intermediate relay UE is per direction or for both directions. And if the configuration is direction-agnostic, what is the correct UE behaviour.

Note that even if we decide the SL RLC channel configuration is direction agnostic, this does not mean the dedicated RLC channel configurations for each relay UE needs to be doubled. It is very possible that one SL RLC channel configuration template represented by one “SL-RLC-ChannelID” can still be used to represent two RLC channels in different directions (i.e., one towards parent node and one towards the child node). Given that the PDB requirements for DRB traffic may be different in UL and DL case, it is better to include both an UL PDB and DL PDB configuration in one template so that when this SL RLC channel is established in DL PC5-RRC signaling, the DL PDB will be used. This can help to reduce the Uu signaling overhead for Uu RRC configuration.

Proposal 3 	Both UL PDB and DL PDB can be provided in SL-RLC-ChannelConfig together in dedicated RRC configuration for intermediate relay UE to use in UL direction or DL direction respectively.

For multi-hop relaying, it will be a lot of redundant signaling for each relay UE to be configured again and again for SRAP mapping for each new local ID, which could be a child of child of a relay UE and becomes burdensome with the increasing number of hops. For good practice, it is wise to introduce a default SRAP mapping concept so that if NW does not provide an explicit SRAP mapping configuration for a “local ID”, then the default mapping is assumed by the relay UE for this local ID. The easiest way to realize this is to designate default PC5 Relay RLC channel configuration(s). Therefore, when one or more RBs are not explicitly configured with a mapping, then the RLC channel setup based on the default RLC channel configuration is assumed to be egress RLC channel.

Proposal 4 	When the SRAP mapping configuration (RB to Relay RLC channel) is not provided for a “local ID”, the default PC5 relay RLC channel(s) are used by the relay UE to forward the SRAP PDU for this “local ID”.

Moreover, NW can choose to configure default RLC channel for upstream traffic, downstream-traffic, SRB or DRB, or all of them, depends on what is suitable for the gNB. It will be a very useful tool for NW to reduce the configuration overhead.

Proposal 5 	Support to label a certain SL-RLC-ChannelConfig or SL-RLC-Channel-ID to be the default egress RLC channel for a certain kind of end-to-end traffic (SRB or DRB, upstream or downstream).

### 2.2 PC5 RLF notification

When a relay UE encounters PC5-RLF, it may occur in either upstream or downstream. If the PC5-RLF is in upstream, then the relay UE may trigger a relay reselection. However, if the PC5-RLF is between the relay UE and its child node, and the relay UE is in RRC_CONNECTED state, then it may report this to the gNB via SUI procedure. However, in multi-hop relay case, even with SUI message sent, its upstream relay node does not know the RLF. So, those nodes may  continue to conduct communication-related functions towards a downstream node which is no longer reachable. Hence, it is important for an intermediate relay UE to also notify the parent relay about PC5-RLF towards a child.

Proposal 6 	Allow Intermediate Relay UE to send/forward PC5-RLF notifications to upstream nodes via PC5-RRC.

### 2.3  Paging Monitoring

As each relay UE is responsible to submit and cancel the SI/Paging request for all its children UE and itself, it makes sense to allow the relay UE to aggregate the requests and send them in one PC5-RRC message. Logically, this can also be done by the relay UE to send multiple PC5-RRC request one-by-one, but this is a wasteful of SL radio resource and create some redundancy. Also, all those requests maybe triggered simultaneously and sending them one by one will add unnecessary latency. For example, when a PC5-RLF occurs, it no longer makes sense to monitor the paging for the child and grandchildren severed by the failed link. In another case, when a relay UE enters RRC_CONNECTED, it need to cancel the paging monitoring for itself and all its children and grandchildren to avoid duplicate paging monitoring. In those cases, it is much better to just send one signalling to cancel the paging monitoring for all those remote UEs. Adding this “aggregated request” has very limited spec impact, but yields much greater flexibility Therefore, we think it is desirable to change ASN.1 to support it.

Proposal 7 	Extend RemoteUEInformationSidelink message to allow aggregation of multiple paging/SI requests together.

It is also worth noting that the paging records forwarded by relay UE can also be aggregated, especially when the relay UE can intercept Uu paging message which contains more than one paging records matching the paging monitoring requests from the path through one child relay. In this case, it is natural to forward all those matching paging records together to this child relay. Therefore, we think similar changes of

Proposal 8 	Allow UuMessageTransferSidleink to contain a list of PagingRecord.

# 3 Conclusion

In this contribution, we discuss some SRAP and RRC remaining issues and have the following proposals:

Proposal 1 	Reflective mapping is used for SRAP entity to derive DL egress PC5 link as it does not require the change of legacy SRAP configuration .

Proposal 2 	RAN2 discuss whether SL-RLC Channel configuration for Intermediate relay UE is per direction or for both directions. And if the configuration is direction-agnostic, what is the correct UE behaviour.

Proposal 3 	Both UL PDB and DL PDB can be provided in SL-RLC-ChannelConfig together in dedicated RRC configuration for intermediate relay UE to use in UL direction or DL direction respectively .

Proposal 4	When the SRAP mapping configuration (RB to Relay RLC channel) is not provided for a “local ID”, the default PC5 relay RLC channel(s) are used by the relay UE to forward the SRAP PDU for this “local ID” .

Proposal 5 	Support to label a certain SL-RLC-ChannelConfig or SL-RLC-Channel-ID to be the default egress RLC channel for a certain kind of end-to-end traffic ( SRB or DRB, upstream or downstream ).

Proposal 6 	Allow Intermediate Relay UE to send/forward PC5-RLF notifications to upstream nodes via PC5-RRC .

Proposal 7 	Extend RemoteUEInformationSidelink message to allow aggregation of multiple paging/SI requests together.

Proposal 8 	Allow UuMessageTransferSidleink to contain a list of PagingRecord.

# 4 References

[1] Chairman’s notes, RAN2#129

[2] Chairman’s notes, RAN2#129bis