3GPP TSG-RAN WG2 Meeting #130	R2-2503743

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	7.0.2.22

Source: 	Fujitsu

Title:	Data loss issue on fast LTM recovery

Document for:	Discussion and Decision

- Introduction

In this contribution, we would like to point out that there is a data loss issue in case of fast LTM recovery to an intra-DU cell after inter-DU LTM failure and discuss how to solve it.

- Discussions

  - Issue on fast LTM recovery to intra-DU cell after inter-DU LTM failure

For Rel-18 LTM, fast LTM recovery after LTM failure is supported, including fast LTM recovery to an intra-DU cell after inter-DU LTM cell switch failure. However, there is a potential data loss issue in case of fast LTM recovery to an intra-DU cell after inter-DU LTM cell switch failure. We explain the issue with Figure 1.

Figure 1

1. The UE receives LTM CSC MAC CE and switches the serving cell from Cell-1 (ltm-ServingCellNoResetId = 1) to Cell-3 (ltm-NoReserId = 2). As ltm-NoResetId in the target cell is different from ltm-ServingCellNoResetId, RLC re-establishment is performed and RLC buffer becomes empty. Also, PDCP data recovery for AM DRB is performed. Then, the LTM cell switch fails. PDCP data recovery also fails.

2. The UE reverts back to the source PCell (Cell-1) configuration. In this reverting back operation, RLC SDUs, SDU segments and PDUs cannot be retrieved as explained in [1]. Therefore, RLC buffer remains empty.

3. During RRC re-establishment procedure, the UE selects Cell-2 (ltm-NoResetId = 1) and performs fast LTM recovery to Cell-2. As ltm-NoResetId in the target cell is the same with ltm-ServingCellNoResetId, RLC re-establishment is not performed. Also, PDCP data recovery for AM DRB is not performed even though RLC buffer is empty. This causes data loss issue.

Observation 1: In the case of fast LTM recovery to an intra-DU cell after inter-DU LTM cell switch failure, data loss issue will be caused. This is because:

1. At the failed inter-DU LTM cell switch, RLC re-establishment and PDCP data recovery for AM DRB is performed. But PDCP data recovery also fails due to LTM cell switch failure.

2. When the UE reverts back to the source PCell configuration, RLC SDUs, SDU segments and PDUs cannot be retrieved as they are already discarded.

3. If the UE performs fast LTM recovery to an intra-DU cell, PDCP data recovery is not performed even though RLC SDUs, SDU segments and PDUs are already discarded.

We see that the issue is not a corner case. Also, data loss on AM DRB will affect application services. We propose RAN2 should confirm this issue needs to be solved.

Proposal 1: RAN2 to confirm the data loss issue caused by fast LTM recovery to an intra-DU cell after inter-DU LTM cell switch failure needs to be solved.

  - Discussion for solving the issue

If Proposal 1 is agreeable, RAN2 should discuss how to solve the issue. Considering that Rel-18 specification is already frozen, the solution should not make any impact to Rel-18 specifications. One possible solution is leaving it to the NW implementation. This means, the NW will be able to configure attemptLTM-Switch only when each candidate cell belongs to separate DU, i.e., ltm-NoResetID and ltm-ServingCellNoReserID are not configured, or all candidate cells belong to the same DU.

Observation 2: One possible solution for Rel-18 is leaving it to the NW implementation, i.e., the NW will be able to configure attemptLTM-Switch only when:

- each candidate cell belongs to separate DU, i.e., ltm-NoResetID and ltm-ServingCellNoReserID are not configured, or

- all candidate cells belong to the same DU.

If RAN2 confirms that the solution in Observation 2 is reasonable, we propose to either add a NOTE to TS 38.331 or TS 38.300. Text proposals are shown in Annex.

Proposal 2: If RAN2 confirms that the solution in Observation 2 is reasonable, RAN2 to decide either add a NOTE to TS 38.331 or TS 38.300.

However, this solution is workaround and will require a restriction of NW configuration. Therefore, it is important to find a different solution for Rel-19. We propose RAN2 should discuss whether a different solution is necessary for Rel-19.

Proposal 3: RAN2 to discuss whether a different solution is necessary or not for Rel-19.

If RAN2 decides a different solution is necessary for Rel-19, we propose to add this issue in Rel-19 RRC open issue list and discuss a solution in Rel-19 mobility session. We also point out the issue in our Rel-19 contribution [2].

Proposal 4: If RAN2 decides a different solution is necessary for Rel-19, RAN2 to add this issue in Rel-19 RRC open issue list and discuss a solution in Rel-19 mobility session.

- Conclusion

We have the following observations and proposals.

2.1	Issue on fast LTM recovery to intra-DU cell after inter-DU LTM failure

Observation 1: In the case of fast LTM recovery to an intra-DU cell after inter-DU LTM cell switch failure, data loss issue will be caused. This is because:

1. At the failed inter-DU LTM cell switch, RLC re-establishment and PDCP data recovery for AM DRB is performed. But PDCP data recovery also fails due to LTM cell switch failure.

2. When the UE reverts back to the source PCell configuration, RLC SDUs, SDU segments and PDUs cannot be retrieved as they are already discarded.

3. If the UE performs fast LTM recovery to an intra-DU cell, PDCP data recovery is not performed even though RLC SDUs, SDU segments and PDUs are already discarded.

Proposal 1: RAN2 to confirm the data loss issue caused by fast LTM recovery to an intra-DU cell after inter-DU LTM cell switch failure needs to be solved.

2.2	Discussion for solving the issue

Observation 2: One possible solution for Rel-18 is leaving it to the NW implementation, i.e., the NW will be able to configure attemptLTM-Switch only when:

- each candidate cell belongs to separate DU, i.e., ltm-NoResetID and ltm-ServingCellNoReserID are not configured, or

- all candidate cells belong to the same DU.

Proposal 2: If RAN2 confirms that the solution in Observation 2 is reasonable, RAN2 to decide either add a NOTE to TS 38.331 or TS 38.300.

Proposal 3: RAN2 to discuss whether a different solution is necessary or not for Rel-19.

Proposal 4: If RAN2 decides a different solution is necessary for Rel-19, RAN2 to add this issue in Rel-19 RRC open issue list and discuss a solution in Rel-19 mobility session.

# References

[1] R2-2402235, “Fast Recovery with LTM Candidates”, MediaTek Inc.

[2] R2-2503747, “Discussion on inter-CU LTM”, Fujitsu

# Annex

## (a) TP for TS 38.331

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

<snip>

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

NOTE 2:	In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute.

NOTE X:	It is left to network implementation to avoid data loss in case of fast LTM recovery after LTM fails.

## (b) TP for TS 38.300

### 9.2.3	Mobility in RRC_CONNECTED

#### 9.2.3.1	Overview

Network controlled mobility applies to UEs in RRC_CONNECTED and is categorized into two types of mobility: cell level mobility and beam level mobility. Beam level mobility includes intra-cell beam level mobility and inter-cell beam level mobility.

<snip>

NOTE:	PDCP SN gap for SRB may exist upon LTM attempt toward the selected cell after LTM fails. It is up to network implementation to avoid the latency caused by the PDCP SN gap. It is up to network implementation to avoid data loss in case of fast LTM recovery after LTM fails.