3GPP TSG-RAN-WG2 Meeting #130	R2-2503744

Malta, MT, 19th– 23rd May, 2025

Agenda item	:		8.6.2

Source	: 	Sharp

Title	: 	Discussion on issues for supporting inter-CU LTM

Document for	:	Discussion and Decision

Introduction

WID [1] for mobility enhancement was approved and RAN2 started discussion on the WI. In RAN2#129bis, RAN2 made agreements as follows [2];

Agreements on inter-CU LTM:

- A list of sk-counters is linked to a Rel-19 ID configured by the SN.

- At RLF and reconfiguration with sync failure:

- If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

- if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

- How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration).

- The indication on whether to allow or not the SN to configure an inter-SN candidate is included in the inter-node RRC message. We include this agreement in the LS to RAN3.

- Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration (no specification change).

- Introducing a new R19 LTM cell switch command MAC CE with indicating the NCC value for R19 inter-CU LTM cell switch. Details will be further discussed during CR phase.

- For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE.

- When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

- No need to define a separate capability for the reference configuration for inter-CU LTM.

- Define per-UE capabilities for security key change of MCG LTM (e.g. ltm-KeyUpdate-MCG-r19) and security key change of SCG LTM (e.g. ltm-KeyUpdate-SCG-r19). UE supports these capabilities should also support ltm-MCG-IntraFreq-r18 or ltm-SCG-IntraFreq-r18 respectively. No new UE capability on inter-CU MCG LTM with SN unchanged and inter-CU MCG LTM SN with SCG addition.

- For security key update in inter-CU LTM, RAN2 agree to include actual NCC value in the LTM cell switch command MAC CE.

- NCC is included in the LTM cell switch command MAC CE if the R19 set ID is different between the target cell and source cell. Conversely, if the R19 set ID is same for both cells, the NCC will not be included.

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

- Option3 (LTM recovery can be applied for all candidates) is not considered.

- Offline discussion to clearly understand how option2 (LTM recovery can be applied if the selected candidate cell belongs to the same Rel-19 with the target cell) works and check if there is a real technical blocking issue. If it works w/o real technical blocking issue, then we can go to option2. If there is a real technical blocking issue for option2, we’ll go to option1 (No LTM recovery is applied for inter-CU LTM).

- From RAN2 point of view, UE-based TA measurement can be also applicable for the synchronized inter-CU LTM scenario, and no additional RAN2 specification impact is expected. Final decision is up to RAN4.

- MIMO 2TA can be configured with inter-CU LTM using PDCCH-order based TA measurement.

- If UE-based TA measurement is supported for inter-CU LTM, it cannot be configured if candidate cell is configured with MIMO 2TA.

- An LTM target cell will not apply cell DTX/DRX until LTM cell switch is completed. RAN2 will not consider any specification impacts needed for LTM RACH-less cell switch to an NES candidate cell.

In this paper, we discuss inter-CU LTM related issues.

Discussion

Inter-CU LTM configuration

RAN2already agreed that intra-CU MCG LTM configuration and inter-CU LTM configuration are simultaneously configured at a UE. It means that ltm-Config (which doesn’t contain inter-CU LTM candidate configuration) for MCG and ltm-ConfigNRDC can be simultaneously configured.

Observation 1. ltm-Config (which doesn’t contain inter-CU LTM candidate configuration) for MCG and ltm-ConfigNRDC can be simultaneously configured.

Here, “ltm-ConfigNRDC” is signalled via MCG RRC as agreed. However, if the UE is configured with ltm-Config by MCG, the UE variables (VarLTM-ServingCellNoResetID, VarLTM-ServingCellUE-MeasuredTA-ID and VarLTM-ServingCellNoSecurityChange) for MCG is already used. If the UE variables for MCG is also used for ltm-ConfigNRDC, the UE overwrites its variables for MCG upon SCG LTM cell switch execution while inter-SN LTM candidate configuration is configured. It causes unexpected behaviour. Therefore, we propose that the UE has independent UE variables for each of ltm-Config and ltm-ConfigNRDC when the UE is configured with intra-CU MCG LTM and inter-CU SCG LTM.

Proposal 1. When the UE is configured with intra-CU MCG LTM and inter-CU SCG LTM, the UE has independent UE variables for each of ltm-Config and ltm-ConfigNRDC.

Here, we’ll consider whether ltm-Config for SCG and ltm-ConfigNRDC are simultaneously configured. RAN2 agreed that intra-SCG LTM candidate configuration can be mixed in MN RRC format, i.e., ltm-ConfigNRDC. It means that 8 intra-SCG LTM candidate configurations can be provided by MN RRC format while 8 intra-SCG LTM candidate configurations are configured by SN RRC format. It seems a violation of the agreement “The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations.” And it is difficult how to differentiate the configuration which is indicated in the LTM CSC/TCI (de)activation MAC CE. Therefore, we suggest prohibiting that ltm-Config for SCG and ltm-ConfigNRDC are simultaneously configured.

Proposal 2. ltm-Config for SCG and ltm-ConfigNRDC cannot be simultaneously configured.

If P2 is agreed, intra-CU SCG LTM candidate configuration should be mixed into ltm-ConfigNRDC if ltm-ConfigNRDC is configured.

Proposal 3. Intra-CU SCG LTM candidate configuration can be mixture into ltm-ConfigNRDC if ltm-ConfigNRDC is configured.

Then, RAN2 needs to discuss how to treat the ltm-Config for SCG when the ltm-ConfigNRDC is configured. According to the current RRC CR and some agreements, inter-CU SCG LTM candidate configurations are provided by MCG and a MAC CE indicating inter-SN LTM cell switch is provided by SCG. If SN provides intra-CU SCG LTM candidate configuration via RRC signalling for the UE, UE holds ltm-Config for SCG and UE variables associated with ltm-Config for SCG. Thus, UE needs to release or replace the configuration and UE variables. If UE needs to release these, two implementation methods can be considered. The first one is that NW (SCG) sends RRC signalling to the UE to release the configurations and variables, and the second one is that UE autonomously releases the configurations and variables. It seems wasteful if UE releases it by explicit signalling from the SCG from the perspective of radio resource efficiency. That is, the UE needs to autonomously release the contents of ltm-Config for SCG.

Observation 2. It is wasteful if UE releases ltm-Config for SCG by explicit signalling from SN before ltm-ConfigNRDC is configured by MN, from the perspective of radio resource efficiency.

Proposal 4. UE autonomously releases the contents of ltm-Config for SCG when the ltm-ConfigNRDC is configured.

Security

In the RRC CR discussion there were some opinions on how to select the sk-Counter upon inter-CU SCG LTM execution. In case of SCPAC, UE selects the first unused sk-Counter in the key stream associated with the securityCellSetId of target configuration as below.

1> if the securityCellSetId is included in the entry in VarConditionalReconfig containing the RRCReconfiguration message: 2> if servingSecurityCellSetId is not included within VarServingSecurityCellSetID; or 2> if the value of the securityCellSetId is not equal to the value of servingSecurityCellSetId within VarServingSecurityCellSetID: 3> consider the first sk-Counter value in the sk-CounterList associated with the securityCellSetId within the VarConditionalReconfig as the selected sk-Counter value, and perform security key update procedure as specified in 5.3.5.7; 3> remove the selected sk-Counter value from the sk-CounterList associated with the securityCellSetId within the VarConditionalReconfig;

This mechanism allows SN to perform early decoding since SN can predict the next SN key. We think this advantage should be applied to inter-CU SCG LTM. Therefore, we provide Text Proposal to specify the mechanism.

<TP1>

1> if the field ltm-NoSecurityChangeID is configured for the LTM-Candidate IE and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or 1> if the value of field ltm-NoSecurityChangeID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID: 2> if the LTM cell switch is triggered on the MCG: 3> UE actions are FFS; 2> else if the LTM cell switch is triggered on the SCG: 3> consider the first sk-Counter value in the sk-CounterList associated with the ltm-NoSecurityChangeID within the VarLTM-ServingCellNoSecurityChange as the selected sk-Counter value, and perform security key update procedure as specified in 5.3.5.7; 3> remove the selected sk-Counter value from the sk-CounterList associated with the ltm-NoSecurityChangeID within the VarLTM-ServingCellNoSecurityChange; Editor’s Note: How to understand which sk-counter the UE shall select is FFS.

Proposal 5. To select the appropriate sk-Counter, UE needs to select the first sk-Counter value in the sk-CounterList associated with the ltm-NoSecurityChangeID within the VarLTM-ServingCellNoSecurityChange as above TP1.

Furthermore, in the email discussion [3], CR Rapporteur added security related behaviour for MCG LTM as below:

1> if the field ltm-NoSecurityChangeID is configured for the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or 1> if the value of field ltm-NoSecurityChangeID contained within the LTM-Candidate IE in ltm-Config or ltm-ConfigNRDC indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID: 2> if the LTM cell switch is triggered on the MCG: 3> perform AS security key update procedure as specified in 5.3.5.7; 3> if the LTM-Candidate IE in ltm-Config indicated by lower layers does not include an mrdc-SecondaryCellGroupConfig set to release: 4> consider the first sk-Counter value in the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChangeID according to the field ltm-NoSecurityChangeID as the selected sk-Counter value, and perform security key update procedure as specified in 5.3.5.7; 4> remove the selected sk-Counter value from the ltm-SK-Counters within the VarLTM-ServingCellNoSecurityChangeID; Editor’s note: FFS whether the list of sk-counters can be used only for the case of inter-MN LTM.

In our understanding, the Rapp’s intention is to support subsequent inter-MN LTM. However, RAN2 already agreed that:

- NW configures the corresponding sk-Counter in all LTM candidate configurations, and UE uses the configured value for generating the SN key when security key update is performed in MCG.

We think this agreement says default sk-Counter is included in each LTM candidate configuration for inter-CU MCG LTM if the LTM configuration includes SCG part. Even if this assumption is followed, subsequent LTMs are supportable. Because UE needs to update MN key upon inter-CU MCG LTM and then, UE derives SN key by using updated MN key. In this case, even if UE uses the same sk-Counter key used once before, since MN key is updated, UE can derive brand new SN key. Thus, there is no security issue if configured sk-Counter is reused. Therefore, the list of sk-Counters is not used for inter-MN LTM.

Proposal 6. The list of sk-Counters is not used for inter-MN LTM.

Fast LTM recovery

At RLF and reconfiguration with sync failure:

If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18).

if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. FFS if fast failure recovery with different Rel-19 IDs is allowed.

In RAN2#129bis, RAN2 made the above agreement. Here, we discuss whether fast failure recovery with different Rel-19 IDs is allowed. One case is that Rel-19 ID of selected candidate cell is the same as Rel-19 ID of source cell of LTM cell switch. We think UE can perform fast recovery in this case. When UE fails LTM cell switch, UE reverts back to the source configuration including security configuration. Hence, from UE perspective, UE can perform fast recovery to the selected candidate cell which has the same Rel-19 ID as the source cell without security key update. To enable this, source MN needs to keep UE context including security context until the UE successfully completes inter-CU LTM cell switch. According to the TS38.401-8.9.4, source gNB releases UE context after target gNB sends an UE CONTEXT RELEASE message to the source gNB. It means that source MN keeps UE context including security context until the UE successfully completes inter-CU LTM cell switch. Thus, UE can perform fast recovery if it is reconfigurationWithSync (inter-CU LTM) failure and if the selected candidate cell has the same Rel-19 ID as source cell.

Proposal 7. UE can perform fast recovery if it is reconfigurationWithSync (inter-CU LTM) failure and if the selected candidate cell has the same Rel-19 ID as source cell.

Another case is that Rel-19 ID of selected candidate cell is different from both of Rel-19 IDs of source cell and target cell. In this case, UE has not valid security configuration for the selected candidate cell. Thus, UE cannot perform fast recovery if it is reconfigurationWithSync (inter-CU LTM) failure and if the selected candidate cell has a different Rel-19 ID from both of source cell and target cell.

Proposal 8. UE cannot perform fast recovery if it is reconfigurationWithSync (inter-CU LTM) failure and if the selected candidate cell has a different Rel-19 ID from both of source cell and target cell.

Completion

In SCPAC, UE sends a selected configuration ID and a selected sk-Counter to MN via RRCReconfigurationComplete.

In intra-CU LTM, UE sends a selected configuration ID to MN if it is MCG LTM and UE sends a selected configuration ID to SN if it is SCG LTM. In case of intra-CU MCG LTM, inter-CU MCG LTM or inter-CU SCG LTM, UE should send the selected configuration ID to MN via complete message since the reconfiguration message is sent from the MN. And MN should indicate the selected PSCell to SN, the same as SCPAC. And, in SCPAC, the selected sk-Counter is informed from UE to MN via complete message, then MN informs the selected sk-Counter to the SN as well as the selected configuration ID. However, the selected configuration ID and selected sk-Counter is not included in SN RRCReconfigurationComplete message. The principle should be applied to inter-CU LTM. Therefore, we suggest RAN2 to approve the following TP.

<TP2>

2> if this RRCReconfiguration message is applied due to an LTM cell switch execution procedure according to clause 5.3.5.18.6: 3> include in the appliedLTM-CandidateId the LTM-CandidateId of the applied LTM candidate configuration; 3> if this RRCReconfiguration message was included in LTM-Candidate within ltm-ConfigNRDC: 4> if a new sk-Counter value has been selected due to the LTM cell switch execution procedure: 5> include selectedSK-Counter and set its value to the selected sk-Counter value; Editor’s Note: FFS if appliedLTM-CandidateId needs to be included in both MCG and SCG RRCReconfigurationComplete message.

Proposal 9. UE sends the selected configuration ID and the selected sk-Counter via MN RRCReconfigurationComplete message upon inter-CU SCG LTM execution as above TP2.

Co-existence issues

#Rel-19 LTM and Rel-19 CLTM

In RAN2#129bis meeting, RAN2 agreed that network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration and no specification change is expected. It means that CLTM candidate configuration can be used for intra-CU LTM configuration. In TS 38.331 running CR [3], the LTM-CandidateId-r18 included in the CLTM candidate configuration (i.e., CLTM-ExecutionConditions-r19) can be associated with the LTM-CandidateId-r18 included in the intra-CU LTM candidate configuration (i.e., LTM-Candidate-r18). Thus, Rel-19 CLTM and Rel-18 LTM can co-exist.

Proposal 10. RAN2 confirms that Rel-19 CLTM can co-exist with Rel-18 LTM.

However, RAN2 hasn’t discussed whether Rel-19 LTM and Rel-19 CLTM can coexist. If the coexistence between Rel-19 LTM and Rel-19 CLTM is not realized, the robustness of intra-CU LTM will be ensured, but mobility scenarios will be significantly restricted. Thus, Rel-19 LTM should coexist with Rel-19 CLTM. However, according to the WID, Rel-19 CLTM is limited to the intra-CU and non-DC case. Therefore, non-DC CLTM configurations can coexist with inter-CU LTM configurations. However, CLTM must not be possible after the UE performs inter-CU LTM due to limitation of Rel-19 Objects. So, some spec impact is expected.

Proposal 11. CLTM configuration can coexist with inter-CU MN LTM configuration in case of non-DC.

#LTM and L3 HO

In RAN2#127, it was agreed that L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE while inter-CU LTM is being configured (w/o DC). It was also agreed that when performing L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations. That is, the UE simultaneously keeps the RRC configuration of L3 mobility (HO and CHO) and inter-CU LTM. Depending on the triggering timing, the UE performs either L3 mobility or inter-CU LTM. However, it is the MAC entity of the UE that receives the LTM CSC MAC CE from the source DU to trigger the inter-CU LTM, while the RRC layer of the UE receives the L3 HO command from the source CU to trigger L3 mobility. Based on Rel-18 intra-CU LTM, when the intra-CU LTM is triggered, the MAC entity of the UE subsequently notifies the RRC layer of the UE. Therefore, we propose that when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE further notifies the RRC layer of the UE, by following the rationale in Rel-18. Afterwards, RAN2 further discuss the cases when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and the execution priority of L3 HO and inter-CU LTM.

Proposal 12. By following the rationale in Rel-18, when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE notifies the RRC layer of the UE.

Proposal 13. RAN2 further discuss:

- The case when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and

- The execution priority of L3 HO and inter-CU LTM.

Conclusion

In this paper, we made the following observations and proposals;

Observation 1. ltm-Config (which doesn’t contain inter-CU LTM candidate configuration) for MCG and ltm-ConfigNRDC can be simultaneously configured.

Proposal 1. When the UE is configured with intra-CU MCG LTM and inter-CU SCG LTM, the UE has independent UE variables for each of ltm-Config and ltm-ConfigNRDC.

Proposal 2. ltm-Config for SCG and ltm-ConfigNRDC cannot be simultaneously configured.

Proposal 3. Intra-CU SCG LTM candidate configuration can be mixture into ltm-ConfigNRDC if ltm-ConfigNRDC is configured.

Observation 2. It is wasteful if UE releases ltm-Config for SCG by explicit signalling from SN before ltm-ConfigNRDC is configured by MN, from the perspective of radio resource efficiency.

Proposal 4. UE autonomously releases the contents of ltm-Config for SCG when the ltm-ConfigNRDC is configured.

Proposal 5. To select the appropriate sk-Counter, UE needs to select the first sk-Counter value in the sk-CounterList associated with the ltm-NoSecurityChangeID within the VarLTM-ServingCellNoSecurityChange as above TP1.

Proposal 6. The list of sk-Counters is not used for inter-MN LTM.

Proposal 7. UE can perform fast recovery if it is reconfigurationWithSync (inter-CU LTM) failure and if the selected candidate cell has the same Rel-19 ID as source cell.

Proposal 8. UE cannot perform fast recovery if it is reconfigurationWithSync (inter-CU LTM) failure and if the selected candidate cell has a different Rel-19 ID from both of source cell and target cell.

Proposal 9. UE sends the selected configuration ID and the selected sk-Counter via MN RRCReconfigurationComplete message upon inter-CU SCG LTM execution as above TP2.

Proposal 10. RAN2 confirms that Rel-19 CLTM can co-exist with Rel-18 LTM.

Proposal 11. CLTM configuration can coexist with inter-CU MN LTM configuration in case of non-DC.

Proposal 12. By following the rationale in Rel-18, when the inter-CU LTM is triggered at the MAC entity of the UE, the MAC entity of the UE notifies the RRC layer of the UE.

Proposal 13. RAN2 further discuss:

- The case when the RRC layer of the UE receives the L3 HO command and the MAC entity of the UE receives the LTM CSC MAC CE to execute the inter-CU LTM, and

- The execution priority of L3 HO and inter-CU LTM.

Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4

- R2-250xxxx, Report from session on V2X/SL, R19 NES and MOB

- R2-250xxxx, [POST129bis][115][MOB] RRC inter-CU and CLTM (Ericsson)