3GPP TSG-RAN-WG2 Meeting #130	R2-2503746

St.Julians, Malta, May 19th – 23rd, 2025

Agenda item	:		8.6.4

Source	: 	Sharp

Title	: 	Discussion on issues for supporting conditional LTM

Document for	:	Discussion and Decision

Introduction

Revised WID [1] for mobility enhancement was approved and RAN2 started discussion on the WI. For conditional LTM, RAN2 starts discussion from the RAN2#127bis meeting;

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this paper, we discuss some issued related to conditional LTM.

Discussion

Evaluation

Cell selection when multiple cells fulfil the condition

In the RAN2#128, it was agreed that L1-based evaluation is performed at MAC level and L3-based evaluation is performed at RRC level. In the last meeting, it was agreed that the network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell. It means that a UE can be simultaneously configured with L3(RRC)-level execution condition and L1(MAC)-level execution condition but not be configured with both for the same candidate cell. (e.g. L1 cond is configured for candidate cell A, and L3 cond is configured for candidate cell B, …)

Observation 1. The RRC evaluates L3 execution conditions, and the MAC evaluates L1 execution conditions simultaneously. (e.g. L1 cond is configured for candidate cell A, and L3 cond is configured for candidate cell B, …)

In the current MAC CR, the MAC entity evaluates L1 conditions and determines that the event for CLTM is satisfied, and indicates to the upper layer that the event for CLTM is satisfied. Therefore, RRC layer knows all fulfilled candidate cells and so, RRC layer can determine the candidate cell on which to execute the LTM cell switch.

Observation 2. The RRC layer knows all fulfilled candidate cells.

Observation 3. The RRC layer needs to determine the one candidate cell that will be executed.

In the current RRC CR, there is an FFS on how to handle the case where more than one LTM candidate cell fulfil the LTM cell switch execution condition. In Rel-18 CHO procedure, if more than one triggered cell exists, the RRC layer selects one of the triggered cells as the selected cell for conditional reconfiguration execution. This principle is simple enough, and it would be reasonable to reuse it in CLTM for consistency over similar features unless there is a critical problem.

Proposal 1. If there are more than one LTM candidate cell fulfil the LTM cell switch execution, the RRC layer selects one of the triggered cells as the selected cell for conditional LTM execution. And this selection is performed by the UE implementation.

Beam selection for L3 based RACH-less CLTM determination

There is the FFS how to determine the selected beam in case almost full information on beam status. Hence, UE implementation can handle properly rather that defining other rules. In other words, UE implementation is the simple enough and can be supported.
Proposal 2. For L3 based RACH-less LTM, it is up to UE implementation to select the target beam.

Conclusion

Proposal 1. If there are more than one LTM candidate cell fulfil the LTM cell switch execution, the RRC layer selects one of the triggered cells as the selected cell for conditional LTM execution. And this selection is performed by the UE implementation.

Proposal 2. For L3 based RACH-less LTM, it is up to UE implementation to select the target beam.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4