3GPP TSG RAN WG2#130                                                                                  R2- 2503750

St.Julians, Malta, May. 19th– 23rd, 2025

Agenda Item:	8.2.4

Source:	OPPO

Title:	Discussion on AIoT data transmission related functionalities

Document for:	Discussion, Agreement

Introduction

- AS ID is applied for inventory + command case;

- AS ID is not included in D2R message except Msg1 (RN16 in Msg1 has been agreed).

- For both CFRA and CBRA, the AS ID size is same as RN16, i.e., 16 bits.

- Do not specify the reader behaviour on how exactly the AS ID is generated.

- The device releases the AS ID upon power off (no stage 3 specification impact);

- The device only keeps one AS ID at a time.

- For CFRA, command message is used for AS ID assignment.

- For CBRA, Msg2 is used for AS ID assignment.

- The device releases the AS ID at least:

  - Upon receiving Paging with new transaction ID for that device, i.e., different session/service

  - When it triggers new Msg1 transmission as a result of receiving Paging message (i.e., it has to generate a random ID for CBRA

  - FFS other cases for release AS ID to avoid keeping it indefinitely.

- For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. FFS whether offset zero is always included.

- FFS whether the reader always includes the command for retransmission of segments

- 1-bit indication is sufficient to indicate whether more D2R data will be sent

- For inventory response, RAN assumes that segmentation is not applied. RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size. Notify SA2 about this assumption.

- The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message.

- RAN2 assumes that the upper layer data SUD is byte-aligned, and an LS can be sent to CT1.

- The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message

- The MAC padding is supported at least for D2R from RAN2 perspective. The device includes padding bits if there is no more data and there is still space available in the TBS.

- In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required. FFS how this is provided (i.e., SDU length field or padding length field). The size of length field is FFS.

In the last RAN2#129bis meeting, following agreements regarding A-IOT data transmission have been made:

In this contribution, we aim to address our view on the aspects e.g., transmission resource determination for A-IOT Msg3 and subsequent messages, segmentation for D2R, MAC PDU format, etc.

Discussion

Consideration on segment retransmission

In the last RAN2#129bis meeting, an agreement that allowing the re-transmission of the first/unsegmented D2R message by re-sending the upper layer command again is achieved, and FFS whether offset zero is always included. Recalled that previously, we also agree to trigger the re-transmission of the D2R message by re-sending the R2D message again. We do not see difference between such two agreements. So, offset zero is not needed to be included in the R2D message triggering re-transmission.

Proposal 1: offset zero shall not be included in the R2D message triggering re-transmission of the first/unsegmented D2R message.

The next question is related to whether or not the command shall be always transmitted with the offset indication together to trigger the D2R message following segments re-transmission. From our point of view, it is reasonable to assume that the memory of an A-IOT device is large enough to accommodate the generated D2R message. Regardless of re-transmission or transmission of the next segment, the Reader will always indicate the offset from beginning of the data to the A-IOT device to trigger it for subsequent D2R message transmission. For the transmission of the next segment, the A-IOT device follows the instruction of the offset indication to transmit data from the memory. Similar implementation applies to re-transmission too. So, we think that upper layer command is not needed to be carried in the R2D message to trigger the D2R message re-transmission.

Proposal 2: the upper layer command is not needed to be carried in the R2D message to trigger the D2R message following segment re-transmission/transmission.

NACK based mechanism for subsequent D2R message

In the last RAN2 #129bis meeting, RAN2 agree that NACK based mechanism is applied only to the Msg3, and may come back for D2R, if the NACK feedback indication is needed for the purpose to stop/terminate the “on-going procedure”.

Recalled that sending NACK message is for the A-IOT device who failed the D2R transmission to re-access to the Reader later when a subsequent A-IOT Paging message with the same transaction ID as current is received. However, if a device does not receive NACK message, it must have a way to know that its previous D2R transmission is successful, otherwise: 1. when the subsequent paging message comes, the device does not know if it needs to perform re-access; 2. when an A-IOT Paging message with the new transaction ID comes, the device does not know if it shall terminate the ongoing procedure to follow the new A-IOT Paging message

Observation 1: the AIOT device needs to determine whether or not it shall perform re-access to the Reader when a subsequent A-IOT Paging message with the same transaction ID is received, or determine if it shall terminate the ongoing procedure to follow the new A-IOT paging message with another transaction ID.

As mentioned in our accompany paper, R2D trigger message is suggested by us to deal with the problem, i.e., let A-IOT device stop waiting for the NACK. Similarly, in our opinion, for the subsequent D2R message after Msg3, still A-IOT devices needs to determine its action, i.e., whether to perform re-access when receiving a subsequent A-IOT Paging message. As a result, for subsequent D2R message, NACK feedback indication is suggested to be applied also. Moreover, the R2D trigger message stops the A-IOT device from waiting for the NACK feedback indication for D2R messages beside Msg3 too.

Proposal 3: NACK feedback indication-based NACK mechanism applies to D2R message also.

Proposal 4: when receiving R2D trigger message, the A-IOT device who previously transmit the D2R message stops monitoring for the NACK message.

Next, we would like to discuss how to solve the problem of missing the NACK feedback at the A-IOT device. If the Reader has not decoded the Msg3 from the A-IOT device, the Reader cannot know the A-IOT device ID but the AS ID, which is either the RN16 generated by the A-IOT device or the AS ID allocated by the Reader in case the collision of the RN16 across different access occasions occurs. So, a reasonable approach is to embed the AS ID of the targeted device in a subsequent Paging message with the same transaction ID, in case the Reader finds that there is device whose D2R transmission was failed does not perform the re-access to the Reader in the prior subsequent Paging message with the same transaction ID. In such case, the A-IOT device needs to transmit the same AS ID when it is paged by the Subsequent Paging message to perform access to the Reader.

On the other hand, for the failure of the subsequent D2R message transmission, since the A-IOT device ID is known at the Reader, the Reader can choose to include the A-IOT device ID in the subsequent Paging message with the same transaction ID to trigger the A-IOT device to perform re-access. An illustration of the example is provided in the following figure.

- A-IOT device with AS ID#5 loses NACK message for the Msg3 transmission

- A-IOT device #5 loses NACK message for the subsequent D2R message transmission

Figure 1: addressing losing of NACK message at A-IOT device by issuing subsequent Paging message with device related ID included.

In Figure 1(a), the device missed the NACK for it. As proposal 4 suggest, the device will stop monitoring the NACK message for it, after receiving a following R2D trigger message. When the first subsequent Paging message with the same transaction ID comes, the device will not perform re-access to the reader, since at this point, it considers the Reader did not issue a NACK message to it. As the reader finds that such device has not made re-access in this paging round, a subsequent Paging message with the device’s AS ID included is transmitted towards the device. By receiving such Paging message, the device can recognize the NACK was transmitted by the Reader but missed by the device. Accordingly, the device will perform re-access to the Reader. Similar procedure applies for the Figure 1(b), the only difference is that the Reader already knows the A-IOT device ID, when the device performs subsequent D2R message transmission, so the subsequent Paging message can include the A-IOT device ID instead to trigger re-access

Proposal 5: the A-IOT device shall perform re-access to the Reader, when it receives a subsequent A-IOT Paging message with the same transaction ID as the previous one including the AS ID or device ID of it.

Proposal 6: the device shall maintain its AS ID until a subsequent A-IOT Paging message with new transaction ID is received.

How to end the current access procedure

Currently, it is still questionable how a A-IOT device behaves when it receives a Paging message with a new transaction ID while one procedure is still ongoing. Generally, there are two options on the table：

- Option A: the device will follow the current procedure and ignore the Paging message.

- Option B: the device will follow the new Paging message and terminate the current procedure.

For Option B, the main problem is that it requires stringent coordination between Readers covering overlapping areas, in order to avoid terminating other Reader’s on-going procedure by triggering a new Paging message. It can be foreseen that this problem will become more serious in Rel-20 when there are also UE Readers deployed in the network, co-ordination between various type of Readers may become an overwhelmingly heavy task due to the nature of mobility of UE Reader.

Observation 2: for option B, stringent coordination between Readers covering overlapping areas is necessary, in order to avoid terminating other Reader’s on-going procedure by triggering a new Paging message.

On the other hand, for Option A, we need to have a method to define the ending point of the current access procedure, either using a R2D message or defining a new timer, for the device to know whether or not it shall follow the Paging message with the new transaction ID.

For the timer-based solution, the timer is started when the device sends the D2R message, and will keep running until it receives a D2R message addressed to the device. Otherwise, when the timer finally expires, the device will consider the current procedure is ended.  In our opinion, due to the bad clock synchronization performance of A-IOT devices, such mechanism is not feasible, considering the high likelihood of the misalignment between the Reader and A-IOT device on whether or not the access procedure is ended at a specific timing moment, and the incurred problem that the A-IOT device will no longer process the R2D message addressed to it when its timer expires but the timer of Reader is still running.

Observation 3: for Option A, the timer-based solution suffers from the bad clock synchronization performance.

On the other hand, the explicit message-based solution can work without any misalignment, i.e, both of the Reader and A-IOT device will consider the procedure is ended when the R2D message is received by the A-IOT device. The Question is which R2D message should play the role of marking the ending point for A-IOT devices. Currently, there are several options on the table:

- Option A-1: the R2D trigger message

- Option A-2: the Paging message

- Option A-3: introduce a new type of R2D message

Actually, Option A-2 is the same as option B, so it should be filtered out. Option A-3 will let the Reader send more R2D message, which leads to more radio resource usage, so it shall be deprioritized. On the other hand, the functionality of the R2D trigger message is to trigger the new set of A-IOT devices to perform access procedure towards the Reader. Naturally, it could be employed to mark the end of the current access procedure.  It should be further noted that for the last set of A-IOT devices, there is no R2D trigger message to trigger another set of A-IOT devices to perform access procedure towards the Reader. We need to find out a method to mark the end of the procedure towards them. In our opinion, a feasible way is to let the Reader send R2D trigger message additionally towards these A-IOT devices. In order to avoid A-IOT devices who missed some of R2D trigger messages wrongly consider this final R2D trigger message is for provide new set of access resources, such additionally sent R2D trigger message should be allowed to carry one-bit flag to indicate the end of the current Paging round.

Proposal 7: the R2D trigger message shall be employed to indicate the end of the access procedure for A-IOT device in the procedure.

In addition, as mentioned in the section 2.2, a subsequent Paging message with the same transaction ID but with the AS ID inside could be used to trigger the A-IOT device missed the NACK message to perform the re-access to the network, so the validity of the AS ID shall not be terminated by the R2D trigger message.

Proposal 8: the validity of the AS ID shall not be terminated by the R2D trigger message.

Design of MAC PDU format

- 2.4.1 R2D trigger message

In the last RAN1 meeting, RAN1 agrees that one R2D transmission triggering message can trigger X=1 and X=2 time domain resources for Msg1 transmission, so it seems that the number of resources triggered by one R2D trigger message is flexible. So, it is possible to embed the number of D2R resources, i.e., the TDMed and FDMed number, or at least the number of TDM in the R2D trigger message.

Proposal 9: the number of TDM or the number of triggered Msg1 resources, i.e., TDM multiplied by FDM number, shall be provided by the R2D trigger message.

- 2.4.2 the overall R2D message MAC PDU format design principle

The first thing we would like to discuss is whether or not R2D message for different A-IOT devices should be multiplexed in the same R2D message. In the CBRA scenario, if R2D messages are needed to be sent to each A-IOT device individually, then messages should be sent one-by-one in time domain. Correspondingly, each A-IOT device shall parse these messages one-by-one to see if one of the messages is destined to itself, which causes heavy burden to these A-IOT devices.

Observation 4:  in the CBRA scenario, if R2D message is needed to be sent to each A-IOT device individually, then messages should be sent one-by-one in time domain. Correspondingly, each A-IOT device shall parse these messages one-by-one to see if one of the messages is destined to itself, which causes heavy burden to these A-IOT devices.

Proposal 10: allow packaging contents for different A-IOT devices in the same R2D message, besides A-IOT Msg2.

In the 5G NR MAC PDU format, Logical channel ID (LCID) in the sub-header is used to identify the message type of the corresponding SDU, MAC CE, etc. For the A-IOT system, since in the RAN2 #129 meeting, we agree to have several message types, and different A-IOT devices shall experience exactly the same R2D message, e.g., they shall receive A-IOT paging message before making access, they shall receive A-IOT Msg4 after performing Msg3 transmission, etc. A common header field consisting of the message type shall be placed in the beginning of the R2D MAC PDU for A-IOT devices to know the purpose of received R2D messages. For AIOT devices not involved in the current access procedure, parsing such common header field only can help them skip this message content part.

Observation 5: setting a common header field could let A-IOT devices not involved in the current access procedure skip parsing the message content part.

Proposal 11: place a common header field, consisting of the message type to indicate the purpose of the message, in the beginning of the 5G A-IOT R2D MAC PDU.

Recalled in 5G NR system, instead of putting all sub-header fields in the beginning of the MAC PDU, an alternative way of interleaving the sub-header fields in-between different SDUs, MAC CEs was adopted. The reasons are summarized as follows:

- Flexible assembly of the PDU: prioritizing to assembly the one with higher priorities or timing constraints. For example, a MAC CE that needs urgent attention can be placed early in the PDU, followed by its sub-header and payload, without waiting for other elements to be processed.

- Reduced Delay: delay could be introduced to have all sub-headers at the beginning, which requires the transmitter to collect all necessary information about each element (e.g., length) before constructing the header block. By interleaving headers and payloads, the transmitter can build the PDU incrementally, which is more efficient for real-time processing. In turn, the receiver can process each element sequentially without waiting for the entire header block.

- Simplified segmentation and padding:

  - Segmentation: Enables on-the-fly segmentation without restructuring the entire PDU, as each sub-header directly precedes its payload.

  - Padding: Padding can be appended at the end with a dedicated subheader, avoiding complex adjustments to a centralized header block.

For A-IOT R2D message, sub-header field is needed for each A-IOT device to at least embrace the length(s) of the variable part(s). the Pros brought by interleaving the sub-header fields in-between different SDUs, such as reduced processing delay, simplified padding, is still needed for the 5G A-IOT system, in our opinion. Also, unify between A-IOT system and 5G NR system regarding MAC PDU processing will ease the Reader, likely to be a gNB, implementation complexity.

As a result, we propose that similar with 5G NR system, MAC subPDU destined to targeted A-IOT devices consisting of the sub-header and payload are put in sequential order

Proposal 12: MAC subPDUs destined to targeted A-IOT devices consisting of the sub-header and payload are put in sequential order in the 5G A-IOT R2D MAC PDU.

Putting Proposals 11 and 12 together, the MAC PDU format for multiplexing the MAC SDUs for different A-IOT devices in the same R2D message is indicated as follows:

Figure 2:  the suggested MAC PDU format for the R2D message multiplexing the MAC SDUs for different A-IOT devices in the process.

Regarding the length field, it has been agreed to be introduced for SDU and variable parts in the RAN2#129 meeting. In our opinion, with the aim of designing simple MAC PDU format, the number of variable part(s) should be reduced as much as possible. In such way, we can alleviate the need of length fields. As a result, we propose RAN2 to employ fixed length of bitstream to represent parameters to be included in the A-IOT R2D message, e.g., No. of total access occasions, No. of FDM for D2R transmission, etc.

Observation 6: the number of variable part(s) included in the R2D message MAC PDU should be reduced as much as possible. In such way, the need of length fields can be alleviated, which also reduces the decoding complexity for A-IOT devices.

Proposal 13: employing fixed number of bitstream to represent variable parts, e.g., number of total access occasions, number of FDM for D2R transmission, AS ID, etc, for all kinds of R2D message, as much as possible.

2.4.2.1 sub-header format

The sub-header of MAC subPDU format of 5G NR is provided as follows:

- Sub-header for Fixed-Size MAC Control Elements (CEs), Padding, and a MAC SDU containing UL CCCH :

  - Format: 1 octet (8 bits)

  - Fields:

    - R (Reserved): 2 bits (set to 0)

    - LCID (Logical Channel ID): 6 bits (identifies the CE type or padding)

- Variable-Size Subheader: used for variable-length elements such as MAC SDUs and some MAC CEs with variable payloads (e.g., Long BSR):

- Format: 3 octets (24 bits)

- Fields:

    - R (Reserved): 2 bits (set to 0)

    - F: indicates the size of the Length field; the value 0/1 indicates 8/16 bits of the Length field。

    - LCID (Logical Channel ID): 6 bits. If the LCID field is set to 33, two additional octets are present in the MAC subheader containing the eLCID field and these two additional octets follow the octet containing LCID field; if the LCID field is set to 34, one additional octet is present in the MAC subheader containing the eLCID field and follow the octet containing LCID field.

    - eLCID: the extended Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU as described in tables 6.2.1-1a, 6.2.1-1b, 6.2.1-2a and 6.2.1-2b for the DL-SCH and UL-SCH respectively. The size of the eLCID field is either 8 bits or 16 bits.

    - Length (L): 8/16 bits (specifies the payload size in bytes).

For the Ambient IOT R2D message, in the RAN2#129 meeting, we have achieved the agreement that at least the following field are required for at least for R2D in the MAC header – message type, length for SDU and variable part(s). For message type, we have already proposed to indicate in the common header for the R2D message. So, we would like to discuss the other potential fields one by one.

- Length: as in the last RAN2#129bis meeting, byte-alignment requirement of MAC PDU is agreed, so the usage of length field should be similar with the current 5G NR, i.e., specifying the payload size in bytes. Regarding the bit-stream length of the length field, 8 bit can indicate SDU length varies in the range of from 0 to 2^8*8 = 2048 bit. For a typical MAC SDU addressed to single A-IOT device, such implementation, from our perspective of view, is enough.

- F: since the length field is fixed, F shall be not included in the R2D messages.

- LCID: the detailed command type is included in the upper layer data packet, so this field is not needed.

- ACK/NACK to the previous transmitted D2R message for performing re-access: this is needed to identify if the R2D message sent to one particular A-IOT device is to confirm the success/unsuccess of the previous D2R message to perform re-access. Only 1 bit is enough, setting to 1 is to indicate NACK and setting to 0 means NACK feedback is disabled.

- AS ID: if AS IDs of targeted A-IOT devices are agreed to be included in the multiplexed R2D message addressed to them, we suggest AS ID to be included in the sub-header field, since each R2D message shall include the information.

- R: could be included for future-proof

- Segment retransmission Offset indicator: this reflects the RAN2 #129 meeting agreement that network instructs to the A-IOT device regarding from which position in the D2R message the device shall perform transmission in the case that segmentation is applied. For example, such indicator is set to 1 if segment retransmission offset is sent to the A-IOT device, otherwise set to 0.

Proposal 14: following fields are considered to be included in the sub-header fields for the R2D message in A-IOT system:

- the Length field for SDU:  set to 1 byte

- the ACK/NACK to the previous transmitted D2R message for triggering re-access (optional): wherein the 1-bit flag being setting to 1 is to indicate NACK and setting to 0 means ACK/NACK feedback is disabled.

- AS IDs for the targeted A-IOT devices are included in the R2D message except the A-IOT paging message.

- the Reserved field could be included in the sub-header field, if any blank is left, for future-proof purpose.

- Segment retransmission enabling indicator

    - the overall D2R message MAC PDU design principle

In our opinion, since only 1 command is included in one R2D message sent to a A-IOT device, only one response is sent to the Reader in the subsequent D2R message. As a result, concatenation of MAC subPDUs for different command is not needed.

Proposal 15: confirm that concatenation of MAC subPDUs including the response for different commands is not needed for the D2R message.

Next, regarding the fields to be included in the sub-header, we discuss them one-by-one, as follows:

- Length: regarding how to indicate how many SDU bits are present in the D2R message, since only 1 SDU, i.e., D2R response, is transmitted in the D2R message towards the Reader, so either the length of the SDU or the padding field solely can be indicated to the Reader. As, in the majority cases, the padding field is shorter than the SDU, the length of length field could be saved compared with the SDU, so we prefer to include in the length field of the padding bits in the sub-header. If the length field is set 0, it means that padding field is absent in the D2R message.

- Format (F) field: regarding Format (F) field, since the suggested length field is static, ‘F’ shall not be included in D2R message sub header field.

- LCID/ eLCID: the message type indication is not needed, since the Reader knows exactly which message is scheduled by itself

- AS ID: if the AS ID of sourcing A-IOT device is agreed to be included in the D2R message for the Reader to identify the A-IOT device, especially for CBRA, we suggest AS ID to be included in the sub header field of D2R message except A-IOT Msg1.

Proposal 16: following fields are considered to be included in the sub-header fields for the D2R message in A-IOT system:

- length field: this field indicates the length of the padding field in bytes. If the length field is set 0, it means that padding field is absent in the D2R message.

- AS ID

- Conclusion

We have the following proposals:

Consideration on segmentation retransmission:

Proposal 1: offset zero shall not be included in the R2D message triggering re-transmission of the first/unsegmented D2R message.

Proposal 2: the upper layer command is not needed to be carried in the R2D message to trigger the D2R message following segment re-transmission/transmission.

NACK based mechanism for subsequent D2R message:

Observation 1: the AIOT device needs to determine whether or not it shall perform re-access to the Reader when a subsequent A-IOT Paging message with the same transaction ID is received, or determine if it shall terminate the ongoing procedure to follow the new A-IOT paging message with another transaction ID.

Proposal 3: NACK feedback indication-based NACK mechanism applies to D2R message also.

Proposal 4: when receiving R2D trigger message, the A-IOT device who previously transmit the D2R message stops monitoring for the NACK message.

Proposal 5: the A-IOT device shall perform re-access to the Reader, when it receives a subsequent A-IOT Paging message with the same transaction ID as the previous one including the AS ID or device ID of it.

Proposal 6: the device shall maintain its AS ID until a subsequent A-IOT Paging message with new transaction ID is received.

How to end the current access procedure:

Observation 2: for option B, stringent coordination between Readers covering overlapping areas is necessary, in order to avoid terminating other Reader’s on-going procedure by triggering a new Paging message.

Observation 3: for Option A, the timer-based solution suffers from the bad clock synchronization performance.

Proposal 7: the R2D trigger message shall be employed to indicate the end of the access procedure for A-IOT device in the procedure.

Proposal 8: the validity of the AS ID shall not be terminated by the R2D trigger message.

Design of MAC PDU format:

Proposal 9: the number of TDM or the number of triggered Msg1 resources, i.e., TDM multiplied by FDM number, shall be provided by the R2D trigger message.

Observation 4:  in the CBRA scenario, if R2D message is needed to be sent to each A-IOT device individually, then messages should be sent one-by-one in time domain. Correspondingly, each A-IOT device shall parse these messages one-by-one to see if one of the messages is destined to itself, which causes heavy burden to these A-IOT devices.

Proposal 10: allow packaging contents for different A-IOT devices in the same R2D message, besides A-IOT Msg2.

Observation 5: setting a common header field could let A-IOT devices not involved in the current access procedure skip parsing the message content part.

Proposal 11: place a common header field, consisting of the message type to indicate the purpose of the message, in the beginning of the 5G A-IOT R2D MAC PDU.

Proposal 12: MAC subPDUs destined to targeted A-IOT devices consisting of the sub-header and payload are put in sequential order in the 5G A-IOT R2D MAC PDU.

Observation 6: the number of variable part(s) included in the R2D message MAC PDU should be reduced as much as possible. In such way, the need of length fields can be alleviated, which also reduces the decoding complexity for A-IOT devices.

Proposal 13: employing fixed number of bitstream to represent variable parts, e.g., number of total access occasions, number of FDM for D2R transmission, AS ID, etc, for all kinds of R2D message, as much as possible.

Proposal 14: following fields are considered to be included in the sub-header fields for the R2D message in A-IOT system:

- the Length field for SDU:  set to 1 byte

- the ACK/NACK to the previous transmitted D2R message for triggering re-access (optional): wherein the 1-bit flag being setting to 1 is to indicate NACK and setting to 0 means ACK/NACK feedback is disabled.

- AS IDs for the targeted A-IOT devices are included in the R2D message except the A-IOT paging message.

- the Reserved field could be included in the sub-header field, if any blank is left, for future-proof purpose.

- Segment retransmission enabling indicator

Proposal 15: confirm that concatenation of MAC subPDUs including the response for different commands is not needed for the D2R message.

Proposal 16: following fields are considered to be included in the sub-header fields for the D2R message in A-IOT system:

- length field: this field indicates the length of the padding field in bytes. If the length field is set 0, it means that padding field is absent in the D2R message.

- AS ID