3GPP TSG-RAN WG2 Meeting #130																					R2-2503755

St Julian, Malta, 19th– 23rd May 2025

Agenda item:		8.5.2

Title: 	Remaining issues of on demand SSB SCell operation

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some understanding on the remaining open issues of on demand SSB SCell per the latest progress in RAN1.

- Discussion

  - On-demand SSB configuration

The following parameters have been included in the latest higher layers parameter list from RAN1 [1]:

| od-ssb-Periodicity | Indicate the periodicity of OD-SSB. Note: It is up to RAN2 whether to introduce new parameter or reusing existing parameter ssb-Periodicity. | Same as ssb-Periodicity |  | per od-ssb-Config |
| --- | --- | --- | --- | --- |
| od-ssb-halfFrameIndex | Indicate whether OD-SSB is in the first half or the second half of the frame. Note: It is up to RAN2 whether to introduce new parameter or reusing existing parameter halfFrameIndex. | ENUMERATED {zero, one} FFS: Whether to define this parameter as a list or not | UE assumes 0 if not configured | per od-ssb-Config |
| od-ssb-nrofBurst | Indicate the number of OD-SSB bursts to be transmitted after on-demand SSB is indicated. | up to N2 integer values N2 = [8] |  | per od-ssb-Config |
| od-ssb-absoluteFrequency | Indicate frequency of OD-SSB. Note: It is up to RAN2 whether to introduce new parameter or reusing existing parameter absoluteFrequencySSB. | Same as absoluteFrequencySSB | For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), if absent, od-ssb-absoluteFrequency is the same as absoluteFrequencySSB configured for always-on SSB. | per od-ssb-Config |
| od-ssb-PositionsInBurst | Indicate the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. Note: It is up to RAN2 whether to introduce new parameter or reusing existing parameter ssb-PositionsInBurst. | Same as ssb-PositionsInBurst provided in ServingCellConfigCommon, i.e., choice of 4-bit shortBitmap, 8-bit mediumBitmap, or 64-bit longBitmap | For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), if absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon. | per od-ssb-Config |
| od-ssbSubcarrierSpacing | Indicate subcarrier spacing of OD-SSB, for Case #1, i.e., no always-on SSB on this serving cell | Same as ssbSubcarrierSpacing that is applicable for SSB | For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), this parameter is absent and sub-carrier spagcing of on-demand SSB is the same as that of always-on SSB. | per od-ssb-Config |
| od-ssb-physCellId | Indicate physical cell identity of a cell in the cell list, for Case #1, i.e., no always-on SSB on this serving cell | Sams as physCellId | For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), this parameter is absent and physical cell ID of on-demand SSB is the same as that of always-on SSB. | per od-ssb-Config |
| od-ss-PBCH-BlockPower | Indicate average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for OD-SSB transmission, see TS 38.213 [13], clause 7, for Case #1, i.e., no always-on SSB on this serving cell | Same as ss-PBCH-BlockPower | For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), this parameter is absent and downlink transmit power of on-demand SSB is the same as that of always-on SSB. | per od-ssb-Config |

Proposal 1: Based on the latest input from RAN1, the following parameters should be included in RRC signaling for each on demand SSB configuration:

- od-ssb-Periodicity

- od-ssb-halfFrameIndex

- od-ssb-nrofBurst

- od-ssb-absoluteFrequency

- od-ssb-PositionsInBurst

- od-ssbSubcarrierSpacing

- od-ssb-physCellId

- od-ss-PBCH-BlockPower

Then another issue is where to put these parameters for on demand SSB SCell. We understand the existing SCell addition/modification procedure can be reused, i.e. the on demand SSB parameters for a SCell can be configured when NW adds or modifies a SCell.

Proposal 2: The SCell addition/modification procedure can be reused to provide the on demand SSB configuration, i.e. the on demand SSB parameters for a SCell can be configured when NW adds or modifies a SCell.

  - Activation and Deactivation of on demand SSB

The following agreements have been achieved at RAN2#129bis:

Agreements on OD-SSB

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

- For explicit activation/deactivation, the OD-SSB MAC-CE supports two formats: one format indicates up to 7 SCells and the other format indicates up to 31 SCells.

- OD-SSB MAC-CE includes a configuration index for each SCell activating OD-SSB.

- RAN2 aims to define single MAC CE format for both explicit and implicit OD-SSB deactivations. Details are FFS.

- Working assumption: When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. FFS if we allow multiple OD-SSBs with the different frequencies for a given SCell.

Issue 1: On demand SSB activation/deactivation MAC CE

Since we have agreed that the OD-SSB MAC CE include a configuration index for each SCell activating OD-SSB, the maximum value of OD-SSB configuration for each SCell would impact the design of the MAC CE as it will impact the size of the configuration index.

Also considering that RAN1 would not discuss the detailed configuration and leave it to RAN2, we can make a decision on the maximum number of OD-SSB configuration for each SCell, e.g. 2 or 4.

Proposal 3: RAN2 make decision on the maximum number of OD-SSB configuration for each SCell, e.g. 2 or 4, which will decide the size of the configuration index in the on demand SSB MAC CE.

Issue 2: Explicit activation and deactivation

Since we have agreed that:

- For explicit activation/deactivation, the OD-SSB MAC-CE includes fixed sized bitmap to indicate whether OD-SSB is activated in each SCell (i.e., similar to legacy SCell A/D MAC-CE).

- A/D bit: “1” means activation, “0” means deactivation for explicit deactivation case.

We would like to clarify the interpretation of activation and deactivation for explicit case.

For activation:

- For case#1, the OD-SSB configuration is activated for a SCell when the corresponding bit is set to 1.

- For case#2, two options can be considered and requires down selection:

  - Option 1: Switch from always-on SSB to OD-SSB is activated when the corresponding bit is set to 1.

  - Option 2: OD-SSB is activated in addition to always-on SSB when the corresponding bit is set to 1.

Proposal 4: Clarify the following understanding on explicit activation:

- For case#1, the OD-SSB configuration is activated for a SCell when the corresponding bit is set to 1.

- For case#2, two options can be considered and need to down select:

  - Option 1: Switch from always-on SSB to OD-SSB is activated when the corresponding bit is set to 1.

  - Option 2: OD-SSB is activated in addition to always-on SSB when the corresponding bit is set to 1.

For deactivation:

- For case#1, the OD-SSB configuration is deactivated for a SCell when the corresponding bit is set to 0.

- For case#2, UE switch back to always-on SSB when the corresponding bit is set to 0.

Proposal 5: Clarify the following understanding on explicit deactivation:

- For case#1, the OD-SSB configuration is deactivated for a SCell when the corresponding bit is set to 0.

- For case#2, UE switch back to always-on SSB when the corresponding bit is set to 0.

Issue 3: Unified design for explicit activation/deactivation and implicit deactivation

RAN1 has agreed to introduce the od-ssb-nrofBurst, i.e Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated, then we need to discuss the expected UE behavior with unified design for explicit activation/deactivation and implicit deactivation.

We understand the following scenarios need to be considered:

- Scenario 1: No further configuration or activation is provided from NW after N OD-SSB bursts have been transmitted from NW and monitored by UE:

- For case#1, UE stop monitor OD-SSB if no further configuration or activation is provided from NW.

- For case#2, UE switch back to always-on SSB if no further configuration or activation is provided from NW.

- Scenario 2: Another activation via MAC CE with a new N for the same SCell before the old N OD-SSB bursts have been transmitted from NW and monitored by UE:

- UE stop counting towards the old N and re-start counting towards the new N for both case#1 and case#2.

- Scenario 3: Another activation via MAC CE without N for the same SCell before N OD-SSB bursts have been transmitted from NW and monitored by UE:

- UE stop counting towards N and follow the latest activation.

- Scenario 4: Deactivation via MAC CE for the same SCell before N OD-SSB bursts have been transmitted from NW and monitored by UE:

- UE stop counting towards N and deactivate the on demand SSB immediately.

Proposal 6: Unified design for explicit activation/deactivation and implicit deactivation and clarify the expected UE behavior:

- Scenario 1: No further configuration or activation is provided from NW after N OD-SSB bursts have been transmitted from NW and monitored by UE:

- For case#1, UE stop monitor OD-SSB if no further configuration or activation is provided from NW.

- For case#2, UE switch back to always-on SSB if no further configuration or activation is provided from NW.

- Scenario 2: Another activation via MAC CE with a new N for the same SCell before the old N OD-SSB bursts have been transmitted from NW and monitored by UE:

- UE stop counting towards the old N and re-start counting towards the new N for both case#1 and case#2.

- Scenario 3: Another activation via MAC CE without N for the same SCell before N OD-SSB bursts have been transmitted from NW and monitored by UE:

- UE stop counting towards N and follow the latest activation.

- Scenario 4: Deactivation via MAC CE for the same SCell before N OD-SSB bursts have been transmitted from NW and monitored by UE:

- UE stop counting towards N and deactivate the on demand SSB immediately.

  - L3 Measurement

For L3 measurement, we have agreed last meeting that RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2, then the remaining issues is how to configure and perform measurements after OD-SSB activation if OD-SSB are to be measured at least for case#1.

We had discussion on SSB adaption measurements with the following agreement reached [2]:

-------------------------------------- RAN2#129 discussion--------------------------------------

R2-2500199	Discussion on adaptation of common signal channel transmission	OPPO	discussion	Rel-19	Netw_Energy_NR_enh-Core

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

-------------------------------------- RAN2#129 discussion--------------------------------------

We do not see the need to differentiate handling for L3 measurement for on demand SSB since it is also SSB adaption which happens in a on demand way.

Thus, we would like to follow the same solution as SSB adaption:

Proposal 7: RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to the OD-SSB for L3 RRM measurement on OD-SSB SCell.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: Based on the latest input from RAN1, the following parameters should be included in RRC signaling for each on demand SSB configuration:

- od-ssb-Periodicity

- od-ssb-halfFrameIndex

- od-ssb-nrofBurst

- od-ssb-absoluteFrequency

- od-ssb-PositionsInBurst

- od-ssbSubcarrierSpacing

- od-ssb-physCellId

- od-ss-PBCH-BlockPower

Proposal 2: The SCell addition/modification procedure can be reused to provide the on demand SSB configuration, i.e. the on demand SSB parameters for a SCell can be configured when NW adds or modifies a SCell.

Proposal 3: RAN2 make decision on the maximum number of OD-SSB configuration for each SCell, e.g. 2 or 4, which will decide the size of the configuration index in the on demand SSB MAC CE.

Proposal 4: Clarify the following understanding on explicit activation:

- For case#1, the OD-SSB configuration is activated for a SCell when the corresponding bit is set to 1.

- For case#2, two options can be considered and need to down select:

  - Option 1: Switch from always-on SSB to OD-SSB is activated when the corresponding bit is set to 1.

  - Option 2: OD-SSB is activated in addition to always-on SSB when the corresponding bit is set to 1.

Proposal 5: Clarify the following understanding on explicit deactivation:

- For case#1, the OD-SSB configuration is deactivated for a SCell when the corresponding bit is set to 0.

- For case#2, UE switch back to always-on SSB when the corresponding bit is set to 0.

Proposal 6: Unified design for explicit activation/deactivation and implicit deactivation and clarify the expected UE behavior:

- Scenario 1: No further configuration or activation is provided from NW after N OD-SSB bursts have been transmitted from NW and monitored by UE:

- For case#1, UE stop monitor OD-SSB if no further configuration or activation is provided from NW.

- For case#2, UE switch back to always-on SSB if no further configuration or activation is provided from NW.

- Scenario 2: Another activation via MAC CE with a new N for the same SCell before the old N OD-SSB bursts have been transmitted from NW and monitored by UE:

- UE stop counting towards the old N and re-start counting towards the new N for both case#1 and case#2.

- Scenario 3: Another activation via MAC CE without N for the same SCell before N OD-SSB bursts have been transmitted from NW and monitored by UE:

- UE stop counting towards N and follow the latest activation.

- Scenario 4: Deactivation via MAC CE for the same SCell before N OD-SSB bursts have been transmitted from NW and monitored by UE:

- UE stop counting towards N and deactivate the on demand SSB immediately.

Proposal 7: RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to the OD-SSB for L3 RRM measurement on OD-SSB SCell.

- Reference

[1] R1-2503155 Consolidated Rel-19 higher layers parameter list Post RAN1#120bis

[2] [2] Draft_RAN2_129_Meeting_Report_v3