3GPP TSG-RAN WG2 Meeting #130																					R2-2503756

St Julian, Malta, 19th – 23rd May 2025

Agenda item:		8.5.3

Title: 	Remaining issues of on-demand SIB1

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In this paper, we share some consideration on the remaining issues for on demand SIB1, with the latest higher parameter list [1] from RAN1 taken into consideration.

- Discussion

  - Alignment with latest RAN1 parameter list

The following SIB has been introduced for UL WUS configuration [2]:

SIBxx information element

-- ASN1START

-- TAG-SIBxx-START

SIBxx-r19 ::=                  SEQUENCE {

od-SIB1-CellConfigList-r19         SEQUENCE (SIZE(1..maxCellODSIB1-r19))  OF OD-SIB1-CellConfig-r19    OPTIONAL,        -- Need R

lateNonCriticalExtension                 OCTET STRING                                      OPTIONAL,

...

}

OD-SIB1-CellConfig-r19 ::=              SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR,

physCellId-r19                      SEQUENCE (SIZE(1..maxPCI-r19))  OF PhysCellId,

od-SIB1-Config-r19                        od-SIB1-Config-r19                               OPTIONAL,        -- Need R

...

}

OD-SIB1-Config-r19  ::= {

sib1-rsrp-ThresholdSSB-r19                       RSRP-Range                                                              OPTIONAL,   -- Need R

prach-RootSequenceIndex-r19        CHOICE {

l839                               INTEGER (0..837),

l139                               INTEGER (0..137)

}                                                                                                                   OPTIONAL, -- Need R

msg1-SubcarrierSpacing-r19               ENUMERATED {kHz1dot25, kHz5, kHz15, kHz30, kHz60, kHz120, spare1, spare2}  OPTIONAL, -- Need R

sib1-tdd-UL-DL-ConfigurationCommon-r19   TDD-UL-DL-ConfigCommon                                                     OPTIONAL, -- Cond TDD

sib1-restrictedSetConfig-r19             ENUMERATED {unrestrictedSet, restrictedSetTypeA, restrictedSetTypeB}       OPTIONAL, -- Need R

offsetToCarrier-r19                          INTEGER (0..2199)                                                          OPTIONAL, -- Need R

absoluteFrequencyPointA-r19                  ARFCN-ValueNR                                                              OPTIONAL, -- Need R

p-Max-r19                                    P-Max                                                                      OPTIONAL, -- Need R

ss-PBCH-BlockPower-r19                       INTEGER (-60..50)                                                          OPTIONAL, -- Need R

ssb-PositionsInBurst-r19                SEQUENCE {

inOneGroup                          BIT STRING (SIZE (8)),

groupPresence                       BIT STRING (SIZE (8))                                   OPTIONAL  -- Cond FR2-Only

},

sib1-RequestConfig-r19                   SIB1-RequestConfig-r19                                                     OPTIONAL  -- Need R

}

SIB1-RequestConfig-r19 ::=                SEQUENCE {

totalNumberOfRA-Preambles-r19           INTEGER (1..63),

rach-OccasionsSIB1-r19                    SEQUENCE {

ssb-perRACH-Occasion-r19                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

sib1-RequestPeriod-r19                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

sib1-RequestResources-r19                 SEQUENCE (SIZE (1..maxSIB1-Message)) OF SIB1-RequestResources-r19,

preambleTransMax-r19                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200}

}

SIB1-RequestResources-r19 ::=             SEQUENCE {

sib1-ra-PreambleStartIndex-r19               INTEGER (0..63),

sib1-ra-AssociationPeriodIndex-r19           INTEGER (0..15)                                                     OPTIONAL,   -- Need R

sib1-ra-ssb-OccasionMaskIndex-r19            INTEGER (0..15)                                                     OPTIONAL    -- Need R

}

-- TAG-SIBxx-STOP

-- ASN1STOP

The following parameter is mentioned in the latest RAN1 parameter list but not included in the running CR:

| frequencyBandList | Provides the frequency band indicator and a list of additionalPmax and additionalSpectrumEmission values as defined in TS 38.101-1 [15], table 6.2.3.1-1, TS 38.101-2 [39], table 6.2.3.1-2, and TS 38.101-5 [75], table 6.2.3.1-1. The UE shall apply the first listed band which it supports in the frequencyBandList field. | SEQUENCE (SIZE (1..maxNrofMultiBands)) OF FreqBandIndicatorNR |
| --- | --- | --- |
| Prach-ConfigurationIndex | PRACH configuration index used for UL-WUS transmission. | INTEGER (0..255) |
| msg1-FDM | The number of PRACH transmission occasions FDMed in one time instance. (see TS 38.211, clause 6.3.3.2). | ENUMERATED {one, two, four, eight}, |
| msg1-FrequencyStart | Msg1-FrequencyStart is with respect to the first RB of the InitialUplinkBWP determined by locationAndBandwidth (same as legacy) | INTEGER (0..maxNrofPhysicalResourceBlocks-1), |
| zeroCorrelationZoneConfig | N-CS configuration, see Table 6.3.3.1-5 in TS 38.211. | INTEGER(0..15) |
| preambleReceivedTargetPower | The target power level at the network receiver side (see TS 38.213, clause 7.4, TS 38.321, clauses 5.1.2, 5.1.3). Only multiples of 2 dBm may be chosen (e.g. -202, -200, -198, ...). | INTEGER (-202..-60) |
| powerRampingStep | Power ramping steps for PRACH (see TS 38.321,5.1.3). | ENUMERATED {dB0, dB2, dB4, dB6} |
| ra-ResponseWindow | Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and a value lower than or equal to 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321, clause 5.1.4). If ra-ResponseWindow-v1610 or ra-ResponseWindow-v1700 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). The field ra-ResponseWindow-v1700 is applicable to SCS 480 kHz and SCS 960 kHz. | ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80} |
| searchSpaceZero | Determines a common search space with ID #0, see TS 38.213, clause 13. | INTEGER (0..15) |
| controlResourceSetZero | Determines a common ControlResourceSet (CORESET) with ID #0, see TS 38.213, clause 13. | INTEGER (0..15) |
| ra-SearchSpace | Search space for random access procedure (see TS 38.213, clause 10.1). | SearchSpace |
| n-TimingAdvanceOffset | The N_TA-Offset to be applied for random access on this cell. If the field is absent, the UE applies the value defined for the duplex mode and frequency range of this cell. See TS 38.133, table 7.1.2-2. | ENUMERATED { n0, n25600, n39936 } |
| ssb-PeriodicityServingCell | The SSB periodicity in ms for at least the purpose of RO validation determination for WUS transmission. | ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160}, |
| od-sib1-WindowDuration | Indicate the duration of the time window of type 0 PDCCH monitoring occasions for SIB1 after transmission of UL-WUS. | FFS |
| od-sib1-windowStartOffset | Indicate the time offset between the reference time point and the starting time for the time window of on-demand SIB1. | INTEGER (0…[positive values] |

Proposal 1: Include the following parameters from the RAN1 parameter list in SIBxx:

- frequencyBandList

- Prach-ConfigurationIndex

- msg1-FDM

- msg1-FrequencyStart

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- powerRampingStep

- ra-ResponseWindow

- searchSpaceZero

- controlResourceSetZero

- ra-SearchSpace

- n-TimingAdvanceOffset

- ssb-PeriodicityServingCell

- od-sib1-WindowDuration

- od-sib1-windowStartOffset

  - Valid cell list of the WUS configuration

The following agreements have been made last meeting on the validity of the WUS configuration:

RAN2#128

SIB-x can be cell specific or area specific, as legacy.

RAN2#129bis

WUS configuration can be associated with a list of cells if the whole WUS configuration is same.

Area id will be used as legacy.

The following editor’s note has been provided by the 38.331 CR rapporteur: FFS: if list of cells is ARFCN&PCI or only PCI

The ARFCN is used to indicate the carrier frequency of the cell and we understand it is unlikely that intra-frequency neighbor NES cells can share the same on demand SIB1 request configuration.

Thus we understand:

Proposal 2: The list of cells sharing the same WUS configuration would be in the same frequency, i.e. the list of cells is only PCI.

Also we notice that PCI-Range is often used in a cell list to describe cells with consecutive physical cell identities, we can also reuse this kind of structure.

But the value range can be limited to n2, n4 and n8 as we do not expect to have that many cells sharing the same on demand SIB1 request configuration.

Proposal 3: Use the following ASN.1 to describe the valid cell list of WUS configuration and the range value of PCI-Range can be n2, n4 and n8.

physCellId-r19		SEQUENCE (SIZE (1.. maxPCI-r19)) OF PCI-Range

  - Maximum number of WUS configuration and the associated cells

The following editor’s note has been provided by the 38.331 CR rapporteur: FFS: value for maxCells, maxSIB1-Message, maxPCI

SIBxx-r19 ::=                  SEQUENCE {

od-SIB1-CellConfigList-r19         SEQUENCE (SIZE(1..maxCellODSIB1-r19))  OF OD-SIB1-CellConfig-r19    OPTIONAL,        -- Need R

lateNonCriticalExtension                 OCTET STRING                                      OPTIONAL,

...

}

OD-SIB1-CellConfig-r19 ::=              SEQUENCE {

carrierFreq-r19                          ARFCN-ValueNR,

physCellId-r19                      SEQUENCE (SIZE(1..maxPCI-r19))  OF PhysCellId,

od-SIB1-Config-r19                        od-SIB1-Config-r19                               OPTIONAL,        -- Need R

...

}

SIB1-RequestConfig-r19 ::=                SEQUENCE {

totalNumberOfRA-Preambles-r19           INTEGER (1..63),

rach-OccasionsSIB1-r19                    SEQUENCE {

ssb-perRACH-Occasion-r19                ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

}                                                                                                       OPTIONAL,   -- Need R

sib1-RequestPeriod-r19                    ENUMERATED {one, two, four, six, eight, ten, twelve, sixteen}       OPTIONAL,   -- Need R

sib1-RequestResources-r19                 SEQUENCE (SIZE (1..maxSIB1-Message)) OF SIB1-RequestResources-r19,

preambleTransMax-r19                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200}

}

As highlighted above, the maxCellODSIB1-r19 refers to the maximum sets of on demand SIB1 request configuration while the maxPCI-r19 refers to the maximum number of cells sharing the same on demand SIB1 request configuration. The definition of these two are not covered in the running CR and should be included.

Proposal 4: Add definition of maxCellODSIB1-r19 and maxPCI-r19 in 6.4 RRC multiplicity and type constraint values.

Also, there is limit of the maximum size of a SIB based on the following:

The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

We understand the maximum sets of on demand SIB1 request configuration should be evaluated after the full set of parameters are included for us to derive the required signaling overhead for each on demand SIB1 request configuration and see how many sets of configuration we can include in a single SIB. Since there is limit of the maximum size of a SIB, there would be an upper bound and the maxCellODSIB1-r19 must be defined to ensure we do not exceed this upper bound.

Proposal 5: The maximum number of on demand SIB1 request configuration, i.e. maxCellODSIB1-r19, should be evaluated and defined after the full set of parameters are included and ready to derive the required signaling overhead and ensure the overall signaling overhead will not go beyond the upper limit of SIB size, i.e. 2976 bits.

The maxPCI-r19 refers to the maximum number of cells sharing the same on demand SIB1 request configuration and would not contribute a lot to the signaling overhead, we can simply define it with a value 8.

- If we keep the current structure, i.e. physCellId-r19		SEQUENCE (SIZE(1..maxPCI-r19))  OF PhysCellId, the maximum number of cells sharing the same on demand SIB1 request configuration is 8.

- If we use the new structure as proposed above, i.e. physCellId-r19		SEQUENCE (SIZE (1.. maxPCI-r19)) OF PCI-Range, the maximum number of cells sharing the same on demand request configuration would be 8*8=64.

Proposal 6: The maxPCI-r19 is 8.

sib1-RequestResources-r19                 SEQUENCE (SIZE (1..maxSIB1-Message)) OF SIB1-RequestResources-r19,

While for maxSIB1-Messge which comes from the following for on demand SI request:

si-RequestResources                 SEQUENCE (SIZE (1..maxSI-Message)) OF SI-RequestResources

The above structure is used to allocate different RACH resources for requesting different SIs while for on demand SIB1 request, there is only one SIB to request, if we allow more than one SIB1-RequestResources-r19, then it means we allocate more than one set of PRACH resources for requesting SIB1 from the same cell.

Proposal 7: Confirm the understanding that having the maxSIB1-Message with value larger than 1 means more than one set of PRACH resources are configured for requesting SIB1 and add the definition of maxSIB1-Message in 6.4 RRC multiplicity and type constraint values.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: Include the following parameters from the RAN1 parameter list in SIBxx:

- frequencyBandList

- Prach-ConfigurationIndex

- msg1-FDM

- msg1-FrequencyStart

- zeroCorrelationZoneConfig

- preambleReceivedTargetPower

- powerRampingStep

- ra-ResponseWindow

- searchSpaceZero

- controlResourceSetZero

- ra-SearchSpace

- n-TimingAdvanceOffset

- ssb-PeriodicityServingCell

- od-sib1-WindowDuration

- od-sib1-windowStartOffset

Proposal 2: The list of cells sharing the same WUS configuration would be in the same frequency, i.e. the list of cells is only PCI.

Proposal 3: Use the following ASN.1 to describe the valid cell list of WUS configuration and the range value of PCI-Range can be n2, n4 and n8.

physCellId-r19		SEQUENCE (SIZE (1.. maxPCI-r19)) OF PCI-Range

Proposal 4: Add definition of maxCellODSIB1-r19 and maxPCI-r19 in 6.4 RRC multiplicity and type constraint values.

Proposal 5: The maximum number of on demand SIB1 request configuration, i.e. maxCellODSIB1-r19, should be evaluated and defined after the full set of parameters are included and ready to derive the required signaling overhead and ensure the overall signaling overhead will not go beyond the upper limit of SIB size, i.e. 2976 bits.

Proposal 6: The maxPCI-r19 is 8.

Proposal 7: Confirm the understanding that having the maxSIB1-Message with value larger than 1 means more than one set of PRACH resources are configured for requesting SIB1 and add the definition of maxSIB1-Message in 6.4 RRC multiplicity and type constraint values.

- Reference

- R1-2503155 Consolidated Rel-19 higher layers parameter list Post RAN1#120bis

[2] R2-250xxxx Running RRC CR NES Post129bis_V03