3GPP TSG-RAN WG2 Meeting #130																				R2-2503760

St Julian, Malta, 19th – 23rd May 2025

Agenda item:		7.0.2.10

Title: 	Way forward of servingCellMO for SSB-less SCell

Source: 			ZTE Corporation, Sanechips, Ericsson, Nokia

Document for: 	Discussion and Decision

- Introduction

This paper aims to discuss a way forward for servingCellMO configuration for SSB-less SCell.

- Background

  - RAN4 LS on neighboring cell measurement

An LS has been sent from RAN4 [1] on neighboring cell measurement for Rel-18 SSB-less:

----------------------------------------------RAN4 LS----------------------------------------------

RAN4 is discussing the SSB based neighbor cell measurement on the carrier frequency of SSB-less SCell. From RAN4 understanding, the definition of intra-frequency/inter-frequency measurement, whether MG is needed and which requirements apply depends on whether servingCellMO is present or not.

Based on the agreements in RAN4#114 meeting, RAN4 provides analysis for the following cases:

- If the servingCellMO indication is configured:

      - Intra-frequency/inter-frequency: The SSB based neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB frequency configured in the MO indicated by servingCellMO; otherwise, it is defined as inter-frequency.

      - UE Requirements:

- The requirements of intra-frequency without measurement gap are defined in Clause 9.2.5 TS 38.133

- The requirements of intra-frequency with measurement gap are defined in Clause 9.2.6 TS 38.133

- The requirements of inter-frequency with measurement gap are defined in Clause 9.3.4 TS 38.133

- The requirements of inter-frequency without measurement gap are defined in Clause 9.3.9 TS 38.133

- If the servingCellMO indication is not configured:

      - Intra-frequency/inter-frequency: Based current RAN4 framework on Intra-frequency/inter-frequency definition, the SSB based neighbor cell measurement is defined as inter-frequency.

      - UE Requirements:

- The requirements of inter-frequency with measurement gap are defined in Clause 9.3.4 TS 38.133

- The requirements of inter-frequency without measurement gap are defined in Clause 9.3.9 TS 38.133

Based on above information, it is up to RAN2 to decide whether servingCellMO can be present or not and detailed signalling design.

The potential conflict (if any) when servingCellMO is present is up to RAN2 to handle.

Note: The servingCellMO is based on the current RAN2 specification, it does not preclude any RAN2 discussion or signalling design for the same purpose.

----------------------------------------------RAN4 LS----------------------------------------------

In summary, RAN4 understand that the presence of servingCellMO for SSB-less SCell will impact neighbor cell measurements:

- If servingCellMO for SSB-less SCell is configured, intra-frequency measurements will be performed on neighbor cells with the same center frequency as the SSB frequency in servingCellMO of a SSB-less SCell.

- If servingCellMO for SSB-less SCell is configured, inter-frequency measurements will be performed on neighbor cells with the same center frequency as the SSB-less SCell.

Observation 1: RAN4 understand that the presence of servingCellMO for SSB-less SCell will impact neighbor cell measurements:

- If servingCellMO for SSB-less SCell is configured, intra-frequency measurements will be performed on neighbor cells with the same center frequency as the SSB frequency in servingCellMO of a SSB-less SCell.

- If servingCellMO for SSB-less SCell is configured, inter-frequency measurements will be performed on neighbor cells with the same center frequency as the SSB-less SCell.

  - Discussion history in RAN2 on servingCellMO for SSB-less Scell

RAN2#127bis [2]

R2-2409392	Report of [AT127bis][008][NES] SSB-less Scell (Huawei)	Huawei, HiSilicon	discussion	Rel-18	Netw_Energy_NR-Core

- Noted

Agreements

1	 From RAN2 perspective, no benefit is identified for configuring servingCellMO for inter-band SSB-less SCell.

2	 No LS is sent to R4.

3	Discuss in the next meeting on whether to capture “For inter-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO, depending on R4 progress.

4	Postpone the discussion on intra-band SSB-less SCell.

RAN2#128 [3]

R2-2409693	Measurements on the carrier of SSB-less SCell	 NEC	discussion	Rel-18

Proposal-1: Adopt the same handling of servingCellMO configuration for both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less SCell operation.

Proposal-2: Capture “For both inter-band and intra-band SSB-less SCell(s), this field is not present” in the conditional presence of servingCellMO of the IE structure BWP-DownlinkDedicated and ServingCellConfig. (see Text proposal in Annex).

Proposal-3: P1 and P2 can be checked according to concurrent RAN4 discussions during the upcoming meeting.

- Noted

R2-2410259	SSBLess handling	Nokia	discussion	Rel-18	Netw_Energy_NR-Core

Proposal 1:In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency.

- Noted

- Wait for next meeting to make decision

RAN2#129 [4]

R2-2500555	SSBLess handling	Nokia	discussion	Rel-18	Netw_Energy_NR-Core

R2-2500693	Discussion on servingCellMO for SSB-less Scell	Huawei, HiSilicon, Apple, Qualcomm Incorporated	discussion	Rel-18	Netw_Energy_NR-Core

R2-2500395	Measurements on the carrier of SSB-less SCell	 NEC	discussion

- Postponed to next meeting

  - Offline discussion at RAN2#129bis

For the servingCellMO for SSB-less SCell, two issues have been discussed offline:

- For the serving SSB-less SCell, all the companies understand that even if the servingCellMO is configured, the SSB measurements on the corresponding SSB-less SCell can be skipped via UE implementation.

- For the neighbour cell measurement, according to the latest RAN4 input, the views are split:

  - Understanding 1: servingCellMO should not be configured for SSB-less SCell, and inter-frequency measurements will be performed on all the neighbour cells in the same frequency as the SSB-less SCell. -> Preferred by Huawei, QC, Apple and MediaTek

  - Understanding 2: servingCellMO is configured for SSB-less SCell based on the existing spec, and UE will perform inter-frequency or intra-frequency neighbour cell measurements accordingly: -> Preferred by Ericsson, Nokia, LG, ZTE and CMCC.

    - center frequency of the SSB of the neighbor cell is same as that of SSB frequency configured in the MO indicated by servingCellMO -> intra-frequency neighbor cell measurements

    - center frequency of the SSB of the neighbor cell is different from that of SSB frequency configured in the MO indicated by servingCellMO -> inter-frequency neighbor cell measurements

  - Compromised solution: servingCellMO is optionally configured for SSB-less SCell and if it is configured UE will perform inter-frequency or intra-frequency neighbour cell measurements accordingly: -> Acceptable to Ericsson, Nokia, LG, ZTE, CMCC, Samsung and OPPO.

    - center frequency of the SSB of the neighbor cell is same as that of SSB frequency configured in the MO indicated by servingCellMO -> intra-frequency neighbor cell measurements

    - center frequency of the SSB of the neighbor cell is different from that of SSB frequency configured in the MO indicated by servingCellMO -> inter-frequency neighbor cell measurements

But we failed to reach consensus as the views from two sides are still strong thus the discussion has been postponed.

  - Possible way forward

One possible way forward to address UE vendors’ concern on the measurement handling for the case when servingCellMO is configured for SSB-less SCell is to introduce UE capability for this so that NW and UE vendors can negotiate the implementation for real deployment.

For example, we can have a capability in MeasAndMobParameters as follows and network would only configure servingCellMO for SSB-less SCell when UE indicate such capability:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| servingCellMOConfigForSSBLess-r18 Indicates whether the UE supports the configuration of servingCellMO for a SSB-less SCell as specified in TS 38.331 [9]. | UE | No | No | FR1 only |

Proposal 1: Introduce a UE capability to indicate whether UE supports the configuration of servingCellMO for a SSB-less SCell and network would only configure servingCellMO for SSB-less SCell if UE indicate such capability.

Since we have supported SSB-less SCell since Rel-15, we need to consider since which release we would like to introduce such capability.

Proposal 2: Discuss and decide since which release we can introduce such a UE capability, Rel-15 or Rel-18.

Proposal 3: Send back the reply LS to RAN4.

- Conclusion and proposals

The following proposals are given:

Proposal 1: Introduce a UE capability to indicate whether UE supports the configuration of servingCellMO for a SSB-less SCell and network would only configure servingCellMO for SSB-less SCell if UE indicate such capability.

Proposal 2: Discuss and decide since which release we can introduce such a UE capability, Rel-15 or Rel-18.

Proposal 3: Send back the reply LS to RAN4.

- Annex – draft reply LS

3GPP TSG-RAN WG2 Meeting #130									R2-25xxxxx

St Julian, Malta, 19th – 23rd May 2025

Title:	Reply LS on neighboring cell measurement for Rel-18 SSB-less

Response to:	R4-2502694 LS on neighboring cell measurement for Rel-18 SSB-less

Release:	Release 18

Work Item:	Netw_Energy_NR

Source:	RAN2

To:	RAN2

Cc:

Contact Person:

#### Name:	Yuan Gao

E-mail Address:	gao.yuan66@zte.com.cn

- Overall Description:

RAN2 have discussed the presence of servingCellMO for SSB-less SCell and the impact on neighbour cell measurements and reached the following agreements:

Introduce a UE capability to indicate whether UE supports the configuration of servingCellMO for a SSB-less SCell and network would only configure servingCellMO for SSB-less SCell if UE indicate such capability.

The agreed CRs are also attached for reference.

2. Actions:

To RAN4:

RAN2 respectfully asks RAN4 to take the above agreements and agreed CRs into account.

3. Date of Next TSG-RAN2 Meetings:

RAN WG2 Meeting #131	August 25 – August 29, 2025	India (TBC), IN

RAN WG2 Meeting #131bis	October 13 – October 17, 2025	Prague, CZ