3GPP TSG-RAN WG2 Meeting #130		R2-2503772

Malta, May 19th – 23rd, 2025

Source:	Baicells

Title:		Discussion on L1 event triggered measurement reporting

Agenda Item:		8.6.3

Document for:	Discussion and Decision

- Introduction

The following agreements to support the L1 event triggered measurement reporting are made in the RAN2#129[1] and RAN2#129bis[2]:

RAN2#129 Agreements on L1 event-triggered MR 1. Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused". 2. Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide). 3. Do not create a new resource configuration just for the event triggered measurements. 4. Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused". 5. Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report. 6. Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue. 7. TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval. 8. Cell specific offset is not supported for LTM L1 event evaluation. 9. The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity. 10. Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling. 11. Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam. 12. R18 differential RSRP encoding is used as baseline. 13. LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant. 14. Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s)) 15. Truncated MR MAC CE includes the triggered beam information: - One report config ID (event config ID) - At least one triggered beam id (SSBRI or CRI) + L1 RSRP 16. Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur. 17. For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used. 18. S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

RAN2#129bis Agreements on L1 event-triggered MR 1. Introduce new MAC CE to activate/deactivate the semi-persistent CSI-RS resource. 2. The new MAC CE includes: A/D indication, Target configuration id or {SP CSI-RS resource set id and/or candidate id}, TCI state id. 3. In L1 reporting configuration, the UE can be configured with a list of candidate IDs. For each listed candidate ID, the network can configure an offset, this offset applies to all the RSs belong the candidate ID that associated with the reporting configuration (the offset value also includes 0dB); If a candidate ID is not provided in the reporting configuration, it means the UE is not required to measure/evaluate the RSs that belonging to the candidate ID for this event, even if these are configured within the L1 resource configuration (to which the L1 reporting configuration points to). 4. In the MR MAC CE, adopt a uniform format for all the reported beams (i.e., using the same format for beams leaving the entering condition, beams met the entering condition, beams not satisfying the event). Introduce two bits per beam. 5. RAN2 will consider only the following options: - Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4} - Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting. Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE. Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE. Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before). Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info) 6. Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4} 7. Intention is that the UE should be able to report the event-triggered beam(s) that were not included in the truncated MR MAC CE by the following grant. Detailed wording can be further discussed as part of the running CR. 8. For co-existence with mTRP, will be revisited in August. If one simple solution is not prepared / agreed until / in August meeting, we will not apply mTRP in Rel-19 event-triggered MR.

In this contribution, we will discuss the remaining open issues and further details of L1 event triggered measurement reporting including:

-	How to design the truncated MAC CE for L1 event triggered measurement reporting.

- Discussion

  - Truncated Measurement report MAC CE format design

The priority of beam type to be  transmitted in the truncated MA CE

According to RAN2#129bis, the following agreements are achieved

- When UL grant is enough to accommodate only limited number of beams in MR MAC CE, the triggered beam should be included in the truncated MR MAC CE.

- RAN2 will consider only the following options:

Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

Option3: To introduce three codepoints to represent one of {type1_or_type3, type2, type4}, i.e., type1 beam and type3 beam are not distinguished in reporting

Type1 beam: a beam that has satisfied the entry condition of the event for TTT and thus triggers the MR MAC CE.

Type2 beam: a beam that has satisfied the leaving condition of the event for TTT and thus triggers the MR MAC CE.

Type3 beam: a beam that is neither type1 nor type2 but has remained in beamsTriggeredList (the beam was reported as type1 beam before).

Type4 beam: a beam that is neither type1 nor type2 and not included in beamsTriggeredList (i.e., just extra beam info)

- Working assumption: Option1: To introduce four codepoints to represent one of {type1, type2, type3, type4}

Thus, the type1 beam and type2 beam should be the highest priority. About the priority between the type3 and type4. The type3 should have higher priority than type4 because the gNB can compare the results between the type1 and type3 to figure out the changes that UE is going through.

Observation 1: The truncated MAC CE can only accommodate a limited number of beams.

Proposal 1: A priority should be determined between the above four types of beams i.e. type1(type2)>type3>type4.

How to transmit the current beam in the truncated MA CE

According to RAN2#129bis, the following agreements are achieved

- The current beam information of SpCell (if configured to be included) is always placed at the end of the normal (not truncated) MAC CE.

According to RAN2#129, the following agreements are achieved

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

Thus, to keep the truncated MAC CE consistent with the normal MAC CE, we suggest that the current beam information of SpCell is always placed at the end of one of the truncated MAC CE which includes the current beam.

Observation 2: RAN#2 129 meeting achieves the agreement that common format for normal MR MAC CE and truncated MR MAC CE is used.

Proposal 2: The current beam information of SpCell is always placed at the end of one of the truncated MAC CE which includes the current beam.

As is depicted above,  the truncated MAC CE can only accommodate a limited number of beams, so there can be serval truncated MAC CE. And in which truncated MAC CE the current beam is included can be unknown to the gNB. So, a certain bit to indicate the occurrence of the current beam is needed.

Observation 3: UE can transmit serval truncated MAC CE to the gNB and gNB doesn’t know in which truncated MAC CE the current beam is included.

Proposal 3: A certain bit to indicate the occurrence of the current beam is needed.

How to determine the best beam in the truncated MA CE

According to RAN2#129bis, the following agreements are achieved

- For differential L1-RSRP reporting, the best quality beam among the beams included in L1 MR MAC CE is taken as the reference beam as the first one. The differential L1-RSRP value is derived based on the absolute L1-RSRP of the reference beam. FFS for truncated MAC CE.

- According to RAN2#129, the following agreements are achieved

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

Again, to keep the truncated MAC CE consistent with the normal MAC CE, we suggest that the best quality beam among the beams to be included in a truncated MR MAC CE is taken as the reference beam as the first one. More specifically, the priority of beam type to be  transmitted in the truncated MAC CE should be determined first. Then UE take out some of them according to the allocated UL resource. Finally, UE puts the best quality beam among the beams taken out as the reference beam as the first one.

Observation 4: RAN#2 129 meeting achieves the agreement that common format for normal MR MAC CE and truncated MR MAC CE is used.

Proposal 4: The best quality beam among the beams to be included in a truncated MR MAC CE is taken as the reference beam as the first one

How to determine the priority between the type1 and type2

A CSI Resource Configuration can contain multiple resources from different cells, there is some possibility that some beams trigger the entering condition, meanwhile some beams trigger the leaving condition. In such case, both the  type1 and the type2 beams are both the triggered beams. Then both should be included in the truncated MR MAC CE. However, if the first truncated MAC CE fail to contain all the two kinds of beams. Then a certain priority should be determined between the type1 and type2. Type2 should have higher priority than type1 because type2 can help the source gNB to avoid making wrong cell switching which can cause ping-pong cell switching.

Observation 5:It’s possible that some beams trigger the entering condition, meanwhile some beams trigger the leaving condition.

Proposal 5: Type2 should have higher priority than type1.

- Conclusion

In this contribution, we have the following Observations and Proposals:

Observation 1: The truncated MAC CE can only accommodate only a limited number of beams.

Proposal 1: A priority should be determined between the above four types of beam i.e. type1(type2)>type3>type4.

Observation 2: RAN#2 129 meeting achieves the agreement that common format for normal MR MAC CE and truncated MR MAC CE is used.

Proposal 2: The current beam information of SpCell is always placed at the end of one of the truncated MAC CE which includes the current beam.

Observation 3: UE can transmit serval truncated MAC CE to the gNB and gNB doesn’t know in which truncated MAC CE the current beam is included.

Proposal 3: A certain bit to indicate the occurrence of the current beam is needed.

Observation 4: RAN#2 129 meeting achieves the agreement that common format for normal MR MAC CE and truncated MR MAC CE is used.

Proposal 4: The best quality beam among the beams to be included in a truncated MR MAC CE is taken as the reference beam as the first one

Observation 5:It’s possible that some beams trigger the entering condition, meanwhile some beams trigger the leaving condition.

Proposal 5: Type2 should have higher priority than type1.

- References

- RAN2 #129 chairman notes;

- RAN2 #129bis chairman notes;