3GPP TSG-RAN WG2 Meeting #130	R2-2503779

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	8.2.4

Source:	NEC

Title:	Ambient-IoT Data transmission

Document for:	Discussion, Decision

Introduction

In RAN2#129bis meeting, a good progress was achieved on AS ID, segmentation and retransmission of data. This contribution further discusses remaining issues of AS ID, retransmission and the related MAC PDU formats.

Discussion

AS ID

RAN2#129bis meeting [1] reached following agreements on AS ID.

Agreements on AS ID 1 AS ID is applied for Inventory + command case. 2 AS ID is not included in D2R message except Msg 1 (RN16 in Msg 1 has been agreed. 3 For both CFRA and CBRA, the AS ID size is same as RN 16, i.e. 16 bits. 4 Do not specify the reader behavior on how exactly the ASID is generated. 5 The device releases the AS ID upon power off (no stage 3 specification impact); 6 The device only keeps one AS ID at a time. 7 For CFRA, command message is used for AS ID assignment. 8 For CBRA, Msg 2 is used for AS ID assignment. 9 The device releases the AS ID at least: - upon receiving Paging with new transaction id for that device, i.e. different session/service - when it triggers new msg1 transmission as a result of receiving Paging message (i.e. it has to generate a random ID for CBRA) - FFS other cases for release ASID to avoid keeping it indefinitely.

The first issue is to revisit the agreement on using command message for AS ID assignment in CFRA. Some discussions related to this issue took place under the random access sub-agenda at the RAN2#129bis meeting, and the corresponding agreement is copied below:

Agreements on msg1 2. RN16 is not included in the first D2R message in the CFRA procedure. AS ID is the only ID needed for addressing the device in R2D command message assuming for CFRA no multiple devices are performing the procedures with the given reader. FFS if we can assume or need to support multiple device scenario.

In the SA2 agreed pCR for the command procedure [3], the AIOTF firstly performs Inventory procedure, and secondly for each successful Inventory response received, the AIOTF sends Command Request message to the AIoT RAN. We observed that the reader is able to know whether there is a follow on command upon reception of the “Follow on Command Indication” included in the Inventory Request transfer.

The reader is able to know whether there is follow on command upon reception of the “Follow on Command Indication” included in the Inventory Request transfer.

When CFRA is applied at A-IoT RAN, steps corresponding to AIOTF Inventory Request and Command Request are summarized below:

Steps corresponding to AIOTF Inventory Request:

- CFRA A-IoT paging message

- the first D2R message

Steps corresponding to AIOTF Command Request:

(3) R2D command message

(4) the second D2R message

On how to assign AS ID assignment in CFRA, we have three candidates:

- assign AS ID in CFRA A-IoT paging message

- including RN16 in the first D2R message

- assign AS ID in the first R2D command message

Based on Observation 1, we think Assigning AS ID in CFRA A-IoT paging message is feasible.

Assigning AS ID in CFRA A-IoT paging message is feasible since the reader knows whether there is follow on command upon reception of the “Follow on Command Indication” included in the Inventory Request transfer.

During RAN2#129 meeting, discussions under the random access sub-agenda revealed concerns from some companies about collision scenarios when the RN16 is included in the first D2R message. In the CBRA case, the reader can reassign a new AS ID using A-IoT Msg.2 associated with an echoed RN16 to address the collision. However, in the CFRA case, there is no proper message to do so.

Furthermore, RAN2#129bis agreed that 'For inventory response, RAN2 assumes that segmentation is not applied.' In the CFRA case, the first D2R message serves as the inventory response. Including the RN16 in this first D2R message means that both the device ID and RN16 are included, which may increase the risk of necessitating segmentation of the inventory response.

The third candidate is to assign AS ID in the first R2D command message as we agreed at RAN1#129bis meeting. Since AS ID can only be used after assigning, it means the assigned AS ID can only be used from the second R2D command message. It is not an efficient way.

Based on the above analysis, we propose assigning the AS ID using the CFRA A-IoT paging message.

Revise RAN2#129bis agreement “For CFRA, command message is used for AS ID assignment.” to “For CFRA, the CFRA A-IoT paging message is used for AS ID assignment.”

The second issue here is “FFS other cases for release ASID to avoid keeping it indefinitely.” In the SA2 agreed pCR for the command procedure [3], following the Inventory procedure, it is our understanding that the CN sends a Command Request message for each successful Inventory response it receives, one by one. When the reader receives a Command Request message for a new successful Inventory response device, it understands that there is no further Command Request for the previous served device. Therefore, an explicit message to release the stored AS ID is preferable to avoid devices keeping AS ID indefinitely.

(Issue 3-3) Support an explicit message to release the AS ID to avoid devices keeping AS ID indefinitely.

Segmentation

RAN2#129bis [1] meeting reached the following agreements on segmentation.

Agreements on segmentation 1 For the retransmission of the first segment/unsegmented D2R message, the reader sends the R2D message by including the upper layer command again. FFS whether offset zero is always included. 2 FFS whether the reader always includes the command for retransmission of segments. 3 1-bit indication is sufficient to indicate whether more D2R data will be sent (Ratio proposal is excluded) 4 For inventory response, RAN2 assumes that segmentation is not applied. RAN2 assumes that the reader can avoid segmentation by reader being aware of inventory response size. Notify SA2 about this assumption.

Regarding 'FFS whether offset zero is always included,' if the offset field is optional, we would need to introduce at least one indicator bit to specify whether the offset field is present. Alternatively, two different types of MAC PDUs could be defined for transmitting the upper layer command: one MAC PDU may include the offset field, while the other may not. Both options would introduce varying formats, requiring the A-IoT device to identify the format first before processing. To simplify processing for the A-IoT device, it would be sufficient to have only one type of MAC PDU that always includes the offset.

To simplify processing for the A-IoT device, only one type of MAC PDU that always includes the offset is sufficient.

For 'FFS whether the reader always includes the command for retransmission of segments,' if the upper layer command is not included in the R2D message for retransmission, the A-IoT device may need to store the command for the subsequent D2R transmission. This could increase the device's cost and complexity. To keep the A-IoT device simple and cost-effective, it is sufficient to have only one type of MAC PDU that always includes the upper layer command.

To keep the A-IoT device simple and cost-effective, only one type of MAC PDU that always includes the upper layer command is sufficient.

(Issue 3-1 and 3-2) Only one type of MAC PDU for the upper layer command that always includes the field of upper layer command and the field of offset is supported.

MAC PDU format

The following agreements on MAC PDU format were captured in RAN2#129bis meeting [1]:

Agreements on MAC PDU format 1 The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message 2 RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1. 3 The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message 4 The MAC padding is supported at least for D2R from RAN2 perspective. The device includes padding bits if there is no more data and there is still space available in the TBS. 5 In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required. FFS how this is provided (i.e. SDU length field or padding length field). The size of length field is FFS.

The first issue on MAC PDU format is about the R2D trigger message. The R2D trigger message is discussed under the random access sub-agenda at the RAN2#129bis meeting [1] and the related agreements are copied as below:

Agreements on new R2D message 1. A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. 2. Assumption: The R2D message does not include slot number/count down number.

According to the agreements, the purpose of R2D trigger message is to indicate the start of a set of MSG1 resources. In addition, the running CR of A-IoT MAC [4] already introduced 3 bits for the MAC PDU type of R2D messages. Thus, the R2D trigger message may only comprise the field of R2D Message Type.

(Issue 2-3) The R2D trigger message comprises only the field of R2D Message Type and does not require byte-aligned.

The second issue on MAC PDU format is how to determine the length of padding in the D2R message, e.g., D2R Upper Layer Data Transfer message [4].

If there is no padding, it means that the MAC PDU with MAC SDU (e.g., data SDU) has filled up the allocated resources. So, the length field is useful for the case where the MAC PDU includes both the MAC SDU and the padding. Therefore, it is more appropriate to indicate the length of padding rather than the length of MAC SDU. With such length information, the reader can determine the length of the MAC SDU by subtracting the indicated length from the scheduled resource size.

The D2R message including both the MAC SDU and the padding comprises only one field of length, indicating the length of the padding.

Although the length information is only useful in the case of padding, considering the simplification of device processing, the D2R message including the MAC SDU only may also carry the length of padding, which can be set as zero. Therefore, only one type of MAC PDU for the MAC SDU (e.g., data SDU), with or without padding, is defined.

To keep the A-IoT device simple, the D2R message including MAC SDU always comprises one field of length of the padding.

(Issue 3-4) The D2R message including MAC SDU always comprises one and only one field of length, which indicates the length of the padding.

As per the following working assumption on the payload size of RAN1#120bis meeting [5], 7 bits can be used for the indication of the length of the padding since the worst case is that there isn’t any byte of MAC SDU.

Working assumption For indicating the payload size (i.e. TBS-like) for PDRCH transmission with variable size: • 7 bits for byte-level D2R payload size indication.

(Issue 3-4) The size of the length field is 7 bits.

Conclusion and Proposal

We have the following observations and proposals:

- Observation 1	The reader is able to know whether there is follow on command upon reception of the “Follow on Command Indication” included in the Inventory Request transfer.

Observation 2	Assigning AS ID in CFRA A-IoT paging message is feasible since the reader knows whether there is follow on command upon reception of the “Follow on Command Indication” included in the Inventory Request transfer.

Observation 3	To simplify processing for the A-IoT device, only one type of MAC PDU that always includes the offset is sufficient.

Observation 4	To keep the A-IoT device simple and cost-effective, only one type of MAC PDU that always includes the upper layer command is sufficient.

Observation 5	The D2R message including both the MAC SDU and the padding comprises only one field of length, indicating the length of the padding.

Observation 6	To keep the A-IoT device simple, the D2R message including MAC SDU always comprises one field of length of the padding.

Proposal 1	Revise RAN2#129bis agreement “For CFRA, command message is used for AS ID assignment.” to “For CFRA, the CFRA A-IoT paging message is used for AS ID assignment.”

Proposal 2	(Issue 3-3) Support an explicit message to release the AS ID to avoid devices keeping AS ID indefinitely.

Proposal 3	(Issue 3-1 and 3-2) Only one type of MAC PDU for the upper layer command that always includes the field of upper layer command and the field of offset is supported.

Proposal 4	(Issue 2-3) The R2D trigger message comprises only the field of R2D Message Type and does not require byte-aligned.

Proposal 5	(Issue 3-4) The D2R message including MAC SDU always comprises one and only one field of length, which indicates the length of the padding.

Proposal 6	(Issue 3-4) The size of the length field is 7 bits.

Reference

- Report of 3GPP TSG RAN WG2 meeting #129bis, Wuhan, China.

- TR 38.769.

- S2-2504286, Update on Command procedure.

- R2-2502259, Initial Text Proposal for A-IoT MAC specification, Huawei, HiSilicon.

- Report of 3GPP TSG RAN WG1 meeting #120bis, Wuhan, China.