3GPP TSG-RAN WG2 Meeting #130	                    R2-2503780

St. Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	8.3.1

Source: 	MediaTek Inc., vivo, CATT, Qualcomm Incorporated, Nokia, Huawei, HiSilicon, Ericsson, Samsung

Title:	Support of Spatial Domain Prediction for AI Mobility

Document for:	Discussion, Decision

Introduction

In RAN2#129bis meeting, we agreed to study the specification and RAN2 impact for the following cases:

|  | UE sided model | Network sided model |
| --- | --- | --- |
| Measurement event prediction | Yes, | No (up to NW implementation) |
| Temporal domain case A | Yes, all sub cases | Yes |
| Temporal domain case B | Yes , all sub cases | Yes |
| Frequency domain | Yes , all sub cases | Yes |
| Spatial domain | Pending | Pending |
| L3 beam level measurement prediction | Yes, | Yes |

The decision to study spatial domain prediction is pending due to differing understandings among companies regarding the spatial level. This includes considerations such as whether it pertains to beam level or cell level, intra-cell or inter-cell prediction, and the model's input and output.

In this contribution, we discuss potential scenarios for spatial domain prediction, provide a definition for it, and make observations on performance evaluation based on simulation results.

Discussion

The spatial level for spatial domain prediction

When defining spatial domain prediction, there are two key dimensions to consider: the beam dimension and the cell dimension, as illustrated in Figures 1 and 2. We understand the motivations and benefits of spatial domain prediction in these dimensions:

- Spatial domain prediction in the beam dimension aims to reduce the reference signal (RS) and measurement effort at the beam level. For example, it can minimize the transmission and measurement requirements for Synchronization Signal Blocks (SSB).

- Spatial domain prediction in the cell dimension focuses on network energy savings. This allows the network to dynamically manage the SSB transmission across different cells, turning it on or off as needed.

In the following subsections, we will discuss the details of spatial domain prediction in these two dimensions.

Spatial domain prediction across beams

In the RAN1 discussion on AI BM use cases, BM-Case 1 is defined as “Spatial-domain Downlink Beam Prediction for Set A of Beams Based on Measurement Results of Set B of Beams,” with the input and output of the spatial-domain downlink beam prediction confined to the same cell.

For spatial domain prediction, RAN2 initially considered extending the research from AI BM-Case 1 (intra-cell inter-beam prediction) to encompass different cells, focusing on spatial domain reference signal (RS) and measurement reduction at the beam level. This scenario is considered a medium priority because the evaluation of prediction accuracy in RAN1 AI-BM Case 1 can be applied, as illustrated in Figure 1.

Consequently, RAN2 has included a description in the Technical Report (TR) stating: “Intra-frequency intra-cell spatial domain prediction is evaluated for the 1st study goal by measuring a subset of configured SSB as input to the model to derive L3 filtered cell-level measurements for every time instance of the same cell. It is only evaluated for FR2 intra-frequency scenario and is applicable for RRM sub-use case 1 and 3.”  Furthermore, MRRS is defined as skipped beams to be measured/ total beams to be measured. Therefore, current description in the TR considers spatial domain prediction across beams.

Figure 1. Intra-frequency Spatial domain prediction in Beam Dimension

Different from AI-BM Case 1, the beams from multiple cells can be measured as inputs for mobility purposes. Therefore, the following question arises: can the measurement results from neighboring cells be used as inputs/outputs for the AI/ML model? Two options are considered, just as described in current TR ‘In cluster approach, measurement results from more than one cells are used as input to the model. Conversely, in single cell approach, measurement results from single cell are used as input to the model.’

•	Option1-single cell approach: By measuring a subset of configured SSB of one cell as input to the model to derive L3 filtered cell-level measurements for every time instance of the same cell.

•	Option 2-cluster approach: By measuring a subset of configured SSB of a set of cells as input to the model.

In our understanding, option 1 is a special case of option 2 when there is only single cell in the set of cells. Furthermore, whether to utilize the measurement of the same cell or a set of cells can be implementation choice. Current TR for intra-frequency intra-cell prediction is more aligned with option 1. Therefore, we confirm the definition of ‘intra-frequency intra-cell spatial domain prediction’ in the TR and clarify that the spatial domain prediction is in beam dimension.

Proposal 1a: Confirm the definition of ‘intra-frequency intra-cell spatial domain prediction (in beam dimenstion)’ in the TR, i.e. Intra-frequency intra-cell spatial domain prediction is evaluated for the 1st study goal by measuring a sub set of configured SSB as input to the model to derive L3 filtered cell-level measurements for every time instance of the same cell.

Proposal 1b: Intra-frequency spatial domain prediction (in beam dimension) can utilize the ‘cluster approach’, i.e., in addition to the measurement of subset of configured SSB of the same cell, measurement results from other cell(s) are used as input to the model.

Spatial domain prediction across cells

Measured

Measured

Predicted

In addition to spatial domain prediction in the beam dimension, some companies[2] believe that spatial domain prediction should also be considered in the cell dimension. This involves measuring a set of cells as input to the model to derive Layer 3 (L3) filtered cell-level measurements for another set of cells, as illustrated in Figure 2.

Figure 2. Intra-frequency Spatial domain prediction in Cell Dimension

We understand that spatial domain prediction in the cell dimension is a different scenario from spatial domain prediction in beam dimension as captured in the TR. This scenario presents significant challenges when supported as a UE-sided model. Using the measurement of one cell to predict the measurement of another cell can be considered a non-colocated case, making accurate prediction difficult based solely on the measurements available at a single UE, especially if the cell on/off pattern is unknown to the UE. Due to these challenges and the lack of performance evaluation for this scenario as a UE-sided model, it is preferred not to further consider the potential specification impact of spatial domain prediction across cells at the UE side.

However, it may be feasible as a network-side model, as the network inherently knows the cell on/off pattern. The network can utilize measurement reports from multiple UEs across different cells to predict the cell-level Reference Signal Received Power (RSRP) for other cells. In this case, we do not foresee any significant RAN2 and specification impact. The network can provide the Radio Resource Management (RRM) measurement configuration to the UEs as usual, and the UEs can perform the legacy RRM measurement procedure and send measurement reports for the detected and measured cells. These reports can then be used by the network as input to the AI model to perform inference and determine the cell-level RSRP of the predicted cells.

Proposal 2: Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for every time instance of another cell(s).

Proposal 3: The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 to discuss whether to consider it by the network-sided model. RAN2 assumes that a network-sided model requires no specification impacts.

Simulation results for spatial domain prediction

In this section, we evaluate the performance of spatial domain prediction in the beam dimension. Table 1 provides both average L1 RSRP difference and L3 RSRP difference for sub-use case 1. We utilize L1 beam-level RSRPs of partial beam-pairs from a set of cells as model inputs to predict the L1 beam-level RSRP of other beam-pairs within the same set of cells.

We observe that as the MRRS increases, both the average L1 RSRP difference and average L3 RSRP difference gradually increase. Specifically, when MRRS is set to 50%, the average L1 RSRP difference and average L3 RSRP difference are 0.97 dB and 0.53 dB, respectively.

Table 1: Evaluation on Intermediate KPIs for spatial domain prediction in beam dimension

| Measurement Reduction rate in spatial domain(1-N/32beams) | 25% | 50% | 87.5% |
| --- | --- | --- | --- |
| Avg. L1-beam RSRP diff | 0.82 | 0.97 | 1.72 |
| Avg. L3-cell RSRP diff | 0.51 | 0.53 | 1.34 |
| Model storage complexity | 0.41MB(CNN) |  |  |
| Model computational complexity | 0.03 GFLOPs (CNN) |  |  |

Observation 1: As MRRS increases, both the average L1 beam-level RSRP difference and average L3 cell-level RSRP difference increase.

According to Nokia’s simulation results [2], the higher the speed of the UE, the larger the error of the predicted measurements.

Table 2: Mean absolute error for three speed classes [2]

|  | Random Forest | Take Max |
| --- | --- | --- |
| 30 km/h | 0.7753 | 0.5410 |
| 60 km/h | 0.7814 | 0.5662 |
| 120 km/h | 0.7985 | 0.6253 |

Observation 2: As UE speed increases, the prediction accuracy decreases.

One key issue for spatial domain prediction in beam dimension is how to choose the measured and reduced beam pattern, we evaluate the reduced beam pattern impact on the spatial domain prediction. We consider the following pattern and evaluate the average L3 cell-level RSRP difference in our simulation results.

Table 3: Average L3 cell-level RSRP difference for spatial domain prediction with different beam pattern

| Intra-freq FR2, 30km/h, Sub use case 1, 50% reduction | AI Input/ output | Avg, L3 Cell RSRP diff (dB) |
| --- | --- | --- |
| Uniform Odd index | Subset of L1 beam RSRP of one cell/ L1 beam RSRP of one cell | 0.52 |
| Uniform Even Index | Subset of L1 beam RSRP of one cell/ L1 beam RSRP of one cell | 0.48 |
| Upper Half index | Subset of L1 beam RSRP of one cell/ L1 beam RSRP of one cell | 0.99 |

Besides, serval companies have similar observation [2] [3] that the way the measured subset of beams is selected can significantly affect the prediction error of the L3 measurements.

Observation 3: Under the same MRRS setting, the selection of measured beam pattern can significantly affect the prediction accuracy (i.e. L3 cell-level RSRP difference) of the spatial domain prediction in beam dimension.

Furthermore, using L1 beam-level RSRPs from multiple cells as model input achieves higher prediction accuracy than only utilizing L1 beam-level RSRPs from the same single cell. In other words, for spatial domain prediction in the beam dimension, the cluster-based approach outperforms the cell-based approach.

Table 4: Average L3 cell-level RSRP difference for spatial domain prediction (cluster-based vs. cell-based)

|  | Measurement level | Beam RSRP differ | Cell RSRP differ |
| --- | --- | --- | --- |
| cell-based (1 cell) | L1L1 | 0.97 | 0.53 |
|  | L1L3 | NA | 2.20 |
| Cluster-based (21 cells) | L1L1 | 0.54 | 0.29 |
|  | L1L3 | NA | 2.08 |

Observation 4: Under the same MRRS setting, sub-use case 1 demonstrates higher prediction accuracy than RRM sub-use case 3.

Observation 5: Under the same MRRS setting, cluster-based approach demonstrates higher prediction accuracy than the single-cell approach.

When comparing the system-level performance of the spatial domain prediction to the legacy mobility method with MRRS=50%, several observations can be made:

- As shown in Figure (a), the average data interruption time caused by HO, HOF, and RLF remains relatively consistent or slightly reduced compared with legacy HO mechanism. This is primarily due to a decrease in the overall number of handovers, although there are slight increases in HOF and RLF occurrences, as indicated in Figure (b).

- Figure (d) illustrates that the average ToS for UEs in each cell increases due to the reduction in the occurrences of handovers and ping-pong effects. Additionally, as UE speed increases, the AI approach significantly enhances ToS compared to the legacy mobility method.

Looking ahead, we aim to achieve even greater reductions in reference signal overhead and UE measurement effort, ensuring system performance is acceptable.

(a) Data interruption time rate                 (b)  HO success/HOF times

(c) Ping-pong times                        (d) Average time of stay(ms)

Figure 3 System-level Performance for spatial domain prediction in beam dimension

Observation 6: By spatial domain prediction in beam dimension, 50% reduction in reference signal and UE measurement effort can be achieved, without any degradation in mobility performance compared with legacy HO in the metrics such as HOF, Ping-pong, data interruption time, and average TOS.

Based on above simulation results, we have following proposal:

Proposal 4: Capture the following observations for intra-frequency spatial domain prediction (in bean dimension) in the TR:

- Increasing MRRS correlates with decreased prediction accuracy.

- Increasing UE speed correlates with decreased prediction accuracy.

- Under the same MRRS setting, different measurement skipping patterns can result in different prediction accuracy.

- Under the same MRRS setting, sub-use case 1 demonstrates higher prediction accuracy than RRM sub-use case 3.

- Under the same MRRS setting, cluster-based approach demonstrates higher prediction accuracy than the single-cell approach.

- For intra-frequency spatial domain prediction (in beam dimension) with MRRS=50%, the AI algorithm can achieve comparable system performance as the legacy mechanism.

Considering the performance gains in spatial domain prediction in beam dimension, spatial domain prediction in beam dimension should not be excluded and the impact should be further studied by RAN2.

Proposal 5: The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

Conclusion

Observation on the performance on spatial domain prediction in beam dimension

Observation 1: As MRRS increases, both the average L1 beam-level RSRP difference and average L3 cell-level RSRP difference increase.

Observation 2: As UE speed increases, the prediction accuracy decreases.

Observation 3: Under the same MRRS setting, the selection of measured beam pattern can significantly affect the prediction accuracy (i.e. L3 cell-level RSRP difference) of the spatial domain prediction in beam dimension.

Observation 4: Under the same MRRS setting, sub-use case 1 demonstrates higher prediction accuracy than RRM sub-use case 3.

Observation 5: Under the same MRRS setting, cluster-based approach demonstrates higher prediction accuracy than the single-cell approach.

Observation 6: By spatial domain prediction in beam dimension, 50% reduction in reference signal and UE measurement effort can be achieved, without any degradation in mobility performance compared with legacy HO in the metrics such as HOF, Ping-pong, data interruption time, and average TOS.

Proposal

Spatial domain prediction across beams

Proposal 1a: Confirm the definition of ‘intra-frequency intra-cell spatial domain prediction (in beam dimenstion)’ in the TR, i.e. Intra-frequency intra-cell spatial domain prediction is evaluated for the 1st study goal by measuring a sub set of configured SSB as input to the model to derive L3 filtered cell-level measurements for every time instance of the same cell.

Proposal 1b: Intra-frequency spatial domain prediction (in beam dimension) can utilize the ‘cluster approach’, i.e., in addition to the measurement of subset of configured SSB of the same cell, measurement results from other cell(s) are used as input to the model.

Spatial domain prediction across cells

Proposal 2: Intra-frequency spatial domain prediction (in cell dimension) involves measuring one or more cells as input to the model to derive L3 filtered cell-level measurements for every time instance of another cell(s).

Proposal 3: The scenario of intra-frequency spatial domain prediction (in cell dimension) is not considered by the UE-side model in 5G. RAN2 to discuss whether to consider it by the network-sided model. RAN2 assumes that a network-sided model requires no specification impacts.

Observation based on current evaluation results

Proposal 4: Capture the following observations for intra-frequency spatial domain prediction (in bean dimension) in the TR:

- Increasing MRRS correlates with decreased prediction accuracy.

- Increasing UE speed correlates with decreased prediction accuracy.

- Under the same MRRS setting, different measurement skipping patterns can result in different prediction accuracy.

- Under the same MRRS setting, sub-use case 1 demonstrates higher prediction accuracy than RRM sub-use case 3.

- Under the same MRRS setting, cluster-based approach demonstrates higher prediction accuracy than the single-cell approach.

- For intra-frequency spatial domain prediction (in beam dimension) with MRRS=50%, the AI algorithm can achieve comparable system performance as the legacy mechanism.

Proposal 5: The specification impact of intra-frequency spatial domain prediction in beam dimension will be studied by RAN2 for both UE-side and network-side model.

Appendix-2: Simulation Assumption

| Parameters | Value for FR2 |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel model | UMi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and with LOSsoft; without UE rotation, Oxygen absorption, Time-varying Doppler shift, Explicit ground reflection model and blockage |
| System BW | 80MHz |
| UE speed | 60,90,120 km/h |
| UE distribution | 100% outdoor |
| BS Antenna Configuration | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ 32 TX beams are assumed |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1) 4RX beams are assumed |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, |
| BS Tx Power | 40 dBm (baseline) |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200 m |
| BS Antenna height | 10m |
| UE Antenna height | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 |
| UE trajectory model | Option #3 |
| UE trajectory boundary processing model | Circle-bouncing model, |
| Sampling period | 80ms |

Reference

[1] R2_129b_ChairNotes_25-04-11_16-45 final

[2] R2-2410474, Simulation results for Temporal, Frequency and Spatial domain RRM measurement predictions, Ericsson

[3] R2-2408929, RRM Measurement Prediction Evaluation Results, Nokia